ARM Bare Metal Programming

Complete Tutorial for Nucleo-F429ZI and QEMU

From Reset Vector to Interrupt Handling

# Introduction

This comprehensive tutorial covers bare metal programming for ARM Cortex-M4 microcontrollers using both real hardware (STM32 Nucleo-F429ZI) and QEMU emulation. You'll learn how to write code that runs directly on the processor without an operating system, understanding every aspect from reset to interrupt handling.

## What You'll Learn

- Understanding ARM Cortex-M4 architecture and memory map

- Writing startup code in assembly and C

- Creating linker scripts for memory layout

- Implementing interrupt handlers and vector tables

- Working with GPIO, UART, and system clocks

- Debugging bare metal code with GDB

- Testing on both QEMU and real hardware

## Prerequisites

- Basic C programming knowledge

- Understanding of computer architecture concepts

- Linux environment (Ubuntu/Debian recommended)

- Familiarity with command line tools

# Chapter 1: Environment Setup

## Installing the ARM Toolchain

The GNU ARM Embedded Toolchain provides everything needed to compile and debug ARM programs.

$ sudo apt-get update $ sudo apt-get install gcc-arm-none-eabi gdb-multiarch $ sudo apt-get install make git build-essential # Verify installation $ arm-none-eabi-gcc --version arm-none-eabi-gcc (GNU Arm Embedded Toolchain 10.3-2021.10) 10.3.1

## Installing QEMU

QEMU allows testing without physical hardware:

$ sudo apt-get install qemu-system-arm # Verify installation $ qemu-system-arm --version QEMU emulator version 6.2.0

## Installing OpenOCD (for Nucleo board)

OpenOCD provides debugging and flashing capabilities for real hardware:

$ sudo apt-get install openocd stlink-tools # Test ST-Link connection (with board connected) $ st-info --probe Found 1 stlink programmers version: V2J37S27 serial: 066DFF485750667567194325 flash: 2097152 (pagesize: 16384) sram: 262144 chipid: 0x0419

## Project Directory Structure

Create a organized workspace:

$ mkdir -p ~/bare-metal-arm/{step-1-minimal,step-2-startup,step-3-uart,step-4-interrupts} $ cd ~/bare-metal-arm/step-1-minimal # Your structure should look like: bare-metal-arm/ ├── step-1-minimal/ # Minimal blinky ├── step-2-startup/ # Full startup code ├── step-3-uart/ # UART communication └── step-4-interrupts/ # Interrupt handling

# Chapter 2: Understanding ARM Cortex-M4 Architecture

## STM32F429ZI Memory Map

The STM32F429ZI has a well-defined memory layout:

| Region | Address Range | Size |
| --- | --- | --- |
| Flash Memory | 0x08000000 - 0x081FFFFF | 2 MB |
| SRAM | 0x20000000 - 0x2002FFFF | 192 KB |
| Peripherals | 0x40000000 - 0x5FFFFFFF | 512 MB region |

## Cortex-M4 Vector Table

The vector table must be placed at address 0x08000000 (start of Flash) and contains:

- Initial Stack Pointer (SP) value at offset 0x00

- Reset Handler address at offset 0x04

- Exception handlers (Hard Fault, NMI, etc.)

- Interrupt handlers (IRQ 0-81 for STM32F429)

Key registers in Cortex-M4:

- R0-R12: General purpose registers

- R13 (SP): Stack pointer

- R14 (LR): Link register (return address)

- R15 (PC): Program counter

- PSR: Program status register

# Chapter 3: Step 1 - Minimal LED Blinky

Let's create the simplest possible bare metal program that blinks an LED. This demonstrates the complete toolchain without unnecessary complexity.

## 3.1 Startup Assembly Code

Create startup.s with the minimal vector table and reset handler:

.syntax unified .cpu cortex-m4 .thumb /* Vector Table */ .section .vector_table,"a",%progbits .type vector_table, %object vector_table: .word _estack /* Initial Stack Pointer */ .word Reset_Handler /* Reset Handler */ .word 0 /* NMI Handler */ .word 0 /* Hard Fault Handler */ /* Add more vectors as needed */ .size vector_table, .-vector_table /* Reset Handler */ .section .text.Reset_Handler .type Reset_Handler, %function Reset_Handler: /* Set up stack pointer */ ldr r0, =_estack mov sp, r0 /* Call main function */ bl main /* Infinite loop if main returns */ hang: b hang .size Reset_Handler, .-Reset_Handler

## 3.2 Main C Code

Create main.c with LED blinking logic:

#include <stdint.h> /* STM32F429ZI Register Definitions */ #define RCC_BASE 0x40023800 #define RCC_AHB1ENR (*(volatile uint32_t *)(RCC_BASE + 0x30)) #define GPIOB_BASE 0x40020400 #define GPIOB_MODER (*(volatile uint32_t *)(GPIOB_BASE + 0x00)) #define GPIOB_ODR (*(volatile uint32_t *)(GPIOB_BASE + 0x14)) #define LED_PIN 7 /* Green LED on PB7 */ static void delay(volatile uint32_t count) { while(count--) { __asm__("nop"); } } int main(void) { /* Enable GPIOB clock */ RCC_AHB1ENR ¦= (1 << 1); /* Configure PB7 as output */ GPIOB_MODER &= ~(3 << (LED_PIN * 2)); GPIOB_MODER ¦= (1 << (LED_PIN * 2)); /* Blink LED forever */ while(1) { GPIOB_ODR ^= (1 << LED_PIN); /* Toggle LED */ delay(1000000); } return 0; }

## 3.3 Linker Script

Create stm32f429zi.ld to define memory layout:

MEMORY { FLASH (rx) : ORIGIN = 0x08000000, LENGTH = 2048K SRAM (rwx) : ORIGIN = 0x20000000, LENGTH = 192K } /* Stack size */ _stack_size = 0x4000; /* 16KB stack */ /* End of RAM */ _estack = ORIGIN(SRAM) + LENGTH(SRAM); SECTIONS { .vector_table : { KEEP(*(.vector_table)) } > FLASH .text : { *(.text*) *(.rodata*) } > FLASH .data : { _sdata = .; *(.data*) _edata = .; } > SRAM AT> FLASH .bss : { _sbss = .; *(.bss*) *(COMMON) _ebss = .; } > SRAM } _sidata = LOADADDR(.data);

## 3.4 Makefile

Create Makefile to automate building:

PROJECT = blinky MCU = cortex-m4 CC = arm-none-eabi-gcc AS = arm-none-eabi-as LD = arm-none-eabi-gcc OBJCOPY = arm-none-eabi-objcopy SIZE = arm-none-eabi-size CFLAGS = -mcpu=${MCU} -mthumb -O2 -Wall -g LDFLAGS = -T stm32f429zi.ld -nostartfiles SRCS = main.c ASM_SRCS = startup.s all: ${PROJECT}.bin ${PROJECT}.elf: ${SRCS} ${ASM_SRCS} ${LD} ${CFLAGS} ${LDFLAGS} -o $@ $^ ${SIZE} $@ ${PROJECT}.bin: ${PROJECT}.elf ${OBJCOPY} -O binary $< $@ clean: rm -f *.o *.elf *.bin flash: ${PROJECT}.bin st-flash write $< 0x08000000 .PHONY: all clean flash

## 3.5 Building and Testing

Build the project:

$ make arm-none-eabi-gcc -mcpu=cortex-m4 -mthumb -O2 -Wall -g -T stm32f429zi.ld \ -nostartfiles -o blinky.elf main.c startup.s text data bss dec hex filename 428 0 0 428 1ac blinky.elf

Flash to Nucleo board:

$ make flash st-flash write blinky.bin 0x08000000 st-flash 1.7.0 Loading device parameters.... Device: STM32F42x/43x Flash size: 2097152 bytes Flash page size: 16384 bytes Wrote 428 bytes to 0x08000000 Press reset button and LED should blink!

# Chapter 4: Running on QEMU

QEMU allows testing without hardware. We'll modify our code to work with QEMU's emulated STM32 board.

## 4.1 QEMU-Compatible Main

Create main_qemu.c for semihosting output:

#include <stdint.h> /* Semihosting for QEMU printf */ extern void initialise_monitor_handles(void); #define GPIOB_BASE 0x40020400 #define GPIOB_ODR (*(volatile uint32_t *)(GPIOB_BASE + 0x14)) #define LED_PIN 7 void delay(volatile uint32_t count) { while(count--); } int main(void) { uint32_t counter = 0; while(1) { GPIOB_ODR ^= (1 << LED_PIN); /* Print every 10 toggles */ if(counter++ % 10 == 0) { /* Use QEMU semihosting for output */ asm("mov r0, #0x04"); /* SYS_WRITE0 */ asm("mov r1, %0" : : "r"("LED Toggle\n")); asm("bkpt 0xAB"); } delay(500000); } }

## 4.2 Running in QEMU

$ qemu-system-arm -M netduinoplus2 -cpu cortex-m4 \ -kernel blinky.elf -nographic -semihosting LED Toggle LED Toggle LED Toggle ^C to exit

## 4.3 Debugging with GDB

Start QEMU with GDB server:

# Terminal 1: Start QEMU with GDB $ qemu-system-arm -M netduinoplus2 -cpu cortex-m4 \ -kernel blinky.elf -s -S -nographic # Terminal 2: Connect GDB $ gdb-multiarch blinky.elf (gdb) target remote :1234 (gdb) break main (gdb) continue (gdb) info registers (gdb) step

# Chapter 5: Complete Startup Code with Data Initialization

A production startup must initialize the .data and .bss sections. Let's implement proper startup code.

## 5.1 Enhanced Startup Assembly

.syntax unified .cpu cortex-m4 .thumb .global Reset_Handler .global Default_Handler /* Vector Table */ .section .vector_table,"a",%progbits vector_table: .word _estack .word Reset_Handler .word NMI_Handler .word HardFault_Handler .word MemManage_Handler .word BusFault_Handler .word UsageFault_Handler .word 0 .word 0 .word 0 .word 0 .word SVC_Handler .word DebugMon_Handler .word 0 .word PendSV_Handler .word SysTick_Handler /* External Interrupts */ .word EXTI0_IRQHandler .word EXTI1_IRQHandler /* ... add all 82 IRQs ... */ /* Default handler - weak aliases */ .weak NMI_Handler .weak HardFault_Handler .weak MemManage_Handler .weak BusFault_Handler .weak UsageFault_Handler .weak SVC_Handler .weak DebugMon_Handler .weak PendSV_Handler .weak SysTick_Handler .thumb_func NMI_Handler: HardFault_Handler: MemManage_Handler: BusFault_Handler: UsageFault_Handler: SVC_Handler: DebugMon_Handler: PendSV_Handler: SysTick_Handler: Default_Handler: b Default_Handler /* Reset Handler with data init */ .section .text.Reset_Handler .type Reset_Handler, %function Reset_Handler: /* Copy .data from flash to SRAM */ ldr r0, =_sdata /* Destination start */ ldr r1, =_edata /* Destination end */ ldr r2, =_sidata /* Source start */ copy_loop: cmp r0, r1 bge copy_done ldmia r2!, {r3} stmia r0!, {r3} b copy_loop copy_done: /* Zero .bss section */ ldr r0, =_sbss ldr r1, =_ebss movs r2, #0 bss_loop: cmp r0, r1 bge bss_done stmia r0!, {r2} b bss_loop bss_done: /* Call main */ bl main hang: b hang

# Chapter 6: UART Communication

Implement UART for debugging and communication. We'll use USART3 on the Nucleo board.

## 6.1 UART Driver Implementation

#include <stdint.h> /* USART3 Registers (connected to ST-Link VCP) */ #define USART3_BASE 0x40004800 #define USART3_SR (*(volatile uint32_t *)(USART3_BASE + 0x00)) #define USART3_DR (*(volatile uint32_t *)(USART3_BASE + 0x04)) #define USART3_BRR (*(volatile uint32_t *)(USART3_BASE + 0x08)) #define USART3_CR1 (*(volatile uint32_t *)(USART3_BASE + 0x0C)) /* GPIO for USART3 (PD8=TX, PD9=RX) */ #define GPIOD_BASE 0x40020C00 #define GPIOD_MODER (*(volatile uint32_t *)(GPIOD_BASE + 0x00)) #define GPIOD_AFRL (*(volatile uint32_t *)(GPIOD_BASE + 0x20)) #define GPIOD_AFRH (*(volatile uint32_t *)(GPIOD_BASE + 0x24)) /* RCC */ #define RCC_APB1ENR (*(volatile uint32_t *)(RCC_BASE + 0x40)) #define RCC_AHB1ENR (*(volatile uint32_t *)(RCC_BASE + 0x30)) void uart_init(uint32_t baudrate) { /* Enable clocks */ RCC_AHB1ENR ¦= (1 << 3); /* GPIOD clock */ RCC_APB1ENR ¦= (1 << 18); /* USART3 clock */ /* Configure GPIO pins */ /* PD8: AF7 (USART3_TX) */ GPIOD_MODER &= ~(3 << 16); GPIOD_MODER ¦= (2 << 16); /* Alternate function */ GPIOD_AFRH &= ~(0xF << 0); GPIOD_AFRH ¦= (7 << 0); /* AF7 */ /* PD9: AF7 (USART3_RX) */ GPIOD_MODER &= ~(3 << 18); GPIOD_MODER ¦= (2 << 18); GPIOD_AFRH &= ~(0xF << 4); GPIOD_AFRH ¦= (7 << 4); /* Configure USART */ /* 16MHz / baudrate */ USART3_BRR = 16000000 / baudrate; /* Enable TX, RX, and USART */ USART3_CR1 = (1 << 13) ¦ (1 << 3) ¦ (1 << 2); } void uart_putc(char c) { /* Wait for TX empty */ while(!(USART3_SR & (1 << 7))); USART3_DR = c; } void uart_puts(const char *s) { while(*s) { if(*s == '\n') { uart_putc('\r'); } uart_putc(*s++); } } char uart_getc(void) { /* Wait for RX not empty */ while(!(USART3_SR & (1 << 5))); return USART3_DR; } int main(void) { uart_init(115200); uart_puts("STM32F429ZI UART Demo\n"); uart_puts("Type characters...\n"); while(1) { char c = uart_getc(); uart_putc(c); /* Echo */ } }

## 6.2 Testing UART

# Build and flash $ make clean && make flash # Connect to serial terminal $ minicom -D /dev/ttyACM0 -b 115200 # Or use screen $ screen /dev/ttyACM0 115200

# Chapter 7: Interrupt Handling

Implement interrupt-driven GPIO and timers for efficient bare metal programming.

## 7.1 SysTick Timer Setup

#include <stdint.h> /* SysTick Registers */ #define SYSTICK_BASE 0xE000E010 #define SYST_CSR (*(volatile uint32_t *)(SYSTICK_BASE + 0x00)) #define SYST_RVR (*(volatile uint32_t *)(SYSTICK_BASE + 0x04)) #define SYST_CVR (*(volatile uint32_t *)(SYSTICK_BASE + 0x08)) volatile uint32_t systick_counter = 0; void systick_init(uint32_t ticks) { SYST_RVR = ticks - 1; SYST_CVR = 0; SYST_CSR = 0x07; /* Enable, interrupt, use processor clock */ } /* SysTick Handler - defined in startup.s */ void SysTick_Handler(void) { systick_counter++; } void delay_ms(uint32_t ms) { uint32_t start = systick_counter; while((systick_counter - start) < ms); } int main(void) { /* Initialize SysTick for 1ms interrupts */ /* 16MHz / 1000 = 16000 ticks */ systick_init(16000); uart_init(115200); uart_puts("SysTick Demo\n"); while(1) { uart_puts("Tick\n"); delay_ms(1000); } }

## 7.2 External Interrupt (Button)

/* EXTI for User Button (PC13) */ #define EXTI_BASE 0x40013C00 #define EXTI_IMR (*(volatile uint32_t *)(EXTI_BASE + 0x00)) #define EXTI_FTSR (*(volatile uint32_t *)(EXTI_BASE + 0x0C)) #define EXTI_PR (*(volatile uint32_t *)(EXTI_BASE + 0x14)) #define SYSCFG_BASE 0x40013800 #define SYSCFG_EXTICR4 (*(volatile uint32_t *)(SYSCFG_BASE + 0x10)) #define NVIC_ISER1 (*(volatile uint32_t *)(0xE000E104)) void button_init(void) { /* Enable SYSCFG clock */ RCC_APB2ENR ¦= (1 << 14); /* Configure PC13 as input */ GPIOC_MODER &= ~(3 << 26); /* Connect EXTI13 to PC13 */ SYSCFG_EXTICR4 &= ~(0xF << 4); SYSCFG_EXTICR4 ¦= (2 << 4); /* Port C */ /* Configure EXTI13 */ EXTI_IMR ¦= (1 << 13); /* Unmask */ EXTI_FTSR ¦= (1 << 13); /* Falling edge */ /* Enable EXTI15_10 interrupt in NVIC */ NVIC_ISER1 ¦= (1 << 8); } void EXTI15_10_IRQHandler(void) { if(EXTI_PR & (1 << 13)) { /* Clear pending bit */ EXTI_PR ¦= (1 << 13); /* Handle button press */ uart_puts("Button pressed!\n"); } }

# Chapter 8: Advanced Topics

## 8.1 Clock Configuration

The STM32F429 can run at 180MHz using the PLL. Here's how to configure it:

void system_clock_config(void) { /* Enable HSE (8MHz external crystal) */ RCC_CR ¦= (1 << 16); while(!(RCC_CR & (1 << 17))); /* Wait for HSE ready */ /* Configure PLL: 8MHz / 8 * 360 / 2 = 180MHz */ RCC_PLLCFGR = (1 << 22) ¦ /* HSE as source */ (8 << 0) ¦ /* PLLM = 8 */ (360 << 6) ¦ /* PLLN = 360 */ (0 << 16); /* PLLP = 2 */ /* Enable PLL */ RCC_CR ¦= (1 << 24); while(!(RCC_CR & (1 << 25))); /* Wait for PLL ready */ /* Configure Flash latency for 180MHz */ FLASH_ACR ¦= 5; /* 5 wait states */ /* Switch to PLL */ RCC_CFGR ¦= (2 << 0); while((RCC_CFGR & (3 << 2)) != (2 << 2)); }

## 8.2 DMA Configuration

/* DMA for UART TX */ void uart_dma_init(void) { RCC_AHB1ENR ¦= (1 << 21); /* DMA1 clock */ /* DMA1 Stream3 Channel4 for USART3_TX */ DMA1_Stream3->CR = 0; /* Disable */ while(DMA1_Stream3->CR & 1); DMA1_Stream3->PAR = (uint32_t)&USART3_DR; DMA1_Stream3->CR = (4 << 25) ¦ /* Channel 4 */ (1 << 6) ¦ /* Memory increment */ (1 << 4); /* Memory to peripheral */ } void uart_dma_send(const char *data, uint32_t len) { DMA1_Stream3->M0AR = (uint32_t)data; DMA1_Stream3->NDTR = len; DMA1_Stream3->CR ¦= 1; /* Enable */ USART3_CR3 ¦= (1 << 7); /* Enable DMA TX */ }

## 8.3 Power Management

void enter_sleep_mode(void) { /* Wait For Interrupt */ __asm__("wfi"); } void enter_stop_mode(void) { /* Configure stop mode */ PWR_CR ¦= (1 << 0); /* LPDS */ SCB_SCR ¦= (1 << 2); /* SLEEPDEEP */ __asm__("wfi"); SCB_SCR &= ~(1 << 2); }

# Appendix A: Complete Project Structure

bare-metal-arm/ ├── common/ │ ├── stm32f429zi.h # Register definitions │ ├── system.c # System initialization │ └── uart.c # UART driver ├── step-1-minimal/ │ ├── main.c │ ├── startup.s │ ├── stm32f429zi.ld │ └── Makefile ├── step-2-startup/ │ ├── main.c │ ├── startup_full.s │ ├── stm32f429zi.ld │ └── Makefile ├── step-3-uart/ │ ├── main.c │ ├── uart.c │ ├── startup.s │ └── Makefile └── step-4-interrupts/ ├── main.c ├── interrupts.c ├── startup.s └── Makefile

# Appendix B: Debugging Tips

- Use openocd for hardware debugging: openocd -f board/st_nucleo_f4.cfg

- Connect GDB: arm-none-eabi-gdb -ex 'target remote :3333'

- View disassembly: arm-none-eabi-objdump -D file.elf

- Check memory sections: arm-none-eabi-nm file.elf

- Analyze with readelf: arm-none-eabi-readelf -a file.elf

# Appendix C: Common Issues

### LED doesn't blink

- Check clock is enabled for GPIO peripheral

- Verify pin configuration (mode, speed)

- Confirm correct LED pin number

### UART not working

- Verify baud rate calculation (depends on clock)

- Check alternate function configuration

- Confirm TX/RX pins match schematic

### Hard Fault

- Enable hard fault handler with useful debug info

- Check for uninitialized pointers

- Verify stack size is adequate

- Look for memory access violations

# Resources and References

## Official Documentation

- STM32F429ZI Reference Manual (RM0090)

- Cortex-M4 Generic User Guide

- STM32F4 Programming Manual (PM0214)

- Nucleo-144 User Manual (UM1974)

## Online Resources

- GNU ARM Embedded Toolchain: developer.arm.com/tools-and-software/open-source-software/developer-tools/gnu-toolchain

- QEMU Documentation: qemu.org/docs/master

- OpenOCD: openocd.org

## Books

- The Definitive Guide to ARM Cortex-M3/M4 Processors by Joseph Yiu

- Mastering STM32 by Carmine Noviello

- Embedded Systems with ARM Cortex-M Microcontrollers in Assembly Language and C by Yifeng Zhu

End of Tutorial

Happy Bare Metal Programming!