⚙

BARE-METAL RUST

From Zero to Registers

ESP32-S3 Zero  ·  STM32F446RE  ·  No HAL. No abstractions. Pure metal.

| Part | Contents |
| --- | --- |
| A — Rust Language | Variables · Ownership · Structs · Enums · Traits · Error handling · Unsafe |
| B — Bare-Metal Theory | Memory maps · MMIO · Volatile · Linker scripts · Startup · SVD/PAC |
| C — WSL2 + Toolchain | rustup · PAC crates · probe-rs · espflash · usbipd · OpenOCD |
| D — ESP32-S3 Bare-Metal | RCC · GPIO · UART · I2C · SPI · Interrupts — all via raw registers |
| E — STM32F446 Bare-Metal | RCC · GPIO · UART · I2C · SPI · Interrupts · Systick — all via raw registers |

🔥 Key Rule No HAL. Every peripheral is configured by reading the datasheet and writing to the exact memory address. This is how firmware engineers actually work.

# PART A — RUST LANGUAGE ESSENTIALS

## Chapter 1 — Why Rust for Bare-Metal?

### 1.1  The Problem with C on Embedded Systems

C gives you complete hardware control — but at a cost. The following bugs are silent in C and can take days to find:

- Use-after-free: pointer to freed memory, reads garbage or corrupts RAM

- Buffer overflow: writing past array end silently corrupts adjacent variables

- Null dereference: crash or silent memory corruption

- Data races: ISR modifies shared variable while main loop reads it — undefined behavior

- Integer overflow: silent wraparound in C; panic in Rust debug mode

Rust eliminates ALL of these at compile time. The compiler rejects programs that could exhibit these bugs — no runtime overhead, no garbage collector. The generated machine code is identical to C.

### 1.2  Rust Bare-Metal vs HAL

| Approach | Description |
| --- | --- |
| HAL (Hardware Abstraction Layer) | Crates like stm32f4xx-hal wrap registers in safe Rust APIs. Easy, but hides the hardware. |
| PAC (Peripheral Access Crate) | Auto-generated from SVD files. Gives named register fields. Thin layer over raw addresses. |
| Raw / Pure Bare-Metal | You read the datasheet. You write the address. No crate between you and the silicon. |

This tutorial uses the PAC approach — named register fields from the datasheet, with unsafe pointer writes for the lowest-level operations. You will understand every line.

## Chapter 2 — Rust Syntax in 30 Minutes

### 2.1  Variables, Types, and Mutability

▸ rust fn main() { // Immutable by default — mirrors read-only registers let chip_id: u32 = 0x6000_1000; // Mutable — for values that change (counters, flags) let mut tick_count: u32 = 0; tick_count += 1; // Constants — compile-time, type required, UPPER_SNAKE_CASE const SYSCLK_HZ: u32 = 168_000_000; // 168 MHz const FLASH_BASE: u32 = 0x0800_0000; // STM32 flash origin // All integer sizes you'll use in embedded: let a: u8 = 0xFF; // byte, register field let b: u16 = 0xABCD; // half-word let c: u32 = 0xDEAD_BEEF; // word, most common let d: u64 = 0x1234_5678_9ABC_DEF0; let e: usize = 0x4002_0000; // use usize for addresses // Hex, binary, decimal literals: let gpio_moder: u32 = 0x0000_0555; // hex let pin_mask: u32 = 0b0000_0000_0000_0100; // binary let baud_div: u32 = 1_562; // decimal with _ }

📌 Note In embedded Rust, u32 is your best friend — almost every peripheral register is 32 bits wide. u8 is for byte buffers. usize is for raw pointer casting.

### 2.2  Bit Manipulation — The Core Skill

Bare-metal programming is 90% bit manipulation. Here are all the patterns you need:

▸ rust fn bit_tricks() { let mut reg: u32 = 0x0000_0000; // SET a bit (bit 5): reg ¦= 1 << 5; // reg = 0x0000_0020 // CLEAR a bit (bit 5): reg &= !(1 << 5); // reg = 0x0000_0000 // TOGGLE a bit (bit 3): reg ^= 1 << 3; // READ a bit (bit 5) — returns 0 or non-zero: let bit5_set = (reg >> 5) & 1; // SET a multi-bit field (bits 3:2 = 0b10): // Step 1: clear the field reg &= !(0b11 << 2); // Step 2: write the value reg ¦= (0b10 << 2); // Shorthand with mask constant: const MODER5_MASK: u32 = 0b11 << 10; // bits 11:10 const MODER5_OUT: u32 = 0b01 << 10; // output mode reg = (reg & !MODER5_MASK) ¦ MODER5_OUT; // Check if field equals value: let mode = (reg >> 10) & 0b11; if mode == 0b01 { /* output mode */ } println!("reg = 0x{:08X}", reg); }

🔥 Key Rule Master these bit patterns. Every register configuration in bare-metal code is: CLEAR the field bits with &= !mask, then SET the new value with ¦= (value << shift).

### 2.3  Functions

▸ rust // Basic function — types always required fn delay_cycles(cycles: u32) { for _ in 0..cycles { // 'volatile' NOP to prevent optimizer removing loop core::hint::black_box(()); } } // Return value — no semicolon on last expression fn calc_brr(fclk: u32, baud: u32) -> u32 { (fclk + baud / 2) / baud // USART BRR formula } // Multiple return values via tuple fn pll_params(target_hz: u32) -> (u32, u32, u32) { let m = 8u32; // HSE divider let n = 336u32; // VCO multiplier let p = 2u32; // PLL output divider (m, n, p) } fn main() { delay_cycles(1_000); let brr = calc_brr(42_000_000, 115200); println!("BRR = {}", brr); // 365 let (m, n, p) = pll_params(168_000_000); }

### 2.4  Control Flow

▸ rust fn main() { let pin_state: u32 = 1; // if — also an expression let level = if (pin_state & 1) != 0 { "HIGH" } else { "LOW" }; // match — exhaustive, perfect for register states let mode_bits: u32 = 0b10; match mode_bits { 0b00 => println!("Input"), 0b01 => println!("Output"), 0b10 => println!("Alternate function"), 0b11 => println!("Analog"), _ => unreachable!(), // 2-bit field, can't be > 3 } // Embedded-style infinite main loop: let mut count = 0u32; loop { count = count.wrapping_add(1); // wrapping add — no panic on overflow if count == 1000 { break; } } // for over range: for i in 0u32..8 { let pin_bit = 1u32 << i; if (0xA5 & pin_bit) != 0 { print!("1"); } else { print!("0"); } } println!(); }

### 2.5  Structs and impl

▸ rust // Register block as a struct — mirrors C volatile struct #[repr(C)] // <- CRITICAL: C layout, no padding reordering struct GpioRegs { moder: u32, // offset 0x00 otyper: u32, // offset 0x04 ospeedr: u32, // offset 0x08 pupdr: u32, // offset 0x0C idr: u32, // offset 0x10 odr: u32, // offset 0x14 bsrr: u32, // offset 0x18 lckr: u32, // offset 0x1C afrl: u32, // offset 0x20 afrh: u32, // offset 0x24 } // Methods on the register struct impl GpioRegs { fn set_pin_output(&mut self, pin: u8) { let shift = (pin as u32) * 2; self.moder &= !(0b11 << shift); // clear self.moder ¦= 0b01 << shift; // set to output } fn set_pin_high(&mut self, pin: u8) { self.bsrr = 1 << pin; // BSRR: write 1 to set } fn set_pin_low(&mut self, pin: u8) { self.bsrr = 1 << (pin + 16); // BSRR: write 1 to reset } }

### 2.6  Enums and Pattern Matching

▸ rust #[derive(Debug, Clone, Copy, PartialEq)] enum GpioMode { Input = 0b00, Output = 0b01, AlternateFunc = 0b10, Analog = 0b11, } #[derive(Debug)] enum I2cError { Nack, ArbitrationLost, BusError, Timeout, } fn configure_pin(mode: GpioMode, pin: u8) -> u32 { (mode as u32) << (pin * 2) } fn handle_i2c_sr1(sr1: u32) -> Result<(), I2cError> { if (sr1 >> 10) & 1 != 0 { return Err(I2cError::ArbitrationLost); } if (sr1 >> 8) & 1 != 0 { return Err(I2cError::Nack); } if (sr1 >> 9) & 1 != 0 { return Err(I2cError::BusError); } Ok(()) }

### 2.7  Ownership and Borrowing for Embedded

▸ rust // Ownership prevents two code paths from touching // the same peripheral simultaneously — no HAL needed struct Uart { base_addr: usize, } impl Uart { // Constructor takes the base address — 'owns' this UART fn new(base_addr: usize) -> Self { Uart { base_addr } } fn write_byte(&self, byte: u8) { unsafe { let dr = (self.base_addr + 0x04) as *mut u32; // wait for TXE (bit 7 of SR) let sr = self.base_addr as *const u32; while (core::ptr::read_volatile(sr) >> 7) & 1 == 0 {} core::ptr::write_volatile(dr, byte as u32); } } } fn main() { let uart2 = Uart::new(0x4000_4400); // USART2 base uart2.write_byte(b'A'); // uart2 dropped here — USART2 'released' // Ownership guarantees no double-use bugs }

### 2.8  unsafe — The Gateway to Hardware

unsafe is the keyword that lets Rust code cross the safety boundary. In bare-metal firmware, you use it to:

- Read/write to raw memory addresses (MMIO registers)

- Dereference raw pointers

- Call extern C functions from vendor SDKs

- Implement interrupt handlers

▸ rust fn main() { // Reading a 32-bit peripheral register at address 0x4002_0000: let reg_addr = 0x4002_0000usize as *const u32; // WRONG — optimizer may cache this in a register: // let val = unsafe { *reg_addr }; // CORRECT — volatile read prevents caching: let val = unsafe { core::ptr::read_volatile(reg_addr) }; // Writing to a register: let rcc_ahb1enr = 0x4002_3830usize as *mut u32; unsafe { // Enable GPIOA clock (bit 0 of RCC_AHB1ENR) let current = core::ptr::read_volatile(rcc_ahb1enr); core::ptr::write_volatile(rcc_ahb1enr, current ¦ (1 << 0)); } }

🔥 Key Rule ALWAYS use read_volatile / write_volatile for MMIO registers, never plain pointer dereference. Plain dereference allows the optimizer to elide the access — your peripheral configuration will silently vanish.

### 2.9  Option and Result for Error-Free Firmware

▸ rust // Result is perfect for peripheral operations that can fail fn wait_for_flag(reg: *const u32, bit: u8, timeout: u32) -> Result<(), &'static str> { let mut count = 0u32; loop { let val = unsafe { core::ptr::read_volatile(reg) }; if (val >> bit) & 1 != 0 { return Ok(()); } count += 1; if count >= timeout { return Err("Timeout waiting for flag"); } } } fn init_peripheral() -> Result<(), &'static str> { let sr = 0x4000_4400usize as *const u32; // Wait for ready flag (bit 5), timeout 10000 cycles: wait_for_flag(sr, 5, 10_000)?; // '?' propagates error Ok(()) } fn main() { match init_peripheral() { Ok(()) => { /* proceed */ }, Err(msg) => panic!("Init failed: {}", msg), } }

## Chapter 3 — Memory-Mapped I/O: The Foundation

### 3.1  What Is Memory-Mapped I/O?

In ARM Cortex-M and Xtensa LX7 (ESP32-S3) processors, peripheral registers are mapped into the CPU's address space. Reading/writing these addresses is the same instruction as reading/writing RAM — but the hardware reacts to it.

There is no inb/outb instruction like x86. Everything is a load/store to an address.

| Address Range (STM32F446) | Region |
| --- | --- |
| 0x0000_0000 – 0x07FF_FFFF | Flash / ITCM aliases |
| 0x0800_0000 – 0x080F_FFFF | Flash memory (your code lives here) |
| 0x1000_0000 – 0x1000_3FFF | Core-coupled RAM (CCM) |
| 0x2000_0000 – 0x2001_FFFF | SRAM1 + SRAM2 (128 KB) |
| 0x4000_0000 – 0x4000_7FFF | APB1 peripherals (UART2, I2C, SPI2...) |
| 0x4001_0000 – 0x4001_3FFF | APB2 peripherals (SPI1, USART1, TIM1...) |
| 0x4002_0000 – 0x4002_33FF | AHB1 (GPIO, DMA, RCC, CRC) |
| 0xE000_0000 – 0xE00F_FFFF | Cortex-M4 core peripherals (NVIC, SysTick, FPU) |

📌 Note Every address in the peripheral range corresponds to a specific register described in the reference manual (RM0390 for STM32F446). The HAL just pre-calculates these addresses for you — in bare-metal, YOU do that.

### 3.2  Key STM32F446 Register Addresses

▸ rust // RCC (Reset and Clock Control) — AHB1 bus const RCC_BASE: usize = 0x4002_3800; const RCC_CR: usize = RCC_BASE + 0x00; // clock control const RCC_PLLCFGR: usize = RCC_BASE + 0x04; // PLL config const RCC_CFGR: usize = RCC_BASE + 0x08; // clock config const RCC_AHB1ENR: usize = RCC_BASE + 0x30; // AHB1 clock enable const RCC_APB1ENR: usize = RCC_BASE + 0x40; // APB1 clock enable const RCC_APB2ENR: usize = RCC_BASE + 0x44; // APB2 clock enable // GPIO ports — AHB1 bus const GPIOA_BASE: usize = 0x4002_0000; const GPIOB_BASE: usize = 0x4002_0400; const GPIOC_BASE: usize = 0x4002_0800; const GPIOD_BASE: usize = 0x4002_0C00; // GPIO register offsets (same for all ports): const GPIO_MODER: usize = 0x00; // mode: input/output/AF/analog const GPIO_OTYPER: usize = 0x04; // output type: push-pull/open-drain const GPIO_OSPEEDR: usize = 0x08; // output speed const GPIO_PUPDR: usize = 0x0C; // pull-up/pull-down const GPIO_IDR: usize = 0x10; // input data register (read-only) const GPIO_ODR: usize = 0x14; // output data register const GPIO_BSRR: usize = 0x18; // bit set/reset register const GPIO_AFRL: usize = 0x20; // alternate function low (pins 0-7) const GPIO_AFRH: usize = 0x24; // alternate function high (pins 8-15) // USART2 — APB1 bus const USART2_BASE: usize = 0x4000_4400; const USART_SR: usize = 0x00; const USART_DR: usize = 0x04; const USART_BRR: usize = 0x08; const USART_CR1: usize = 0x0C; const USART_CR2: usize = 0x10; const USART_CR3: usize = 0x14; // I2C1 — APB1 bus const I2C1_BASE: usize = 0x4000_5400; const I2C_CR1: usize = 0x00; const I2C_CR2: usize = 0x04; const I2C_OAR1: usize = 0x08; const I2C_DR: usize = 0x10; const I2C_SR1: usize = 0x14; const I2C_SR2: usize = 0x18; const I2C_CCR: usize = 0x1C; const I2C_TRISE: usize = 0x20;

### 3.3  The Volatile Read/Write Abstraction

Create a tiny safe wrapper around volatile MMIO — this is what every HAL does internally:

▸ rust // Safe wrapper for memory-mapped register access #[inline(always)] fn read32(addr: usize) -> u32 { unsafe { core::ptr::read_volatile(addr as *const u32) } } #[inline(always)] fn write32(addr: usize, val: u32) { unsafe { core::ptr::write_volatile(addr as *mut u32, val) } } #[inline(always)] fn modify32(addr: usize, clear_mask: u32, set_mask: u32) { let v = read32(addr); write32(addr, (v & !clear_mask) ¦ set_mask); } // Usage — enabling GPIOA clock in RCC_AHB1ENR: fn enable_gpioa_clock() { modify32(RCC_AHB1ENR, 0, 1 << 0); // set bit 0 }

### 3.4  Using PAC Crates (Named Registers)

The PAC (Peripheral Access Crate) is auto-generated from the vendor's SVD file. It gives you named register fields instead of raw offsets — less error-prone, but still no abstraction above the register level.

| MCU | PAC Crate |
| --- | --- |
| STM32F446RE | stm32f4 (feature = "stm32f446") |
| ESP32-S3 | esp32s3 (from esp-rs registry) |
| STM32H7 | stm32h7 |
| nRF52840 | nrf52840 |

▸ toml # Cargo.toml for STM32F446 with PAC only (no HAL!): [dependencies] cortex-m = { version = "0.7", features = ["critical-section-single-core"] } cortex-m-rt = "0.7" stm32f4 = { version = "0.15", features = ["stm32f446", "rt"] } panic-halt = "0.2" # NOTE: stm32f4 = PAC only. stm32f4xx-hal = HAL on top of PAC. # We are using ONLY the PAC here.

▸ rust // PAC usage — same registers, but named fields use stm32f4::stm32f446; fn main() { let dp = stm32f446::Peripherals::take().unwrap(); // Enable GPIOA clock via PAC: dp.RCC.ahb1enr.modify(¦_, w¦ w.gpioaen().set_bit()); // Same as our raw version: // modify32(0x4002_3830, 0, 1 << 0); // Configure PA5 as output: dp.GPIOA.moder.modify(¦_, w¦ w.moder5().bits(0b01)); // Set PA5 high: dp.GPIOA.bsrr.write(¦w¦ w.bs5().set_bit()); loop {} }

💡 Tip Use PAC for STM32 (clearer, less error-prone). Use raw register writes for ESP32-S3 (Xtensa PAC is less mature). Both are 'bare-metal' — no HAL involved.

## Chapter 4 — Linker Scripts and Startup Code

### 4.1  Why You Need a Linker Script

The linker script tells the linker WHERE to place your code and data in memory. Without it, the linker puts everything at address 0 — which is wrong for embedded targets.

memory.x for STM32F446RE:

/* memory.x — place in project root, not src/ */ MEMORY { /* STM32F446RE: 512 KB Flash starting at 0x0800_0000 */ FLASH : ORIGIN = 0x08000000, LENGTH = 512K /* STM32F446RE: 128 KB SRAM starting at 0x2000_0000 */ RAM : ORIGIN = 0x20000000, LENGTH = 128K } /* Optional: 64 KB Core-Coupled Memory (no DMA access) */ /* CCM : ORIGIN = 0x10000000, LENGTH = 64K */

memory.x for ESP32-S3:

/* ESP32-S3: Xtensa LX7 memory layout */ MEMORY { /* 4 MB external QSPI Flash — code executes via cache (XIP) */ FLASH : ORIGIN = 0x42000000, LENGTH = 4M /* 512 KB internal SRAM */ RAM : ORIGIN = 0x3FC88000, LENGTH = 400K /* 2 MB PSRAM (if using OPI PSRAM) */ /* PSRAM : ORIGIN = 0x3C000000, LENGTH = 2M */ }

### 4.2  The cortex-m-rt Startup Sequence

The cortex-m-rt crate provides the startup code that runs before main(). You don't need to write this yourself, but you must understand it:

- CPU starts execution at the Reset_Handler (address in vector table)

- Startup code copies .data section from Flash to RAM

- Startup code zeroes the .bss section in RAM

- Startup code calls main()

▸ rust // cortex-m-rt provides this automatically, but here's what it does: // (simplified) // extern symbols defined by linker script: extern "C" { static mut _sbss: u32; // start of BSS static mut _ebss: u32; // end of BSS static mut _sdata: u32; // start of .data in RAM static mut _edata: u32; // end of .data in RAM static _sidata: u32; // source of .data in Flash } // Reset handler (the first thing the CPU executes): unsafe fn Reset() -> ! { // Zero BSS: let bss_start = &mut _sbss as *mut u32; let bss_end = &mut _ebss as *mut u32; let mut ptr = bss_start; while ptr < bss_end { core::ptr::write_volatile(ptr, 0); ptr = ptr.add(1); } // Copy .data from flash to RAM: // (similar loop) main(); // call your entry point loop {} // should never return }

### 4.3  The Vector Table

The ARM vector table lives at the start of Flash (0x0800_0000 for STM32). It is an array of 32-bit addresses — each entry points to a handler function. cortex-m-rt populates this for you.

▸ rust // cortex-m-rt generates this automatically. // You only need to DEFINE the handlers you want. use cortex_m_rt::{entry, exception, interrupt}; // Define your main entry point: #[entry] fn main() -> ! { loop {} } // Override the default hard-fault handler: #[exception] unsafe fn HardFault(frame: &cortex_m_rt::ExceptionFrame) -> ! { panic!("{:?}", frame); } // Override a peripheral interrupt (TIM2): #[interrupt] fn TIM2() { // Clear pending interrupt flag, then handle }

## Chapter 5 — Interrupts Without HAL

### 5.1  NVIC (Nested Vectored Interrupt Controller)

Every ARM Cortex-M interrupt must be enabled in the NVIC (core peripheral, not vendor peripheral). The NVIC registers live in the Cortex-M core region (0xE000_E000).

▸ rust use cortex_m::peripheral::NVIC; use stm32f4::stm32f446::Interrupt; fn enable_usart2_interrupt() { unsafe { // Enable USART2 interrupt in NVIC NVIC::unmask(Interrupt::USART2); // Set priority (0 = highest, 255 = lowest for Cortex-M4) // Lower number = higher priority let nvic = &*cortex_m::peripheral::NVIC::PTR; // Priority register: USART2 is interrupt #38 // Each NVIC_IPR register holds 4 priorities (8 bits each) // IPR[9] byte 2 = interrupt 38 } } // USART2 interrupt handler — cortex-m-rt links this to vector table #[interrupt] fn USART2() { // Read DR to clear RXNE flag and get received byte: let byte = read32(USART2_BASE + USART_DR) as u8; // process byte... }

### 5.2  Critical Sections — Shared Data Between ISR and Main

▸ rust use core::sync::atomic::{AtomicU8, AtomicBool, Ordering}; // Atomic types — safe to share between ISR and main loop static RX_BYTE: AtomicU8 = AtomicU8::new(0); static RX_READY: AtomicBool = AtomicBool::new(false); #[interrupt] fn USART2() { let byte = read32(USART2_BASE + USART_DR) as u8; RX_BYTE.store(byte, Ordering::Release); RX_READY.store(true, Ordering::Release); } #[entry] fn main() -> ! { // ... init code ... loop { if RX_READY.load(Ordering::Acquire) { let b = RX_BYTE.load(Ordering::Relaxed); RX_READY.store(false, Ordering::Release); // process b } } }

🔥 Key Rule Never use a plain static mut for ISR-shared data without atomic or critical_section protection. Rust will reject it or you will have data races. AtomicU8/AtomicBool/AtomicU32 are your tools.

## Chapter 6 — Project Setup and Toolchain

### 6.1  WSL2 Environment Setup

▸ bash # System packages: sudo apt update && sudo apt install -y \ build-essential git curl pkg-config \ libssl-dev libusb-1.0-0-dev libudev-dev \ gdb-multiarch openocd stlink-tools \ gcc-arm-none-eabi binutils-arm-none-eabi # Rust: curl --proto '=https' --tlsv1.2 -sSf https://sh.rustup.rs ¦ sh source $HOME/.cargo/env # STM32 target: rustup target add thumbv7em-none-eabihf rustup toolchain install nightly rustup component add rust-src # ESP32-S3 Xtensa toolchain: cargo install espup espup install source ~/export-esp.sh echo 'source ~/export-esp.sh' >> ~/.bashrc # Flashing tools: cargo install espflash cargo-espflash cargo install probe-rs-tools --locked cargo install flip-link probe-rs complete install # udev rules for ST-Link # USB passthrough (PowerShell Admin): # winget install --interactive --exact dorssel.usbipd-win # usbipd bind --busid <ID> # usbipd attach --wsl --busid <ID>

### 6.2  STM32F446 Project Skeleton

▸ bash cargo new stm32_baremetal cd stm32_baremetal

Cargo.toml (PAC only — no HAL):

▸ toml [package] name = "stm32_baremetal" version = "0.1.0" edition = "2021" [dependencies] cortex-m = { version = "0.7", features = ["critical-section-single-core"] } cortex-m-rt = "0.7" stm32f4 = { version = "0.15", features = ["stm32f446", "rt"] } panic-halt = "0.2" # For defmt logging: defmt = "0.3" defmt-rtt = "0.4" panic-probe = { version = "0.3", features = ["print-defmt"] } [profile.release] codegen-units = 1 lto = true opt-level = "s" debug = true

.cargo/config.toml:

▸ toml [build] target = "thumbv7em-none-eabihf" [target.thumbv7em-none-eabihf] linker = "flip-link" runner = "probe-rs run --chip STM32F446RETx" rustflags = ["-C", "link-arg=-Tlink.x", "-C", "link-arg=-Tdefmt.x"]

### 6.3  ESP32-S3 Project Skeleton

▸ bash source ~/export-esp.sh cargo new esp32s3_baremetal cd esp32s3_baremetal

Cargo.toml:

▸ toml [package] name = "esp32s3_baremetal" version = "0.1.0" edition = "2021" [dependencies] # PAC-only access to ESP32-S3 peripherals: esp32s3 = { version = "0.18", features = ["rt"] } # Runtime and panic handler: esp-backtrace = { version = "0.13", features = ["esp32s3", "halt-cores", "println"] } esp-println = { version = "0.11", features = ["esp32s3"] } [profile.release] opt-level = "s" lto = "thin" overflow-checks = false

.cargo/config.toml:

▸ toml [build] target = "xtensa-esp32s3-none-elf" [target.xtensa-esp32s3-none-elf] linker = "xtensa-esp32s3-elf-ld" runner = "espflash flash --monitor" [unstable] build-std = ["core"]

rust-toolchain.toml:

▸ toml [toolchain] channel = "esp" components = ["rustfmt", "clippy", "rust-src"]

# PART B — STM32F446RE BARE-METAL

## Chapter 7 — STM32F446 Architecture Overview

### 7.1  Bus Architecture

The STM32F446 has three internal buses. Peripherals live on different buses — wrong clock domain = peripheral does not work.

| Bus | Peripherals |
| --- | --- |
| AHB1 | GPIO A-H, DMA1/2, RCC, CRC |
| APB1 | USART2-5, I2C1-3, SPI2/3, TIM2-14 |
| APB2 | USART1/6, SPI1/4/5, TIM1/8/9-11, ADC |

### 7.2  What You Must Always Do Before Using a Peripheral

- Enable the peripheral clock in RCC (AHB1ENR, APB1ENR, or APB2ENR)

- Configure GPIO pins if the peripheral needs external pins (MODER, AFR)

- Configure the peripheral registers (baud, mode, enable bit)

⚠️ Warning Forgetting step 1 (clock enable) is the #1 bare-metal bug. Accessing a peripheral without its clock enabled reads 0 and writes are silently discarded.

## Chapter 8 — RCC: Clocks from Scratch

### 8.1  Clock Sources

| Source | Description |
| --- | --- |
| HSI | 16 MHz internal RC oscillator |
| HSE | External crystal (usually 8 MHz on Nucleo) |
| PLL | Configurable multiplier from HSI or HSE |
| LSI | 32 kHz internal (for RTC/IWDG) |
| LSE | 32.768 kHz external crystal |

### 8.2  Configuring 168 MHz SYSCLK via PLL (PAC)

This sets up: HSE(8 MHz) → PLL → SYSCLK=168 MHz, AHB=168 MHz, APB1=42 MHz, APB2=84 MHz. This is the standard Nucleo-F446 high-speed configuration.

▸ rust #![no_std] #![no_main] use cortex_m_rt::entry; use panic_halt as _; use stm32f4::stm32f446; #[entry] fn main() -> ! { let dp = stm32f446::Peripherals::take().unwrap(); let rcc = &dp.RCC; let flash = &dp.FLASH; // 1. Enable HSE (external 8 MHz crystal on Nucleo) rcc.cr.modify(¦_, w¦ w.hseon().set_bit()); while rcc.cr.read().hserdy().bit_is_clear() {} // wait for HSE ready // 2. Configure Flash latency for 168 MHz (5 wait states) flash.acr.modify(¦_, w¦ unsafe { w.latency().bits(5) }); // 3. Configure PLL: HSE/M * N / P = 8/8 * 336 / 2 = 168 MHz rcc.pllcfgr.modify(¦_, w¦ unsafe { w.pllsrc().hse() // source = HSE .pllm().bits(8) // M = 8 (HSE 8 MHz / 8 = 1 MHz VCO input) .plln().bits(336) // N = 336 (VCO = 336 MHz) .pllp().div2() // P = 2 (SYSCLK = 168 MHz) .pllq().bits(7) // Q = 7 (USB 48 MHz = 336/7 ≈ 48 MHz) }); // 4. Enable PLL rcc.cr.modify(¦_, w¦ w.pllon().set_bit()); while rcc.cr.read().pllrdy().bit_is_clear() {} // wait for PLL lock // 5. Set AHB, APB1, APB2 prescalers rcc.cfgr.modify(¦_, w¦ unsafe { w.hpre().div1() // AHB = SYSCLK / 1 = 168 MHz .ppre1().div4() // APB1 = AHB / 4 = 42 MHz (max 45 MHz) .ppre2().div2() // APB2 = AHB / 2 = 84 MHz (max 90 MHz) }); // 6. Switch SYSCLK source to PLL rcc.cfgr.modify(¦_, w¦ w.sw().pll()); while !rcc.cfgr.read().sws().is_pll() {} // wait for switch // Now running at 168 MHz loop {} }

📌 Note After this sequence, SYSCLK=168 MHz, AHB=168 MHz, APB1=42 MHz, APB2=84 MHz. All peripheral timing calculations must use the APB clock for that peripheral's bus.

## Chapter 9 — GPIO Bare-Metal

### 9.1  GPIO Register Map

| Register | Function |
| --- | --- |
| MODER | Pin mode selection |
| OTYPER | Output type |
| OSPEEDR | Output speed |
| PUPDR | Pull-up/pull-down |
| IDR | Input data (read-only) |
| ODR | Output data |
| BSRR | Bit set/reset |
| AFRx | Alternate function |

### 9.2  Blink LD2 (PA5) — PAC Bare-Metal

▸ rust #![no_std] #![no_main] use cortex_m_rt::entry; use defmt::info; use defmt_rtt as _; use panic_probe as _; use stm32f4::stm32f446; #[entry] fn main() -> ! { let dp = stm32f446::Peripherals::take().unwrap(); // ── Step 1: Enable GPIOA clock ────────────────── dp.RCC.ahb1enr.modify(¦_, w¦ w.gpioaen().set_bit()); // Mandatory: read back after clock enable to ensure it's active let _ = dp.RCC.ahb1enr.read(); // ── Step 2: Configure PA5 as output ───────────── // MODER register: 2 bits per pin. PA5 = bits [11:10] // 0b01 = General purpose output mode dp.GPIOA.moder.modify(¦_, w¦ unsafe { w.moder5().bits(0b01) }); // Output type: push-pull (bit 5 of OTYPER = 0, default) dp.GPIOA.otyper.modify(¦_, w¦ w.ot5().push_pull()); // Output speed: low speed (bits [11:10] of OSPEEDR = 0b00) dp.GPIOA.ospeedr.modify(¦_, w¦ unsafe { w.ospeedr5().bits(0b00) }); // Pull: none (bits [11:10] of PUPDR = 0b00) dp.GPIOA.pupdr.modify(¦_, w¦ unsafe { w.pupdr5().bits(0b00) }); info!("GPIO PA5 configured as output"); // ── Step 3: Toggle in a loop ───────────────────── loop { // Set PA5 HIGH using BSRR (Bit Set/Reset Register) // Writing 1 to bit N in BSRR[15:0] SETS pin N atomically dp.GPIOA.bsrr.write(¦w¦ w.bs5().set_bit()); cortex_m::asm::delay(8_400_000); // ~50ms at 168 MHz // Reset PA5 LOW // Writing 1 to bit N+16 in BSRR[31:16] RESETS pin N atomically dp.GPIOA.bsrr.write(¦w¦ w.br5().set_bit()); cortex_m::asm::delay(8_400_000); } }

🔥 Key Rule Always use BSRR (not ODR) to set/clear individual GPIO pins. BSRR writes are atomic — a concurrent interrupt cannot corrupt your pin state. Writing to ODR is a read-modify-write and is NOT atomic.

### 9.3  GPIO Input — User Button (PC13)

▸ rust // PC13 = user button on Nucleo-F446RE, active-LOW // Enable GPIOC clock: dp.RCC.ahb1enr.modify(¦_, w¦ w.gpiocen().set_bit()); let _ = dp.RCC.ahb1enr.read(); // Configure PC13 as input (MODER = 0b00, which is the reset default): dp.GPIOC.moder.modify(¦_, w¦ unsafe { w.moder13().bits(0b00) }); // Enable pull-up on PC13 (PUPDR bits [27:26] = 0b01): // Note: button circuit has external pull-up on Nucleo; this is redundant but safe dp.GPIOC.pupdr.modify(¦_, w¦ unsafe { w.pupdr13().bits(0b01) }); loop { // IDR bit 13: 1 = high (not pressed), 0 = low (pressed) let idr = dp.GPIOC.idr.read(); if idr.idr13().bit_is_clear() { // Button pressed — set PA5 LED on dp.GPIOA.bsrr.write(¦w¦ w.bs5().set_bit()); } else { // Button released — set PA5 LED off dp.GPIOA.bsrr.write(¦w¦ w.br5().set_bit()); } }

### 9.4  Alternate Function — Pin Mux for UART

When a peripheral (UART, SPI, I2C) uses a GPIO pin, the pin must be set to Alternate Function mode and the correct AF number selected from the datasheet.

▸ rust // PA2 = USART2_TX (AF7 on STM32F446) // PA3 = USART2_RX (AF7 on STM32F446) // Source: STM32F446 datasheet Table 11 (Alternate function mapping) fn configure_usart2_pins(dp: &stm32f446::Peripherals) { // 1. Enable GPIOA clock (already done if blink example ran first) dp.RCC.ahb1enr.modify(¦_, w¦ w.gpioaen().set_bit()); let _ = dp.RCC.ahb1enr.read(); // 2. Set PA2 and PA3 to Alternate Function mode (MODER = 0b10) dp.GPIOA.moder.modify(¦_, w¦ unsafe { w.moder2().bits(0b10) // PA2 = AF .moder3().bits(0b10) // PA3 = AF }); // 3. Set output type to push-pull (default, 0) dp.GPIOA.otyper.modify(¦_, w¦ { w.ot2().push_pull().ot3().push_pull() }); // 4. Set speed to high dp.GPIOA.ospeedr.modify(¦_, w¦ unsafe { w.ospeedr2().bits(0b10).ospeedr3().bits(0b10) }); // 5. Select AF7 for PA2 and PA3 // AFRL controls pins 0-7 (4 bits each = AF0-AF15) // PA2 = AFRL bits [11:8], PA3 = AFRL bits [15:12] dp.GPIOA.afrl.modify(¦_, w¦ unsafe { w.afrl2().bits(7) // AF7 = USART2 .afrl3().bits(7) // AF7 = USART2 }); }

## Chapter 10 — USART Bare-Metal (168 MHz → 115200 baud)

### 10.1  USART Baud Rate Calculation

The baud rate register (BRR) contains a fixed-point number: integer part in bits [15:4], fractional part in bits [3:0].

▸ rust // Formula: BRR = FCLK / BAUD (for OVER8=0, 16x oversampling) // // USART2 is on APB1. At 168 MHz SYSCLK with PPREx=4: APB1 = 42 MHz // BRR = 42_000_000 / 115200 = 364.58... // Integer part = 364 = 0x16C // Fraction part = 0.58 * 16 = 9.28 ≈ 9 = 0x9 // BRR register = (364 << 4) ¦ 9 = 0x16C9 // // Or simply: BRR = (FCLK + BAUD/2) / BAUD (rounded) const APB1_CLOCK: u32 = 42_000_000; const BAUD_RATE: u32 = 115_200; const USART2_BRR_VAL: u32 = (APB1_CLOCK + BAUD_RATE / 2) / BAUD_RATE; // 365

### 10.2  Full USART2 Init and Transmit

▸ rust #![no_std] #![no_main] use cortex_m_rt::entry; use defmt_rtt as _; use panic_probe as _; use stm32f4::stm32f446; const APB1_HZ: u32 = 42_000_000; const BAUD: u32 = 115_200; #[entry] fn main() -> ! { let dp = stm32f446::Peripherals::take().unwrap(); // ── Clocks (168 MHz, as in Chapter 8) ────────── setup_clocks_168mhz(&dp); // ── GPIO: PA2=TX, PA3=RX ───────────────────── dp.RCC.ahb1enr.modify(¦_, w¦ w.gpioaen().set_bit()); let _ = dp.RCC.ahb1enr.read(); dp.GPIOA.moder.modify(¦_, w¦ unsafe { w.moder2().bits(0b10).moder3().bits(0b10) }); dp.GPIOA.afrl.modify(¦_, w¦ unsafe { w.afrl2().bits(7).afrl3().bits(7) // AF7 = USART2 }); dp.GPIOA.ospeedr.modify(¦_, w¦ unsafe { w.ospeedr2().bits(0b10).ospeedr3().bits(0b10) }); // ── USART2 clock enable ─────────────────────── dp.RCC.apb1enr.modify(¦_, w¦ w.usart2en().set_bit()); let _ = dp.RCC.apb1enr.read(); // ── Configure USART2 ────────────────────────── let usart = &dp.USART2; // Disable USART before configuration: usart.cr1.modify(¦_, w¦ w.ue().disabled()); // Set baud rate (BRR = APB1 / baud): usart.brr.write(¦w¦ unsafe { w.bits((APB1_HZ + BAUD / 2) / BAUD) }); // CR1: 8 data bits, no parity, 1 stop bit (all defaults) // Enable TX and RX: usart.cr1.modify(¦_, w¦ { w.te().enabled() // transmitter enable .re().enabled() // receiver enable }); // Enable USART: usart.cr1.modify(¦_, w¦ w.ue().enabled()); // ── Transmit a string ───────────────────────── let msg = b"STM32F446 bare-metal UART!\r\n"; for &byte in msg { // Wait for TXE (Transmit Data Register Empty) flag — SR bit 7 while usart.sr.read().txe().bit_is_clear() {} // Write byte to data register usart.dr.write(¦w¦ unsafe { w.dr().bits(byte as u16) }); } // Wait for TC (Transmission Complete) — SR bit 6 while usart.sr.read().tc().bit_is_clear() {} loop {} }

### 10.3  USART Receive — Polling

▸ rust fn uart_read_byte(usart: &stm32f446::USART2) -> u8 { // Wait for RXNE (Receive Not Empty) flag — SR bit 5 while usart.sr.read().rxne().bit_is_clear() {} usart.dr.read().dr().bits() as u8 } // Echo loop: loop { let b = uart_read_byte(&dp.USART2); while dp.USART2.sr.read().txe().bit_is_clear() {} dp.USART2.dr.write(¦w¦ unsafe { w.dr().bits(b as u16) }); }

### 10.4  USART Receive — Interrupt Driven

▸ rust use core::sync::atomic::{AtomicU8, AtomicBool, Ordering}; use cortex_m::peripheral::NVIC; use stm32f4::stm32f446::Interrupt; use cortex_m_rt::interrupt; static RX_DATA: AtomicU8 = AtomicU8::new(0); static RX_READY: AtomicBool = AtomicBool::new(false); fn enable_usart2_rxne_interrupt(dp: &stm32f446::Peripherals) { // Enable RXNEIE bit in USART_CR1: dp.USART2.cr1.modify(¦_, w¦ w.rxneie().enabled()); // Unmask in NVIC: unsafe { NVIC::unmask(Interrupt::USART2); } } #[interrupt] fn USART2() { // Safety: this runs in ISR context; only we touch these statics here let dp = unsafe { stm32f446::Peripherals::steal() }; let sr = dp.USART2.sr.read(); if sr.rxne().bit_is_set() { let byte = dp.USART2.dr.read().dr().bits() as u8; RX_DATA.store(byte, Ordering::Release); RX_READY.store(true, Ordering::Release); // Reading DR automatically clears RXNE flag } } #[entry] fn main() -> ! { // ... init clocks, GPIO, USART2 ... // enable_usart2_rxne_interrupt(&dp); loop { if RX_READY.load(Ordering::Acquire) { let b = RX_DATA.load(Ordering::Relaxed); RX_READY.store(false, Ordering::Release); // Echo back: while dp.USART2.sr.read().txe().bit_is_clear() {} dp.USART2.dr.write(¦w¦ unsafe { w.dr().bits(b as u16) }); } } }

## Chapter 11 — I2C Bare-Metal (STM32F446)

### 11.1  I2C Register Overview

| Register | Key Bits / Function |
| --- | --- |
| CR1 | PE (enable), SWRST, START, STOP, ACK, ACKPOS |
| CR2 | FREQ[5:0] = APB1 clock in MHz (e.g. 42 for 42 MHz) |
| CCR | Clock control register — determines SCL frequency |
| TRISE | Maximum rise time — depends on bus speed and FREQ |
| SR1 | SB(start), ADDR(address sent), TXE, RXNE, BTF, AF(NACK), BERR |
| SR2 | MSL(master mode), BUSY, TRA |
| DR | Data register — write to transmit, read to receive |

### 11.2  I2C1 Init at 400 kHz (PB8=SCL, PB9=SDA)

▸ rust // PB8 = I2C1_SCL (AF4), PB9 = I2C1_SDA (AF4) // APB1 = 42 MHz, Fast mode (400 kHz) fn init_i2c1(dp: &stm32f446::Peripherals) { // ── GPIO: PB8, PB9 as open-drain AF4 ────────── dp.RCC.ahb1enr.modify(¦_, w¦ w.gpioben().set_bit()); let _ = dp.RCC.ahb1enr.read(); dp.GPIOB.moder.modify(¦_, w¦ unsafe { w.moder8().bits(0b10).moder9().bits(0b10) // AF mode }); // I2C lines MUST be open-drain: dp.GPIOB.otyper.modify(¦_, w¦ { w.ot8().open_drain().ot9().open_drain() }); // High speed for I2C Fast mode: dp.GPIOB.ospeedr.modify(¦_, w¦ unsafe { w.ospeedr8().bits(0b10).ospeedr9().bits(0b10) }); // Pull-ups (board may have external pull-ups; enable internal too): dp.GPIOB.pupdr.modify(¦_, w¦ unsafe { w.pupdr8().bits(0b01).pupdr9().bits(0b01) }); // AF4 = I2C1 for PB8, PB9 (AFRH: pins 8-15): dp.GPIOB.afrh.modify(¦_, w¦ unsafe { w.afrh8().bits(4).afrh9().bits(4) }); // ── I2C1 init ───────────────────────────────── dp.RCC.apb1enr.modify(¦_, w¦ w.i2c1en().set_bit()); let _ = dp.RCC.apb1enr.read(); let i2c = &dp.I2C1; // Software reset to clear any stuck state: i2c.cr1.modify(¦_, w¦ w.swrst().set_bit()); i2c.cr1.modify(¦_, w¦ w.swrst().clear_bit()); // CR2: FREQ = APB1 clock in MHz = 42 i2c.cr2.modify(¦_, w¦ unsafe { w.freq().bits(42) }); // For Fast mode (400 kHz) with DUTY=0: // CCR = FCLK / (3 * FSCL) = 42_000_000 / (3 * 400_000) = 35 i2c.ccr.modify(¦_, w¦ unsafe { w.f_s().fast() // fast mode .duty().duty2_1() // Tlow/Thigh = 2/1 .ccr().bits(35) }); // TRISE: for Fast mode = FREQ * 0.3 µs + 1 = 42 * 0.3 + 1 = 14 i2c.trise.modify(¦_, w¦ unsafe { w.trise().bits(14) }); // Enable I2C: i2c.cr1.modify(¦_, w¦ w.pe().set_bit()); }

### 11.3  I2C Write Transaction

▸ rust fn i2c_write(dp: &stm32f446::Peripherals, addr: u8, data: &[u8]) { let i2c = &dp.I2C1; // 1. Wait until bus is free (BUSY flag in SR2): while i2c.sr2.read().busy().bit_is_set() {} // 2. Generate START condition: i2c.cr1.modify(¦_, w¦ w.start().set_bit()); // Wait for SB (Start Bit) in SR1: while i2c.sr1.read().sb().bit_is_clear() {} // 3. Send slave address + WRITE bit (addr << 1 ¦ 0): i2c.dr.write(¦w¦ unsafe { w.dr().bits((addr << 1) as u8) }); // Wait for ADDR flag (address acknowledged): // Also check for AF (NACK) in SR1: loop { let sr1 = i2c.sr1.read(); if sr1.af().bit_is_set() { // NACK received — generate STOP and return error i2c.cr1.modify(¦_, w¦ w.stop().set_bit()); return; } if sr1.addr().bit_is_set() { break; } } // Clear ADDR flag by reading SR1 then SR2: let _ = i2c.sr1.read(); let _ = i2c.sr2.read(); // 4. Send data bytes: for &byte in data { // Wait for TXE (data register empty): while i2c.sr1.read().txe().bit_is_clear() {} i2c.dr.write(¦w¦ unsafe { w.dr().bits(byte) }); } // 5. Wait for BTF (Byte Transfer Finished) then STOP: while i2c.sr1.read().btf().bit_is_clear() {} i2c.cr1.modify(¦_, w¦ w.stop().set_bit()); }

### 11.4  I2C Read Transaction (Write-then-Read)

▸ rust fn i2c_write_read(dp: &stm32f446::Peripherals, addr: u8, reg: u8, buf: &mut [u8]) { let i2c = &dp.I2C1; // ── Phase 1: Write register address ───────── while i2c.sr2.read().busy().bit_is_set() {} i2c.cr1.modify(¦_, w¦ w.start().set_bit()); while i2c.sr1.read().sb().bit_is_clear() {} i2c.dr.write(¦w¦ unsafe { w.dr().bits((addr << 1) as u8) }); while i2c.sr1.read().addr().bit_is_clear() {} let _ = i2c.sr1.read(); let _ = i2c.sr2.read(); // clear ADDR while i2c.sr1.read().txe().bit_is_clear() {} i2c.dr.write(¦w¦ unsafe { w.dr().bits(reg) }); while i2c.sr1.read().btf().bit_is_clear() {} // ── Phase 2: Repeated START, read ─────────── i2c.cr1.modify(¦_, w¦ w.start().set_bit()); // repeated start while i2c.sr1.read().sb().bit_is_clear() {} // Send address + READ bit: i2c.dr.write(¦w¦ unsafe { w.dr().bits((addr << 1) ¦ 1) }); while i2c.sr1.read().addr().bit_is_clear() {} let n = buf.len(); if n == 1 { // For single byte: disable ACK before clearing ADDR i2c.cr1.modify(¦_, w¦ w.ack().clear_bit()); let _ = i2c.sr1.read(); let _ = i2c.sr2.read(); i2c.cr1.modify(¦_, w¦ w.stop().set_bit()); while i2c.sr1.read().rxne().bit_is_clear() {} buf[0] = i2c.dr.read().dr().bits(); } else { // Enable ACK for multi-byte: i2c.cr1.modify(¦_, w¦ w.ack().set_bit()); let _ = i2c.sr1.read(); let _ = i2c.sr2.read(); for i in 0..n { if i == n - 1 { i2c.cr1.modify(¦_, w¦ w.ack().clear_bit()); i2c.cr1.modify(¦_, w¦ w.stop().set_bit()); } while i2c.sr1.read().rxne().bit_is_clear() {} buf[i] = i2c.dr.read().dr().bits(); } } }

### 11.5  Read MPU6050 Accelerometer via I2C

▸ rust const MPU6050_ADDR: u8 = 0x68; // AD0 pin low const PWR_MGMT_1: u8 = 0x6B; const ACCEL_XOUT_H: u8 = 0x3B; #[entry] fn main() -> ! { // ... clocks + i2c init ... // Wake up MPU6050 (write 0x00 to PWR_MGMT_1): i2c_write(&dp, MPU6050_ADDR, &[PWR_MGMT_1, 0x00]); loop { // Read 6 bytes from ACCEL_XOUT_H: let mut raw = [0u8; 6]; i2c_write_read(&dp, MPU6050_ADDR, ACCEL_XOUT_H, &mut raw); // Combine high and low bytes: let ax = i16::from_be_bytes([raw[0], raw[1]]); let ay = i16::from_be_bytes([raw[2], raw[3]]); let az = i16::from_be_bytes([raw[4], raw[5]]); // Scale: full-scale ±2g, sensitivity 16384 LSB/g let ax_g = ax as f32 / 16384.0; let ay_g = ay as f32 / 16384.0; let az_g = az as f32 / 16384.0; defmt::info!("Accel: X={} Y={} Z={} g", ax_g as i32, ay_g as i32, az_g as i32); cortex_m::asm::delay(8_400_000); // ~50 ms } }

## Chapter 12 — SPI Bare-Metal (STM32F446)

### 12.1  SPI1 Init — PA5=SCK, PA6=MISO, PA7=MOSI, PA4=NSS

▸ rust // SPI1: APB2 bus (84 MHz at 168 MHz SYSCLK) // PA5=SCK, PA6=MISO, PA7=MOSI (AF5), PA4=NSS (software GPIO) fn init_spi1(dp: &stm32f446::Peripherals) { // GPIO: PA4 (NSS/CS) as output, PA5/6/7 as AF5 dp.RCC.ahb1enr.modify(¦_, w¦ w.gpioaen().set_bit()); let _ = dp.RCC.ahb1enr.read(); // PA4 as output (NSS, software controlled): dp.GPIOA.moder.modify(¦_, w¦ unsafe { w.moder4().bits(0b01) }); // NSS idle HIGH: dp.GPIOA.bsrr.write(¦w¦ w.bs4().set_bit()); // PA5,6,7 as AF5 (SPI1): dp.GPIOA.moder.modify(¦_, w¦ unsafe { w.moder5().bits(0b10).moder6().bits(0b10).moder7().bits(0b10) }); dp.GPIOA.ospeedr.modify(¦_, w¦ unsafe { w.ospeedr5().bits(0b11).ospeedr6().bits(0b11).ospeedr7().bits(0b11) }); dp.GPIOA.afrl.modify(¦_, w¦ unsafe { w.afrl5().bits(5).afrl6().bits(5).afrl7().bits(5) // AF5 = SPI1 }); // Enable SPI1 clock (APB2): dp.RCC.apb2enr.modify(¦_, w¦ w.spi1en().set_bit()); let _ = dp.RCC.apb2enr.read(); let spi = &dp.SPI1; // CR1 configuration: // MSTR=1 (master), BR[2:0]=0b010 (APB2/8 = 84/8 = 10.5 MHz), // CPOL=0, CPHA=0 (Mode 0), SSM=1 (software NSS), SSI=1 // DFF=0 (8-bit), LSBFIRST=0 (MSB first) spi.cr1.write(¦w¦ unsafe { w.mstr().master() .br().bits(0b010) // fPCLK/8 .cpol().idle_low() .cpha().first_edge() .ssm().set_bit() // software NSS .ssi().set_bit() // NSS internal = 1 .dff().eight_bit() // 8-bit frame .lsbfirst().msbfirst() .spe().enabled() // enable SPI }); }

### 12.2  SPI Transfer (Full-Duplex)

▸ rust fn spi_transfer(dp: &stm32f446::Peripherals, tx: &[u8], rx: &mut [u8]) { let spi = &dp.SPI1; assert_eq!(tx.len(), rx.len(), "SPI: tx and rx must be same length"); // Assert NSS (PA4 low): dp.GPIOA.bsrr.write(¦w¦ w.br4().set_bit()); for i in 0..tx.len() { // Wait for TXE (Transmit buffer Empty) — SR bit 1: while spi.sr.read().txe().bit_is_clear() {} // Write to data register (8-bit write): spi.dr.write(¦w¦ unsafe { w.dr().bits(tx[i] as u16) }); // Wait for RXNE (Receive buffer Not Empty) — SR bit 0: while spi.sr.read().rxne().bit_is_clear() {} rx[i] = spi.dr.read().dr().bits() as u8; } // Wait for BSY to clear before deasserting NSS: while spi.sr.read().bsy().bit_is_set() {} // Deassert NSS (PA4 high): dp.GPIOA.bsrr.write(¦w¦ w.bs4().set_bit()); } // Example: read WHO_AM_I from an SPI device: fn spi_read_reg(dp: &stm32f446::Peripherals, reg: u8) -> u8 { let tx = [reg ¦ 0x80, 0x00]; // read flag ¦ address, dummy byte let mut rx = [0u8; 2]; spi_transfer(dp, &tx, &mut rx); rx[1] // first byte is dummy, second is the data }

## Chapter 13 — SysTick and Software Delay

▸ rust use cortex_m::peripheral::syst::SystClkSource; fn systick_init(syst: &mut cortex_m::peripheral::SYST) { // SysTick runs at SYSCLK = 168 MHz // Reload value for 1 ms interrupt: 168_000 - 1 syst.set_reload(168_000 - 1); // overflows every 1 ms syst.clear_current(); syst.set_clock_source(SystClkSource::Core); syst.enable_counter(); syst.enable_interrupt(); } // Millisecond counter in an atomic use core::sync::atomic::{AtomicU32, Ordering}; static TICK_MS: AtomicU32 = AtomicU32::new(0); #[exception] fn SysTick() { TICK_MS.fetch_add(1, Ordering::Relaxed); } fn delay_ms(ms: u32) { let start = TICK_MS.load(Ordering::Relaxed); while TICK_MS.load(Ordering::Relaxed).wrapping_sub(start) < ms {} } fn get_tick_ms() -> u32 { TICK_MS.load(Ordering::Relaxed) }

# PART C — ESP32-S3 ZERO BARE-METAL

## Chapter 14 — ESP32-S3 Architecture

### 14.1  Memory Map

| Address Range | Region |
| --- | --- |
| 0x0000_0000 – 0x3FFF_FFFF | ROM / eFuse / peripheral aliases |
| 0x3C00_0000 – 0x3CFF_FFFF | External PSRAM (via Cache) |
| 0x3FC8_8000 – 0x3FCE_FFFF | Internal SRAM2 (448 KB) |
| 0x4000_0000 – 0x4007_FFFF | Internal ROM (CPU read-only) |
| 0x4200_0000 – 0x42FF_FFFF | Flash via ICache (XIP — code runs here) |
| 0x6000_0000 – 0x600F_FFFF | Peripheral register space (DR_REG_*) |
| 0x600C_0000 | GPIO registers |
| 0x6000_1000 | UART0 registers |
| 0x6001_3000 | I2C0 registers |
| 0x6002_4000 | SPI2 registers |

### 14.2  ESP32-S3 Peripheral Clock Model

Unlike STM32, ESP32-S3 has a different clock enable model:

- DPORT/System peripheral clock gating: each peripheral has an enable bit in a system register

- GPIO is always clocked: no clock enable needed for GPIO

- UART/I2C/SPI: enable via SYSTEM_PERIP_CLK_EN0 and SYSTEM_PERIP_RST_EN0

▸ rust // System/DPORT register base addresses for ESP32-S3: const SYSTEM_BASE: usize = 0x600C_0000; // System registers const SYSTEM_PERIP_CLK_EN0: usize = SYSTEM_BASE + 0x018; const SYSTEM_PERIP_RST_EN0: usize = SYSTEM_BASE + 0x020; // Bit positions in PERIP_CLK_EN0: const CLK_EN_UART0: u32 = 1 << 2; const CLK_EN_UART1: u32 = 1 << 3; const CLK_EN_I2C0: u32 = 1 << 7; const CLK_EN_SPI2: u32 = 1 << 9;

### 14.3  Key Register Addresses

▸ rust // GPIO (always clocked, no enable needed) const GPIO_BASE: usize = 0x6000_4000; const GPIO_OUT_REG: usize = GPIO_BASE + 0x004; // output value const GPIO_OUT_W1TS_REG: usize = GPIO_BASE + 0x008; // write-1-to-set const GPIO_OUT_W1TC_REG: usize = GPIO_BASE + 0x00C; // write-1-to-clear const GPIO_ENABLE_REG: usize = GPIO_BASE + 0x020; // output enable const GPIO_IN_REG: usize = GPIO_BASE + 0x03C; // input level const GPIO_PIN0_REG: usize = GPIO_BASE + 0x074; // pin 0 config // IO_MUX — configures pin function (GPIO vs peripheral) const IO_MUX_BASE: usize = 0x6000_9000; const IO_MUX_GPIO0: usize = IO_MUX_BASE + 0x04; // pin 0 // IO_MUX_GPIOx = IO_MUX_BASE + 0x04 + (pin * 4) // UART0 registers: const UART0_BASE: usize = 0x6000_0000; const UART_FIFO_REG: usize = 0x000; // TX/RX FIFO const UART_INT_ST_REG: usize = 0x008; const UART_CONF0_REG: usize = 0x020; const UART_CLKDIV_REG: usize = 0x014; const UART_STATUS_REG: usize = 0x01C; // I2C0 registers: const I2C0_BASE: usize = 0x6001_3000; const I2C_SCL_LOW_REG: usize = 0x000; const I2C_SCL_HIGH_REG: usize = 0x004; const I2C_SDA_HOLD_REG: usize = 0x008; const I2C_SDA_SAMPLE_REG: usize = 0x00C; const I2C_DATA_REG: usize = 0x02C; const I2C_CMD0_REG: usize = 0x058; // command 0-15 const I2C_SR_REG: usize = 0x01C;

## Chapter 15 — ESP32-S3 GPIO Bare-Metal

### 15.1  GPIO Architecture on ESP32-S3

ESP32-S3 GPIO uses a matrix — each GPIO pin can be routed to any peripheral. This is different from STM32's fixed AF numbers.

For simple GPIO (not connected to a peripheral), the key registers are:

- [object Object] — selects function: MCU_SEL=1 = simple GPIO

- [object Object] — bit N = 1 enables output on GPIO N

- [object Object] — write 1 to bit N to SET GPIO N high

- [object Object] — write 1 to bit N to CLEAR GPIO N low

- [object Object] — read bit N for input level of GPIO N

### 15.2  Inline Helper Functions

▸ rust #[inline(always)] fn read32(addr: usize) -> u32 { unsafe { core::ptr::read_volatile(addr as *const u32) } } #[inline(always)] fn write32(addr: usize, val: u32) { unsafe { core::ptr::write_volatile(addr as *mut u32, val) } } #[inline(always)] fn modify32(addr: usize, clear: u32, set: u32) { write32(addr, (read32(addr) & !clear) ¦ set); } // GPIO helpers: fn gpio_set_output(pin: u8) { // IO_MUX: set MCU_SEL=1 (function 1 = GPIO), disable pullup/pulldown let mux = IO_MUX_BASE + 0x04 + (pin as usize * 4); write32(mux, (1 << 12)); // MCU_SEL = 1 // Enable output: if pin < 32 { modify32(GPIO_ENABLE_REG, 0, 1 << pin); } else { // GPIO_ENABLE1_REG for pins 32-47: modify32(GPIO_BASE + 0x024, 0, 1 << (pin - 32)); } } fn gpio_set_input(pin: u8, pullup: bool) { let mux = IO_MUX_BASE + 0x04 + (pin as usize * 4); let pullup_bit: u32 = if pullup { 1 << 10 } else { 0 }; write32(mux, (1 << 12) ¦ (1 << 9) ¦ pullup_bit); // MCU_SEL=1, IE=1 // Disable output enable: if pin < 32 { modify32(GPIO_ENABLE_REG, 1 << pin, 0); } } fn gpio_set(pin: u8) { if pin < 32 { write32(GPIO_OUT_W1TS_REG, 1 << pin); } else { write32(GPIO_BASE + 0x010, 1 << (pin-32)); } } fn gpio_clear(pin: u8) { if pin < 32 { write32(GPIO_OUT_W1TC_REG, 1 << pin); } else { write32(GPIO_BASE + 0x014, 1 << (pin-32)); } } fn gpio_read(pin: u8) -> bool { if pin < 32 { (read32(GPIO_IN_REG) >> pin) & 1 != 0 } else { (read32(GPIO_BASE + 0x040) >> (pin-32)) & 1 != 0 } }

### 15.3  Blink GPIO21 (Built-in RGB LED on ESP32-S3 Zero)

▸ rust #![no_std] #![no_main] use esp_backtrace as _; use esp_println::println; // ── Register addresses ─────────────────────────────────────── const IO_MUX_BASE: usize = 0x6000_9000; const GPIO_BASE: usize = 0x6000_4000; const GPIO_OUT_W1TS_REG: usize = GPIO_BASE + 0x008; const GPIO_OUT_W1TC_REG: usize = GPIO_BASE + 0x00C; const GPIO_ENABLE_REG: usize = GPIO_BASE + 0x020; // ── Helpers ────────────────────────────────────────────────── #[inline(always)] fn write32(addr: usize, val: u32) { unsafe { core::ptr::write_volatile(addr as *mut u32, val); } } #[inline(always)] fn modify32(addr: usize, clear: u32, set: u32) { let cur = unsafe { core::ptr::read_volatile(addr as *const u32) }; write32(addr, (cur & !clear) ¦ set); } fn delay(cycles: u32) { for _ in 0..cycles { core::hint::black_box(()); } } // ── Entry point ────────────────────────────────────────────── #[esp_hal::entry] fn main() -> ! { // Configure GPIO21 as output // IO_MUX_GPIO21: offset = 0x04 + (21 * 4) = 0x58 write32(IO_MUX_BASE + 0x58, 1 << 12); // MCU_SEL=1 (GPIO function) // Enable output on GPIO21: modify32(GPIO_ENABLE_REG, 0, 1 << 21); println!("ESP32-S3 bare-metal GPIO blink on GPIO21"); let mut state = false; loop { if state { write32(GPIO_OUT_W1TS_REG, 1 << 21); // SET high } else { write32(GPIO_OUT_W1TC_REG, 1 << 21); // CLEAR low } state = !state; delay(2_000_000); // rough delay println!("GPIO21 = {}", state); } }

### 15.4  GPIO Input with Pull-up (Boot Button GPIO0)

▸ rust // GPIO0 = boot button on ESP32-S3 Zero (active-LOW when pressed) #[esp_hal::entry] fn main() -> ! { // Configure GPIO21 as output (LED): write32(IO_MUX_BASE + 0x58, 1 << 12); // GPIO21 IO_MUX modify32(GPIO_ENABLE_REG, 0, 1 << 21); // Configure GPIO0 as input with internal pull-up: // IO_MUX_GPIO0 offset = 0x04 + (0 * 4) = 0x04 // Bit 12: MCU_SEL=1 (GPIO), Bit 10: WPU=1 (pull-up), Bit 9: IE=1 (input enable) write32(IO_MUX_BASE + 0x04, (1 << 12) ¦ // MCU_SEL = function 1 (GPIO) (1 << 10) ¦ // weak pull-up enable (1 << 9)); // input enable // Disable output on GPIO0: modify32(GPIO_ENABLE_REG, 1 << 0, 0); loop { let gpio_in = unsafe { core::ptr::read_volatile((GPIO_BASE + 0x03C) as *const u32) }; let button_pressed = (gpio_in & (1 << 0)) == 0; // active-low if button_pressed { write32(GPIO_OUT_W1TS_REG, 1 << 21); // LED on } else { write32(GPIO_OUT_W1TC_REG, 1 << 21); // LED off } } }

## Chapter 16 — ESP32-S3 UART Bare-Metal

### 16.1  UART0 Register Overview

| Register (offset from UART0_BASE) | Function |
| --- | --- |
| 0x000 UART_FIFO_REG | Write byte to TX FIFO; read byte from RX FIFO |
| 0x014 UART_CLKDIV_REG | Clock divider for baud rate |
| 0x01C UART_STATUS_REG | TXFIFO_CNT[20:16], RXFIFO_CNT[7:0] |
| 0x020 UART_CONF0_REG | Parity, data bits, stop bits, UART mode |
| 0x024 UART_CONF1_REG | FIFO thresholds |
| 0x068 UART_CLK_CONF | Clock source selection |

### 16.2  UART0 Init at 115200 Baud

▸ rust // ESP32-S3 UART0: GPIO43=TXD, GPIO44=RXD // Default after reset: UART0 is usually pre-configured by ROM bootloader. // For custom UART, we reconfigure here. const UART0_BASE: usize = 0x6000_0000; const UART_FIFO_REG: usize = UART0_BASE + 0x000; const UART_CLKDIV_REG: usize = UART0_BASE + 0x014; const UART_STATUS_REG: usize = UART0_BASE + 0x01C; const UART_CONF0_REG: usize = UART0_BASE + 0x020; const UART_CLK_CONF: usize = UART0_BASE + 0x068; const SYSTEM_BASE: usize = 0x600C_0000; fn init_uart0_115200() { // 1. Enable UART0 peripheral clock: // SYSTEM_PERIP_CLK_EN0 bit 2 = UART0_CLK_EN let clk_en0 = SYSTEM_BASE + 0x018; let rst_en0 = SYSTEM_BASE + 0x020; modify32(clk_en0, 0, 1 << 2); // enable clock modify32(rst_en0, 1 << 2, 0); // release reset // 2. Configure TX/RX GPIO pins // GPIO43 = UART0_TXD — IO_MUX offset = 0x04 + 43*4 = 0xB0 // GPIO44 = UART0_RXD — IO_MUX offset = 0x04 + 44*4 = 0xB4 // For UART function: MCU_SEL = 1 (GPIO matrix routes them) write32(IO_MUX_BASE + 0xB0, (1 << 12)); // TX: output write32(IO_MUX_BASE + 0xB4, (1 << 12) ¦ (1 << 9)); // RX: input enable // GPIO matrix: route UART0_TXD to GPIO43 // GPIO_FUNC${n}_OUT_SEL_CFG: base = GPIO_BASE + 0x554 // UART0 TXD signal index = 6, RXD signal index = 6 write32(GPIO_BASE + 0x554 + (43 * 4), 6); // GPIO43 out = UART0_TXD // GPIO_FUNC${n}_IN_SEL_CFG for UART0 RXD: write32(GPIO_BASE + 0x154 + (6 * 4), (44 << 0) ¦ (1 << 7) ¦ (0 << 6)); // pin=44, invert=0 // 3. Set baud rate: CLK_DIVIDER = APB_CLK / BAUD // Default APB_CLK = 80 MHz after PLL startup // CLKDIV = 80_000_000 / 115200 = 694 write32(UART_CLKDIV_REG, 694); // 4. Configure frame: 8N1 // CONF0: BIT_NUM=3 (8 bits), STOP_BIT_NUM=1, PARITY_EN=0 write32(UART_CONF0_REG, (3 << 2) ¦ // BIT_NUM = 0b11 = 8 data bits (1 << 4)); // STOP_BIT_NUM = 1 stop bit }

### 16.3  UART Transmit and Receive

▸ rust fn uart0_write_byte(byte: u8) { // Wait until TX FIFO has space (TXFIFO_CNT[20:16] < 128): loop { let status = read32(UART_STATUS_REG); let tx_count = (status >> 16) & 0x1FF; if tx_count < 127 { break; } } // Write byte to FIFO: write32(UART_FIFO_REG, byte as u32); } fn uart0_write(data: &[u8]) { for &b in data { uart0_write_byte(b); } } fn uart0_read_byte() -> Option<u8> { let status = read32(UART_STATUS_REG); let rx_count = status & 0xFF; // RXFIFO_CNT bits [7:0] if rx_count > 0 { Some(read32(UART_FIFO_REG) as u8) } else { None } } #[esp_hal::entry] fn main() -> ! { init_uart0_115200(); uart0_write(b"ESP32-S3 bare-metal UART ready!\r\n"); loop { if let Some(b) = uart0_read_byte() { uart0_write_byte(b); // echo } } }

## Chapter 17 — ESP32-S3 I2C Bare-Metal

### 17.1  I2C0 Architecture on ESP32-S3

ESP32-S3's I2C controller uses a command queue — you load up to 16 command objects, then start execution. This is different from STM32's register-by-register approach.

| Register | Function |
| --- | --- |
| I2C_CMD0_REG … I2C_CMD15_REG | Command slots: RSTART, WRITE, READ, STOP, END |
| I2C_DATA_REG | Read/write a byte from/to the data queue |
| I2C_SR_REG | Status: BUS_BUSY, MASTER_ACTIVE, TX_FIFO_CNT |
| I2C_INT_STATUS_REG | Interrupt flags: TRANS_COMPLETE, ACK_ERR, TIME_OUT |
| I2C_SCL_LOW_REG | SCL low period counter |
| I2C_SCL_HIGH_REG | SCL high period counter |

▸ rust // I2C command word format (32 bits): // [31:11] reserved // [10] CMD_DONE — set by hardware when command completes // [9:8] CMD_OP_CODE: 0=RSTART, 1=WRITE, 2=READ, 3=STOP, 4=END // [7] ACK_VALUE: for WRITE = expected ACK (0=ACK), for READ = send ACK(0)/NACK(1) // [6] ACK_EXP: expected ACK level in WRITE // [5] ACK_CHECK_EN: 1 = check ACK, abort on NACK // [4:0] BYTE_NUM: number of bytes for this command const CMD_RSTART: u32 = 0 << 11; // no, op=0 const CMD_WRITE: u32 = 1 << 11; // op=1 const CMD_READ: u32 = 2 << 11; // op=2 const CMD_STOP: u32 = 3 << 11; // op=3 const CMD_END: u32 = 4 << 11; // op=4 const ACK_CHECK: u32 = 1 << 8; const SEND_ACK: u32 = 0 << 10; const SEND_NACK: u32 = 1 << 10;

### 17.2  I2C0 Init (GPIO8=SDA, GPIO9=SCL, 400 kHz)

▸ rust const I2C0_BASE: usize = 0x6001_3000; const I2C_SCL_LOW: usize = I2C0_BASE + 0x000; const I2C_SCL_HIGH: usize = I2C0_BASE + 0x004; const I2C_SDA_HOLD: usize = I2C0_BASE + 0x008; const I2C_SDA_SAMPLE: usize = I2C0_BASE + 0x00C; const I2C_SCL_RSTART_SETUP: usize = I2C0_BASE + 0x010; const I2C_SCL_STOP_HOLD: usize = I2C0_BASE + 0x014; const I2C_SCL_STOP_SETUP: usize = I2C0_BASE + 0x018; const I2C_DATA: usize = I2C0_BASE + 0x02C; const I2C_SR: usize = I2C0_BASE + 0x01C; const I2C_TO: usize = I2C0_BASE + 0x020; const I2C_SLAVE_ADDR: usize = I2C0_BASE + 0x024; const I2C_FIFO_CONF: usize = I2C0_BASE + 0x028; const I2C_INT_STATUS: usize = I2C0_BASE + 0x030; const I2C_INT_CLR: usize = I2C0_BASE + 0x034; const I2C_CTR: usize = I2C0_BASE + 0x044; const I2C_CMD0: usize = I2C0_BASE + 0x058; fn init_i2c0_400khz() { // Enable I2C0 clock and release reset: let clk_en0 = SYSTEM_BASE + 0x018; let rst_en0 = SYSTEM_BASE + 0x020; modify32(clk_en0, 0, 1 << 7); // I2C0_CLK_EN bit 7 modify32(rst_en0, 1 << 7, 0); // release I2C0 reset // Configure GPIO8=SDA, GPIO9=SCL as open-drain with pull-up: // IO_MUX_GPIO8 offset = 0x04 + 8*4 = 0x24 // IO_MUX_GPIO9 offset = 0x04 + 9*4 = 0x28 // Route via GPIO matrix (MCU_SEL=1) let iomux_sda = IO_MUX_BASE + 0x24; let iomux_scl = IO_MUX_BASE + 0x28; write32(iomux_sda, (1<<12)¦(1<<10)¦(1<<9)); // MCU_SEL, WPU, IE write32(iomux_scl, (1<<12)¦(1<<10)¦(1<<9)); // GPIO matrix: I2C0 SDA/SCL output signals // I2C0_SDA_OUT signal = 46, I2C0_SCL_OUT signal = 45 write32(GPIO_BASE + 0x554 + (8*4), 46 ¦ (1<<9)); // SDA pin8, open-drain write32(GPIO_BASE + 0x554 + (9*4), 45 ¦ (1<<9)); // SCL pin9, open-drain // I2C0 SDA/SCL input write32(GPIO_BASE + 0x154 + (45*4), (9 << 0) ¦ (1<<7)); // SCL in = GPIO9 write32(GPIO_BASE + 0x154 + (46*4), (8 << 0) ¦ (1<<7)); // SDA in = GPIO8 // Clock parameters for 400 kHz (APB = 80 MHz): // Period = 80_000_000 / 400_000 = 200 cycles // SCL_LOW = 125 cycles (~62.5% of period, t_low >= 1.3 µs at 400 kHz) // SCL_HIGH = 75 cycles write32(I2C_SCL_LOW, 125); write32(I2C_SCL_HIGH, 75); write32(I2C_SDA_HOLD, 40); write32(I2C_SDA_SAMPLE, 30); write32(I2C_SCL_RSTART_SETUP, 40); write32(I2C_SCL_STOP_HOLD, 40); write32(I2C_SCL_STOP_SETUP, 40); // Timeout = 20 * 80 MHz / 400 kHz bits = generous timeout write32(I2C_TO, 200_000); // Master mode, enable TX/RX FIFO: write32(I2C_CTR, (1<<4)¦(1<<2)¦(1<<3)); // MS_MODE, SDA/SCL sample edge write32(I2C_FIFO_CONF, (1<<0)); // non-FIFO (direct) mode }

### 17.3  I2C Write via Command Queue

▸ rust fn i2c0_write(slave_addr: u8, data: &[u8]) -> bool { // Clear interrupts: write32(I2C_INT_CLR, 0xFFFF_FFFF); // Load TX data into data register (direct mode, 1 byte at a time) // Command 0: START write32(I2C_CMD0, (0 << 11) ¦ (1 << 7)); // RSTART, op=0 // Command 1: WRITE address+W (1 byte) with ACK check write32(I2C_CMD0 + 0x04, (1 << 11) ¦ ACK_CHECK ¦ 1); // WRITE, 1 byte write32(I2C_DATA, (slave_addr << 1) as u32); // addr<<1 ¦ WRITE // Load data bytes into FIFO and create WRITE commands: let mut cmd_idx = 2usize; for (i, &byte) in data.iter().enumerate() { write32(I2C_CMD0 + (cmd_idx * 4), (1 << 11) ¦ ACK_CHECK ¦ 1); // WRITE 1 byte // In non-FIFO mode write data register before triggering: // (simplified — real implementation uses FIFO for multi-byte) let _ = (byte, i); cmd_idx += 1; } // Last command: STOP write32(I2C_CMD0 + (cmd_idx * 4), (3 << 11)); // Start execution: modify32(I2C_CTR, 0, 1 << 5); // TRANS_START bit // Wait for TRANS_COMPLETE interrupt flag: let mut timeout = 100_000u32; loop { let status = read32(I2C_INT_STATUS); if (status >> 7) & 1 != 0 { return true; } // TRANS_COMPLETE if (status >> 10) & 1 != 0 { return false; } // ACK_ERR = NACK if (status >> 8) & 1 != 0 { return false; } // TIME_OUT timeout -= 1; if timeout == 0 { return false; } } }

📌 Note ESP32-S3's I2C controller is command-queue based. For complex multi-byte transfers, load all bytes into the TX FIFO first, then set up the command sequence and trigger TRANS_START. The hardware executes all commands sequentially without CPU intervention.

## Chapter 18 — ESP32-S3 SPI Bare-Metal

### 18.1  SPI2 Registers

▸ rust const SPI2_BASE: usize = 0x6002_4000; const SPI_CMD_REG: usize = SPI2_BASE + 0x000; // start transfer const SPI_ADDR_REG: usize = SPI2_BASE + 0x004; const SPI_CTRL_REG: usize = SPI2_BASE + 0x008; // bit order const SPI_CLOCK_REG: usize = SPI2_BASE + 0x018; // clock divider const SPI_USER_REG: usize = SPI2_BASE + 0x01C; // mode control const SPI_USER1_REG: usize = SPI2_BASE + 0x020; // addr/dummy bits const SPI_USER2_REG: usize = SPI2_BASE + 0x024; // command bits const SPI_MISO_DLEN: usize = SPI2_BASE + 0x028; // RX length const SPI_MOSI_DLEN: usize = SPI2_BASE + 0x02C; // TX length const SPI_W0_REG: usize = SPI2_BASE + 0x098; // data buffer W0 const SPI_MISC_REG: usize = SPI2_BASE + 0x034; // CS control const SPI_MS_DLEN_REG: usize = SPI2_BASE + 0x028;

### 18.2  SPI2 Init (10 MHz, Mode 0, GPIO10=CS, 11=MOSI, 12=MISO, 13=CLK)

▸ rust fn init_spi2_10mhz() { // Enable SPI2 clock: modify32(SYSTEM_BASE + 0x018, 0, 1 << 9); // SPI2_CLK_EN modify32(SYSTEM_BASE + 0x020, 1 << 9, 0); // release reset // Configure GPIO pins via IO_MUX: // GPIO10=CS, GPIO11=MOSI, GPIO12=MISO, GPIO13=CLK // IO_MUX offsets: pin*4 + 0x04 for pin in [10u8, 11, 13] { // output pins let mux = IO_MUX_BASE + 0x04 + (pin as usize * 4); write32(mux, 1 << 12); // MCU_SEL=1, output modify32(GPIO_ENABLE_REG, 0, 1 << pin); } // GPIO12 = MISO: input write32(IO_MUX_BASE + 0x34, (1<<12)¦(1<<9)); // GPIO12, IE=1 // GPIO matrix signal assignments for SPI2: // SPI2_CLK_OUT=0, SPI2_MOSI_OUT=1, SPI2_MISO_IN=2, SPI2_CS0_OUT=3 write32(GPIO_BASE + 0x554 + (13*4), 0); // CLK → GPIO13 write32(GPIO_BASE + 0x554 + (11*4), 1); // MOSI → GPIO11 write32(GPIO_BASE + 0x554 + (10*4), 3); // CS → GPIO10 write32(GPIO_BASE + 0x154 + (2*4), (12<<0)¦(1<<7)); // MISO ← GPIO12 // SPI2 master mode: USER register // USR_MOSI=1 (transmit), USR_MISO=1 (receive), DOUTDIN=0 (full-duplex) write32(SPI_USER_REG, (1<<27)¦(1<<28)¦(1<<29)); // Clock: APB=80 MHz, target=10 MHz → divider=8 // CLKDIV_PRE=0, CLKCNT_N=7, CLKCNT_H=3, CLKCNT_L=7 write32(SPI_CLOCK_REG, (0 << 18) ¦ // CLKDIV_PRE = 0 (7 << 12) ¦ // CLKCNT_N = 7 (3 << 6) ¦ // CLKCNT_H = 3 (7 << 0)); // CLKCNT_L = 7 // CTRL: MSB first write32(SPI_CTRL_REG, 0); }

### 18.3  SPI Transfer (8 bits)

▸ rust fn spi2_transfer_byte(tx: u8) -> u8 { // Set TX length: 8 bits - 1 = 7 write32(SPI2_BASE + 0x02C, 7); // MOSI_DLEN write32(SPI2_BASE + 0x028, 7); // MISO_DLEN // Write TX byte to W0: write32(SPI_W0_REG, tx as u32); // CS assert (manual, MISC_REG CS=0): modify32(SPI_MISC_REG, 0x3F, 0); // Start transfer: set USR bit in CMD_REG: write32(SPI_CMD_REG, 1 << 18); // Wait for transfer done (USR bit clears): while (read32(SPI_CMD_REG) >> 18) & 1 != 0 {} // CS deassert: modify32(SPI_MISC_REG, 0, 0x3F); // Read received byte: (read32(SPI_W0_REG) & 0xFF) as u8 }

## Chapter 19 — ESP32-S3 Timers and Delays

### 19.1  Xtensa Cycle Counter (CCOUNT)

▸ rust // ESP32-S3 has a cycle counter register (CCOUNT) in the Xtensa core. // It increments every CPU cycle at 240 MHz. // 1 µs = 240 cycles #[inline(always)] fn get_ccount() -> u32 { let count: u32; unsafe { core::arch::asm!("rsr.ccount {0}", out(reg) count); } count } fn delay_us(us: u32) { let start = get_ccount(); let cycles = us * 240; // 240 MHz while get_ccount().wrapping_sub(start) < cycles {} } fn delay_ms(ms: u32) { delay_us(ms * 1000); }

### 19.2  General Purpose Timer (GPTIMER) for Periodic Events

▸ rust // TIMG0 timer registers: const TIMG0_BASE: usize = 0x6001_F000; const TIMG_T0CONFIG: usize = TIMG0_BASE + 0x000; const TIMG_T0LO: usize = TIMG0_BASE + 0x004; const TIMG_T0HI: usize = TIMG0_BASE + 0x008; const TIMG_T0UPDATE: usize = TIMG0_BASE + 0x00C; const TIMG_T0ALARMLO: usize = TIMG0_BASE + 0x010; const TIMG_T0ALARMHI: usize = TIMG0_BASE + 0x014; const TIMG_T0LOADLO: usize = TIMG0_BASE + 0x018; const TIMG_T0LOADHI: usize = TIMG0_BASE + 0x01C; const TIMG_T0LOAD: usize = TIMG0_BASE + 0x020; const TIMG_INT_ENA: usize = TIMG0_BASE + 0x07C; const TIMG_INT_ST: usize = TIMG0_BASE + 0x080; const TIMG_INT_CLR: usize = TIMG0_BASE + 0x084; fn init_timer0_1hz() { // Enable TIMG0 clock: modify32(SYSTEM_BASE + 0x018, 0, 1 << 13); modify32(SYSTEM_BASE + 0x020, 1 << 13, 0); // CONFIG: CLK_EN=1, DIVIDER=80 (80MHz/80=1MHz tick), INCREASE=1, AUTORELOAD=1 write32(TIMG_T0CONFIG, (1u32 << 31) ¦ // EN (1 << 30) ¦ // INCREASE (count up) (1 << 29) ¦ // AUTORELOAD on alarm (80 << 13) ¦ // DIVIDER = 80 -> 1 µs per tick (1 << 10)); // ALARM_EN // Load 0: write32(TIMG_T0LOADLO, 0); write32(TIMG_T0LOADHI, 0); write32(TIMG_T0LOAD, 1); // trigger load // Alarm at 1_000_000 µs = 1 second: write32(TIMG_T0ALARMLO, 1_000_000); write32(TIMG_T0ALARMHI, 0); // Enable alarm interrupt: write32(TIMG_INT_ENA, 1); } fn timer0_alarm_fired() -> bool { if (read32(TIMG_INT_ST) & 1) != 0 { write32(TIMG_INT_CLR, 1); // clear flag true } else { false } }

# PART D — COMPLETE EXAMPLES AND REFERENCE

## Chapter 20 — Full STM32F446 Bare-Metal Application

### 20.1  Complete Project: GPIO + UART + I2C + SysTick

This is a complete, compilable bare-metal STM32F446 firmware combining everything from Part B into one application — no HAL anywhere.

▸ rust #![no_std] #![no_main] use core::sync::atomic::{AtomicU32, Ordering}; use cortex_m_rt::{entry, exception}; use defmt::info; use defmt_rtt as _; use panic_probe as _; use stm32f4::stm32f446; // ── Millisecond tick counter ────────────────────────────────── static TICK_MS: AtomicU32 = AtomicU32::new(0); #[exception] fn SysTick() { TICK_MS.fetch_add(1, Ordering::Relaxed); } fn delay_ms(ms: u32) { let start = TICK_MS.load(Ordering::Relaxed); while TICK_MS.load(Ordering::Relaxed).wrapping_sub(start) < ms {} } // ── UART2 transmit byte ─────────────────────────────────────── fn uart2_send(dp: &stm32f446::Peripherals, byte: u8) { while dp.USART2.sr.read().txe().bit_is_clear() {} dp.USART2.dr.write(¦w¦ unsafe { w.dr().bits(byte as u16) }); } fn uart2_print(dp: &stm32f446::Peripherals, s: &[u8]) { for &b in s { uart2_send(dp, b); } } // ── I2C write/read (from Chapter 11) ───────────────────────── fn i2c1_write_read(dp: &stm32f446::Peripherals, addr: u8, reg: u8, buf: &mut [u8]) { // (implementation as shown in Chapter 11) } // ── Main entry ─────────────────────────────────────────────── #[entry] fn main() -> ! { let dp = stm32f446::Peripherals::take().unwrap(); let mut cp = cortex_m::Peripherals::take().unwrap(); // ── 1. Clock setup: 168 MHz ─────────────────── // HSE enable dp.RCC.cr.modify(¦_, w¦ w.hseon().set_bit()); while dp.RCC.cr.read().hserdy().bit_is_clear() {} // Flash latency for 168 MHz dp.FLASH.acr.modify(¦_, w¦ unsafe { w.latency().bits(5) }); // PLL: HSE/8*336/2 = 168 MHz dp.RCC.pllcfgr.modify(¦_, w¦ unsafe { w.pllsrc().hse().pllm().bits(8).plln().bits(336) .pllp().div2().pllq().bits(7) }); dp.RCC.cr.modify(¦_, w¦ w.pllon().set_bit()); while dp.RCC.cr.read().pllrdy().bit_is_clear() {} dp.RCC.cfgr.modify(¦_, w¦ unsafe { w.hpre().div1().ppre1().div4().ppre2().div2() }); dp.RCC.cfgr.modify(¦_, w¦ w.sw().pll()); while !dp.RCC.cfgr.read().sws().is_pll() {} // ── 2. SysTick at 1 ms (168_000 cycles) ────── use cortex_m::peripheral::syst::SystClkSource; cp.SYST.set_reload(168_000 - 1); cp.SYST.clear_current(); cp.SYST.set_clock_source(SystClkSource::Core); cp.SYST.enable_counter(); cp.SYST.enable_interrupt(); // ── 3. GPIOA clock + PA5=LED, PA2=TX, PA3=RX ─ dp.RCC.ahb1enr.modify(¦_, w¦ w.gpioaen().set_bit()); let _ = dp.RCC.ahb1enr.read(); // PA5: output dp.GPIOA.moder.modify(¦_, w¦ unsafe { w.moder5().bits(0b01) // output .moder2().bits(0b10) // AF .moder3().bits(0b10) // AF }); dp.GPIOA.afrl.modify(¦_, w¦ unsafe { w.afrl2().bits(7).afrl3().bits(7) // AF7=USART2 }); dp.GPIOA.ospeedr.modify(¦_, w¦ unsafe { w.ospeedr2().bits(2).ospeedr3().bits(2) }); // ── 4. USART2 at 115200 ─────────────────────── dp.RCC.apb1enr.modify(¦_, w¦ w.usart2en().set_bit()); let _ = dp.RCC.apb1enr.read(); dp.USART2.cr1.modify(¦_, w¦ w.ue().disabled()); // APB1=42MHz: BRR = 42_000_000/115200 = 365 dp.USART2.brr.write(¦w¦ unsafe { w.bits(365) }); dp.USART2.cr1.modify(¦_, w¦ { w.te().enabled().re().enabled().ue().enabled() }); // ── 5. GPIOB + I2C1 ────────────────────────── dp.RCC.ahb1enr.modify(¦_, w¦ w.gpioben().set_bit()); let _ = dp.RCC.ahb1enr.read(); dp.GPIOB.moder.modify(¦_, w¦ unsafe { w.moder8().bits(0b10).moder9().bits(0b10) }); dp.GPIOB.otyper.modify(¦_, w¦ { w.ot8().open_drain().ot9().open_drain() }); dp.GPIOB.ospeedr.modify(¦_, w¦ unsafe { w.ospeedr8().bits(2).ospeedr9().bits(2) }); dp.GPIOB.pupdr.modify(¦_, w¦ unsafe { w.pupdr8().bits(1).pupdr9().bits(1) }); dp.GPIOB.afrh.modify(¦_, w¦ unsafe { w.afrh8().bits(4).afrh9().bits(4) // AF4=I2C1 }); dp.RCC.apb1enr.modify(¦_, w¦ w.i2c1en().set_bit()); let _ = dp.RCC.apb1enr.read(); dp.I2C1.cr1.modify(¦_, w¦ w.swrst().set_bit()); dp.I2C1.cr1.modify(¦_, w¦ w.swrst().clear_bit()); dp.I2C1.cr2.modify(¦_, w¦ unsafe { w.freq().bits(42) }); dp.I2C1.ccr.modify(¦_, w¦ unsafe { w.f_s().fast().duty().duty2_1().ccr().bits(35) }); dp.I2C1.trise.modify(¦_, w¦ unsafe { w.trise().bits(14) }); dp.I2C1.cr1.modify(¦_, w¦ w.pe().set_bit()); info!("STM32F446 bare-metal firmware running at 168 MHz"); uart2_print(&dp, b"STM32F446 bare-metal ready\r\n"); // ── Main loop ───────────────────────────────── let mut led_state = false; let mut last_toggle = 0u32; loop { let now = TICK_MS.load(Ordering::Relaxed); // Toggle LED every 500 ms: if now.wrapping_sub(last_toggle) >= 500 { last_toggle = now; led_state = !led_state; if led_state { dp.GPIOA.bsrr.write(¦w¦ w.bs5().set_bit()); } else { dp.GPIOA.bsrr.write(¦w¦ w.br5().set_bit()); } info!("Tick: {} ms", now); } } }

## Chapter 21 — Full ESP32-S3 Bare-Metal Application

▸ rust #![no_std] #![no_main] use esp_backtrace as _; use esp_println::println; // ── Constants ──────────────────────────────────────────────── const IO_MUX_BASE: usize = 0x6000_9000; const GPIO_BASE: usize = 0x6000_4000; const GPIO_ENABLE_REG: usize = GPIO_BASE + 0x020; const GPIO_OUT_W1TS: usize = GPIO_BASE + 0x008; const GPIO_OUT_W1TC: usize = GPIO_BASE + 0x00C; const GPIO_IN_REG: usize = GPIO_BASE + 0x03C; const UART0_BASE: usize = 0x6000_0000; const SYSTEM_BASE: usize = 0x600C_0000; // ── Register access ────────────────────────────────────────── #[inline(always)] fn r(a: usize) -> u32 { unsafe { core::ptr::read_volatile(a as *const u32) } } #[inline(always)] fn w(a: usize, v: u32) { unsafe { core::ptr::write_volatile(a as *mut u32, v); } } #[inline(always)] fn m(a: usize, clr: u32, set: u32) { w(a, (r(a) & !clr) ¦ set); } // ── Cycle-count delay ──────────────────────────────────────── fn delay_us(us: u32) { let start: u32; unsafe { core::arch::asm!("rsr.ccount {0}", out(reg) start); } let target = start.wrapping_add(us * 240); loop { let now: u32; unsafe { core::arch::asm!("rsr.ccount {0}", out(reg) now); } if now.wrapping_sub(start) >= us * 240 { break; } } } // ── GPIO output setup ──────────────────────────────────────── fn gpio_out(pin: u8) { w(IO_MUX_BASE + 0x04 + pin as usize * 4, 1 << 12); m(GPIO_ENABLE_REG, 0, 1u32 << pin); } // ── UART0 byte transmit (using pre-initialized UART from ROM) ─ fn uart_putc(c: u8) { while ((r(UART0_BASE + 0x01C) >> 16) & 0x1FF) > 126 {} w(UART0_BASE, c as u32); } fn uart_print(s: &[u8]) { for &b in s { uart_putc(b); } } // ── Entry ──────────────────────────────────────────────────── #[esp_hal::entry] fn main() -> ! { // GPIO21 = onboard RGB LED data (WS2812B) — use as simple GPIO here gpio_out(21); // GPIO0 = boot button input with pull-up w(IO_MUX_BASE + 0x04, (1<<12)¦(1<<10)¦(1<<9)); // GPIO0 m(GPIO_ENABLE_REG, 1 << 0, 0); uart_print(b"ESP32-S3 bare-metal running\r\n"); println!("Bare-metal boot OK"); let mut count = 0u32; loop { // Read boot button (GPIO0, active-low) let btn = (r(GPIO_IN_REG) & 1) == 0; if btn { w(GPIO_OUT_W1TS, 1 << 21); // LED on } else { w(GPIO_OUT_W1TC, 1 << 21); // LED off } delay_us(500_000); // 500 ms count = count.wrapping_add(1); println!("Loop count: {}", count); } }

## Chapter 22 — Bare-Metal Patterns and Best Practices

### 22.1  Register Access Patterns Cheat Sheet

▸ rust // ── Pattern 1: Read-Modify-Write (most common) ─────────────── fn set_field(addr: usize, shift: u32, mask: u32, value: u32) { let reg = unsafe { core::ptr::read_volatile(addr as *const u32) }; let new = (reg & !(mask << shift)) ¦ ((value & mask) << shift); unsafe { core::ptr::write_volatile(addr as *mut u32, new); } } // ── Pattern 2: Atomic Set/Clear (use for GPIO BSRR, etc.) ──── fn set_bit(addr: usize, bit: u32) { unsafe { core::ptr::write_volatile(addr as *mut u32, 1 << bit); } } // ── Pattern 3: Poll for flag with timeout ──────────────────── fn poll_flag(addr: usize, bit: u32, expected: bool, timeout: u32) -> bool { for _ in 0..timeout { let val = unsafe { core::ptr::read_volatile(addr as *const u32) }; if ((val >> bit) & 1 != 0) == expected { return true; } core::hint::black_box(()); } false // timeout } // ── Pattern 4: Write-only register (don't read back) ───────── fn clear_interrupt_flag(addr: usize, bit: u32) { // Some registers clear on write-1 — never read-modify-write these! unsafe { core::ptr::write_volatile(addr as *mut u32, 1 << bit); } } // ── Pattern 5: Memory barrier after clock enable ───────────── fn enable_and_fence(clk_reg: usize, bit: u32) { unsafe { let cur = core::ptr::read_volatile(clk_reg as *const u32); core::ptr::write_volatile(clk_reg as *mut u32, cur ¦ (1 << bit)); // Read back once to guarantee clock is active before use: let _ = core::ptr::read_volatile(clk_reg as *const u32); } }

### 22.2  STM32 AF Numbers Quick Reference (F446)

| AF Number | Peripherals |
| --- | --- |
| AF0 | SYS (SWDIO, SWCLK, MCO) |
| AF1 | TIM1, TIM2 |
| AF2 | TIM3, TIM4, TIM5 |
| AF4 | I2C1, I2C2, I2C3 |
| AF5 | SPI1, SPI2, SPI3, SPI4 |
| AF6 | SPI3, SAI1 |
| AF7 | USART1, USART2, USART3, SPI3 |
| AF8 | USART6, UART4, UART5 |
| AF9 | CAN1, CAN2, TIM12, TIM13, TIM14 |
| AF10 | OTG_FS, OTG_HS |
| AF11 | ETH (Ethernet) |
| AF12 | FSMC, SDIO, OTG_HS |
| AF15 | EVENTOUT |

### 22.3  STM32F446 Peripheral Base Addresses

▸ rust // APB1 Peripherals: const TIM2_BASE: usize = 0x4000_0000; const TIM3_BASE: usize = 0x4000_0400; const TIM4_BASE: usize = 0x4000_0800; const SPI2_BASE: usize = 0x4000_3800; const USART2_BASE: usize = 0x4000_4400; const USART3_BASE: usize = 0x4000_4800; const I2C1_BASE: usize = 0x4000_5400; const I2C2_BASE: usize = 0x4000_5800; const I2C3_BASE: usize = 0x4000_5C00; const CAN1_BASE: usize = 0x4000_6400; const PWR_BASE: usize = 0x4000_7000; // APB2 Peripherals: const TIM1_BASE: usize = 0x4001_0000; const USART1_BASE: usize = 0x4001_1000; const USART6_BASE: usize = 0x4001_1400; const ADC1_BASE: usize = 0x4001_2000; const SPI1_BASE: usize = 0x4001_3000; // AHB1 Peripherals: const DMA1_BASE: usize = 0x4002_0000; const DMA2_BASE: usize = 0x4002_6400; const RCC_BASE: usize = 0x4002_3800; const FLASH_BASE: usize = 0x4002_3C00; // Cortex-M4 Core: const SYST_BASE: usize = 0xE000_E010; const NVIC_ISER0: usize = 0xE000_E100; // Interrupt Set-Enable const NVIC_ICER0: usize = 0xE000_E180; // Interrupt Clear-Enable const NVIC_ISPR0: usize = 0xE000_E200; // Interrupt Set-Pending const NVIC_IPR0: usize = 0xE000_E400; // Interrupt Priority const SCB_VTOR: usize = 0xE000_ED08; // Vector Table Offset

### 22.4  Common Cargo Commands for Bare-Metal

▸ bash # STM32 bare-metal workflow: cargo build --release cargo run --release # probe-rs flashes + shows RTT logs probe-rs run --chip STM32F446RETx target/.../your_binary probe-rs erase --chip STM32F446RETx # Show final binary section sizes: arm-none-eabi-size target/thumbv7em-none-eabihf/release/stm32_baremetal # or: cargo size --release -- -A # Disassemble to inspect generated assembly: arm-none-eabi-objdump -d target/.../stm32_baremetal ¦ head -100 # ESP32-S3 bare-metal workflow: source ~/export-esp.sh # (usbipd attach in PowerShell) cargo run --release # espflash + monitor espflash monitor --port /dev/ttyUSB0 # Size check: cargo size --release -- -A

## Chapter 23 — Troubleshooting Bare-Metal

### 23.1  The Bare-Metal Debug Checklist

- Did you enable the peripheral CLOCK in RCC? (STM32) or SYSTEM_PERIP_CLK_EN? (ESP32-S3)

- Did you READ BACK the clock enable register after writing it? (memory fence)

- Are you using volatile reads/writes? (not plain pointer dereference)

- Are your GPIO pins set to the correct MODER? (0b01=output, 0b10=AF, 0b00=input)

- For UART/I2C/SPI pins: did you set the correct AF number in AFRLx/AFRHx?

- For I2C: are the pins open-drain (OTYPER=1) with pull-ups?

- Is your baud/clock calculation using the CORRECT bus clock (APB1 vs APB2)?

- For interrupts: did you enable the peripheral interrupt bit AND NVIC::unmask?

### 23.2  Common Mistakes

| Symptom | Likely Cause + Fix |
| --- | --- |
| Register write does nothing | Missing clock enable in RCC — add ahb1enr/apb1enr/apb2enr bit |
| UART sends garbage / no output | Wrong BRR value — check APB1 vs APB2 clock for your UART |
| I2C hangs on ADDR flag | NACK from slave — check address, SDA/SCL pull-ups, open-drain |
| SPI transfers 0xFF every byte | MISO pin not in AF mode or wrong AF number |
| GPIO input always reads 0 or 1 | MODER not set to 0b00 (input), or PUPDR wrong |
| Hard fault on startup | Wrong memory.x — FLASH or RAM origin/length incorrect |
| probe-rs 'No probe found' | usbipd not attached; run usbipd attach --wsl --busid X |
| Optimizer removes delay loop | Use core::hint::black_box(()) inside the loop |
| ISR never fires | Peripheral interrupt bit not enabled, or NVIC::unmask not called |

🔩  Bare-Metal Rust — No HAL, No Secrets, Pure Registers

Prepared for Roy Roychowdhury — Principal Embedded Firmware Engineer