# Register Bit Manipulation

A Complete Guide for Embedded Systems

# What are Bits and Registers?

Bits: The Fundamental Unit

A bit is a binary digit - it can only be 0 or 1. Everything in computers is built from bits.

0 = OFF = False = Low voltage (0V)

1 = ON  = True  = High voltage (3.3V)

Registers: Collections of Bits

A register is a storage location made of multiple bits. In STM32F4, most registers are 32 bits wide.

32-bit register:

[Bit 31][Bit 30]...[Bit 2][Bit 1][Bit 0]

MSB (Most Significant Bit)  LSB (Least Significant Bit)

Each bit can control a different hardware feature. For example, in a GPIO register:

Bit 0 controls Pin 0

Bit 1 controls Pin 1

...

Bit 15 controls Pin 15

# Understanding Binary Numbers

## Decimal vs Binary

Humans use decimal (base-10): 0-9. Computers use binary (base-2): 0-1.

Decimal:  0   1   2   3   4   5   6   7   8   9  10

Binary:   0   1  10  11 100 101 110 111 1000 1001 1010

## Binary Place Values

Bit position:  7   6   5   4   3   2   1   0

Value:       128  64  32  16   8   4   2   1

Binary:        1   0   1   0   1   1   0   1

Calculation: 128 + 32 + 8 + 4 + 1 = 173 (decimal)

## Hexadecimal Notation

Hex (base-16) is commonly used because 4 bits = 1 hex digit.

Hex:    0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F

Binary: 0000 0001 0010 0011 0100 0101 0110 0111

1000 1001 1010 1011 1100 1101 1110 1111

Example: 0xA5 = 0b10100101

A = 1010, 5 = 0101

# Bitwise Operators Explained

## 1. Left Shift Operator (<<)

Purpose: Move bits to the left, filling with zeros

1 << 0  =  0b00000001  =  1

1 << 1  =  0b00000010  =  2

1 << 2  =  0b00000100  =  4

1 << 3  =  0b00001000  =  8

1 << 4  =  0b00010000  =  16

1 << 5  =  0b00100000  =  32

Think of it as "create a mask with bit N set".

(1 << 5) creates: 00100000 - only bit 5 is set

## 2. OR Operator (|)

Purpose: Set bits to 1 (turn ON)

Rule: 0 | 0 = 0    Any bit OR 0 stays unchanged

0 | 1 = 1    Any bit OR 1 becomes 1

1 | 0 = 1

1 | 1 = 1

Example:

Register = 0b00010010  (bits 1 and 4 are set)

Mask     = 0b00100000  (want to set bit 5)

--------------------------------- OR

Result   = 0b00110010  (bits 1, 4, and 5 set)

## 3. AND Operator (&)

Purpose: Check bits or keep only certain bits

Rule: 0 & 0 = 0    Only 1 & 1 gives 1

0 & 1 = 0    Everything else gives 0

1 & 0 = 0

1 & 1 = 1

Example:

Register = 0b10110101

Mask     = 0b00000100  (check if bit 2 is set)

--------------------------------- AND

Result   = 0b00000100  (non-zero, so bit 2 is set)

## 4. NOT Operator (~)

Purpose: Flip all bits (0→1, 1→0)

Value = 0b00100000

-------------------- NOT

~Value = 0b11011111  (all bits flipped)

Used to create inverted masks for clearing bits.

## 5. XOR Operator (^)

Purpose: Toggle bits (flip their state)

Rule: 0 ^ 0 = 0    Same bits give 0

0 ^ 1 = 1    Different bits give 1

1 ^ 0 = 1

1 ^ 1 = 0

Example:

Register = 0b00010010

Mask     = 0b00100010  (toggle bits 1 and 5)

--------------------------------- XOR

Result   = 0b00110000  (bit 1 cleared, bit 5 set)

# Operation 1: SET BIT (Turn ON)

## Formula: REG |= (1 << bit)

What it Does:

Sets bit to 1 without affecting other bits.

## Step-by-Step Example: Set bit 5

Initial register value:

REG = 0b00010010  (bits 1 and 4 are already set)

Step 1: Create mask

(1 << 5) = 0b00100000  (only bit 5 is set)

Step 2: OR with register

0b00010010  (original REG)

| 0b00100000  (mask)

-------------

0b00110010  (result)

Result: Bit 5 is now set, others unchanged!

## Real STM32 Example: Turn on LED

// Turn on LED on PD12 (bit 12 of GPIOD)

GPIOD->ODR |= (1 << 12);

Before: GPIOD->ODR = 0b00000000 00000000

Mask:                0b00010000 00000000

After:  GPIOD->ODR = 0b00010000 00000000

↑

bit 12 now ON

## Setting Multiple Bits

// Set bits 5, 7, and 12

REG |= (1 << 5) | (1 << 7) | (1 << 12);

// Or use hex mask directly

REG |= 0x10A0;  // bits 5, 7, 12 in hex

# Operation 2: CLEAR BIT (Turn OFF)

## Formula: REG &= ~(1 << bit)

What it Does:

Clears bit to 0 without affecting other bits.

## Step-by-Step Example: Clear bit 4

Initial register value:

REG = 0b00110010  (bits 1, 4, and 5 are set)

Step 1: Create mask

(1 << 4) = 0b00010000  (bit 4 set)

Step 2: Invert mask with NOT

~(1 << 4) = 0b11101111  (bit 4 clear, all others set)

Step 3: AND with register

0b00110010  (original REG)

& 0b11101111  (inverted mask)

-------------

0b00100010  (result)

Result: Bit 4 is now clear, others unchanged!

## Why AND with Inverted Mask?

AND truth table:

1 & 1 = 1  (bits stay set)

1 & 0 = 0  (bit gets cleared)

0 & 1 = 0  (bits stay clear)

So: ANDing with 1 keeps bit unchanged

ANDing with 0 clears the bit

## Real STM32 Example: Turn off LED

// Turn off LED on PD12

GPIOD->ODR &= ~(1 << 12);

Before: GPIOD->ODR = 0b00010000 00000000

Mask:   ~(1 << 12) = 0b11101111 11111111

After:  GPIOD->ODR = 0b00000000 00000000

↑

bit 12 now OFF

## Clearing Multiple Bits

// Clear bits 5, 7, and 12

REG &= ~((1 << 5) | (1 << 7) | (1 << 12));

Step 1: Create mask with bits to clear

(1<<5) | (1<<7) | (1<<12) = 0b00010000 10100000

Step 2: Invert

~0b00010000 10100000      = 0b11101111 01011111

Step 3: AND with register

Clears those 3 bits, keeps everything else

# Operation 3: TOGGLE BIT (Flip State)

## Formula: REG ^= (1 << bit)

What it Does:

Flips bit: 0→1 or 1→0, without affecting other bits.

## Step-by-Step Example: Toggle bit 5

Initial register value:

REG = 0b00110010  (bits 1, 4, and 5 are set)

Step 1: Create mask

(1 << 5) = 0b00100000  (bit 5 set)

Step 2: XOR with register

0b00110010  (original REG, bit 5 = 1)

^ 0b00100000  (mask)

-------------

0b00010010  (result, bit 5 = 0)

If we toggle again:

0b00010010  (REG, bit 5 = 0)

^ 0b00100000  (mask)

-------------

0b00110010  (result, bit 5 = 1 again!)

## XOR Toggle Property

XOR with 1: Flips the bit

0 ^ 1 = 1  (0 becomes 1)

1 ^ 1 = 0  (1 becomes 0)

XOR with 0: Keeps bit unchanged

0 ^ 0 = 0  (0 stays 0)

1 ^ 0 = 1  (1 stays 1)

## Real STM32 Example: Blink LED

// Toggle LED on PD12 (blinks LED)

GPIOD->ODR ^= (1 << 12);

First call:  OFF → ON

Second call: ON  → OFF

Third call:  OFF → ON

...

## Toggle Multiple Bits

// Toggle bits 5, 7, and 12

REG ^= (1 << 5) | (1 << 7) | (1 << 12);

Each bit flips independently:

If bit was 0 → becomes 1

If bit was 1 → becomes 0

# Operation 4: READ BIT (Check State)

## Formula: (REG & (1 << bit))

What it Does:

Checks if a specific bit is set (1) or clear (0).

## Step-by-Step Example: Read bit 5

Register value:

REG = 0b00110010  (bits 1, 4, and 5 are set)

Step 1: Create mask

(1 << 5) = 0b00100000  (only bit 5 set)

Step 2: AND with register

0b00110010  (register)

& 0b00100000  (mask)

-------------

0b00100000  (result = 32, non-zero = TRUE)

Bit 5 is SET!

Now check bit 3 (which is clear):

0b00110010  (register)

& 0b00001000  (mask for bit 3)

-------------

0b00000000  (result = 0 = FALSE)

Bit 3 is CLEAR!

## Why This Works

AND keeps only the bit we are checking:

If bit is SET (1):

1 & 1 = 1  → Non-zero result

If bit is CLEAR (0):

0 & 1 = 0  → Zero result

All other bits become 0 (AND with 0)

## Using in if Statements

// Check if button pressed (bit 0 of GPIOA->IDR)

if (GPIOA->IDR & (1 << 0)) {

// Bit 0 is set (button pressed)

LED_On();

} else {

// Bit 0 is clear (button not pressed)

LED_Off();

}

## Extracting Bit Value

// Get bit value as 0 or 1

uint8_t bit_value = (REG & (1 << 5)) >> 5;

Step 1: Mask the bit

REG & (1 << 5) gives either 0b00100000 or 0b00000000

Step 2: Shift right to position 0

0b00100000 >> 5 = 0b00000001 = 1

0b00000000 >> 5 = 0b00000000 = 0

# Common Patterns in STM32

## Pattern 1: Set Specific Bits, Clear Others

// Problem: Configure bits 4-6 to 101b

// Others bits must stay unchanged

Step 1: Clear the bits

REG &= ~((1 << 4) | (1 << 5) | (1 << 6));

// This makes bits 4-6 = 000

Step 2: Set the desired pattern

REG |= (1 << 4) | (1 << 6);

// This makes bits 4-6 = 101

// Or combine in one line:

REG = (REG & ~0x70) | 0x50;  // 0x70 = 0b01110000

// 0x50 = 0b01010000

## Pattern 2: Modify Bit Field

// Set 3-bit field (bits 4-6) to value 5

uint32_t value = 5;  // 0b101

uint32_t position = 4;

uint32_t mask = 0x7;  // 0b111 (3 bits)

REG &= ~(mask << position);  // Clear field

REG |= (value << position);  // Set new value

## Pattern 3: Atomic Bit Set/Clear (BSRR)

// STM32 has special BSRR register for atomic GPIO

// Set bit 12 (turn on)

GPIOD->BSRR = (1 << 12);

// Lower 16 bits set corresponding GPIO pin

// Clear bit 12 (turn off)

GPIOD->BSRR = (1 << (12 + 16));

// Upper 16 bits clear corresponding GPIO pin

// Why BSRR? It is atomic - no read-modify-write!

// Safe to use in interrupts

## Pattern 4: Wait for Flag

// Wait for USART TX empty flag (bit 7)

while (!(USART2->SR & USART_SR_TXE)) {

// Wait until bit 7 is set

}

// Equivalent to:

while ((USART2->SR & (1 << 7)) == 0) {

// Continue waiting

}

# Complete Worked Examples

## Example 1: Configure GPIO Pin as Output

// Goal: Set PD12 as output (push-pull, low speed)

// MODER register: 2 bits per pin

// Pin 12 uses bits 24-25

// 00 = Input, 01 = Output, 10 = AF, 11 = Analog

Step 1: Clear bits 24-25

GPIOD->MODER &= ~(0b11 << 24);

// 0b11 = 3 = mask for 2 bits

Step 2: Set bits 24-25 to 01 (output mode)

GPIOD->MODER |= (0b01 << 24);

// Combined:

GPIOD->MODER &= ~(3 << 24);  // Clear

GPIOD->MODER |= (1 << 24);   // Set to output

## Example 2: Configure USART Baud Rate

// Goal: Set baud rate to 115200

// BRR = APB_clock / (16 × baud)

// BRR = 42000000 / (16 × 115200) = 22.786

// Split into mantissa and fraction

uint32_t mantissa = 22;      // Integer part

uint32_t fraction = 12;      // 0.786 × 16 ≈ 12

// BRR format: [15:4]=mantissa, [3:0]=fraction

USART2->BRR = (mantissa << 4) | fraction;

// Shift mantissa to bits 4-15, fraction in bits 0-3

## Example 3: Read Multi-Bit Status

// Read 2-bit field from bits 5-6

uint32_t field_mask = 0b11;  // 2-bit mask

uint32_t position = 5;

// Extract field

uint32_t value = (REG >> position) & field_mask;

// Step by step:

// If REG = 0b01100000 (bits 5-6 = 11)

// Step 1: Shift right by 5

//   0b01100000 >> 5 = 0b00000011

// Step 2: Mask with 0b11

//   0b00000011 & 0b11 = 0b11 = 3

# Common Mistakes and How to Avoid Them

## Mistake 1: Forgetting Parentheses

WRONG:

REG |= 1 << 5 | 1 << 7;  // Parsed as: REG |= (1 << 5 | 1) << 7

CORRECT:

REG |= (1 << 5) | (1 << 7);  // Clear operator precedence

## Mistake 2: Using = Instead of |=

WRONG:

GPIOD->ODR = (1 << 12);  // Sets ONLY bit 12, clears ALL others!

CORRECT:

GPIOD->ODR |= (1 << 12);  // Sets bit 12, keeps others unchanged

## Mistake 3: Not Using Volatile

WRONG:

uint32_t* reg = (uint32_t*)0x40020C00;

*reg |= (1 << 5);  // Compiler may optimize away!

CORRECT:

volatile uint32_t* reg = (volatile uint32_t*)0x40020C00;

*reg |= (1 << 5);  // Forces actual hardware access

## Mistake 4: Wrong Bit Number

// Bits are 0-indexed!

WRONG:

GPIOD->ODR |= (1 << 16);  // Bit 16 - may not exist!

CORRECT:

GPIOD->ODR |= (1 << 15);  // Bit 15 (0-15 for 16 pins)

## Mistake 5: Race Conditions in Interrupts

WRONG: (not thread-safe)

// Main code:

REG |= (1 << 5);  // Read-modify-write

// Interrupt (can happen during above operation):

REG |= (1 << 6);  // Race condition!

CORRECT:

// Use atomic operations or disable interrupts

__disable_irq();

REG |= (1 << 5);

__enable_irq();

// Or use hardware atomic operations (like BSRR)

GPIOD->BSRR = (1 << 5);  // Atomic, no read-modify-write

# Practice Exercises

## Exercise 1: Basic Operations

Given: REG = 0b10110100

a) Set bit 0

b) Clear bit 5

c) Toggle bit 7

d) Check if bit 4 is set

Answers:

a) REG |= (1 << 0);    Result: 0b10110101

b) REG &= ~(1 << 5);   Result: 0b10010100

c) REG ^= (1 << 7);    Result: 0b00110100

d) if (REG & (1 << 4)) → TRUE (bit 4 is set)

## Exercise 2: Configure 3-Bit Field

Set bits 6-8 to 0b101 in register that is 0b11111111

Solution:

REG &= ~(0b111 << 6);  // Clear bits 6-8

REG |= (0b101 << 6);   // Set to 101

Result: 0b10111111 (bits 6-8 = 101)

## Exercise 3: Extract Bit Field

Extract 4-bit value from bits 8-11 of 0xF3A5

Solution:

uint32_t value = (0xF3A5 >> 8) & 0xF;

Step 1: 0xF3A5 = 0b1111001110100101

Step 2: Shift right 8: 0b11110011

Step 3: Mask with 0xF: 0b00000011 = 3

Master bit manipulation for embedded success!