# C++ for Embedded Systems

From C to Modern C++ - A Practical Guide for Firmware Engineers

# Introduction

This tutorial is designed for experienced embedded C developers transitioning to C++. You'll learn how to apply C++ effectively in resource-constrained embedded systems while avoiding common pitfalls.

Key Topics Covered:

- Classes and objects for hardware abstraction

- RAII (Resource Acquisition Is Initialization)

- Templates for type-safe, zero-overhead abstractions

- Modern C++ features (C++11/14/17) safe for embedded

- What to avoid: exceptions, dynamic memory, RTTI

- Best practices for embedded C++ development

# 1. Why C++ for Embedded Systems?

## Benefits Over Pure C

- Type Safety: Stronger compile-time checking prevents runtime errors

- Abstraction Without Overhead: Templates provide zero-cost abstractions

- Better Code Organization: Classes encapsulate hardware drivers cleanly

- RAII for Resource Management: Automatic cleanup prevents resource leaks

- Code Reusability: Inheritance and templates enable efficient code reuse

## What to Avoid in Embedded C++

- Exceptions: Add significant code size and unpredictable timing

- Dynamic Memory (new/delete): Can cause fragmentation in long-running systems

- RTTI (Runtime Type Information): Increases binary size unnecessarily

- Heavy STL Usage: Many STL containers use dynamic allocation

- Virtual Functions (use sparingly): Add vtable overhead and indirection

# 2. Classes for Hardware Abstraction

One of the most powerful uses of C++ in embedded systems is using classes to encapsulate hardware peripherals. This provides clean interfaces and prevents misuse.

## Example: I2C Sensor Driver

C Approach (what you're used to):

// C-style approach

typedef struct {

i2c_port_t port;

uint8_t address;

bool initialized;

} imu_sensor_t;

esp_err_t imu_sensor_init(imu_sensor_t* sensor, i2c_port_t port, uint8_t addr);

esp_err_t imu_sensor_read_accel(imu_sensor_t* sensor, float* x, float* y, float* z);

C++ Approach (cleaner, safer):

// C++ class approach

class IMUSensor {

public:

IMUSensor(i2c_port_t port, uint8_t address);

~IMUSensor();  // Automatic cleanup!

bool init();

bool readAccel(float& x, float& y, float& z);  // References avoid pointers

bool isInitialized() const { return initialized_; }  // Inline getter

private:

i2c_port_t port_;

uint8_t address_;

bool initialized_;

bool writeRegister(uint8_t reg, uint8_t value);

bool readRegister(uint8_t reg, uint8_t& value);

};

Benefits:

- Encapsulation: Private members prevent accidental misuse

- Constructor initializes, destructor cleans up automatically

- References (float&) are safer and clearer than pointers

- Const methods prevent accidental state modification

- No global state needed

# 3. RAII (Resource Acquisition Is Initialization)

RAII is one of C++'s most powerful features for embedded systems. Resources are acquired in constructors and released in destructors, guaranteeing cleanup even if errors occur.

## Example: Mutex Lock Guard

Without RAII (C style - error prone):

void updateSensorData(sensor_data_t* data) {

xSemaphoreTake(dataMutex, portMAX_DELAY);

if (data->valid) {

processData(data);

if (errorCondition) {

xSemaphoreGive(dataMutex);  // Easy to forget!

return;  // Early return

}

}

xSemaphoreGive(dataMutex);  // Must remember to unlock

}

With RAII (C++ - automatic, bulletproof):

class MutexGuard {

public:

MutexGuard(SemaphoreHandle_t mutex) : mutex_(mutex) {

xSemaphoreTake(mutex_, portMAX_DELAY);

}

~MutexGuard() {

xSemaphoreGive(mutex_);  // ALWAYS called!

}

private:

SemaphoreHandle_t mutex_;

};

void updateSensorData(SensorData& data) {

MutexGuard lock(dataMutex);  // Automatic lock/unlock!

if (data.valid) {

processData(data);

if (errorCondition) {

return;  // Mutex automatically released!

}

}

// Mutex automatically released when lock goes out of scope

}

Key Point:

The destructor is ALWAYS called when the object goes out of scope, even with early returns or errors. This makes resource management bulletproof.

# 4. Templates: Zero-Cost Abstractions

Templates provide compile-time polymorphism with zero runtime overhead. The code is generated at compile time, so there's no performance penalty.

## Example: Type-Safe Ring Buffer

template<typename T, size_t SIZE>

class RingBuffer {

public:

RingBuffer() : head_(0), tail_(0), count_(0) {}

bool push(const T& item) {

if (count_ >= SIZE) return false;

buffer_[head_] = item;

head_ = (head_ + 1) % SIZE;

count_++;

return true;

}

bool pop(T& item) {

if (count_ == 0) return false;

item = buffer_[tail_];

tail_ = (tail_ + 1) % SIZE;

count_--;

return true;

}

size_t size() const { return count_; }

bool isEmpty() const { return count_ == 0; }

bool isFull() const { return count_ >= SIZE; }

private:

T buffer_[SIZE];  // Stack allocated, no heap!

size_t head_;

size_t tail_;

size_t count_;

};

Usage:

// Type-safe buffers with compile-time size

RingBuffer<float, 100> accelBuffer;  // 100 float buffer

RingBuffer<uint32_t, 50> timestampBuffer;  // 50 uint32_t buffer

float accel_x;

accelBuffer.push(3.14f);

accelBuffer.pop(accel_x);  // Type safe!

Benefits:

- Type safety at compile time - no void* casting

- Stack allocated - no heap fragmentation

- Zero runtime overhead - fully optimized by compiler

- Reusable for any data type

# 5. Modern C++ Features Safe for Embedded

## Auto Keyword

Type inference makes code cleaner and safer. The compiler determines the type at compile time.

// Instead of:

std::map<uint32_t, sensor_data_t>::iterator it = sensorMap.begin();

// Use:

auto it = sensorMap.begin();  // Much cleaner!

## Range-Based For Loops

float samples[100];

// Old way:

for (size_t i = 0; i < 100; i++) {

processSample(samples[i]);

}

// Modern way:

for (auto& sample : samples) {

processSample(sample);  // Cleaner and safer

}

## Constexpr - Compile-Time Computation

Compute values at compile time instead of runtime:

constexpr float PI = 3.14159265359f;

constexpr uint32_t BUFFER_SIZE = 1024;

// Compile-time function

constexpr uint32_t frequencyToTicks(uint32_t hz) {

return 80000000 / hz;  // Computed at compile time!

}

constexpr auto TICKS_100HZ = frequencyToTicks(100);  // No runtime cost!

## Strongly Typed Enums (enum class)

// Old C-style enum (not type safe)

enum SensorType { ACCEL, GYRO, MAG };

enum MotorType { ACCEL, STEPPER };  // ERROR: ACCEL redefined!

// Modern enum class (type safe)

enum class SensorType { Accel, Gyro, Mag };

enum class MotorType { Accel, Stepper };  // OK! Scoped

SensorType type = SensorType::Accel;  // Must use scope

## nullptr Instead of NULL

// Old way:

void* ptr = NULL;  // Actually just 0

// Modern way:

void* ptr = nullptr;  // Type-safe null pointer

# 6. When to Use Virtual Functions

Virtual functions add overhead (vtable lookup), but sometimes they're worth it for polymorphism.

## Good Use Case: Sensor Interface

class ISensor {  // Interface

public:

virtual ~ISensor() = default;

virtual bool init() = 0;

virtual bool read(float& x, float& y, float& z) = 0;

};

class AccelSensor : public ISensor {

bool init() override { /* ... */ }

bool read(float& x, float& y, float& z) override { /* ... */ }

};

class GyroSensor : public ISensor {

bool init() override { /* ... */ }

bool read(float& x, float& y, float& z) override { /* ... */ }

};

Benefits:

- Can swap sensors without changing application code

- Easy to add new sensor types

- Testable with mock sensors

## Alternative: Templates (No Runtime Overhead)

If you know the sensor type at compile time, templates are better:

template<typename SensorType>

class SensorManager {

public:

void processSensor() {

float x, y, z;

sensor_.read(x, y, z);  // No vtable lookup!

}

private:

SensorType sensor_;

};

# 7. Practical Example: Complete GPIO Class

Here's a real-world example combining multiple C++ features:

class GPIO {

public:

enum class Mode { Input, Output, InputPullup };

enum class Level { Low = 0, High = 1 };

GPIO(gpio_num_t pin, Mode mode) : pin_(pin) {

gpio_config_t config = {};

config.pin_bit_mask = (1ULL << pin);

switch(mode) {

case Mode::Output:

config.mode = GPIO_MODE_OUTPUT;

break;

case Mode::Input:

config.mode = GPIO_MODE_INPUT;

break;

case Mode::InputPullup:

config.mode = GPIO_MODE_INPUT;

config.pull_up_en = GPIO_PULLUP_ENABLE;

break;

}

gpio_config(&config);

}

~GPIO() {

gpio_reset_pin(pin_);  // RAII cleanup

}

void write(Level level) const {

gpio_set_level(pin_, static_cast<uint32_t>(level));

}

Level read() const {

return static_cast<Level>(gpio_get_level(pin_));

}

void toggle() const {

auto current = read();

write(current == Level::High ? Level::Low : Level::High);

}

private:

gpio_num_t pin_;

};

Usage:

GPIO led(GPIO_NUM_2, GPIO::Mode::Output);

led.write(GPIO::Level::High);  // Turn on

vTaskDelay(pdMS_TO_TICKS(1000));

led.toggle();  // Turn off

// Automatic cleanup when led goes out of scope!

# 8. Compiler Settings for Embedded C++

Proper compiler flags are essential for embedded C++:

## Essential Flags

-fno-exceptions        # Disable exceptions

-fno-rtti              # Disable RTTI

-fno-use-cxa-atexit    # Don't register destructors

-std=c++17             # Use C++17 standard

-O2 or -Os             # Optimize for size

-Wall -Wextra          # All warnings

## ESP-IDF CMakeLists.txt Example

idf_component_register(

SRCS "main.cpp" "sensor.cpp"

INCLUDE_DIRS "."

)

target_compile_options(

${COMPONENT_LIB} PRIVATE

-fno-exceptions

-fno-rtti

-std=c++17

)

# 9. Interview-Ready Code Examples

Here are code snippets you can discuss in your interview:

## 1. Interrupt-Safe Circular Buffer

template<typename T, size_t SIZE>

class ISRCircularBuffer {

public:

// ISR-safe push

bool pushFromISR(const T& item) {

size_t next = (head_ + 1) % SIZE;

if (next == tail_) return false;  // Full

buffer_[head_] = item;

head_ = next;  // Atomic on 32-bit

return true;

}

bool pop(T& item) {

if (head_ == tail_) return false;  // Empty

item = buffer_[tail_];

tail_ = (tail_ + 1) % SIZE;

return true;

}

private:

T buffer_[SIZE];

volatile size_t head_ = 0;

volatile size_t tail_ = 0;

};

## 2. Singleton Pattern for Hardware

class I2CBus {

public:

static I2CBus& getInstance() {

static I2CBus instance;  // Thread-safe in C++11

return instance;

}

// Delete copy/move

I2CBus(const I2CBus&) = delete;

I2CBus& operator=(const I2CBus&) = delete;

bool write(uint8_t addr, const uint8_t* data, size_t len) {

// Implementation...

}

private:

I2CBus() { /* Initialize I2C */ }

~I2CBus() { /* Deinitialize */ }

};

# 10. Key Takeaways for Interview

## What You Should Emphasize

- You understand C++ is C with better type safety and abstraction

- You know to avoid exceptions, dynamic memory, and RTTI

- RAII is your friend for automatic resource management

- Templates provide zero-cost abstractions

- Classes encapsulate hardware cleanly

- Modern C++ features (auto, constexpr, enum class) are safe and helpful

## Interview Talking Points

When asked about C++ experience:

"While my embedded work has primarily been in C, I have fundamental C++ knowledge and understand how to apply it effectively in embedded systems. I know C++ provides powerful features like RAII for resource management, templates for type-safe abstractions, and classes for hardware encapsulation - all without runtime overhead when used properly."

"I'm committed to quickly strengthening this skill. I understand the embedded C++ best practices: avoiding exceptions, dynamic memory allocation, and RTTI, while leveraging modern features like auto, constexpr, and strong types that improve code safety without performance cost."

"In my current project, I've started refactoring some sensor drivers using C++ classes, which has improved code organization and made the interfaces much cleaner and safer than my previous C implementations."

## Learning Resources

- Book: "Effective Modern C++" by Scott Meyers

- Book: "Modern C++ for Embedded Systems" by Dan Saks

- Website: cpp reference.com for language reference

- YouTube: CppCon talks on embedded C++

- Practice: Refactor existing C code to C++ incrementally

# Practice Exercises

Try these hands-on exercises to reinforce your learning:

## Exercise 1: Convert C Struct to C++ Class

Take a C struct from your current project and convert it to a C++ class with:

- Private member variables

- Constructor for initialization

- Destructor for cleanup

- Public methods instead of functions

- Const correctness

## Exercise 2: Implement RAII Wrapper

Create RAII wrappers for:

- Mutex locks (FreeRTOS semaphores)

- Critical sections

- SPI transactions

## Exercise 3: Template Data Structure

Implement a template-based:

- Moving average filter

- Median filter

- Fixed-size vector

## Exercise 4: Hardware Abstraction

Create a class hierarchy for:

- Base Sensor interface

- Derived Accelerometer class

- Derived Gyroscope class

# Final Notes

C++ in embedded systems is about using the right features wisely. Focus on zero-cost abstractions (templates, inline functions, classes), RAII for resource management, and type safety. Avoid the heavyweight features that desktop C++ uses freely.

With your 12+ years of embedded C experience, you already understand the concepts - you're just learning a different syntax and some powerful new tools. Most importantly, you understand when NOT to use features, which is just as valuable as knowing when to use them.

Good luck with your interview!