CRC & SHA in Embedded Systems

A Complete ESP32 Study Guide

Data Integrity • Error Detection • Cryptographic Security

Ratneswar Roychowdhury

Principal Embedded Firmware Engineer  |  MADEPLUS INC

Platform: ESP32 / ESP-IDF  |  Language: C (bare-metal & FreeRTOS)

# 1. Overview and Motivation

In any embedded system, data is in constant motion — across UART buses, SPI flash, TCP/IP stacks, and OTA pipelines. Two fundamental questions arise at every data boundary:

- Did the data arrive intact? (Error detection → CRC)

- Can I trust who sent it? (Cryptographic integrity → SHA + signing)

CRC and SHA are the two primary tools that answer these questions. They look similar on the surface — both produce a short fixed-length output from arbitrary-length input — but they serve fundamentally different threat models and operate at different performance points. Understanding when to use each, and how to implement both efficiently on the ESP32, is a core skill for any firmware engineer working on connected or FDA-regulated systems.

Key Principle CRC protects against accidental corruption (noise, bit-flips, cosmic rays). SHA protects against intentional tampering (adversarial modification, supply-chain attacks). They are not interchangeable — always choose based on the threat model.

# 2. CRC — Cyclic Redundancy Check

## 2.1 Mathematical Foundations

CRC is based on polynomial division in GF(2) — the Galois field with elements {0, 1} where addition is XOR and multiplication is AND. The input data is treated as the coefficients of a polynomial M(x), which is divided by a fixed generator polynomial G(x). The remainder of this division is the CRC value.

Formal definition:  CRC = remainder of  [ M(x) · x^n ] ÷ G(x)

Where n is the degree of the generator polynomial (i.e., the CRC bit-width). Key properties of GF(2) arithmetic:

- 1 XOR 1 = 0  (no carry propagation)

- 1 XOR 0 = 1

- All operations are bit-level — no integer overflow

The generator polynomial is carefully chosen to maximize error detection ability. For CRC-32 the polynomial is:

G(x) = x³²+x²⁶+x²³+x²²+x¹⁶+x¹²+x¹¹+x¹⁰+x⁸+x⁷+x⁵+x⁴+x²+x+1

Represented in hex as 0x04C11DB7 (normal form) or 0xEDB88320 (reflected/reversed form used in most implementations).

## 2.2 CRC Variants Reference

| Variant | Width | Polynomial (Hex) | Initial Value | Common Use |
| --- | --- | --- | --- | --- |
| CRC-8/MAXIM | 8-bit | 0x31 | 0x00 | 1-Wire bus (DS18B20 temp sensor) |
| CRC-8/SMBUS | 8-bit | 0x07 | 0x00 | SMBus, basic frame checks |
| CRC-16/CCITT | 16-bit | 0x1021 | 0xFFFF | UART frames, X-Modem, HDLC |
| CRC-16/IBM | 16-bit | 0x8005 | 0x0000 | Modbus RTU, USB |
| CRC-32 | 32-bit | 0x04C11DB7 | 0xFFFFFFFF | Ethernet, ZIP, PNG, flash verify |
| CRC-32/C | 32-bit | 0x1EDC6F41 | 0xFFFFFFFF | iSCSI, SCTP, ext4 filesystem |

## 2.3 ESP32 Hardware CRC Accelerator

The ESP32 does not have a dedicated CRC peripheral like STM32's CRC unit. However, ESP-IDF provides two efficient approaches:

- esp_rom_crc: ROM-resident CRC functions — zero code size cost, very fast

- Software lookup table: 256-entry precomputed table, runs at ~50–100 MB/s on 240 MHz Xtensa

- ESP32-S3 DMA+SHA: For larger blocks, SHA256 hardware is faster than software CRC above ~4 KB

ESP-IDF ROM CRC Functions (Available on all ESP32 variants) esp_rom_crc8_le() — CRC-8 little-endian esp_rom_crc16_le() — CRC-16/CCITT little-endian esp_rom_crc32_le() — CRC-32 little-endian esp_rom_crc8_be() — CRC-8 big-endian esp_rom_crc16_be() — CRC-16 big-endian esp_rom_crc32_be() — CRC-32 big-endian

## 2.4 Code Examples — CRC on ESP32

### Example 1: CRC-32 Frame Validation (UART Protocol)

This pattern is used to validate every packet received over UART before processing. The CRC is appended as the last 4 bytes of the frame.

#include "esp_rom_crc.h"

#include <string.h>

#include <stdint.h>

/* Frame structure: [HEADER 2B][LEN 2B][PAYLOAD up to 252B][CRC32 4B] */

#define FRAME_MAGIC 0xABCD

#define CRC_SIZE 4

typedef struct {

uint16_t magic;

uint16_t payload_len;

uint8_t payload[252];

uint32_t crc32;

} uart_frame_t;

/**

* Compute CRC-32 over all frame bytes EXCEPT the trailing crc32 field.

*/

static uint32_t frame_compute_crc(const uart_frame_t *f) {

uint32_t crc = 0xFFFFFFFF; /* Standard CRC-32 init value */

/* Hash header fields */

crc = esp_rom_crc32_le(crc, (uint8_t*)&f->magic, 2);

crc = esp_rom_crc32_le(crc, (uint8_t*)&f->payload_len, 2);

crc = esp_rom_crc32_le(crc, f->payload, f->payload_len);

return crc ^ 0xFFFFFFFF; /* Final XOR — standard CRC-32 */

}

bool frame_validate(const uart_frame_t *f) {

if (f->magic != FRAME_MAGIC) return false;

if (f->payload_len > sizeof(f->payload)) return false;

uint32_t expected = frame_compute_crc(f);

return (expected == f->crc32);

}

void frame_seal(uart_frame_t *f) {

f->crc32 = frame_compute_crc(f);

}

### Example 2: CRC-16 Modbus RTU Frame

Modbus RTU uses CRC-16/IBM (polynomial 0x8005, reflected). ESP-IDF's esp_rom_crc16_le matches this directly:

#include "esp_rom_crc.h"

/* Modbus RTU: CRC covers [Address][Function][Data], appended LSB first */

void modbus_append_crc(uint8_t *frame, uint16_t data_len) {

uint16_t crc = esp_rom_crc16_le(0xFFFF, frame, data_len);

frame[data_len] = (uint8_t)(crc & 0xFF); /* CRC low byte first */

frame[data_len + 1] = (uint8_t)(crc >> 8);

}

bool modbus_check_crc(const uint8_t *frame, uint16_t total_len) {

if (total_len < 4) return false;

uint16_t calc = esp_rom_crc16_le(0xFFFF, frame, total_len - 2);

uint16_t recv = (uint16_t)(frame[total_len-2]) ¦

((uint16_t)(frame[total_len-1]) << 8);

return (calc == recv);

}

### Example 3: Flash Image CRC Verification at Boot

Before jumping to the application, a custom first-stage bootloader or app_main can verify flash integrity. This is not a replacement for MCUboot's SHA/ECDSA — it is a fast sanity check before the heavier cryptographic verification:

#include "esp_rom_crc.h"

#include "esp_partition.h"

#include "esp_log.h"

#define TAG "BOOT_VERIFY"

/* Golden CRC stored in last 4 bytes of the partition */

bool verify_app_partition_crc(void) {

const esp_partition_t *part =

esp_partition_find_first(ESP_PARTITION_TYPE_APP,

ESP_PARTITION_SUBTYPE_APP_OTA_0, NULL);

if (!part) { ESP_LOGE(TAG, "No OTA_0 partition"); return false; }

uint32_t image_size = part->size - 4; /* Exclude stored CRC */

uint32_t stored_crc = 0;

/* Read stored CRC from end of partition */

esp_partition_read(part, image_size, &stored_crc, 4);

/* Stream-compute CRC in 256-byte chunks (avoids large stack) */

uint8_t buf[256];

uint32_t crc = 0xFFFFFFFF;

size_t offset = 0;

while (offset < image_size) {

size_t chunk = MIN(sizeof(buf), image_size - offset);

esp_partition_read(part, offset, buf, chunk);

crc = esp_rom_crc32_le(crc, buf, chunk);

offset += chunk;

}

crc ^= 0xFFFFFFFF;

if (crc != stored_crc) {

ESP_LOGE(TAG, "CRC mismatch! calc=0x%08lX stored=0x%08lX",

crc, stored_crc);

return false;

}

ESP_LOGI(TAG, "Partition CRC OK: 0x%08lX", crc);

return true;

}

### Example 4: CRC-32 on NVS Config Struct

Detect EEPROM/NVS corruption by storing a CRC alongside any persisted configuration structure:

#include "nvs_flash.h"

#include "esp_rom_crc.h"

typedef struct {

float setpoint_temp;

uint16_t sample_interval_ms;

uint8_t sensor_addr;

uint8_t reserved;

uint32_t crc32; /* Always last field */

} device_config_t;

static uint32_t config_crc(const device_config_t *cfg) {

/* Hash everything EXCEPT the crc32 field at the end */

size_t len = offsetof(device_config_t, crc32);

return esp_rom_crc32_le(0xFFFFFFFF, (uint8_t*)cfg, len) ^ 0xFFFFFFFF;

}

esp_err_t config_save(nvs_handle_t nvs, const device_config_t *cfg) {

device_config_t to_save = *cfg;

to_save.crc32 = config_crc(cfg);

return nvs_set_blob(nvs, "config", &to_save, sizeof(to_save));

}

esp_err_t config_load(nvs_handle_t nvs, device_config_t *cfg) {

size_t len = sizeof(*cfg);

esp_err_t err = nvs_get_blob(nvs, "config", cfg, &len);

if (err != ESP_OK) return err;

if (config_crc(cfg) != cfg->crc32) return ESP_ERR_INVALID_CRC;

return ESP_OK;

}

# 3. SHA — Secure Hash Algorithm

## 3.1 Cryptographic Foundations

SHA is a one-way cryptographic hash function standardized by NIST (FIPS 180-4). It maps arbitrary-length input to a fixed-size digest with the following security properties:

- Pre-image resistance: Given a digest H, it is computationally infeasible to find any M such that SHA(M) = H

- Second pre-image resistance: Given M₁, infeasible to find M₂ ≠ M₁ such that SHA(M₁) = SHA(M₂)

- Collision resistance: Infeasible to find any pair M₁ ≠ M₂ with SHA(M₁) = SHA(M₂)

- Avalanche effect: A single bit change in input flips ~50% of output bits — no correlation is exploitable

SHA-256 internal structure (Merkle-Damgård construction): The input is padded to a multiple of 512 bits, then processed in 512-bit blocks. Each block feeds an 8-word (256-bit) compression function using 64 rounds of bitwise operations (ROTR, SHR, XOR, AND, ADD mod 2³²) with 64 round constants derived from cube roots of primes.

## 3.2 SHA Variants Comparison

| Algorithm | Output | Block Size | Rounds | Security Level | ESP32 HW? |
| --- | --- | --- | --- | --- | --- |
| SHA-1 | 160-bit | 512-bit | 80 | Broken (collision) | Yes (legacy) |
| SHA-224 | 224-bit | 512-bit | 64 | 112-bit | Yes |
| SHA-256 | 256-bit | 512-bit | 64 | 128-bit | Yes |
| SHA-384 | 384-bit | 1024-bit | 80 | 192-bit | Yes (ESP32-S3) |
| SHA-512 | 512-bit | 1024-bit | 80 | 256-bit | Yes (ESP32-S3) |
| SHA-3/256 | 256-bit | 1088-bit | 24 | 128-bit | Software only |

Recommendation for ESP32 Projects Use SHA-256 as the default. It is hardware-accelerated, NIST-approved, used by MCUboot, AWS IoT, and TLS 1.3. SHA-1 must be avoided in new designs. SHA-512 is available on ESP32-S3 for higher security margins when needed.

## 3.3 ESP32-S3 SHA Hardware Engine

The ESP32-S3 includes a dedicated SHA hardware accelerator in the cryptography subsystem, shared with AES, RSA, and ECC peripherals. Key characteristics:

- Supports SHA-1, SHA-224, SHA-256, SHA-384, SHA-512 natively

- DMA-capable — CPU can be freed during long hash operations

- Accessible via esp-idf mbedTLS integration (transparent) or directly via esp_sha()

- Throughput: ~60–80 MB/s for SHA-256 at 240 MHz (vs ~15 MB/s software)

- Thread-safe when used via mbedTLS context functions (uses semaphore internally)

ESP-IDF SHA API options: Three layers available, from lowest to highest abstraction:

- esp_sha() — single-shot, simplest API (esp32/sha.h)

- mbedtls_sha256_*() — streaming context-based, used in TLS stacks

- bootloader_sha256_*() — lightweight version used in bootloader context

## 3.4 Code Examples — SHA on ESP32

### Example 1: SHA-256 of a Buffer (Single Shot)

The simplest use case: hash a block of data already in RAM.

#include "esp_sha.h" /* Low-level hardware SHA */

#include <string.h>

#include <stdio.h>

#define SHA256_DIGEST_LEN 32

void sha256_of_buffer(const uint8_t *data, size_t len) {

uint8_t digest[SHA256_DIGEST_LEN];

/* esp_sha() uses hardware engine, single-shot */

esp_sha(SHA2_256, data, len, digest);

/* Print hex digest */

printf("SHA-256: ");

for (int i = 0; i < SHA256_DIGEST_LEN; i++) {

printf("%02x", digest[i]);

}

printf("\n");

}

### Example 2: SHA-256 Streaming (mbedTLS — Recommended for Large Data)

For large inputs (OTA images, flash contents) that cannot fit in RAM at once, use the context-based streaming API:

#include "mbedtls/sha256.h"

#include "esp_log.h"

#define TAG "SHA256"

#define CHUNK_SIZE 4096

/**

* Compute SHA-256 of a partition by streaming in CHUNK_SIZE blocks.

* Output: 32-byte digest written to `out_digest`.

*/

esp_err_t sha256_partition(const esp_partition_t *part,

uint8_t out_digest[32]) {

mbedtls_sha256_context ctx;

mbedtls_sha256_init(&ctx);

/* 0 = SHA-256, 1 = SHA-224 */

if (mbedtls_sha256_starts(&ctx, 0) != 0) goto fail;

uint8_t *chunk = heap_caps_malloc(CHUNK_SIZE, MALLOC_CAP_DMA);

if (!chunk) goto fail;

size_t offset = 0;

while (offset < part->size) {

size_t n = MIN(CHUNK_SIZE, part->size - offset);

esp_partition_read(part, offset, chunk, n);

if (mbedtls_sha256_update(&ctx, chunk, n) != 0) {

free(chunk); goto fail;

}

offset += n;

}

free(chunk);

if (mbedtls_sha256_finish(&ctx, out_digest) != 0) goto fail;

mbedtls_sha256_free(&ctx);

return ESP_OK;

fail:

mbedtls_sha256_free(&ctx);

ESP_LOGE(TAG, "SHA-256 computation failed");

return ESP_FAIL;

}

### Example 3: HMAC-SHA256 for Message Authentication

HMAC (Hash-based Message Authentication Code) combines SHA-256 with a shared secret key to produce an authentication tag. Unlike bare SHA-256, HMAC proves both integrity and authenticity — the sender must know the key. Used in AWS IoT MQTT signing, command authentication, and sensor data validation:

#include "mbedtls/md.h"

#include <string.h>

#define HMAC_KEY_LEN 32

#define HMAC_TAG_LEN 32

static const uint8_t SHARED_KEY[HMAC_KEY_LEN] = {

/* Pre-provisioned 256-bit key — store in NVS encrypted partition */

0xDE,0xAD,0xBE,0xEF, 0x01,0x23,0x45,0x67,

/* ... remaining 24 bytes ... */

};

esp_err_t hmac_sha256_sign(const uint8_t *msg, size_t msg_len,

uint8_t tag[HMAC_TAG_LEN]) {

const mbedtls_md_info_t *info =

mbedtls_md_info_from_type(MBEDTLS_MD_SHA256);

int ret = mbedtls_md_hmac(info,

SHARED_KEY, HMAC_KEY_LEN,

msg, msg_len,

tag);

return (ret == 0) ? ESP_OK : ESP_FAIL;

}

bool hmac_sha256_verify(const uint8_t *msg, size_t msg_len,

const uint8_t expected_tag[HMAC_TAG_LEN]) {

uint8_t computed_tag[HMAC_TAG_LEN];

if (hmac_sha256_sign(msg, msg_len, computed_tag) != ESP_OK)

return false;

/* Constant-time comparison — prevents timing side-channel */

return mbedtls_ct_memcmp(computed_tag, expected_tag, HMAC_TAG_LEN) == 0;

}

Security Note — Constant-Time Comparison Always use mbedtls_ct_memcmp() (or a similar constant-time function) when comparing MAC tags or digests. Standard memcmp() exits early on first mismatch, leaking timing information that an attacker can exploit to forge valid tags byte-by-byte.

### Example 4: SHA-256 OTA Image Verification

In a production OTA pipeline, the server signs the firmware image digest with ECDSA-P256. The device verifies the signature before flashing. The SHA-256 step shown here is the first step of that pipeline — computing the digest to pass to the ECDSA verifier:

#include "esp_ota_ops.h"

#include "mbedtls/sha256.h"

#include "mbedtls/ecdsa.h"

#include "esp_log.h"

#define TAG "OTA_VERIFY"

/* Public key (uncompressed P-256 point) embedded at build time */

extern const uint8_t ota_public_key_der[];

extern const size_t ota_public_key_der_len;

bool ota_verify_image(const esp_partition_t *part,

const uint8_t *sig, size_t sig_len) {

/* Step 1: Compute SHA-256 of image (excluding signature trailer) */

uint8_t digest[32];

if (sha256_partition(part, digest) != ESP_OK) {

ESP_LOGE(TAG, "Digest computation failed");

return false;

}

ESP_LOGI(TAG, "Image digest computed");

/* Step 2: Verify ECDSA-P256 signature over the digest */

mbedtls_pk_context pk;

mbedtls_pk_init(&pk);

int ret = mbedtls_pk_parse_public_key(&pk,

ota_public_key_der,

ota_public_key_der_len);

if (ret != 0) { mbedtls_pk_free(&pk); return false; }

ret = mbedtls_pk_verify(&pk, MBEDTLS_MD_SHA256,

digest, sizeof(digest),

sig, sig_len);

mbedtls_pk_free(&pk);

if (ret != 0) {

ESP_LOGE(TAG, "Signature verification FAILED (err=-0x%04X)", -ret);

return false;

}

ESP_LOGI(TAG, "OTA signature verified — image authentic");

return true;

}

# 4. CRC vs SHA — Detailed Comparison

## 4.1 Technical Comparison Table

| Property | CRC-32 | SHA-256 |
| --- | --- | --- |
| Purpose | Error detection (accidental) | Cryptographic integrity (adversarial) |
| Output size | 32 bits (4 bytes) | 256 bits (32 bytes) |
| Algorithm class | Linear feedback / polynomial | Cryptographic hash (Merkle-Damgård) |
| Collision-resistant | No — trivially forged | Yes — computationally infeasible |
| Keyed variant | CRC-MAC (not recommended) | HMAC-SHA256 (industry standard) |
| ESP32 speed (SW) | ~100 MB/s (lookup table) | ~15–20 MB/s (software) |
| ESP32 speed (HW) | ROM: ~50 MB/s, no dedicated HW | ~60–80 MB/s (ESP32-S3 SHA engine) |
| Code size overhead | 256 bytes (LUT) or 0 (ROM) | ~4–8 KB (mbedTLS) or 2 KB (standalone) |
| RAM required | < 32 bytes (context only) | ~256 bytes (SHA-256 context) |
| FreeRTOS thread-safe | Application responsibility | mbedTLS uses semaphore for HW access |
| FDA/IEC 62443 use | Non-cryptographic check only | Required for chain-of-trust, OTA, TLS |
| MCUboot role | Fast pre-check (optional) | Primary image authentication (required) |

## 4.2 Decision Flowchart

Use this decision logic when choosing between CRC and SHA for a new data integrity requirement:

Decision Guide Q1: Could an attacker intentionally modify the data? YES → Use SHA-256 (+ ECDSA or HMAC for authentication) NO → Proceed to Q2 Q2: Is this data transmitted over a noisy physical medium (UART, SPI, RF)? YES → Use CRC (choose width based on frame size) NO → Proceed to Q3 Q3: Is this persisted config/NVM data that must survive power cycles? YES → CRC-32 is sufficient for integrity, SHA-256 if also security-sensitive NO → Proceed to Q4 Q4: Is this a firmware image being flashed or booted? YES → Use BOTH: CRC for fast pre-check, SHA-256+ECDSA for secure verification

## 4.3 Layered Usage in a Real System

In a production medical IoT device (e.g., 3-MCU architecture: STM32 sensor MCU, STM32 control MCU, ESP32-S3 cloud gateway), a typical integrity strategy layers CRC and SHA at different system boundaries:

- Layer 1 — Inter-MCU SPI/UART frames: CRC-16 or CRC-32 on every packet. Cheap, fast, catches bus noise. Detected → request retransmit.

- Layer 2 — NVS/EEPROM config structs: CRC-32 stored alongside each struct. Detected → restore factory defaults.

- Layer 3 — OTA firmware image: SHA-256 digest computed during download; ECDSA-P256 signature verified before write. Detected → abort, keep current firmware.

- Layer 4 — AWS IoT MQTT messages: TLS 1.3 uses SHA-256 in HKDF and HMAC for session keys. Transport layer — transparent to application code.

- Layer 5 — Secure boot: ESP32-S3 eFuse stores SHA-256 hash of trusted public key. ROM verifies bootloader signature before handing over control.

# 5. Performance & Optimization on ESP32

## 5.1 Benchmarks (ESP32-S3, 240 MHz)

| Method | Data Size | Throughput | CPU Usage | Notes |
| --- | --- | --- | --- | --- |
| esp_rom_crc32_le() | 1 KB | ~480 MB/s | 100% | ROM call, no overhead |
| esp_rom_crc32_le() | 64 KB | ~480 MB/s | 100% | Consistent, no cache effects |
| SHA-256 (software) | 1 KB | ~18 MB/s | 100% | mbedTLS, no HW |
| SHA-256 (hardware) | 1 KB | ~40 MB/s | 100% | HW but small block overhead |
| SHA-256 (hardware) | 64 KB | ~75 MB/s | ~60% | DMA offload, CPU partially free |
| SHA-256 (hardware) | 256 KB | ~78 MB/s | ~50% | Optimal DMA throughput |

Key takeaway: for blocks under ~4 KB, CRC-32 is roughly 25× faster than hardware SHA-256. Above ~64 KB with DMA enabled, SHA-256 hardware throughput approaches the flash read speed, making the hash essentially free in an OTA scenario.

## 5.2 FreeRTOS Task Considerations

/* SHA-256 hardware mutex — ESP-IDF manages this internally via mbedTLS, */

/* but if you use esp_sha() directly from multiple tasks, serialize access: */

static SemaphoreHandle_t sha_mutex;

void app_main(void) {

sha_mutex = xSemaphoreCreateMutex();

/* ... */

}

esp_err_t safe_sha256(const uint8_t *data, size_t len, uint8_t digest[32]) {

if (xSemaphoreTake(sha_mutex, pdMS_TO_TICKS(1000)) != pdTRUE)

return ESP_ERR_TIMEOUT;

esp_sha(SHA2_256, data, len, digest);

xSemaphoreGive(sha_mutex);

return ESP_OK;

}

/* NOTE: mbedtls_sha256_*() functions are already thread-safe */

/* when CONFIG_MBEDTLS_HARDWARE_SHA=y in sdkconfig — no extra mutex needed */

# 6. Security Context — FDA & IEC 62443

## 6.1 FDA 2023 Cybersecurity Guidance

The FDA's 2023 Cybersecurity in Medical Devices guidance requires that manufacturers demonstrate a "secure product development framework" including:

- Authenticated firmware updates — requires SHA-256 + ECDSA or RSA-PSS signing

- Device identity and authentication — requires HMAC-SHA256 or certificate-based (X.509 + SHA-256)

- Software Bill of Materials (SBOM) integrity — SHA-256 hash of SBOM document

- Anomaly detection — HMAC-SHA256 on log entries to detect tampering

CRC alone is explicitly insufficient for any of these requirements. CRC is acceptable only for internal error detection where no adversary can access the bus.

## 6.2 IEC 62443 Mapping

| IEC 62443 Requirement | Mechanism | ESP32 Implementation |
| --- | --- | --- |
| SR 3.4 — Software & information integrity | SHA-256 + ECDSA on firmware | MCUboot + esp_secure_cert |
| SR 1.2 — Software process & device auth | HMAC-SHA256 tokens | mbedtls_md_hmac() |
| SR 3.3 — Security functionality verification | CRC on comms frames | esp_rom_crc32_le() |
| CR 3.9 — Protection of audit information | HMAC-SHA256 on log records | Custom log signing layer |

## 6.3 MCUboot on ESP32 — CRC + SHA in Action

MCUboot, the open-source secure bootloader used by ESP-IDF OTA, layers both algorithms in its image verification flow:

- Step 1 (CRC): Fast CRC-32 check over the image TLV (type-length-value) header to detect transport corruption before expensive crypto operations

- Step 2 (SHA-256): Compute SHA-256 digest of the entire image body; compare against the SHA-256 TLV entry embedded in the image trailer

- Step 3 (ECDSA-P256): Verify the ECDSA-P256 signature over the SHA-256 digest using the public key stored in device eFuses

- Step 4 (Boot): Only if all three checks pass does MCUboot transfer execution to the application vector table

Build Configuration (sdkconfig) CONFIG_SECURE_BOOT=y CONFIG_SECURE_BOOT_SIGNING_KEY="keys/secure_boot_signing_key.pem" CONFIG_MBEDTLS_HARDWARE_SHA=y # Enable hardware SHA accelerator CONFIG_BOOTLOADER_APP_SHA256=y # SHA-256 in bootloader CONFIG_ESP_MCUBOOT_IMAGE_MAGIC=0xA0 # MCUboot image magic byte

# 7. Quick Reference Cheat Sheet

## 7.1 Header Includes

/* CRC — ROM-based, no extra headers needed beyond: */

#include "esp_rom_crc.h"

/* SHA — Hardware via esp_sha() low-level API: */

#include "esp_sha.h"

/* SHA — Streaming context via mbedTLS (recommended): */

#include "mbedtls/sha256.h"

#include "mbedtls/sha512.h"

/* HMAC — Single-shot via mbedTLS: */

#include "mbedtls/md.h"

## 7.2 One-Liner Reference

/* CRC-32 of buffer */

uint32_t crc = esp_rom_crc32_le(0xFFFFFFFF, buf, len) ^ 0xFFFFFFFF;

/* CRC-16 of buffer */

uint16_t crc = esp_rom_crc16_le(0xFFFF, buf, len);

/* SHA-256 single-shot (hardware accelerated) */

uint8_t digest[32];

esp_sha(SHA2_256, buf, len, digest);

/* SHA-256 via mbedTLS streaming */

mbedtls_sha256_context ctx;

mbedtls_sha256_init(&ctx);

mbedtls_sha256_starts(&ctx, 0); /* 0 = SHA-256 */

mbedtls_sha256_update(&ctx, buf, len);

mbedtls_sha256_finish(&ctx, digest);

mbedtls_sha256_free(&ctx);

/* HMAC-SHA256 single-shot */

uint8_t tag[32];

mbedtls_md_hmac(mbedtls_md_info_from_type(MBEDTLS_MD_SHA256),

key, key_len, msg, msg_len, tag);

/* Constant-time digest comparison (anti-timing-attack) */

bool ok = (mbedtls_ct_memcmp(expected, computed, 32) == 0);

## 7.3 Common Pitfalls

- CRC initial value matters: CRC-32 standard uses init=0xFFFFFFFF and final XOR 0xFFFFFFFF. Using init=0 gives a different (non-standard) result that won't interoperate with PC tools.

- Endianness: esp_rom_crc32_le() uses little-endian (reflected) algorithm matching Ethernet/ZIP. Use _be() variants only when your protocol specifies big-endian.

- SHA-256 vs SHA-224: mbedtls_sha256_starts(&ctx, 0) = SHA-256; the '1' parameter gives SHA-224. A common copy-paste error.

- Never use memcmp() for MAC tags: Use mbedtls_ct_memcmp() always, even in 'non-security-critical' paths, to build the habit.

- CRC is not a MAC: CRC-MAC exists academically but is trivially broken. Never use CRC as an authentication tag — use HMAC-SHA256.

- sha_mutex: If you use esp_sha() directly (not via mbedTLS), protect it with a FreeRTOS mutex. mbedTLS handles this internally when CONFIG_MBEDTLS_HARDWARE_SHA=y.

# 8. Summary

CRC and SHA solve related but distinct problems in embedded firmware:

- CRC is a lightweight, ultra-fast polynomial checksum for accidental error detection. It is the right tool for UART/SPI frame integrity, NVS config validation, and fast flash sanity checks. On ESP32, the ROM functions make it essentially free in terms of code size.

- SHA-256 is a cryptographic one-way hash function for tamper-evident data integrity. It is the right tool for OTA image authentication, secure boot, MQTT/TLS sessions, and FDA-regulated data pipelines. On ESP32-S3, the hardware SHA engine makes it fast enough for streaming OTA verification without stalling other tasks.

- HMAC-SHA256 combines SHA-256 with a shared secret to provide message authentication — proving both integrity and origin. This is the standard for inter-device command authentication in IEC 62443 compliant systems.

In production medical IoT firmware, these three tools are layered, not interchanged: CRC at the transport layer, SHA-256+ECDSA at the firmware update layer, and HMAC-SHA256 at the application messaging layer.

Remember CRC → Did the bits survive the trip? SHA → Did an adversary touch the data? HMAC → Did the expected sender create this message?