FSM & STM32 ISR

Complete Learning Path

From Fundamentals to Production Code

⏱ Total Time: 3 Hours

📚 3 Tutorial Files + 1 Complete Example

# 📋 Table of Contents

Overview

- What You Will Learn

- Prerequisites

- Required Files

- Study Environment Setup

Hour 1: Finite State Machines (60 minutes)

- Session 1.1: FSM Fundamentals (15 min)

- Session 1.2: Switch-Case Pattern (15 min)

- Session 1.3: Button Debouncing (20 min)

- Session 1.4: Protocol Parsing (10 min)

Hour 2: STM32 Interrupts (60 minutes)

- Session 2.1: Interrupt Fundamentals (20 min)

- Session 2.2: GPIO Interrupts (15 min)

- Session 2.3: Timer Interrupts (10 min)

- Session 2.4: UART Interrupts (10 min)

- Session 2.5: ISR Best Practices (15 min)

Hour 3: Complete System (60 minutes)

- Session 3.1: System Architecture (15 min)

- Session 3.2: Button Control System (20 min)

- Session 3.3: Communication System (15 min)

- Session 3.4: Integration & Testing (10 min)

Practical Labs

- Lab Setup Instructions

- Lab 1: Build Button Debouncer

- Lab 2: Implement Timer ISR

- Lab 3: Create Command Parser

Reference Material

- Quick Reference Cards

- Common Mistakes Checklist

- Debugging Guide

- Next Steps

# 📚 Overview

## What You Will Learn

By completing this learning path, you will master:

Finite State Machines:

- What FSMs are and when to use them

- 4 implementation patterns (Switch-Case, State Table, Function Pointers, State Pattern)

- Real-world applications: debouncing, protocols, motor control

- Best practices for embedded systems

STM32 Interrupts:

- Interrupt fundamentals and architecture

- STM32 NVIC priority system

- Setting up GPIO, Timer, UART, and DMA interrupts

- ISR best practices and common pitfalls

- Critical sections and data protection

Integration:

- Combining FSM + ISR for production code

- Building complete multi-function systems

- Professional embedded programming patterns

💡 Note: This is a hands-on learning path. You will read theory, study code, and practice!

## Prerequisites

Before starting, you should have:

☐ ✓ Basic C programming knowledge (variables, functions, loops, if-else)

☐ ✓ Understanding of binary and hexadecimal numbers

☐ ✓ Familiarity with microcontroller concepts (registers, GPIO, timers)

☐ ✓ STM32 Nucleo board (F446RE or similar F4 series) - optional but recommended

☐ ✓ STM32CubeIDE installed - for testing code

☐ ✓ Basic C++ knowledge helpful but not required

✓ Tip: If you're new to C++, don't worry! The tutorials explain all C++ features used.

## Required Files

Make sure you have these 4 files:

1. FSM_Complete_Tutorial.cpp

Complete FSM theory and implementation patterns

2. STM32_ISR_Complete_Tutorial.cpp

Complete interrupt theory and STM32 examples

3. FSM_ISR_Complete_Example.cpp

Working multi-function system combining FSM + ISR

4. FSM_ISR_Learning_Path.docx

This document - your study guide

## Study Environment Setup

For the best learning experience:

Step 1: Organize Your Files

- Create a folder: FSM_ISR_Tutorial

- Place all 4 files in this folder

- Keep this document open while studying

Step 2: Code Editor

- Open .cpp files in your favorite code editor

- Recommended: Visual Studio Code, Sublime Text, or Notepad++

- Enable C++ syntax highlighting

Step 3: Note-Taking

- Have a notebook or digital notes ready

- Write down key concepts as you learn

- Draw state diagrams by hand - very helpful!

Step 4: Optional - Hardware Setup

- Connect STM32 Nucleo board to computer

- Open STM32CubeIDE

- You'll test code in Hour 3

✓ Tip: Don't try to rush through. Take breaks between hours to let concepts sink in.

# ⏱ HOUR 1: Finite State Machines (60 minutes)

Open file: FSM_Complete_Tutorial.cpp

## Session 1.1: FSM Fundamentals (15 minutes)

Goal: Understand what FSMs are and why we use them

What to Read:

- Part 1: FSM Fundamentals section

- Look at the real-world examples (traffic light, door, phone)

- Understand the 4 FSM components: States, Events, Transitions, Actions

Key Questions to Answer:

- What is a Finite State Machine in your own words?

- Why are FSMs useful in embedded systems?

- What are the 4 components of an FSM?

Exercise:

Draw a simple FSM for a vending machine with states: IDLE, COIN_INSERTED, DISPENSING

What are the events? What are the transitions?

☐ □ Read fundamentals section

☐ □ Answered all 3 questions

☐ □ Drew vending machine FSM

💡 Note: If you can explain FSM to someone else after this session, you're ready to continue!

## Session 1.2: Switch-Case Pattern (15 minutes)

Goal: Master the most common FSM implementation pattern

What to Read:

- Part 2: Pattern 1 - Switch-Case FSM

- Study the LedBlinkerFsm example carefully

- Understand the stateChanged_ flag pattern

Key Concepts:

- Each case = one state

- Entry actions (run once when entering state)

- Transition checks (when to change state)

- transitionTo() helper function

Code Study:

Look at this pattern:

switch (currentState_) {

case State::OFF:

if (stateChanged_) {

turnLedOff();  // Entry action

stateChanged_ = false;

}

if (timerExpired()) {  // Transition check

transitionTo(State::ON);

}

break;

}

Practice:

Modify the LED blinker to have 3 states: OFF (2s) → ON (1s) → BLINK (0.5s) → OFF

Write the pseudocode for this FSM

☐ □ Understand switch-case structure

☐ □ Understand entry actions vs transition checks

☐ □ Wrote 3-state LED FSM pseudocode

✓ Tip: Switch-case is used in 90% of embedded FSMs. Master this pattern!

## Session 1.3: Button Debouncing FSM (20 minutes)

Goal: Learn a critical real-world FSM application

What to Read:

- Part 3: Example 1 - Button Debouncing FSM

- Read the problem description carefully

- Study the ButtonDebouncer class implementation

- Understand all 4 states and transitions

The Problem:

Mechanical buttons "bounce" - the signal is noisy:

Button press:  ___/‾\/\/‾‾‾‾\___  (bouncing!)

What we want: ___/‾‾‾‾‾‾‾‾\___  (clean)

The Solution - FSM States:

- IDLE: Waiting for button press

- DEBOUNCE_PRESS: Wait 20ms, ignore bouncing

- PRESSED: Button confirmed pressed

- DEBOUNCE_RELEASE: Wait 20ms before confirming release

Key Learning Points:

- Debouncing is ESSENTIAL in embedded systems

- FSM provides clean solution to timing problem

- 20ms debounce time is typical

- State machine handles all edge cases

Deep Dive Exercise:

Trace through the code:

1. User presses button (bouncing signal)

2. What happens in each state?

3. How does 20ms timer work?

4. When is onButtonPressed() called?

☐ □ Understand the bouncing problem

☐ □ Can explain all 4 states

☐ □ Traced through button press sequence

☐ □ Understand why debouncing is needed

⚠ Warning: Every button in your projects needs debouncing! Save this code.

## Session 1.4: Protocol Parsing FSM (10 minutes)

Goal: See how FSMs handle communication protocols

What to Read:

- Part 3: Example 2 - UART Protocol FSM

- Understand the protocol format

- Study how FSM parses incoming bytes

Protocol Format:

<START_BYTE> <LENGTH> <DATA...> <CHECKSUM>

Example: 0xAA 0x04 0x12 0x34 0x56 0x78 0xB4

FSM States for Parsing:

- WAIT_START: Looking for 0xAA start byte

- READ_LENGTH: Next byte is data length

- READ_DATA: Receiving N data bytes

- READ_CHECKSUM: Verify checksum, process packet

Why FSM is Perfect for This:

- Handles bytes arriving one at a time

- Easy error recovery (just reset to WAIT_START)

- Clear, maintainable code

- Can handle incomplete or corrupt packets

Quick Exercise:

What happens if:

• Wrong start byte received? → Stay in WAIT_START

• Invalid length (0 or >32)? → Reset to WAIT_START

• Checksum mismatch? → Discard packet, reset to WAIT_START

☐ □ Understand protocol format

☐ □ Can trace byte reception through FSM

☐ □ Understand error handling

✓ HOUR 1 COMPLETE!

Take a 10-minute break before Hour 2!

# ⏱ HOUR 2: STM32 Interrupts (60 minutes)

Open file: STM32_ISR_Complete_Tutorial.cpp

## Session 2.1: Interrupt Fundamentals (20 minutes)

Goal: Understand interrupts and the STM32 NVIC

What to Read:

- Part 1: Interrupt Fundamentals

- Part 2: STM32 NVIC Architecture

- Focus on the priority system explanation

Key Questions to Answer:

- What is an interrupt? Use the phone analogy.

- What are the 3 advantages of interrupts over polling?

- In the priority system, is 0 high or low priority?

- What is priority preemption?

Priority System Deep Dive:

Study this example carefully:

Priority 0: System Timer (highest - must run!)

Priority 2: Button (high - user interaction)

Priority 5: UART (medium - communication)

Priority 10: Background task (low)

Scenario: Background task running, UART byte arrives, then button pressed.

Question: What happens?

Answer: Button (priority 2) preempts UART (priority 5), which preempted background (priority 10).

Priority Grouping (Important!):

With Group 4 (4.0 split):

- 16 preempt priority levels (0-15)

- Higher priority can interrupt lower

- No sub-priorities

☐ □ Understand polling vs interrupts

☐ □ Understand priority numbers (0 = highest)

☐ □ Understand preemption

☐ □ Can explain priority grouping

💡 Note: Priority system is crucial! Many embedded bugs come from wrong priorities.

## Session 2.2: GPIO External Interrupts (15 minutes)

Goal: Master GPIO interrupts for button detection

What to Read:

- Part 3: Setting Up Interrupts

- Part 4: GPIO External Interrupt section

- Study the complete setup example

6 Steps to Configure GPIO Interrupt:

- Configure GPIO pin (input with pull-up)

- Enable SYSCFG clock

- Map GPIO pin to EXTI line

- Configure trigger (rising/falling edge)

- Enable EXTI line in EXTI->IMR

- Set priority and enable in NVIC

Critical Code Pattern:

extern "C" void EXTI15_10_IRQHandler(void) {

if (EXTI->PR & (1 << 13)) {  // Check flag

EXTI->PR = (1 << 13);    // Clear flag (MUST!)

// Handle interrupt

}

}

Important Notes:

- EXTI lines 5-9 share one ISR (EXTI9_5_IRQn)

- EXTI lines 10-15 share one ISR (EXTI15_10_IRQn)

- Must check which line triggered in shared ISR

- MUST clear interrupt flag or ISR runs forever!

Practice:

Look at the code and identify:

- Where is SYSCFG enabled?

- Where is PC13 mapped to EXTI13?

- Where is falling edge configured?

- Where is the interrupt flag cleared?

☐ □ Understand all 6 setup steps

☐ □ Know how to clear interrupt flag

☐ □ Understand shared EXTI lines

⚠ Warning: Forgetting to clear the flag is the #1 interrupt bug!

## Session 2.3: Timer Interrupts (10 minutes)

Goal: Use timers for periodic tasks

What to Read:

- Part 4: Timer Interrupt section

- Study the 1ms timer setup

Timer Interrupt Setup:

For 1ms interrupt on TIM6:

TIM6->PSC = 45000 - 1;   // Prescaler

TIM6->ARR = 1 - 1;        // Period (auto-reload)

TIM6->DIER = TIM_DIER_UIE; // Enable update interrupt

TIM6->CR1 = TIM_CR1_CEN;   // Start timer

How it Works:

- Timer counts from 0 to ARR

- When reaches ARR, generates update event

- Update event triggers interrupt

- Timer resets to 0 and continues

Prescaler Calculation:

If APB1 clock = 45 MHz:

- PSC = 45000 → Timer clock = 45MHz / 45000 = 1 kHz

- ARR = 1 → Interrupt every 1ms

ISR Pattern:

extern "C" void TIM6_DAC_IRQHandler(void) {

if (TIM6->SR & TIM_SR_UIF) {

TIM6->SR &= ~TIM_SR_UIF;  // Clear flag

systemTicks++;  // Increment counter

// Update FSMs, check timeouts, etc.

}

}

Common Uses:

- System tick counter (like millis() in Arduino)

- Periodic sensor reading

- LED blinking

- Timeout detection

- FSM updates

☐ □ Understand PSC and ARR calculation

☐ □ Can calculate timer period

☐ □ Know common timer uses

✓ Tip: A 1ms timer ISR is the heart of many embedded systems!

## Session 2.4: UART Interrupts (10 minutes)

Goal: Handle serial communication with interrupts

What to Read:

- Part 4: UART Interrupt section

- Study the circular buffer implementation

UART Interrupt Types:

- RXNE: Receive buffer Not Empty (byte received)

- TXE: Transmit buffer Empty (can send next byte)

- TC: Transmission Complete

- IDLE: Line idle (end of packet)

Most Common: RXNE Interrupt

USART2->CR1 |= USART_CR1_RXNEIE;  // Enable RX interrupt

extern "C" void USART2_IRQHandler(void) {

if (USART2->SR & USART_SR_RXNE) {

uint8_t data = USART2->DR;  // Read clears flag

// Store in circular buffer

}

}

Circular Buffer (Important!):

Why? UART bytes arrive unpredictably, need buffering.

uint8_t rxBuffer[256];

uint16_t rxHead = 0;  // Write position

uint16_t rxTail = 0;  // Read position

// In ISR: Write to buffer

rxBuffer[rxHead] = data;

rxHead = (rxHead + 1) % 256;  // Wrap around

// In main: Read from buffer

if (rxHead != rxTail) {

uint8_t data = rxBuffer[rxTail];

rxTail = (rxTail + 1) % 256;

}

☐ □ Understand RXNE interrupt

☐ □ Know how circular buffer works

☐ □ Can explain head/tail pointers

💡 Note: Circular buffers are essential for asynchronous communication!

## Session 2.5: ISR Best Practices (15 minutes)

Goal: Learn professional ISR programming

What to Read:

- Part 5: ISR Best Practices (Read carefully!)

- Part 6: Critical Sections

- Study both good and bad examples

Golden Rules:

1. KEEP ISR SHORT

Target: < 10 microseconds

Why: Long ISR blocks other interrupts

2. SET FLAG, DON'T PROCESS

// GOOD ISR

dataReady = true;

receivedData = USART2->DR;

// Done! Exit fast

// BAD ISR

uint8_t data = USART2->DR;

processData(data);  // ✗ Too slow!

printf("Data: %d", data);  // ✗ Very slow!

3. ALWAYS CLEAR FLAGS

Forgetting = ISR runs forever!

EXTI->PR = (1 << 13);  // Clear by writing 1

TIM6->SR &= ~TIM_SR_UIF;  // Clear by writing 0

4. USE VOLATILE

For any variable shared between ISR and main:

volatile bool dataReady;

volatile uint32_t systemTicks;

Why: Prevents compiler optimization that breaks code

5. PROTECT SHARED DATA

// Disable interrupts for critical section

__disable_irq();

sharedCounter++;  // Safe

__enable_irq();

What NOT to Do in ISR:

- ✗ printf() or sprintf() - VERY slow

- ✗ delay() functions

- ✗ Complex calculations

- ✗ Calling many functions

- ✗ Memory allocation (malloc/new)

- ✗ Blocking operations

Exercise:

Look at the good vs bad ISR examples in the file.

For each bad practice, explain WHY it's bad.

☐ □ Memorized all 5 golden rules

☐ □ Understand volatile keyword

☐ □ Know how to protect shared data

☐ □ Can identify bad ISR practices

⚠ Warning: ISR best practices are CRITICAL. Many embedded bugs come from violating these rules!

✓ HOUR 2 COMPLETE!

Take a 10-minute break before Hour 3!

# ⏱ HOUR 3: Complete System Integration (60 minutes)

Open file: FSM_ISR_Complete_Example.cpp

💡 Note: This is a complete, production-ready system. Study it carefully!

## Session 3.1: System Architecture (15 minutes)

Goal: Understand how all pieces fit together

System Overview:

Multi-function embedded system with:

- Advanced button control (single/double/long press)

- LED pattern generator (solid/blink/pulse)

- UART command processor (text commands)

- Sensor reading system (ADC with averaging)

Interrupt Architecture:

TIM6 ISR (Priority 0 - Highest)

↓ Runs every 1ms

├─ Update button FSM

├─ Update LED FSM

└─ Update sensor FSM (every 100ms)

EXTI13 ISR (Priority 2 - High)

↓ Button press/release

└─ Notify button FSM

USART2 ISR (Priority 3 - Medium)

↓ Byte received

└─ Store in command buffer

Main Loop

├─ Process UART commands

└─ Sleep (__WFI)

Why This Architecture Works:

- Timer provides precise timing for FSMs

- EXTI provides instant button response

- UART handles communication in background

- Main loop coordinates everything

- CPU can sleep when nothing to do

Data Flow:

[Hardware] → [ISR] → [Flag/Buffer] → [FSM/Main] → [Action]

Example: Button press

Button pressed → EXTI ISR → Set flag → Button FSM → LED toggle

☐ □ Understand interrupt priorities

☐ □ Know what each ISR does

☐ □ Understand data flow

## Session 3.2: Button Control System (20 minutes)

Goal: Master advanced button detection

What to Study:

- Project 1: Advanced Button Control

- Study all states in AdvancedButtonFsm class

- Understand how interrupts and FSM work together

Button Detection Features:

- Single Click: Quick press and release

- Double Click: Two clicks within 300ms

- Long Press: Hold for 1 second

FSM States:

IDLE

↓ Button pressed (EXTI ISR sets flag)

DEBOUNCE_PRESS

↓ Wait 20ms (timer ISR counts)

PRESSED

↓ Check: Hold 1s? → Long press

↓ Release? → Continue

DEBOUNCE_RELEASE

↓ Wait 20ms

↓ Short press → Check for double click

WAIT_DOUBLE_CLICK

↓ Second press within 300ms? → Double click!

↓ Timeout? → Single click

IDLE

Key Implementation Details:

- EXTI ISR: Detects press/release instantly

- Timer ISR: Updates FSM every 1ms

- FSM: Handles timing and logic

- Callbacks: onSingleClick(), onDoubleClick(), onLongPress()

Trace Exercise:

User double-clicks button. Trace through:

- What happens in EXTI ISR?

- What happens in Timer ISR?

- How does FSM detect double click?

- When is onDoubleClick() called?

☐ □ Understand all button states

☐ □ Know role of EXTI ISR

☐ □ Know role of Timer ISR

☐ □ Traced double-click sequence

✓ Tip: This button system is production-ready. You can use it in your projects!

## Session 3.3: Communication System (15 minutes)

Goal: Understand command processing with FSM

What to Study:

- Project 3: UART Command Processor

- Study UartCommandFsm class

- See how commands are parsed and executed

Command Protocol:

ASCII commands terminated by newline:

LED:ON\n

LED:OFF\n

LED:BLINK\n

LED:PULSE\n

STATUS\n

How It Works:

UART ISR:

↓ Byte received

├─ Store in buffer

└─ If \n: Set newCommand flag

Main Loop:

↓ Check newCommand flag

├─ Parse command string

├─ Execute action

└─ Send response

Key Design Decisions:

- ISR: Only stores bytes (fast!)

- Main loop: Processes commands (slow operations OK)

- FSM: Could be added for complex protocols

- Commands: Human-readable for debugging

Try It Yourself:

When you flash the code:

- Open serial terminal (115200 baud)

- Type: LED:BLINK

- Press Enter

- LED should start blinking!

☐ □ Understand command protocol

☐ □ Know ISR vs main loop responsibilities

☐ □ Can add new commands

## Session 3.4: Integration & Testing (10 minutes)

Goal: See how everything works together

Complete System Flow:

Scenario 1: User presses button

1. Button pressed (hardware)

2. EXTI13 ISR triggers (< 1 µs)

3. ISR sets flag in button FSM

4. Timer ISR updates FSM (1ms later)

5. FSM detects stable press after 20ms

6. LED toggles (action)

Scenario 2: User sends UART command

1. User types "LED:BLINK" + Enter

2. USART ISR receives each byte

3. Bytes stored in buffer

4. Newline detected → set flag

5. Main loop processes command

6. LED FSM changes pattern

7. Response sent via UART

Scenario 3: Sensor reading

1. Timer ISR every 100ms

2. Sensor FSM starts sampling

3. ADC reads value

4. 10 samples averaged

5. If changed → report via UART

Key Insights:

- Multiple things happening "simultaneously"

- Interrupts provide fast response

- FSMs provide clean logic

- Main loop coordinates

- Everything non-blocking

Testing Checklist:

☐ □ Compile code without errors

☐ □ Flash to Nucleo board

☐ □ Button press toggles LED

☐ □ Double-click detected

☐ □ Long press works

☐ □ UART commands work

☐ □ System runs smoothly

🎉 CONGRATULATIONS!

You've completed the FSM & ISR learning path!

# 🔬 Practical Labs

Now that you understand the concepts, practice by building these from scratch!

## Lab Setup Instructions

Required Hardware:

- STM32 Nucleo-F446RE (or similar)

- USB cable

- Breadboard (optional)

- LED + resistor (optional - board has built-in)

- Button (optional - board has built-in)

Software Setup:

- Install STM32CubeIDE

- Create new STM32 project

- Select Nucleo-F446RE board

- Set language to C++ (rename main.c to main.cpp)

- Add compiler flags: -std=c++17 -fno-exceptions -fno-rtti

## Lab 1: Build a Button Debouncer

Objective:

Implement a button debouncer FSM from scratch

Requirements:

- Use PC13 (built-in button)

- Debounce time: 20ms

- Toggle LED on PA5 when button pressed

- Print "Button pressed" to UART

Steps:

- Set up 1ms timer (TIM6)

- Create ButtonDebouncer class

- Implement 4 states: IDLE, DEBOUNCE_PRESS, PRESSED, DEBOUNCE_RELEASE

- Call update() from timer ISR

- Test: Press button rapidly, should only register once

Success Criteria:

☐ □ Button works reliably

☐ □ No false triggers from bouncing

☐ □ LED toggles on each press

☐ □ Code is clean and commented

✓ Tip: Reference: FSM_Complete_Tutorial.cpp, Example 1

## Lab 2: Implement Timer ISR System

Objective:

Create a timer-based system with multiple periodic tasks

Requirements:

- 1ms timer ISR

- Task 1: Blink LED every 500ms

- Task 2: Print "Tick" every 1 second

- Task 3: Toggle another pin every 100ms (for oscilloscope)

Steps:

- Configure TIM6 for 1ms interrupt

- Create global tick counter

- In ISR: increment counter, check for task periods

- Execute tasks at correct intervals

Success Criteria:

☐ □ All tasks run at correct intervals

☐ □ ISR is short (< 10 µs)

☐ □ System runs smoothly

☐ □ Can add new tasks easily

✓ Tip: Reference: STM32_ISR_Complete_Tutorial.cpp, Part 4

## Lab 3: Create Command Parser FSM

Objective:

Build a UART command parser with FSM

Protocol:

Binary protocol: <START> <CMD> <DATA> <CHECKSUM>

- START: 0xAA

- CMD: 1 byte command code

- DATA: 0-8 bytes

- CHECKSUM: Sum of all bytes

Commands:

- 0x01: LED ON

- 0x02: LED OFF

- 0x03: LED TOGGLE

- 0x10: GET STATUS (respond with system info)

Steps:

- Create ParserFSM class

- States: WAIT_START, READ_CMD, READ_DATA, READ_CHECKSUM

- UART ISR: Pass bytes to parser

- Main loop: Execute commands when valid packet received

Success Criteria:

☐ □ Parser handles valid packets correctly

☐ □ Parser rejects invalid packets

☐ □ Checksum verified

☐ □ Commands executed

☐ □ Status response sent

✓ Tip: Reference: FSM_Complete_Tutorial.cpp, Example 2

# 📖 Quick Reference Material

## FSM Switch-Case Template

enum class State {

STATE_A,

STATE_B

};

class MyFsm {

public:

void update() {

switch (state_) {

case State::STATE_A:

if (stateChanged_) {

// Entry action

stateChanged_ = false;

}

if (condition) {

transitionTo(State::STATE_B);

}

break;

case State::STATE_B:

// ...

break;

}

}

private:

State state_ = State::STATE_A;

bool stateChanged_ = true;

void transitionTo(State newState) {

state_ = newState;

stateChanged_ = true;

}

};

## GPIO EXTI Interrupt Template

// Setup

RCC->AHB1ENR |= RCC_AHB1ENR_GPIOCEN;

RCC->APB2ENR |= RCC_APB2ENR_SYSCFGEN;

SYSCFG->EXTICR[3] |= (0x02 << 4);  // PC13

EXTI->FTSR |= (1 << 13);  // Falling edge

EXTI->IMR |= (1 << 13);   // Enable

NVIC_SetPriority(EXTI15_10_IRQn, 2);

NVIC_EnableIRQ(EXTI15_10_IRQn);

// ISR

extern "C" void EXTI15_10_IRQHandler(void) {

if (EXTI->PR & (1 << 13)) {

EXTI->PR = (1 << 13);  // Clear flag

// Handle interrupt

}

}

## Timer Interrupt Template

// Setup for 1ms interrupt

RCC->APB1ENR |= RCC_APB1ENR_TIM6EN;

TIM6->PSC = 45000 - 1;  // Prescaler

TIM6->ARR = 1 - 1;      // Period

TIM6->DIER = TIM_DIER_UIE;

NVIC_SetPriority(TIM6_DAC_IRQn, 0);

NVIC_EnableIRQ(TIM6_DAC_IRQn);

TIM6->CR1 = TIM_CR1_CEN;  // Start

// ISR

extern "C" void TIM6_DAC_IRQHandler(void) {

if (TIM6->SR & TIM_SR_UIF) {

TIM6->SR &= ~TIM_SR_UIF;  // Clear flag

systemTicks++;

}

}

## Common Mistakes Checklist

FSM Mistakes:

☐ □ Checked: Using enum class (not plain enum)

☐ □ Checked: Handling all events in all states

☐ □ Checked: Not blocking in state handlers

☐ □ Checked: Entry actions vs state updates separated

ISR Mistakes:

☐ □ Checked: ISR is short (< 10 µs)

☐ □ Checked: Interrupt flag cleared

☐ □ Checked: Not using printf() in ISR

☐ □ Checked: Shared variables declared volatile

☐ □ Checked: Critical sections protected

Integration Mistakes:

☐ □ Checked: Priorities set correctly

☐ □ Checked: All ISRs have extern "C"

☐ □ Checked: No race conditions

☐ □ Checked: System tested under load

## Debugging Tips

FSM Not Working?

- Add printf() to each state entry

- Log all state transitions

- Draw actual vs expected state diagram

- Check if update() being called

Interrupt Not Triggering?

- Check if peripheral interrupt enabled

- Check if NVIC interrupt enabled

- Check priority is set

- Verify pin configuration

- Check if ISR name is correct

System Hanging?

- Interrupt flag not cleared?

- Stack overflow?

- Infinite loop in ISR?

- Priority inversion?

# 🚀 Next Steps

Congratulations on completing the learning path! Here's how to continue:

## Practice Projects

- Build a digital clock with button-set time

- Create a menu system with LCD display

- Implement a motor controller with soft start/stop

- Build a data logger with SD card

## Advanced Topics

- Hierarchical State Machines

- RTOS with FreeRTOS

- DMA for high-speed data transfer

- Low-power modes and sleep

- Watchdog timers

## Resources

- STM32 Reference Manual (your specific chip)

- STM32 Programming Manual (PM0214)

- Quantum Leaps State Machine framework

- Real-Time Systems book by Jane Liu

Happy Embedded Programming! 🎉

You now have the skills to build professional embedded systems!