Finite State Machines

Theory & Practice for Embedded Systems

From Mathematical Foundations to Production Code

# PART 1: THEORETICAL FOUNDATIONS

## 1.1 What is a Finite State Machine?

Formal Mathematical Definition:

A Finite State Machine (FSM) is a 5-tuple (Q, Σ, δ, q₀, F) where:

- Q = Finite set of STATES

- Σ = Finite set of INPUT symbols (EVENTS)

- δ = TRANSITION function: δ: Q × Σ → Q

- q₀ = INITIAL state (q₀ ∈ Q)

- F = Set of FINAL/ACCEPTING states (F ⊆ Q)

● In Plain English

A system that can only be in ONE state at a time. Events cause transitions between states following predefined rules.

## 1.2 Real-World Mathematical Examples

Example 1: Traffic Light

Q  = {RED, YELLOW, GREEN}

Σ  = {Timer_Expired}

q₀ = RED

δ: RED + Timer_Expired → GREEN

GREEN + Timer_Expired → YELLOW

YELLOW + Timer_Expired → RED

Example 2: Door Lock

Q  = {LOCKED, UNLOCKED}

Σ  = {Correct_Code, Wrong_Code, Timeout}

q₀ = LOCKED

δ: LOCKED + Correct_Code → UNLOCKED / Open_Lock()

LOCKED + Wrong_Code → LOCKED / Beep_Error()

UNLOCKED + Timeout → LOCKED / Close_Lock()

## 1.3 Why FSM Theory Matters for Engineers

### Reason 1: Predictability

Every possible system state is formally defined. Behavior is deterministic - no surprises. Critical for safety systems in medical devices, automotive, and aerospace applications.

### Reason 2: Verifiability

You can mathematically prove correctness. Model checking tools can verify:

- All states are reachable (no dead code)

- No deadlock states exist

- System satisfies safety properties

- Real-time constraints are met

### Reason 3: Documentation

State diagram = visual specification. Clear communication with non-programmers. Code reviews become easier. System behavior maintainable over years.

### Reason 4: Testability

Each state can be tested independently. Each transition can be verified. Test coverage is measurable. Edge cases are easy to identify.

### Reason 5: Complexity Management

Complex behavior broken into simple pieces. Each state handles one condition. Reduces cognitive load. Scales better than nested if-else chains.

## 1.4 Types of Finite State Machines

### Mealy Machine

Output depends on: CURRENT STATE + INPUT

Actions occur on TRANSITIONS

More common in embedded systems

Example:

State: IDLE

Event: Button_Press

Transition: IDLE → ACTIVE

Action during transition: Turn_On_LED()

### Moore Machine

Output depends on: CURRENT STATE only

Actions occur WITHIN STATES

May require more states than Mealy

Example:

State: LED_ON_STATE

Output: LED is on (continuous while in state)

### Deterministic FSM (DFA)

For each (state, input) pair: exactly ONE next state

Most embedded systems use DFA

Predictable, testable, reliable

### Non-Deterministic FSM (NFA)

Multiple possible next states for same input

Useful in theoretical computer science

Can be converted to DFA

Rarely used in embedded systems

## 1.5 Spaghetti Code vs FSM - Quantitative Comparison

Approach 1: Multiple Flags (BAD)

bool buttonPressed = false;

bool longPressHandled = false;

bool ledIsOn = false;

uint32_t pressTime = 0;

if (readButton() && !buttonPressed) {

buttonPressed = true;

pressTime = millis();

longPressHandled = false;

}

if (!readButton() && buttonPressed) {

buttonPressed = false;

if (!longPressHandled) {

ledIsOn = !ledIsOn;

}

}

if (buttonPressed && (millis()-pressTime>1000)

&& !longPressHandled) {

longPressHandled = true;

blinkLED();

}

Problems:

- Hard to see overall behavior

- Multiple variables track "state"

- Easy to introduce bugs (forgot to reset flag?)

- Difficult to add features

- Not clear what "state" system is in

- Race conditions possible

Approach 2: FSM (GOOD)

enum class ButtonState {

IDLE, PRESSED, LONG_PRESS

};

ButtonState state = ButtonState::IDLE;

switch (state) {

case ButtonState::IDLE:

if (readButton()) {

state = ButtonState::PRESSED;

pressStartTime = millis();

}

break;

case ButtonState::PRESSED:

if (!readButton()) {

toggleLED();

state = ButtonState::IDLE;

} else if (millis()-pressStartTime > 1000) {

state = ButtonState::LONG_PRESS;

blinkLED();

}

break;

case ButtonState::LONG_PRESS:

if (!readButton()) {

state = ButtonState::IDLE;

}

break;

}

Benefits:

- Clear states (IDLE, PRESSED, LONG_PRESS)

- One variable tracks everything

- Easy to understand at a glance

- Easy to extend (add DOUBLE_CLICK state)

- Matches state diagram exactly

- Self-documenting

# PART 2: FSM COMPONENTS

## 2.1 States

Definition:

Distinct conditions or configurations of the system

Characteristics:

- System is in exactly ONE state at any time

- States should be mutually exclusive

- States should be meaningful and distinct

Examples:

- Motor: STOPPED, STARTING, RUNNING, STOPPING

- Connection: DISCONNECTED, CONNECTING, CONNECTED, ERROR

- Battery: CHARGING, FULL, DISCHARGING, LOW, CRITICAL

## 2.2 Events (Inputs)

Definition:

Triggers that cause state changes

Types:

- External events: Button press, sensor trigger, message received

- Internal events: Timer expiry, counter threshold

- Conditional events: Temperature > 80°C

Properties:

- Events are instantaneous

- Events may carry data (button_press with button_id)

- Events may be queued or immediate

## 2.3 Transitions

Definition:

Rules for changing from one state to another

Format:

[Current State] + [Event] [Condition] → [Next State]

Example:

IDLE + button_press [if armed] → ACTIVE

Properties:

- Can have guard conditions

- Should be deterministic

- Can be self-transitions (stay in same state)

## 2.4 Actions

Definition:

Things that happen during transitions or in states

Types:

a) Entry Actions: Execute when entering a state

Example: Enter RUNNING → start_motor()

b) Exit Actions: Execute when leaving a state

Example: Exit RUNNING → stop_motor()

c) Transition Actions: Execute during state change

Example: IDLE → ACTIVE: turn_on_led()

d) State Actions: Execute while in a state

Example: While in RUNNING: monitor_temperature()

# PART 3: STATE DIAGRAM NOTATION

Standard UML Notation Elements:

- ● = Initial state marker

- ╔═══╗ = Initial state (bold border)

- ┌───┐ = Normal state

- ──> = Transition arrow

- event/action = Transition label

- [condition] = Guard condition

- ◉ = Final/accepting state (double circle)

Complete Button FSM Example:

┌──────────────┐

│              │

↓              │ button_released

╔════════╗         │

●─>║  IDLE  ║         │

╚════════╝         │

│              │

│ button_pressed

│ /start_timer()

↓              │

┌────────────┐     │

│  PRESSED   │     │

└────────────┘     │

│              │

│ timer>1000ms │

│ /long_press_action()

↓              │

┌────────────┐     │

│ LONG_PRESS │     │

└────────────┘     │

│              │

│ button_released

└──────────────┘

This diagram shows:

- Initial state: IDLE (marked with ●)

- Three states with clear transitions

- Events labeled on arrows (button_pressed, button_released)

- Actions on transitions (/start_timer(), /long_press_action())

- Guard condition (timer>1000ms)

- Return paths to IDLE state

# PART 4: WHEN TO USE FSM

## 4.1 Use FSM When:

- ✓ System has multiple operating modes

- ✓ Event-driven behavior

- ✓ Complex timing sequences

- ✓ Protocol implementation

- ✓ User interface logic

- ✓ Safety-critical state management

- ✓ Anything with "modes," "phases," or "stages"

## 4.2 Do NOT Use FSM When:

- ✗ Simple sequential code (1-2 states)

- ✗ Pure mathematical calculations

- ✗ One-time initialization sequences

- ✗ Simple if-else logic

## 4.3 Real-World Application Examples

Example 1: Communication Protocol

Parsing incoming data stream with specific format

WAIT_START → READ_LENGTH → READ_DATA → READ_CHECKSUM

FSM ensures correct parsing even with errors or incomplete data

Example 2: Motor Control

Safe startup and shutdown sequences

STOPPED → ACCELERATING → RUNNING → DECELERATING → STOPPED

FSM prevents sudden motor starts/stops, manages acceleration curves

Example 3: User Interface

Menu system with multiple screens

MAIN_MENU → SETTINGS → BRIGHTNESS → MAIN_MENU

FSM tracks current screen, handles navigation

Example 4: Battery Management

Safe charging with multiple states

IDLE → TRICKLE → FAST_CHARGE → TOP_OFF → FULL

FSM manages charging algorithm, prevents overcharge

# SUMMARY

Key Theoretical Concepts:

- FSM is a formal mathematical model (5-tuple)

- Provides predictability, verifiability, testability

- State diagrams visually document behavior

- Mealy (actions on transitions) vs Moore (actions in states)

- Deterministic FSMs guarantee predictable behavior

Practical Benefits:

- Cleaner code than multiple flags

- Easy to understand and maintain

- Straightforward to add features

- Natural match for real-world systems

- Industry standard for complex control logic

When to Apply:

- Multiple operating modes

- Event-driven systems

- Protocol parsers

- Control systems

- User interfaces

Master FSM theory and you'll write better embedded software!