PRODUCT REQUIREMENTS DOCUMENT

MADEPLUS Smart Shoe
Advanced Gait Analysis & Activity Recognition System

| Product Name | MADEPLUS SMARTSHOE |
| --- | --- |
| Version | 1.1 (with added features) |
| Date | January 2026 |
| Company | MADEPLUS INC |
| Hardware | ESP32-S3 + GY-86 (MPU6050 + MS5611 + Mag optional) |

Owner: Ratneswar Roychowdhury

# Revision History

| Version | Date | Author | Notes |
| --- | --- | --- | --- |
| 1.0 | Jan 2026 | Ratneswar Roychowdhury | Initial PRD draft |
| 1.1 | Jan 2026 | Ratneswar Roychowdhury | Added model bundles, label filters, calibration mode, and app-controlled device tuning |

# 1. Executive Summary

The MADEPLUS Smart Shoe is a foot-mounted wearable system that delivers high-quality gait metrics and activity recognition by combining an ESP32-S3 module with a GY-86 sensor suite and a Flutter mobile application. The system streams real-time biomechanics over BLE and supports on-phone training and inference for activity classification.

## 1.1 Key Differentiators

ZUPT (Zero-Velocity Update) stride estimation without GPS dependency

Per-step barometric elevation change (Δh) for terrain and stair analysis

On-device sensing + on-phone machine learning pipeline (training + inference)

High-rate 200 Hz sensor fusion and stance/swing event detection

Comprehensive Flutter app: live dashboard, charts, training sessions, exports

# 2. Problem Statement

Wrist wearables often miss step dynamics and stair/elevation changes due to arm swing and loose coupling. A foot-mounted sensor captures stronger, more repeatable signals for step timing, stance/swing segmentation, stride dynamics, and vertical motion. The product targets fitness, rehab, and research workflows that need actionable, exportable gait signals.

# 3. Goals & Success Metrics

## 3.1 Product Goals

Stable BLE streaming from shoe to phone with auto-reconnect

Accurate step count, cadence, stride estimate, and Δh per step in real time

Labeled data capture to CSV with sharing/export

On-phone training, saving, loading, and live prediction from models

## 3.2 Initial Success Targets

| Metric | Target | Notes |
| --- | --- | --- |
| Step count error | ≤ ±3–5% | vs manual count (walking/running, flat ground) |
| Cadence error | ≤ ±5 spm | steady-state walking/running |
| BLE stability | < 1 disconnect/hour | auto-reconnect success > 90% |
| ML accuracy (4 labels) | Macro F1 ≥ 0.80 (baseline) | user-collected data; improves with calibration |
| Training time | < 15 seconds | typical session set on phone |

# 4. Personas

Everyday user: wants simple steps/cadence/stride on phone.

Runner/fitness user: wants cadence/stride trends and session exports.

Physio/rehab user: wants repeatable measurements and calibration sessions.

Research/engineering user: wants raw logs, feature control, and model portability.

# 5. System Architecture

## 5.1 Hardware

MCU: ESP32-S3

Sensors: GY-86 module (MPU6050 accel/gyro, MS5611 barometer + temperature, magnetometer optional)

Placement: heel/foot-mounted enclosure to ensure foot-fixed reference and stronger gait signals

## 5.2 Embedded Firmware

IMU sampling at 200 Hz with filtering and bias calibration

Roll/pitch estimation via complementary filter (gyro integration + accel tilt)

Stance/swing segmentation using gyro magnitude and accel variance window

ZUPT-aided swing integration to estimate horizontal stride length

Event-triggered barometer sampling at heel-strike for Δh per step

BLE GATT server using NimBLE with fixed UUIDs and configurable notification rate

## 5.3 Mobile App (Flutter)

Cross-platform iOS/Android

Tabs: Dashboard, Graphs, Training, Tutorial

Live charts and session recording to CSV

On-phone model training, saving/loading, and live predictions

# 6. Core Algorithms

## 6.1 ZUPT-based Stride Length Estimation

Stride length is estimated by integrating linear acceleration during the swing phase and applying a zero-velocity correction at heel-strike (ZUPT). Stance is detected when the foot is stationary (gyro magnitude below threshold and accelerometer variance below threshold).

## 6.2 Complementary Filter

Roll/pitch are computed using a complementary filter combining high-pass gyro integration with low-pass accelerometer tilt for drift-reduced orientation.

## 6.3 Gait Event Detection

Toe-off: stance → swing transition

Heel-strike: swing → stance transition (step event)

Cadence: steps per minute computed over a sliding window (e.g., 8 steps)

## 6.4 Barometric Altitude

Barometer reads are triggered at heel-strike to compute per-step elevation change (Δh), reducing power consumption versus continuous sampling while improving terrain/stairs inference.

# 7. BLE Communication Protocol

Primary Service UUID: 0000feed-0000-1000-8000-00805f9b34fb

| Signal | Characteristic UUID | Type | Unit | Default Rate |
| --- | --- | --- | --- | --- |
| Roll | 0000a001-0000-1000-8000-00805f9b34fb | float32 LE | deg | 10 Hz |
| Pitch | 0000a002-0000-1000-8000-00805f9b34fb | float32 LE | deg | 10 Hz |
| Yaw | 0000a003-0000-1000-8000-00805f9b34fb | float32 LE | deg | 10 Hz |
| Temperature | 0000b001-0000-1000-8000-00805f9b34fb | float32 LE | °C | 10 Hz |
| Steps | 0000c001-0000-1000-8000-00805f9b34fb | int32 LE | count | 10 Hz |
| Cadence | 0000c002-0000-1000-8000-00805f9b34fb | float32 LE | spm | 10 Hz |
| Stride Length | 0000c003-0000-1000-8000-00805f9b34fb | float32 LE | m | 10 Hz |
| Δh per step | 0000b002-0000-1000-8000-00805f9b34fb | float32 LE | m | 10 Hz |

## 7.1 Control & Debug Channels (Recommended)

These characteristics enable app-controlled tuning and debug logging:

| Characteristic | UUID | Purpose |
| --- | --- | --- |
| Command (Write) | 0000d001-0000-1000-8000-00805f9b34fb | ASCII commands: reset, notify_hz, debug, thresholds |
| Debug ASCII (Notify) | 0000a0ee-0000-1000-8000-00805f9b34fb | Human-readable status and algorithm telemetry |

# 8. Mobile Application Requirements

## 8.1 Dashboard (P0)

Scan/filter by service UUID; show device list with RSSI

Connect/disconnect; show connection state clearly

Auto-reconnect to last device; manual override supported

Live tiles for: steps, cadence, stride, Δh; optional roll/pitch/yaw + temperature

Live prediction panel (if model enabled): label + confidence

## 8.2 Graphs (P0)

Multi-axis time-series plots for roll/pitch/yaw and optional cadence/stride/Δh

Pause/resume plotting; clear buffer

Export snapshot or CSV (optional)

## 8.3 Training (P1)

Label picker: walking, running, stairs_up, stairs_down (editable list in settings)

Start/stop recording; store CSV in app storage (sessions/)

Session history view with metadata (duration, label, avg cadence, notes)

Share/export CSV via share sheet

Optional route tracking via OpenStreetMap (if GPS enabled)

## 8.4 Machine Learning (P1)

Feature extraction using sliding windows (default 3.0s window, 1.5s hop)

Train a lightweight multiclass model (softmax/logistic regression baseline)

Model evaluation summary (accuracy, confusion matrix optional)

Save/load models locally; support multiple active models (ensemble)

# 9. Added Features (Requested)

## 9.1 Model Bundle Export / Import (P1)

Users can export a complete 'Model Bundle' (ZIP) containing: model JSON, label set, feature configuration, training notes, and version metadata. Users can import bundles to move models between devices or share with clinicians/teammates.

### Acceptance Criteria

Export creates a single .zip file from the app and opens share sheet.

Import validates bundle schema and shows warnings for mismatched feature sets.

Imported models appear in the Models list and can be activated immediately.

## 9.2 Per-Label Enable/Disable for Live Predictions (P1)

Provide toggles to enable/disable specific labels during live inference. This helps reduce false positives (e.g., ignore 'sitting' while outdoors). Disabled labels are excluded from the final decision and confidence display.

### Acceptance Criteria

User can toggle labels on/off in a simple list UI.

Disabled labels never show as top prediction.

Settings persist across app restarts.

## 9.3 Calibration Session Mode (P1)

Add a dedicated Calibration mode for clinical and high-accuracy scenarios. Calibration guides the user through standardized protocols (e.g., 2-min walk, 10m walk test, stair segment) and updates parameters for stride estimation and/or model thresholds. Calibration output is stored as a profile per user/device.

### Acceptance Criteria

Calibration wizard with at least 2 protocols (flat walk + stairs).

Calibration produces a saved profile (timestamped) and indicates whether calibration is active.

Stride and cadence metrics show improved stability after calibration (qualitative pass).

## 9.4 App-Controlled Device Tuning & Reset (P1.1)

Expose device command controls in the app to adjust notification rate, reset counters, and tune key thresholds (gyro_th, variance_th, filter gain). Provide safe ranges and a 'restore defaults' option.

### Acceptance Criteria

User can send reset and notify_hz commands from the app.

User can adjust thresholds within defined limits; changes take effect within 2 seconds.

Restore defaults returns the device to known baseline parameters.

# 10. Non-Functional Requirements

Latency: notify-to-display < 300 ms (typical).

App stability: no UI freezes during recording; training runs without crashes.

Data integrity: CSV sessions are closed/flushed correctly on stop.

Privacy: local-only by default; explicit user action required to share/export.

# 11. Development Roadmap

| Phase | Scope | Target |
| --- | --- | --- |
| Phase 1 (Core) | BLE streaming + dashboard/graphs + basic metrics | Completed / baseline |
| Phase 2 (Enhancement) | Model bundles, label filters, calibration mode, device tuning UI | Q1 2026 |
| Phase 3 (Clinical) | Protocol library, reporting, partnerships, regulatory prep | Q2–Q3 2026 |
| Phase 4 (Commercial) | Manufacturing, distribution, B2B + D2C channels | Q4 2026 |

# 12. Intellectual Property Notes

Event-triggered barometric sampling at heel-strike for power-efficient Δh per step.

Dual-threshold stance detection using gyro magnitude + accelerometer variance with hysteresis.

Adaptive power management via duty cycling + BLE connection optimization.

# Appendix A: UUID Summary

Service: 0000feed-0000-1000-8000-00805f9b34fb

Roll: 0000a001-0000-1000-8000-00805f9b34fb

Pitch: 0000a002-0000-1000-8000-00805f9b34fb

Yaw: 0000a003-0000-1000-8000-00805f9b34fb

Temp: 0000b001-0000-1000-8000-00805f9b34fb

Steps: 0000c001-0000-1000-8000-00805f9b34fb

Cadence: 0000c002-0000-1000-8000-00805f9b34fb

Stride: 0000c003-0000-1000-8000-00805f9b34fb

Δh: 0000b002-0000-1000-8000-00805f9b34fb

Command (Write): 0000d001-0000-1000-8000-00805f9b34fb

Debug ASCII: 0000a0ee-0000-1000-8000-00805f9b34fb