PHASE 2 · DAYS 31–60 Security, Testing & Architecture TrustZone-M & Secure Boot · pytest/HIL Testing · AUTOSAR · CAN-FD

Phase 2 deepens your security architecture, firmware verification ownership, and systems design vocabulary. These skills are directly tested in principal-level medtech interviews and are increasingly demanded by FDA cybersecurity guidance (2023 final). Work through Skills 5–7 in order.

| Item | Detail |
| --- | --- |
| Skill 5 | TrustZone-M & Secure Boot — hardware-rooted security ¦ Days 31–42 |
| Skill 6 | pytest-embedded & HIL Testing — automated firmware verification ¦ Days 43–53 |
| Skill 7 | AUTOSAR Architecture + CAN-FD — systems vocabulary ¦ Days 54–60 |
| Total | ~22 hours · 3 portfolio artifacts · interview-ready security & testing depth |

| 05 | TrustZone-M & Hardware Security for Medical Devices Days 31–42 · ~10 hours |
| --- | --- |

# Skill 5: TrustZone-M, Secure Boot, and FDA Cybersecurity

## 5.1 Why Hardware Security Is Now a Regulatory Requirement

The FDA's 2023 Final Guidance on Cybersecurity in Medical Devices (enacted under the Consolidated Appropriations Act 2023, Section 3305) makes pre-market cybersecurity submissions mandatory for any device that contains software. The guidance explicitly requires: (1) a Software Bill of Materials (SBOM), (2) a threat model, (3) evidence of secure coding practices, (4) a post-market vulnerability disclosure policy, and (5) a patch management plan with SLA commitments.

For firmware engineers, this translates to concrete technical requirements: secure boot to verify firmware integrity at power-on, signed OTA updates to prevent malicious firmware installation, hardware-protected key storage, and debug access controls that prevent unauthorized access in the field. TrustZone-M provides the hardware foundation for all of these.

| NOTE | The FDA's 2023 guidance has teeth: devices submitted after March 2023 that lack a cybersecurity plan can be refused receipt — meaning FDA will not begin their review until the submission is compliant. This is a direct schedule risk for any medtech company. |
| --- | --- |

## 5.2 ARM TrustZone-M Architecture

TrustZone-M is an ARM security extension available on Cortex-M23, M33, M35P, and M55 cores. It partitions the MCU into two worlds: Secure World and Non-Secure World. Each world has its own code, data, and peripherals. Secure World code can access Non-Secure World resources, but Non-Secure World code cannot access Secure World resources — enforced in hardware.

| Item | Detail |
| --- | --- |
| Secure World (S) | Boots first. Contains: crypto keys, attestation credentials, secure boot logic, TRNG, OTP fuse access, privileged operations. Runs the BL1/BL2 bootloaders and security service functions. |
| Non-Secure World (NS) | Application firmware runs here. Can call Secure World functions only through NSC (Non-Secure Callable) veneer gates — predefined entry points that the Secure World explicitly exposes. |
| NSC Functions | Veneers compiled into a special NSC region. The NS application calls them like normal functions. TrustZone hardware automatically elevates privilege and switches to Secure World, executes the function, then returns. |
| SAU (Security Attribution Unit) | Configures which address ranges are Secure, Non-Secure, or NSC. Configured by the Secure World bootloader before transferring control to NS application. |

## 5.3 TrustZone-M Demo on STM32L5

The STM32L5 is the most accessible TrustZone-M development platform. It has a Cortex-M33 with TrustZone support, integrated STSAFE-A security element, and an extensive STM32CubeL5 HAL with TFM (Trusted Firmware-M) examples.

C

/* ── Secure World: expose TRNG as an NSC function ────────────────────*/

/* File: secure_services.c (compiled with -mcmse for NSC region) */

#include "arm_cmse.h"

#include "stm32l5xx_hal.h"

/* NSC entry point — NS app can call this to get 32 random bytes */

/* __attribute__((cmse_nonsecure_entry)) marks this as NSC veneer */

__attribute__((cmse_nonsecure_entry))

uint32_t NSC_GetRandom32(void) {

uint32_t rng_val = 0U;

/* Access RNG peripheral — mapped to Secure World only */

if (HAL_RNG_GenerateRandomNumber(&hrng, &rng_val) != HAL_OK) {

return 0U; /* caller must handle 0 as an error indicator */

}

return rng_val;

}

/* ── Non-Secure World: call the NSC function ─────────────────────────*/

/* File: app_main.c (Non-Secure application) */

extern uint32_t NSC_GetRandom32(void); /* declaration only */

void generate_session_nonce(uint8_t *nonce_buf, size_t len) {

uint32_t val;

for (size_t i = 0U; i < len; i += 4U) {

val = NSC_GetRandom32(); /* TrustZone switches to S, runs */

if (val == 0U) { trigger_fault(); return; } /* never use 0 */

memcpy(&nonce_buf[i], &val, 4U);

}

}

/* The NS app never has access to the RNG peripheral directly. */

/* Even a fully compromised NS app cannot forge random numbers. */

## 5.4 STM32 Read-Out Protection (RDP)

Read-Out Protection prevents an attacker from reading your firmware or secret keys from flash memory by connecting a JTAG debugger to a field device. STM32 has three RDP levels — configuring the correct level before manufacturing release is a fundamental security hygiene step.

| Item | Detail |
| --- | --- |
| RDP Level 0 | No protection. Flash fully readable via debugger. ONLY for development boards — never ship a product at RDP0. |
| RDP Level 1 | Flash read protection active. Debugger cannot read flash. Debug connection allowed (you can attach a debugger, step code, read registers, but not dump flash). JTAG/SWD still functional. Recommended for most production devices. |
| RDP Level 2 | Full protection. Flash read/write protected. JTAG/SWD permanently disabled. RDP2 is irreversible — the device cannot be recovered to RDP0/1. Use with extreme caution — wrong configuration bricks the device permanently. |

C

/* ── Set RDP Level 1 at manufacturing (do ONCE, then lock) ──────────*/

#include "stm32f4xx_hal_flash_ex.h"

HAL_StatusTypeDef set_rdp_level1(void) {

FLASH_OBProgramInitTypeDef ob = {0};

HAL_FLASHEx_OBGetConfig(&ob);

if (ob.RDPLevel == OB_RDP_LEVEL_1) {

return HAL_OK; /* already set */

}

HAL_FLASH_OB_Unlock();

ob.OptionType = OPTIONBYTE_RDP;

ob.RDPLevel = OB_RDP_LEVEL_1;

HAL_StatusTypeDef ret = HAL_FLASHEx_OBProgram(&ob);

HAL_FLASH_OB_Lock();

if (ret == HAL_OK) {

HAL_FLASH_OB_Launch(); /* Resets device to apply */

}

return ret;

}

/* WARNING: This function should only be callable from a secure */

/* manufacturing provisioning mode — never from normal application code */

## 5.5 MCUboot Secure Boot Chain

MCUboot is an open-source, hardware-agnostic secure bootloader used in Zephyr RTOS, ESP-IDF, Apache Mynewt, and many commercial medical device platforms. It verifies the digital signature of the application image before executing it — preventing unauthorized or corrupted firmware from running.

### Boot Chain of Trust

TEXT

ROM Bootloader (built into MCU silicon, unmodifiable)

└── Verifies MCUboot signature using key burned into OTP fuses

└── MCUboot BL2 (your bootloader, in flash slot 0)

├── Reads application image header

├── Verifies ECDSA-P256 signature against embedded public key

├── Checks version, magic bytes, CRC of image

└── If valid: transfers execution to application

└── Application firmware (your Class C medical device code)

BASH

/* ── MCUboot image signing workflow ─────────────────────────────────*/

# Step 1: Generate ECDSA-P256 signing key pair (done ONCE, offline)

imgtool keygen --key signing_key.pem --type ecdsa-p256

# Step 2: Extract public key to embed in bootloader

imgtool getpub --key signing_key.pem >> bootloader/keys/signing_pubkey.c

# Step 3: Build unsigned application firmware

west build -b esp32s3_devkitc # produces app.bin

# Step 4: Sign the firmware image

imgtool sign \

--key signing_key.pem \

--align 4 \

--version 1.2.3+0 \

--header-size 0x20 \

--slot-size 0x100000 \

build/zephyr/zephyr.bin \

build/zephyr/zephyr_signed.bin

# Step 5: Flash signed image — MCUboot will verify before executing

west flash --bin-file build/zephyr/zephyr_signed.bin

# SECURITY NOTE: signing_key.pem must NEVER leave your secure build

# environment. Compromise of the private key = device takeover.

## 5.6 Generating an SBOM for FDA Submission

The FDA 2023 guidance requires an SBOM — a machine-readable inventory of all software components. The SBOM must include every third-party library, RTOS component, and development tool used to build the final firmware image. The CycloneDX and SPDX formats are both accepted by FDA reviewers.

BASH

# Generate SBOM from Zephyr build using west + sbom plugin

pip install west-sbom

# After 'west build':

west sbom --output sbom.json --format cyclonedx

# For CMake-based STM32 project, use syft:

syft dir:./Src --output cyclonedx-json > sbom.json

# Validate SBOM structure:

cyclonedx validate --input-file sbom.json --input-version v1.4

# SBOM JSON will contain entries like:

# { "name": "FreeRTOS", "version": "10.5.1",

# "licenses": [{"license":{"id":"MIT"}}],

# "hashes": [{"alg":"SHA-256","content":"abc123..."}] }

# FDA submission: include sbom.json in your cybersecurity documentation

# section of the 510(k) or De Novo submission package.

## 5.7 Week 5–6 Exercises

- Configure RDP Level 1 on an STM32 development board. Document the procedure as a manufacturing SOP with screenshots: before state (RDP0), configuration steps, reboot, verify (RDP1 confirmed via STM32CubeProgrammer).

- Build a minimal TrustZone-M demo on STM32L5 or nRF9160-DK: put one NSC function (e.g., TRNG read or AES encrypt) in Secure World, call it from Non-Secure application. Verify with debugger that NS code cannot directly access the peripheral.

- Sign a firmware image using imgtool (MCUboot). Flash it. Then attempt to flash an unsigned image and confirm MCUboot rejects it. Document the test as TP-SEC-001.

- Generate a CycloneDX SBOM for one of your projects. Open it and verify all SOUP items from your SOUP Register appear in the SBOM. This is your FDA cybersecurity documentation artifact.

| GOAL | DAYS 31–42 DELIVERABLE: RDP configuration SOP + TrustZone-M demo write-up + MCUboot sign/reject test report + CycloneDX SBOM file. |
| --- | --- |

| 06 | pytest-embedded & HIL Test Automation Days 43–53 · ~10 hours |
| --- | --- |

# Skill 6: Automated Firmware Testing with pytest-embedded

## 6.1 Why Automated Firmware Testing Is a Principal-Level Skill

Writing firmware is necessary but insufficient at principal level. IEC 62304 requires documented unit tests with results, integration tests with results, and traceability from tests back to requirements — for every Class C software unit. Manually running tests on hardware and filling in spreadsheets does not scale and is not reproducible.

pytest-embedded is Espressif's open-source pytest plugin that automates the process of flashing firmware onto ESP32 targets over USB-serial, running the embedded test suite, parsing the output, and reporting pass/fail results — all from a Python test script that runs in CI. The same pattern extends to STM32 targets using OpenOCD.

| NOTE | Test automation is also a differentiator in interviews. Being able to say 'I own the HIL test pipeline' — including the Python test harness, the CI integration, and the traceability matrix linking test IDs to IEC 62304 requirements — signals readiness to lead a medical device firmware team. |
| --- | --- |

## 6.2 Unity C Test Framework on the Embedded Target

Unity is a lightweight C unit test framework designed for embedded systems. It generates structured output (PASS/FAIL/IGNORE per test case) that pytest-embedded parses. Write Unity tests in C, compile them into your firmware, and run them automatically.

C

/* ── test_dose_calculator.c ──────────────────────────────────────────*/

#include "unity.h"

#include "dose_calculator.h"

/* setUp() and tearDown() are called before/after each test case */

void setUp(void) { dose_calculator_reset(); }

void tearDown(void) { /* nothing */ }

/* ── Test: normal operation ──────────────────────────────────────────*/

void test_dose_calc_normal_range(void) {

dose_input_t in = { .sensor_mg = 1000U, .setpoint_mg = 1000U };

dose_output_t out = {0};

dose_calc_err_t err = dose_calculator_update(&in, &out);

TEST_ASSERT_EQUAL(DOSE_OK, err);

/* Expect PWM near 50% when sensor == setpoint */

TEST_ASSERT_UINT16_WITHIN(100U, 2048U, out.pwm_counts);

}

/* ── Test: sensor underrange triggers safe state ─────────────────────*/

void test_dose_calc_sensor_underrange(void) {

dose_input_t in = { .sensor_mg = 50U, .setpoint_mg = 1000U };

dose_output_t out = {0};

dose_calc_err_t err = dose_calculator_update(&in, &out);

TEST_ASSERT_EQUAL(DOSE_ERR_SENSOR_LOW, err);

TEST_ASSERT_EQUAL_UINT16(0U, out.pwm_counts); /* safe: actuator off */

}

/* ── Test: dose limit enforcement ───────────────────────────────────*/

void test_dose_calc_accumulator_limit(void) {

dose_input_t in = { .sensor_mg = 1000U, .setpoint_mg = 5000U };

dose_output_t out = {0};

/* Drive accumulator to max over 15 cycles */

for (uint8_t i = 0U; i < 15U; i++) { dose_calculator_update(&in, &out); }

dose_calc_err_t err = dose_calculator_update(&in, &out);

TEST_ASSERT_EQUAL(DOSE_ERR_LIMIT_EXCEEDED, err);

TEST_ASSERT_EQUAL_UINT16(0U, out.pwm_counts);

}

/* ── Test runner (called from app_main or test task) ─────────────────*/

void run_dose_calculator_tests(void) {

UNITY_BEGIN();

RUN_TEST(test_dose_calc_normal_range);

RUN_TEST(test_dose_calc_sensor_underrange);

RUN_TEST(test_dose_calc_accumulator_limit);

UNITY_END(); /* prints: 3 Tests 0 Failures 0 Ignored */

}

## 6.3 pytest-embedded Setup

| 1 | Install pytest and pytest-embedded BASH. pip install pytest pytest-embedded pytest-embedded-serial-esp. # For STM32 targets (uses OpenOCD):. pip install pytest-embedded # base. # You'll also need openocd installed: sudo apt install openocd. |
| --- | --- |

| 2 | Create conftest.py — the firmware flash fixture PYTHON. # conftest.py — runs once per test session. import pytest. from pytest_embedded import Dut. @pytest.fixture(scope='session'). def flash_firmware(request):. """Flash latest build to target before any tests run.""". import subprocess. result = subprocess.run(. ['west', 'flash', '--runner', 'esp32s3', '--bin-file',. 'build/zephyr/zephyr_signed.bin'],. capture_output=True, text=True. ). if result.returncode != 0:. pytest.fail(f'Flash failed:\n{result.stderr}'). @pytest.fixture. def dut(flash_firmware):. """Return a connected DUT after firmware is flashed.""". with Dut(port='/dev/ttyUSB0', baudrate=115200) as dut:. yield dut. |
| --- | --- |

| 3 | Write pytest test cases that drive Unity tests on target PYTHON. # test_dose_calculator.py. import pytest. import re. class TestDoseCalculator:. def test_normal_range(self, dut):. """IEC 62304 Traceability: TEST-UNIT-007 -> SW-REQ-007""". dut.write(b'run_test dose_normal\n'). output = dut.read_until(b'UNITY_END', timeout=10.0).decode(). assert '0 Failures' in output, f'Test failed:\n{output}'. def test_sensor_underrange_safe_state(self, dut):. """IEC 62304 Traceability: TEST-UNIT-008 -> SW-REQ-011""". dut.write(b'run_test dose_underrange\n'). output = dut.read_until(b'UNITY_END', timeout=10.0).decode(). assert '0 Failures' in output. def test_accumulator_limit(self, dut):. """IEC 62304 Traceability: TEST-UNIT-009 -> SW-REQ-014""". dut.write(b'run_test dose_limit\n'). output = dut.read_until(b'UNITY_END', timeout=10.0).decode(). assert '0 Failures' in output. def test_wcet_budget(self, dut):. """WCET budget check: dose_calculator_update must complete < 200us""". dut.write(b'run_test dose_wcet\n'). output = dut.read_until(b'UNITY_END', timeout=15.0).decode(). # Parse WCET from output: 'WCET: 145 us'. m = re.search(r'WCET:\s*(\d+)\s*us', output). assert m, f'No WCET output found:\n{output}'. wcet_us = int(m.group(1)). assert wcet_us < 200, f'WCET {wcet_us}us exceeds 200us budget'. |
| --- | --- |

| 4 | Run tests and generate JUnit XML report for CI BASH. # Run all firmware tests, generate JUnit XML (importable by Jenkins/GitLab CI). pytest tests/ \. --junitxml=results/firmware_test_results.xml \. --html=results/firmware_test_report.html \. -v 2>&1 ¦ tee results/test_run.log. # In GitHub Actions:. # - uses: actions/upload-artifact@v3. # with:. # name: firmware-test-results. # path: results/firmware_test_results.xml. |
| --- | --- |

## 6.4 IEC 62304 Traceability Matrix for Tests

Each pytest test function must map to an IEC 62304 software requirement. This creates bidirectional traceability required by the standard. Maintain this in a living document updated as tests are added.

TEXT

UNIT TEST TRACEABILITY MATRIX v1.0 ¦ 2026-04-14

Device: ClO2 Controller ¦ Author: R. Roychowdhury

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

Test ID pytest Function SW-REQ Result Date

──────────────────────────────────────────────────────────────────────

TEST-U-007 test_dose_calc_normal_range SW-REQ-007 PASS 2026-04-14

TEST-U-008 test_sensor_underrange_safe SW-REQ-011 PASS 2026-04-14

TEST-U-009 test_accumulator_limit SW-REQ-014 PASS 2026-04-14

TEST-U-010 test_wcet_budget_200us SW-REQ-022 PASS 2026-04-14

TEST-U-011 test_pid_antiwindup SW-REQ-009 PASS 2026-04-14

TEST-U-012 test_safety_monitor_500ms SW-REQ-019 PASS 2026-04-14

TEST-U-013 test_stuck_sensor_detect SW-REQ-021 PASS 2026-04-14

TEST-U-014 test_dose_crc_verification SW-REQ-025 PASS 2026-04-14

──────────────────────────────────────────────────────────────────────

Summary: 8/8 tests PASS ¦ 0 FAIL ¦ 0 SKIPPED

Test run artifact: firmware_test_results.xml (CI build #147)

## 6.5 Week 6–7 Exercises

- Write 5 Unity test cases in C for one of your embedded firmware modules. Compile them into the firmware and verify they run on target hardware with correct PASS/FAIL output over UART.

- Set up pytest-embedded (or a custom pytest conftest.py with serial communication) on your development machine. Write 3 pytest test functions that flash, run, and validate the Unity test output automatically.

- Add a GitHub Actions workflow (or Makefile target) that runs pytest tests as part of CI. Commit a test failure intentionally to verify the CI gate rejects the commit.

- Create a Test Traceability Matrix (use template in 6.4) mapping each pytest test to an IEC 62304 software requirement. Minimum 8 rows.

| GOAL | DAYS 43–53 DELIVERABLE: pytest-embedded demo running on hardware + JUnit XML test report + Test Traceability Matrix with 8+ entries. |
| --- | --- |

| 07 | AUTOSAR Architecture Concepts + CAN-FD Days 54–60 · ~8 hours |
| --- | --- |

# Skill 7: AUTOSAR Architecture and CAN-FD

## 7.1 AUTOSAR for Non-Automotive Engineers

AUTOSAR (AUTomotive Open System ARchitecture) is a standardized software architecture originally developed for automotive ECUs. While you will not implement a full AUTOSAR stack in most medical device companies, understanding AUTOSAR architectural vocabulary is a strong signal of principal-level systems thinking. Many medtech companies, especially those making capital equipment (infusion pumps, surgical robots, imaging systems), use AUTOSAR-inspired layering.

### AUTOSAR Classic Platform Layers

TEXT

Application Layer

└─ SWC (Software Components): Your application logic, sensor fusion,

control algorithms. SWCs communicate ONLY through the RTE.

RTE — Runtime Environment

└─ Generated middleware: routes data between SWCs, abstracts BSW.

Equivalent to a statically-configured publish-subscribe bus.

You never access hardware directly from a SWC — always via RTE.

Basic Software (BSW)

├─ Service Layer: OS (OSEK/VDX), NVM, Diagnostics (UDS/OBD), ComStack

├─ ECU Abstraction Layer: Abstract interface to peripherals

│ (ADC, PWM, SPI, I2C, CAN) independent of MCU silicon

└─ MCAL — Microcontroller Abstraction Layer

└─ Vendor-specific drivers: STM32 HAL, ESP-IDF drivers, NXP SDK

This is the ONLY layer that knows the MCU part number.

### Why This Matters for Medical Device Firmware

The AUTOSAR layering philosophy — separating application logic from BSW, and BSW from silicon — directly improves IEC 62304 compliance. When your application SWCs have no direct hardware access, you can test them on a host PC without target hardware (Model-in-the-Loop, PIL testing). The MCAL layer can be swapped when you change MCU vendors without touching application code. This reduces scope of change control and re-verification effort.

C

/* ── AUTOSAR-inspired layering in a medical device ──────────────────*/

/* BAD: Application code with direct hardware access (not AUTOSAR) */

void dose_calculator_update(void) {

uint16_t raw = ADC1->DR; /* direct register — MCAL only! */

TIM3->CCR1 = compute_pwm(raw); /* direct register — MCAL only! */

}

/* GOOD: AUTOSAR-inspired — application uses abstraction layer only */

void dose_calculator_update(void) {

uint16_t raw = Adc_ReadChannel(ADC_SENSOR_CHANNEL); /* ECU Abstr. */

uint16_t pwm = compute_pwm_duty(raw);

Pwm_SetDutyCycle(PWM_ACTUATOR_CHANNEL, pwm); /* ECU Abstr. */

}

/* Now dose_calculator_update() is testable on a PC by providing

mock implementations of Adc_ReadChannel() and Pwm_SetDutyCycle(). */

## 7.2 CAN / CAN-FD for Medical Capital Equipment

Controller Area Network (CAN) is the dominant communication protocol for capital medical equipment — surgical robots, infusion pump arrays, imaging system subsystems, patient transport systems. CAN-FD (Flexible Data Rate) extends CAN to 8 Mbps data rate and 64-byte payloads. Understanding CAN is required to work on Class III devices and capital equipment at companies like Boston Scientific, Medtronic, and Insulet.

### CAN Frame Format

TEXT

CAN 2.0B Extended Frame (29-bit ID):

SOF │ Identifier (29 bits) │ RTR │ IDE │ r0 │ DLC │ Data (0-8 B) │ CRC │ ACK │ EOF

1 │ 29 │ 1 │ 1 │ 1 │ 4 │ 0–64 bytes │ 15 │ 2 │ 7

CAN-FD Frame (ISO 11898-1:2015):

├─ Arbitration phase: up to 1 Mbps (same as classic CAN)

└─ Data phase: up to 8 Mbps (with BRS bit set)

└─ Payload: up to 64 bytes (vs 8 bytes in classic CAN)

Key protocol properties:

Priority: Lower arbitration ID = higher priority (wired-AND bus)

Error detection: CRC-15, bit stuffing, ACK check, form check

Error confinement: TEC/REC counters → Error Active → Error Passive → Bus-Off

### STM32 FDCAN Configuration

C

/* ── FDCAN init for CAN-FD on STM32G4/H7 ────────────────────────────*/

FDCAN_HandleTypeDef hfdcan1;

HAL_StatusTypeDef fdcan_init(void) {

hfdcan1.Instance = FDCAN1;

hfdcan1.Init.ClockDivider = FDCAN_CLOCK_DIV1;

hfdcan1.Init.FrameFormat = FDCAN_FRAME_FD_BRS; /* CAN-FD + BRS */

hfdcan1.Init.Mode = FDCAN_MODE_NORMAL;

/* Arbitration: 1 Mbps at 160MHz: prescaler=2, Tseg1=59, Tseg2=20 */

hfdcan1.Init.NominalPrescaler = 2U;

hfdcan1.Init.NominalSyncJumpWidth = 16U;

hfdcan1.Init.NominalTimeSeg1 = 59U;

hfdcan1.Init.NominalTimeSeg2 = 20U;

/* Data phase: 5 Mbps: prescaler=2, Tseg1=11, Tseg2=4 */

hfdcan1.Init.DataPrescaler = 2U;

hfdcan1.Init.DataSyncJumpWidth = 4U;

hfdcan1.Init.DataTimeSeg1 = 11U;

hfdcan1.Init.DataTimeSeg2 = 4U;

hfdcan1.Init.StdFiltersNbr = 8U;

hfdcan1.Init.ExtFiltersNbr = 4U;

hfdcan1.Init.TxFifoQueueMode = FDCAN_TX_FIFO_OPERATION;

return HAL_FDCAN_Init(&hfdcan1);

}

/* ── Send a CAN-FD frame (64-byte payload at 5 Mbps data rate) ───────*/

HAL_StatusTypeDef fdcan_send_sensor_data(const sensor_data_t *data) {

FDCAN_TxHeaderTypeDef hdr = {

.Identifier = 0x101U, /* sensor data CAN ID */

.IdType = FDCAN_EXTENDED_ID,

.TxFrameType = FDCAN_DATA_FRAME,

.DataLength = FDCAN_DLC_BYTES_64,/* CAN-FD max: 64 bytes */

.ErrorStateIndicator = FDCAN_ESI_ACTIVE,

.BitRateSwitch = FDCAN_BRS_ON, /* switch to fast phase */

.FDFormat = FDCAN_FD_CAN,

.TxEventFifoControl = FDCAN_NO_TX_EVENTS,

.MessageMarker = 0U,

};

return HAL_FDCAN_AddMessageToTxFifoQ(&hfdcan1, &hdr, (uint8_t *)data);

}

### CANopen — The Medical Equipment Application Layer

CANopen is an application layer protocol built on top of CAN, widely used in medical capital equipment. Key concepts:

| Item | Detail |
| --- | --- |
| Object Dictionary | A standardized table of parameters, each with a 16-bit index and 8-bit subindex. Every CANopen device has one. Defines all configuration parameters, process data, and diagnostic information. |
| PDO (Process Data Object) | Real-time data exchange: sensor values, setpoints, status. Transmitted cyclically or event-driven. Up to 8 bytes per PDO in CAN 2.0, up to 64 bytes in CANopen-FD. |
| SDO (Service Data Object) | Configuration and diagnostics: read/write any entry in the Object Dictionary over the bus. Used during commissioning, parameterization, and fault investigation. |
| NMT (Network Management) | Controls node state machine: Initializing → Pre-Operational → Operational → Stopped. NMT master controls all nodes on the network. |
| EMCY (Emergency Object) | Device reports fault condition via 8-byte emergency message: error code + device error register + manufacturer-specific data. |

## 7.3 Week 7–8 Exercises

- Draw a 1-page AUTOSAR-inspired layered architecture diagram for your smart shoe project: identify your MCAL (ESP-IDF IMU driver, BLE driver), ECU Abstraction Layer (sensor_read(), ble_notify()), and Application SWCs (gait_analysis, stride_detection). Identify which SWCs can be unit tested on a host PC using mocks.

- Build a CAN-FD loopback demo on an STM32G4 Nucleo or similar board: send a 64-byte frame to yourself using internal loopback mode, receive it, verify the data is intact. Document timing measurements with DWT (arbitration phase + data phase duration).

- Write a 1-page CANopen Device Profile document for a hypothetical medical sensor: define 5 Object Dictionary entries (identity object, sensor value PDO, alarm status PDO, configuration SDO, EMCY mapping). Use the standard CiA 301 index assignments.

| GOAL | DAYS 54–60 DELIVERABLE: AUTOSAR layering diagram for a real project + CAN-FD loopback demo with timing measurements + CANopen OD document. |
| --- | --- |