QEMU Emulation for Embedded Development

ESP32 Families  •  STM32 Families  •  WSL2

Complete Installation & Tutorial Guide

Roy Roychowdhury — Senior Embedded Firmware Engineer

MADEPLUS INC  |  Glen Burnie, Maryland  |  March 2026

# 1. Overview & Architecture

QEMU (Quick EMUlator) is an open-source machine emulator and virtualizer that lets you run firmware binaries targeting ARM Cortex-M (STM32) and Xtensa/RISC-V (ESP32) processors entirely on your WSL2 Linux environment — without physical hardware. This guide covers installation, machine models, toolchain integration, and practical workflow for both MCU families.

📌 NOTE WSL2 runs a real Linux kernel, so native QEMU for ARM/Xtensa works without emulation overhead for the host CPU. You get genuine syscall-level isolation and KVM is NOT required for MCU emulation (we target bare-metal, not Linux guests).

## 1.1  How QEMU Relates to Embedded Development

In embedded development, QEMU fills a specific niche:

- Unit-test firmware logic (drivers, FSMs, ring buffers) without flashing hardware

- CI/CD pipelines — run regression tests on every git push

- Debug with GDB over a virtual serial connection

- Prototype BSP bringup before silicon arrives

- Run Zephyr RTOS, FreeRTOS, or bare-metal apps in a controlled sandbox

## 1.2  QEMU Machine Support Matrix

The table below summarises QEMU's current official and community-supported machine models relevant to your platform stack:

| MCU / Family | QEMU Binary | Machine Flag | Support Level | Key Peripherals |
| --- | --- | --- | --- | --- |
| ESP32 (Xtensa LX6) | qemu-xtensa | esp32 | Partial — community fork | CPU, UART, Timer |
| ESP32-S2 (Xtensa LX7) | qemu-xtensa | esp32s2 | Partial — community fork | CPU, UART, USB CDC |
| ESP32-S3 (Xtensa LX7) | qemu-xtensa | esp32s3 | Community / WIP | CPU, UART, USB CDC |
| ESP32-C3 (RISC-V) | qemu-riscv32 | esp32c3 | Partial — community fork | CPU, UART, Timer |
| STM32F103 (Cortex-M3) | qemu-arm (system) | netduino2 | Official upstream | CPU, UART, SysTick, GPIO |
| STM32F407 (Cortex-M4) | qemu-arm (system) | stm32f4-discovery | xPack / community | CPU, FPU, UART, SysTick |
| STM32F446 (Cortex-M4F) | qemu-arm (system) | custom/GDB stub | STM32 xPack | CPU, FPU, GDB remote |
| STM32H743 (Cortex-M7) | qemu-arm (system) | netduinoplus2 / custom | Limited | CPU core only |
| STM32WB (Cortex-M4+M0) | qemu-arm | Partial BLE stub | Community WIP | CPU, limited radio stub |

🔴 IMPORTANT ESP32 official QEMU support lives in Espressif's fork: https://github.com/espressif/qemu. For STM32, xPack QEMU is the gold standard for Cortex-M emulation with peripheral support.

# 2. WSL2 Prerequisites & System Check

Before installing QEMU, ensure your WSL2 environment meets the following requirements.

## 2.1  Verify WSL2 Version

# PowerShell — Check WSL2 version

# In PowerShell (Windows side)

wsl --version

# Expected output example:

# WSL version: 2.1.5.0

# Kernel version: 5.15.146.1

# WSLg version: 1.0.65

⚠️ WARNING If you see 'WSL 1' as the default, upgrade: wsl --set-default-version 2 (run in PowerShell as Administrator). Then convert your Ubuntu distro: wsl --set-version Ubuntu 2

## 2.2  Check Ubuntu Version Inside WSL

# WSL2 shell

# Inside WSL2 terminal

lsb_release -a

uname -r

# Recommended: Ubuntu 22.04 LTS or 24.04 LTS

## 2.3  Update System Packages

# WSL2 shell — System update

sudo apt update && sudo apt upgrade -y

sudo apt install -y build-essential git curl wget python3-pip

sudo apt install -y libglib2.0-dev libfdt-dev libpixman-1-dev zlib1g-dev

sudo apt install -y ninja-build pkg-config libssl-dev

## 2.4  Install ARM & RISC-V Toolchains

You will need cross-compilers to build firmware before emulating it.

# WSL2 shell — Toolchain install

# ARM Cortex-M (for STM32)

sudo apt install -y gcc-arm-none-eabi binutils-arm-none-eabi

arm-none-eabi-gcc --version   # Verify

# RISC-V (for ESP32-C3 / C6 in QEMU)

sudo apt install -y gcc-riscv64-unknown-elf

riscv64-unknown-elf-gcc --version

# Xtensa (for ESP32 / S2 / S3) — via ESP-IDF

# See Section 4 for ESP-IDF full setup

# 3. QEMU for STM32 Families

STM32 emulation on QEMU uses the ARM System emulation mode (qemu-system-arm). We cover three paths: upstream QEMU (limited peripherals), xPack QEMU (recommended for STM32), and building from source.

## 3.1  Install xPack QEMU ARM (Recommended for STM32)

xPack QEMU is a distribution of QEMU specifically tuned for Cortex-M emulation, packaged by Liviu Ionescu. It provides stm32f4-discovery, stm32f0discovery, nucleo boards, and more.

### 3.1.1  Install via xpm (Node.js package manager)

# WSL2 shell — xPack QEMU install

# Install Node.js first (if not present)

curl -fsSL https://deb.nodesource.com/setup_20.x | sudo -E bash -

sudo apt install -y nodejs

node --version   # Should be v18 or v20

# Install xpm globally

sudo npm install -g xpm@latest

xpm --version

# Install xPack QEMU ARM

xpm install --global @xpack-dev-tools/qemu-arm@latest

# Add to PATH (append to ~/.bashrc)

echo 'export PATH="$HOME/.local/xPacks/@xpack-dev-tools/qemu-arm/$(ls ~/.local/xPacks/@xpack-dev-tools/qemu-arm/)/bin:$PATH"' >> ~/.bashrc

source ~/.bashrc

# Verify

qemu-system-arm --version

qemu-system-arm -machine help | grep -i stm

### 3.1.2  Alternative: Download Pre-built Binary Directly

# WSL2 shell — Direct binary install

# Check latest release at: https://github.com/xpack-dev-tools/qemu-arm-xpack/releases

QEMU_VER="7.2.0-1"

wget https://github.com/xpack-dev-tools/qemu-arm-xpack/releases/download/v${QEMU_VER}/xpack-qemu-arm-${QEMU_VER}-linux-x64.tar.gz

tar -xzf xpack-qemu-arm-${QEMU_VER}-linux-x64.tar.gz -C $HOME/.local/

mv $HOME/.local/xpack-qemu-arm-${QEMU_VER} $HOME/.local/qemu-arm

echo 'export PATH="$HOME/.local/qemu-arm/bin:$PATH"' >> ~/.bashrc

source ~/.bashrc

qemu-system-arm --version

## 3.2  Install Upstream QEMU from Apt (Simpler, Fewer Peripherals)

# WSL2 shell — Apt install

sudo apt install -y qemu-system-arm

qemu-system-arm --version

qemu-system-arm -machine help | grep -E 'stm|nucleo|lm3s'

# Available upstream machines (sample):

# netduino2         STM32 / AT91SAM7X based

# stm32vldiscovery  STM32F100 VL Discovery

## 3.3  Build QEMU STM32 from Source (Maximum Peripheral Support)

For advanced STM32 peripheral emulation (ADC, SPI, I2C, DMA), build the community STM32 fork:

# WSL2 shell — Build from source

# Clone Beckus STM32 QEMU fork (widely used for STM32 research)

git clone https://github.com/beckus/qemu_stm32.git

cd qemu_stm32

# Install additional build dependencies

sudo apt install -y libsdl2-dev libcurl4-openssl-dev

# Configure for ARM only (faster build)

./configure --target-list=arm-softmmu --disable-kvm --enable-debug

# Build (use -j$(nproc) to parallelize)

make -j$(nproc)

# Test

./arm-softmmu/qemu-system-arm -machine stm32-p103 -nographic -kernel blink.elf

✅ TIP The Beckus fork supports STM32F103 (Blue Pill, Maple) very well. For STM32F4 (your STM32F446RE), xPack is more maintained and reliable as of 2026.

## 3.4  Running STM32 Firmware with QEMU

### 3.4.1  Minimal Blink / Hello-World (UART)

Compile a minimal STM32F407 firmware, then launch under QEMU. The UART output redirects to your WSL terminal.

# WSL2 shell — STM32F407 run

# Assuming you have a compiled .elf from STM32CubeIDE or Makefile project

# Machine: stm32f4-discovery (STM32F407 @ 168 MHz)

qemu-system-arm \

-machine stm32f4-discovery \

-cpu cortex-m4 \

-nographic \

-semihosting-config enable=on,target=native \

-kernel build/my_firmware.elf

# Exit QEMU: Ctrl+A, then X

# WSL2 shell — STM32F103 UART

# STM32F103 (Blue Pill / Nucleo-F103RB) example

qemu-system-arm \

-machine stm32vldiscovery \

-cpu cortex-m3 \

-nographic \

-kernel build/f103_blink.elf

# With UART redirect to PTY (for minicom/picocom)

qemu-system-arm \

-machine stm32vldiscovery \

-serial pty \

-kernel build/f103_uart.elf &

# Connect with picocom to the PTY that QEMU reports (e.g., /dev/pts/3)

picocom -b 115200 /dev/pts/3

### 3.4.2  GDB Debugging Inside WSL

# WSL2 shell — GDB Debug session

# Terminal 1 — Launch QEMU with GDB server on port 1234

qemu-system-arm \

-machine stm32f4-discovery \

-cpu cortex-m4 \

-nographic \

-S -gdb tcp::1234 \

-kernel build/my_firmware.elf

# Terminal 2 — Launch GDB and connect

arm-none-eabi-gdb build/my_firmware.elf

# Inside GDB:

(gdb) target remote localhost:1234

(gdb) monitor reset halt

(gdb) load

(gdb) break main

(gdb) continue

(gdb) info registers

(gdb) x/10x 0x20000000   # Inspect SRAM

✅ TIP Add 'set architecture armv7-m' in your .gdbinit to prevent GDB from guessing the wrong ISA. For Cortex-M7 (STM32H7) use armv7e-m or armv8-m.

## 3.5  STM32 Family Quick-Reference

| Family | Core | Best QEMU Machine | QEMU Support | Peripheral Coverage |
| --- | --- | --- | --- | --- |
| STM32F0xx | Cortex-M0 | stm32f0discovery | xPack / upstream | Basic UART, Timer |
| STM32F103 | Cortex-M3 | stm32vldiscovery / netduino2 | Upstream + Beckus fork | UART, GPIO, SysTick, ADC |
| STM32F407/429 | Cortex-M4F | stm32f4-discovery | xPack (best coverage) | FPU, UART, SysTick, DMA stub |
| STM32F446RE | Cortex-M4F | stm32f4-discovery (close match) | xPack | FPU, UART, SysTick |
| STM32G0xx | Cortex-M0+ | Generic armv6m | Limited | CPU core, GDB stub |
| STM32H743/H750 | Cortex-M7 | netduinoplus2 / generic | Limited | CPU core only |
| STM32WBxx | Cortex-M4+M0+ | Partial custom | Community WIP | CPU, UART stub |
| STM32U5xx | Cortex-M33 | Generic armv8m | Very limited | CPU core, Trustzone stub |

# 4. QEMU for ESP32 Families

ESP32 emulation requires Espressif's official QEMU fork, which adds Xtensa LX6/LX7 CPU cores and ESP-specific peripheral stubs. ESP32-C3 (RISC-V) can use the upstream RISC-V QEMU target.

## 4.1  Install Espressif's QEMU Fork

### 4.1.1  Via ESP-IDF (Recommended — All-in-One)

ESP-IDF 5.x ships QEMU as a managed component. This is the easiest path for ESP32 / ESP32-S2 / ESP32-S3 emulation.

# WSL2 shell — ESP-IDF + QEMU install

# Install prerequisites

sudo apt install -y git wget flex bison gperf python3 python3-pip cmake

sudo apt install -y ninja-build ccache libffi-dev libssl-dev dfu-util

# Clone ESP-IDF v5.x (stable)

mkdir -p ~/esp && cd ~/esp

git clone --recursive https://github.com/espressif/esp-idf.git

cd esp-idf

git checkout v5.2.1  # Or latest stable

# Run the install script — downloads toolchain + QEMU

./install.sh esp32,esp32s3,esp32c3

# Source the environment (add to ~/.bashrc for persistence)

. ~/esp/esp-idf/export.sh

# Verify QEMU is available

which qemu-system-xtensa

qemu-system-xtensa --version

📌 NOTE The ESP-IDF install script places QEMU binaries under ~/.espressif/tools/qemu-xtensa/. The export.sh script adds this to PATH automatically.

### 4.1.2  Build Espressif QEMU from Source

For latest features or ESP32-S3 support, build the fork directly:

# WSL2 shell — Build ESP QEMU from source

# Clone Espressif QEMU fork

cd ~/esp

git clone https://github.com/espressif/qemu.git qemu-esp

cd qemu-esp

git checkout esp-develop   # Active development branch

# Install build deps

sudo apt install -y libglib2.0-dev libpixman-1-dev libfdt-dev

sudo apt install -y libslirp-dev python3-sphinx

# Configure — build only Xtensa and RISC-V targets

./configure \

--target-list="xtensa-softmmu,riscv32-softmmu" \

--enable-slirp \

--disable-sdl \

--disable-gtk \

--prefix=$HOME/.local/qemu-esp

make -j$(nproc)

make install

echo 'export PATH="$HOME/.local/qemu-esp/bin:$PATH"' >> ~/.bashrc

source ~/.bashrc

qemu-system-xtensa --version

## 4.2  Running ESP32 Firmware

### 4.2.1  ESP-IDF idf.py qemu (One-Command Launch)

When using ESP-IDF, you can launch QEMU directly from the project directory with idf.py:

# WSL2 shell — ESP-IDF QEMU workflow

cd ~/esp/esp-idf/examples/get-started/hello_world

# Set target chip

idf.py set-target esp32

# Build

idf.py build

# Launch in QEMU (replaces 'idf.py flash')

idf.py qemu monitor

# For ESP32-S3

idf.py set-target esp32s3

idf.py build

idf.py qemu monitor

# Exit: Ctrl+]

### 4.2.2  Manual QEMU Launch (ESP32)

If you want full control over QEMU flags:

# WSL2 shell — Manual ESP32 QEMU

# ESP32 — Run merged firmware binary

# First, merge partitions into single binary:

esptool.py --chip esp32 merge_bin \

-o merged_flash.bin \

--flash_mode dio \

--flash_size 4MB \

0x1000 bootloader/bootloader.bin \

0x8000 partition_table/partition-table.bin \

0x10000 hello_world.bin

# Launch QEMU with merged binary

qemu-system-xtensa \

-nographic \

-machine esp32 \

-drive file=merged_flash.bin,if=mtd,format=raw \

-serial mon:stdio

# Alternatively: run .elf directly (no bootloader)

qemu-system-xtensa \

-nographic \

-machine esp32 \

-kernel build/hello_world.elf

### 4.2.3  ESP32-C3 (RISC-V) with QEMU

# WSL2 shell — ESP32-C3 RISC-V

# ESP32-C3 uses RISC-V (rv32imc) — use riscv32 QEMU

# ESP-IDF handles this automatically via idf.py qemu

idf.py set-target esp32c3

idf.py build

idf.py qemu monitor

# Manual RISC-V QEMU invocation:

qemu-system-riscv32 \

-nographic \

-machine esp32c3 \

-drive file=merged_flash.bin,if=mtd,format=raw

## 4.3  GDB Debugging for ESP32

# WSL2 shell — ESP32 GDB

# Terminal 1 — Launch QEMU with GDB stub

idf.py qemu --gdb

# OR manual:

qemu-system-xtensa \

-machine esp32 \

-nographic \

-gdb tcp::1234 -S \

-kernel build/hello_world.elf

# Terminal 2 — Connect xtensa GDB

xtensa-esp32-elf-gdb build/hello_world.elf

(gdb) target remote :1234

(gdb) break app_main

(gdb) continue

(gdb) info registers

# For ESP32-C3 use RISC-V gdb:

riscv32-esp-elf-gdb build/hello_world.elf

## 4.4  ESP32 Family Quick-Reference

| Chip | CPU Core | QEMU Machine | QEMU Fork | Peripheral Support |
| --- | --- | --- | --- | --- |
| ESP32 (D0WDQ6) | Xtensa LX6 dual-core | esp32 | Espressif fork | UART, Timer, RTC, WiFi stub |
| ESP32-S2 | Xtensa LX7 single-core | esp32s2 | Espressif fork | UART, USB CDC, Timer |
| ESP32-S3 | Xtensa LX7 dual-core | esp32s3 | Espressif fork (WIP) | UART, USB OTG stub, Timer |
| ESP32-C3 | RISC-V rv32imc | esp32c3 | Espressif + upstream | UART, Timer, BLE stub |
| ESP32-C6 | RISC-V rv32imac | esp32c6 | Espressif fork (WIP) | UART, WiFi6/BLE stub |
| ESP32-H2 | RISC-V rv32imac | esp32h2 | Espressif fork (WIP) | UART, IEEE 802.15.4 stub |
| ESP8266 | Xtensa L106 | esp8266 (limited) | Community | UART only, very limited |

# 5. Running RTOS Firmware on QEMU

## 5.1  Zephyr RTOS on STM32 (QEMU)

Zephyr has first-class QEMU support via its 'qemu_cortex_m3' and 'mps2_an385' boards. This is ideal for your Zephyr gait analysis work.

# WSL2 shell — Zephyr on QEMU

# Install Zephyr SDK + west (if not already done)

pip3 install west --break-system-packages

west init ~/zephyrproject

cd ~/zephyrproject

west update

west zephyr-export

# Install Zephyr SDK

cd ~

wget https://github.com/zephyrproject-rtos/sdk-ng/releases/download/v0.16.8/zephyr-sdk-0.16.8_linux-x86_64.tar.xz

tar xvf zephyr-sdk-0.16.8_linux-x86_64.tar.xz

cd zephyr-sdk-0.16.8

./setup.sh -t arm-zephyr-eabi

# Build hello_world for QEMU Cortex-M3

cd ~/zephyrproject/zephyr

west build -b qemu_cortex_m3 samples/hello_world

# Run in QEMU (Zephyr handles this automatically)

west build -t run

# Output visible in terminal — no hardware needed!

✅ TIP For your smart shoe gait analysis system, you can test all BLE GATT logic and sensor fusion algorithms using the 'qemu_cortex_m3' board — just mock the IMU data via a UART feed. This is perfect for CI regression testing.

## 5.2  FreeRTOS on STM32 QEMU

# WSL2 shell — FreeRTOS on STM32

# FreeRTOS demo for STM32F407 (semihosting output)

# Assumes FreeRTOS project compiled with arm-none-eabi-gcc

qemu-system-arm \

-machine stm32f4-discovery \

-cpu cortex-m4 \

-nographic \

-semihosting-config enable=on,target=native \

-kernel FreeRTOS_Demo.elf

# For UART output instead of semihosting:

qemu-system-arm \

-machine stm32f4-discovery \

-serial stdio \

-kernel FreeRTOS_Demo_UART.elf

## 5.3  ESP-IDF FreeRTOS on QEMU

ESP-IDF applications run on FreeRTOS by default. idf.py qemu handles everything:

# WSL2 shell — ESP-IDF FreeRTOS QEMU

# Any ESP-IDF project with FreeRTOS tasks runs transparently

idf.py set-target esp32

idf.py build

idf.py qemu monitor

# You will see FreeRTOS scheduler start, tasks printing via UART

# BLE and WiFi stubs return errors (no real radio) — handle gracefully

# 6. Development Workflow & VS Code Integration

## 6.1  Recommended Directory Layout

# Directory structure

~/embedded/

├── esp/

│   ├── esp-idf/          # ESP-IDF framework

│   └── projects/         # Your ESP32 projects

├── stm32/

│   ├── cube/             # STM32CubeIDE workspace

│   └── qemu/             # QEMU test scripts

├── zephyr/               # Zephyr workspace

└── scripts/

├── run_esp32.sh      # QEMU launch scripts

└── run_stm32.sh

## 6.2  Shell Launch Scripts

### run_stm32.sh — STM32 QEMU Launcher

# ~/scripts/run_stm32.sh

#!/bin/bash

# Usage: ./run_stm32.sh <elf_file> [gdb]

ELF=${1:?"Usage: $0 <elf_file> [gdb]"}

GDB_FLAG=${2:-""}

MACHINE="stm32f4-discovery"

CPU="cortex-m4"

GDB_ARGS=""

if [ "$GDB_FLAG" == "gdb" ]; then

GDB_ARGS="-S -gdb tcp::1234"

echo "[QEMU] GDB server waiting on port 1234..."

fi

qemu-system-arm \

-machine $MACHINE \

-cpu $CPU \

-nographic \

-semihosting-config enable=on,target=native \

$GDB_ARGS \

-kernel "$ELF"

# ~/scripts/run_esp32.sh

#!/bin/bash

# Usage: ./run_esp32.sh <merged_flash.bin> [gdb]

FLASH=${1:?"Usage: $0 <flash.bin> [gdb]"}

GDB_FLAG=${2:-""}

GDB_ARGS=""

if [ "$GDB_FLAG" == "gdb" ]; then

GDB_ARGS="-gdb tcp::1234 -S"

echo "[QEMU] GDB waiting on port 1234"

fi

qemu-system-xtensa \

-nographic \

-machine esp32 \

-drive file="$FLASH",if=mtd,format=raw \

-serial mon:stdio \

$GDB_ARGS

## 6.3  VS Code Integration (WSL2)

Install these VS Code extensions for the best QEMU workflow inside WSL:

- Remote - WSL (ms-vscode-remote.remote-wsl)

- Cortex-Debug (marus25.cortex-debug) — wraps arm-none-eabi-gdb with QEMU

- C/C++ Extension Pack (ms-vscode.cpptools-extension-pack)

- ESP-IDF (espressif.esp-idf-extension) — includes native QEMU launch UI

### launch.json for STM32 + QEMU + Cortex-Debug

# .vscode/launch.json

{

"version": "0.2.0",

"configurations": [

{

"name": "QEMU STM32F4 Debug",

"type": "cortex-debug",

"request": "launch",

"servertype": "qemu",

"cpu": "cortex-m4",

"machine": "stm32f4-discovery",

"executable": "${workspaceFolder}/build/my_firmware.elf",

"runToEntryPoint": "main",

"showDevDebugOutput": "raw"

}

]

}

# 7. Troubleshooting & Common Issues

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| qemu-system-arm: command not found | xPack/apt QEMU not in PATH | source ~/.bashrc or add export PATH to ~/.bashrc; check which qemu-system-arm |
| Error: Unknown machine type 'stm32f4-discovery' | Using upstream apt QEMU (older) | Install xPack QEMU which supports this machine; run qemu-system-arm -machine help |
| Firmware loads but hangs at startup | Missing SysTick or clock config | Ensure firmware configures SystemClock before using peripherals; add -semihosting flag |
| GDB: Remote communication error | QEMU not started with -S -gdb | Verify QEMU started with -S -gdb tcp::1234; check port not in use: ss -tlnp ¦ grep 1234 |
| ESP32 QEMU: Flash image errors | Incorrect binary merging | Re-run esptool merge_bin; verify offsets match your partition table |
| idf.py qemu: QEMU not found | ESP-IDF export.sh not sourced | Run: . ~/esp/esp-idf/export.sh; add to ~/.bashrc |
| WSL2: /dev/kvm not available | KVM not needed for MCU emulation | MCU emulation (Cortex-M / Xtensa) does not use KVM — this error is harmless, ignore it |
| Zephyr west build fails for qemu_cortex_m3 | Zephyr SDK arm target not installed | Re-run: cd zephyr-sdk-0.16.8 && ./setup.sh -t arm-zephyr-eabi |
| UART output garbled in QEMU | Baud rate mismatch or wrong -serial flag | Use -serial stdio for direct output; verify firmware configures 115200 8N1 |
| Build error: libglib not found | Missing glib dev package | sudo apt install -y libglib2.0-dev |

# 8. Quick-Reference Cheat Sheet

## Most-Used Commands

# QEMU Embedded Cheat Sheet

# ── STM32 (xPack QEMU) ────────────────────────────────────────────

qemu-system-arm -machine stm32f4-discovery -cpu cortex-m4 -nographic \

-semihosting-config enable=on,target=native -kernel firmware.elf

# STM32 + GDB server

qemu-system-arm -machine stm32f4-discovery -cpu cortex-m4 -nographic \

-S -gdb tcp::1234 -kernel firmware.elf

# List available ARM machines

qemu-system-arm -machine help

# ── ESP32 (Espressif QEMU) ────────────────────────────────────────

idf.py qemu monitor                  # ESP-IDF project

idf.py qemu --gdb                    # with GDB server

# Manual ESP32 with merged flash

qemu-system-xtensa -nographic -machine esp32 \

-drive file=merged_flash.bin,if=mtd,format=raw -serial mon:stdio

# ── GDB quick connect ─────────────────────────────────────────────

arm-none-eabi-gdb firmware.elf -ex 'target remote :1234'

xtensa-esp32-elf-gdb firmware.elf -ex 'target remote :1234'

# ── Zephyr ────────────────────────────────────────────────────────

west build -b qemu_cortex_m3 samples/hello_world && west build -t run

# ── Merge ESP32 flash ─────────────────────────────────────────────

esptool.py --chip esp32 merge_bin -o merged.bin --flash_size 4MB \

0x1000 bootloader.bin 0x8000 partitions.bin 0x10000 app.bin

## Support Level Legend

| Support Level | Meaning |
| --- | --- |
| Official upstream | Works out of the box with apt QEMU; long-term maintenance guaranteed |
| xPack | Pre-built binaries by Liviu Ionescu; best STM32 Cortex-M coverage; well maintained |
| Espressif fork | Espressif's official fork; required for ESP32 Xtensa; bundled in ESP-IDF 5.x |
| Community / WIP | Works for basic CPU emulation; peripheral support varies; expect rough edges |
| Limited | CPU core emulation only; useful for unit testing pure logic code; no peripherals |

🔴 IMPORTANT For your smart shoe platform (ESP32-S3 Zero + GY-86 IMU), the recommended path is: ESP-IDF 5.x + idf.py qemu for BLE GATT logic testing, and xPack QEMU + Zephyr qemu_cortex_m3 board for cross-validating sensor fusion algorithms on a well-supported QEMU machine.

# Appendix A — Useful Links & Resources

Espressif QEMU Fork (ESP32/S2/S3/C3): https://github.com/espressif/qemu

xPack QEMU ARM Releases: https://github.com/xpack-dev-tools/qemu-arm-xpack/releases

Zephyr RTOS QEMU Boards Doc: https://docs.zephyrproject.org/latest/boards/qemu/

Beckus STM32 QEMU Fork: https://github.com/beckus/qemu_stm32

ESP-IDF Programming Guide (QEMU): https://docs.espressif.com/projects/esp-idf/en/latest/esp32/api-guides/tools/qemu.html

QEMU Upstream Documentation: https://www.qemu.org/docs/master/

Cortex-Debug VS Code Extension: https://github.com/Marus/cortex-debug

# Appendix B — Environment Variables Summary

# ~/.bashrc additions

# Add all of these to ~/.bashrc

# xPack QEMU (ARM/STM32)

export PATH="$HOME/.local/xPacks/@xpack-dev-tools/qemu-arm/$(ls ~/.local/xPacks/@xpack-dev-tools/qemu-arm/)/bin:$PATH"

# ESP-IDF + Espressif QEMU

. $HOME/esp/esp-idf/export.sh

# Zephyr (if using separate SDK)

export ZEPHYR_SDK_INSTALL_DIR=$HOME/zephyr-sdk-0.16.8

export ZEPHYR_TOOLCHAIN_VARIANT=zephyr

# ARM bare-metal GDB

export PATH="/usr/bin:$PATH"   # arm-none-eabi-gdb usually here after apt install