RUST EMBEDDED DEVELOPMENT

on WSL with ESP32-S3 Zero & STM32F446RE

A Hands-On Firmware Engineer's Guide

| Target Board | Framework / Approach |
| --- | --- |
| ESP32-S3 Zero (Waveshare) | esp-hal (no_std, bare-metal Rust) |
| STM32F446RE (Nucleo-64) | embassy-stm32 / stm32f4xx-hal |
| Host Environment | WSL2 (Ubuntu 22.04 LTS) |
| Rust Toolchain | nightly + target-specific |
| Flashing | espflash / probe-rs / OpenOCD |

# PART 1 — WSL2 ENVIRONMENT SETUP

## 1.1  Prerequisites & WSL2 Installation

All embedded Rust development in this tutorial runs entirely inside WSL2 (Windows Subsystem for Linux 2). USB passthrough from Windows to WSL2 is handled via usbipd-win.

### Step 1 — Enable WSL2 in Windows

# In PowerShell (as Administrator) wsl --install wsl --set-default-version 2 # Reboot, then install Ubuntu 22.04 from the Microsoft Store wsl --install -d Ubuntu-22.04

### Step 2 — Install usbipd-win for USB passthrough

# In PowerShell (as Administrator) — installs usbipd-win 4.x winget install --interactive --exact dorssel.usbipd-win

After installation, plug in your board and enumerate USB devices:

# In PowerShell usbipd list # Example output: # BUSID VID:PID DEVICE STATE # 1-4 303a:1001 USB Serial Device (COM5) Not shared <- ESP32-S3 # 2-1 0483:374b ST-Link Debug + Mass Storage + VCP Not shared <- STM32 # Attach your device (replace BUSID): usbipd bind --busid 1-4 usbipd attach --wsl --busid 1-4

💡 TIP Run 'usbipd attach' each time you reconnect the board. For automation, add it to a PowerShell profile or create a .bat shortcut.

### Step 3 — Verify device inside WSL2

# Inside WSL2 Ubuntu terminal lsusb ls /dev/ttyUSB* /dev/ttyACM* # ESP32 -> ttyUSB0, STM32 -> ttyACM0 # Add yourself to dialout group (one-time) sudo usermod -aG dialout $USER newgrp dialout

## 1.2  Rust Toolchain Installation

### Install rustup

curl --proto '=https' --tlsv1.2 -sSf https://sh.rustup.rs ¦ sh source $HOME/.cargo/env # Verify rustc --version cargo --version

### Add Nightly (required for ESP32-S3)

rustup toolchain install nightly rustup component add rust-src --toolchain nightly

### Add STM32 Cortex-M4 target (stable works)

rustup target add thumbv7em-none-eabihf # For Cortex-M0+ (e.g. STM32G0) use: thumbv6m-none-eabi

## 1.3  Essential System Packages

sudo apt update && sudo apt install -y \ build-essential git pkg-config libssl-dev \ libudev-dev libusb-1.0-0-dev udev \ gdb-multiarch openocd stlink-tools curl

### Install espflash (ESP32 flashing tool)

cargo install espflash cargo-espflash # Verify espflash --version

### Install probe-rs (STM32 flashing + debugging)

cargo install probe-rs-tools --locked # Verify probe-rs list

### Install ldproxy (required by esp-idf-sys)

cargo install ldproxy

# PART 2 — ESP32-S3 ZERO IN RUST

## 2.1  Hardware Overview

The Waveshare ESP32-S3 Zero is a compact (25.4 x 25.4 mm) development board built around the ESP32-S3FH4R2 SoC — Xtensa LX7 dual-core @ 240 MHz, 512 KB SRAM, 4 MB Flash, 2 MB PSRAM, USB-OTG, and integrated 2.4 GHz Wi-Fi + BLE 5.0.

| Feature | Specification |
| --- | --- |
| CPU | Xtensa LX7 Dual-Core @ 240 MHz |
| Flash / PSRAM | 4 MB QSPI Flash + 2 MB OPI PSRAM |
| GPIO | 27 usable (3.3 V tolerant) |
| Connectivity | Wi-Fi 802.11 b/g/n + BLE 5 (LE) |
| USB | USB-C (native USB-OTG) |
| Board Power | 3.3 V (onboard LDO) or USB 5 V |
| Onboard LED | WS2812B RGB on GPIO21 |

## 2.2  Rust Ecosystem for ESP32-S3

Espressif officially supports two Rust approaches for ESP32:

- no_std / bare-metal (esp-hal): Uses esp-hal + embassy-executor, no OS. Best for size and determinism.

- std (esp-idf-svc): Wraps ESP-IDF via FFI. Full std library available, Wi-Fi/BLE easier. Larger binary.

This tutorial uses esp-hal (no_std) — the preferred approach for firmware engineers wanting Rust without a hidden C runtime underneath.

## 2.3  Create an ESP32-S3 Rust Project

### Use the official esp template

cargo install cargo-generate cargo generate esp-rs/esp-idf-template cargo # OR for no_std / esp-hal: cargo generate esp-rs/esp-hal-template # Prompts: # Project Name: my_esp32s3_project # MCU: esp32s3 # Framework: esp-hal (no_std)

📦 TEMPLATE NOTE The esp-hal-template auto-configures .cargo/config.toml, rust-toolchain.toml, and Cargo.toml with the correct Xtensa target and linker.

### Key generated files

my_esp32s3_project/ ├── .cargo/ │ └── config.toml # target, linker, runner (espflash) ├── src/ │ └── main.rs # your firmware entry point ├── Cargo.toml └── rust-toolchain.toml # pins nightly + xtensa toolchain

### .cargo/config.toml — understand it

[build] target = "xtensa-esp32s3-none-elf" [target.xtensa-esp32s3-none-elf] linker = "xtensa-esp32s3-elf-ld" # Xtensa-specific linker runner = "espflash flash --monitor" # cargo run -> flash + serial monitor [unstable] build-std = ["core"] # build core from source (no_std)

## 2.4  Blink: Your First ESP32-S3 Firmware

The Waveshare ESP32-S3 Zero has a WS2812B RGB LED on GPIO21. We will blink it using the RMT peripheral via esp-hal.

### Update Cargo.toml

[package] name = "esp32s3_blink" version = "0.1.0" edition = "2021" [dependencies] esp-hal = { version = "0.18", features = ["esp32s3"] } esp-backtrace = { version = "0.13", features = ["esp32s3", "halt-cores", "println"] } esp-println = { version = "0.11", features = ["esp32s3", "log"] } smart-leds = "0.4" ws2812-spi = "0.5" # or use esp-hal RMT driver [profile.release] opt-level = "s" overflow-checks = false lto = "thin"

### src/main.rs — WS2812B blink via RMT

#![no_std] #![no_main] use esp_backtrace as _; use esp_hal::{ clock::ClockControl, delay::Delay, gpio::IO, peripherals::Peripherals, prelude::*, rmt::{PulseCode, Rmt, TxChannel, TxChannelConfig, TxChannelCreatorAsync}, }; use smart_leds::{SmartLedsWrite, RGB8}; #[entry] fn main() -> ! { let peripherals = Peripherals::take(); let system = peripherals.SYSTEM.split(); let clocks = ClockControl::max(system.clock_control).freeze(); let io = IO::new(peripherals.GPIO, peripherals.IO_MUX); let delay = Delay::new(&clocks); // RMT channel 0 on GPIO21 (WS2812B data pin) let rmt = Rmt::new(peripherals.RMT, 80u32.MHz(), &clocks).unwrap(); let channel = rmt.channel0 .configure( io.pins.gpio21.into_push_pull_output(), TxChannelConfig { clk_divider: 1, ..TxChannelConfig::default() }, ).unwrap(); let mut ws = ws2812_rmt::Ws2812Rmt::new(channel); let colors = [ RGB8 { r: 10, g: 0, b: 0 }, // dim red RGB8 { r: 0, g: 10, b: 0 }, // dim green RGB8 { r: 0, g: 0, b: 10 }, // dim blue RGB8 { r: 0, g: 0, b: 0 }, // off ]; loop { for &color in colors.iter() { ws.write([color].iter().cloned()).unwrap(); delay.delay_ms(500u32); } } }

### Build & Flash

# Attach USB first (usbipd in PowerShell), then inside WSL: cargo build --release # Flash and open serial monitor (115200 baud): cargo run --release # Or manually: espflash flash target/xtensa-esp32s3-none-elf/release/esp32s3_blink \ --port /dev/ttyUSB0 --monitor

⚠️ PORT NOTE If you see 'permission denied on /dev/ttyUSB0', run: sudo chmod 666 /dev/ttyUSB0 (or ensure you added yourself to the dialout group and relogged).

## 2.5  UART Logging with esp-println

// Add to main.rs, after peripherals init: use esp_println::println; // In your loop: println!("Tick: color index = {:?}", color); // Serial monitor (Ctrl+C to exit): // espflash monitor --port /dev/ttyUSB0 // or: cargo run --release (runner auto-opens monitor)

## 2.6  GPIO Input — Button Debounce

#![no_std] #![no_main] use esp_backtrace as _; use esp_hal::{ clock::ClockControl, delay::Delay, gpio::{Input, IO, PullUp}, peripherals::Peripherals, prelude::*, }; use esp_println::println; #[entry] fn main() -> ! { let peripherals = Peripherals::take(); let system = peripherals.SYSTEM.split(); let clocks = ClockControl::max(system.clock_control).freeze(); let io = IO::new(peripherals.GPIO, peripherals.IO_MUX); let delay = Delay::new(&clocks); // Button on GPIO0 (boot button on ESP32-S3 Zero) let button: Input<PullUp> = io.pins.gpio0.into_pull_up_input(); let mut last_state = true; // pulled high = not pressed let mut count: u32 = 0; loop { let state = button.is_high(); if state != last_state { delay.delay_ms(20u32); // debounce if button.is_high() != last_state { last_state = state; if !state { count += 1; } println!("Button press count: {}", count); } } } }

## 2.7  I2C Sensor Read (MPU6050)

This example reads acceleration from an MPU6050 connected via I2C (SDA=GPIO8, SCL=GPIO9) — matching your GY-86 module pinout.

# Cargo.toml additions: mpu6050 = "0.1" embedded-hal = "1.0"

use esp_hal::i2c::I2C; use mpu6050::Mpu6050; let i2c = I2C::new( peripherals.I2C0, io.pins.gpio8, // SDA io.pins.gpio9, // SCL 400u32.kHz(), &clocks, ); let mut mpu = Mpu6050::new(i2c); mpu.init(&mut delay).unwrap(); loop { let accel = mpu.get_acc().unwrap(); println!("Accel X={:.3} Y={:.3} Z={:.3}", accel.x, accel.y, accel.z); delay.delay_ms(100u32); }

## 2.8  BLE Advertisement (esp-hal + bleps)

# Cargo.toml: bleps = { version = "0.1", features = ["async"] } esp-wifi = { version = "0.6", features = ["esp32s3", "ble"] } embassy-executor = { version = "0.5", features = ["arch-xtensa"] }

// Minimal BLE advertiser (iBeacon-style) use esp_wifi::ble::controller::asynch::BleConnector; use bleps::{ad_structure::*, asynch::Ble, HciConnector}; #[esp_hal::entry] fn main() -> ! { // ... peripherals, clocks, timer init ... let wifi_init = esp_wifi::initialize(/* ... */).unwrap(); let connector = BleConnector::new(&wifi_init, peripherals.BT); let hci = HciConnector::new(connector, esp_hal::current_time); let mut ble = Ble::new(&hci); ble.init().await.unwrap(); ble.cmd_set_le_advertising_parameters().await.unwrap(); ble.cmd_set_le_advertising_data( create_advertising_data(&[ AdStructure::Flags(LE_GENERAL_DISCOVERABLE ¦ BR_EDR_NOT_SUPPORTED), AdStructure::CompleteLocalName(b"ESP32S3-Rust"), ]) ).await.unwrap(); ble.cmd_set_le_advertise_enable(true).await.unwrap(); loop { /* handle connections */ } }

# PART 3 — STM32F446RE IN RUST

## 3.1  Hardware Overview — Nucleo-F446RE

The NUCLEO-F446RE is an ST Microelectronics development board featuring the STM32F446RET6 — ARM Cortex-M4F @ up to 180 MHz, 512 KB Flash, 128 KB SRAM, FPU, DSP instructions, and an onboard ST-Link/V2-1 debugger over USB.

| Feature | Specification |
| --- | --- |
| CPU | ARM Cortex-M4F @ 180 MHz + FPU |
| Flash / SRAM | 512 KB / 128 KB |
| Peripherals | 3x SPI, 3x I2C, 4x USART, 2x USB OTG, CAN, SAI |
| GPIO | 51 (3.3 V, 5 V-tolerant on select pins) |
| Debugger | Onboard ST-Link/V2-1 (SWD), USB-C |
| Arduino Headers | Yes (Uno R3 compatible) |
| User LED | LD2 (green) on PA5 |
| User Button | B1 (blue) on PC13 |

## 3.2  Rust Target & Toolchain for STM32F446

# The STM32F446 uses Cortex-M4F (hard-float ABI) rustup target add thumbv7em-none-eabihf # .cargo/config.toml for STM32 project: [build] target = "thumbv7em-none-eabihf" [target.thumbv7em-none-eabihf] linker = "flip-link" # or arm-none-eabi-ld runner = "probe-rs run --chip STM32F446RETx" rustflags = [ "-C", "link-arg=-Tlink.x", "-C", "link-arg=-Tdefmt.x", # if using defmt logging ]

### Install ARM linker & flip-link

sudo apt install -y gcc-arm-none-eabi binutils-arm-none-eabi cargo install flip-link

## 3.3  Create STM32 Project with stm32f4xx-hal

### Cargo.toml

[package] name = "stm32f446_blink" version = "0.1.0" edition = "2021" [dependencies] cortex-m = { version = "0.7", features = ["critical-section-single-core"] } cortex-m-rt = "0.7" stm32f4xx-hal = { version = "0.21", features = ["stm32f446"] } panic-halt = "0.2" [profile.release] codegen-units = 1 debug = true lto = true opt-level = "s"

### memory.x — linker script for STM32F446

/* memory.x — place in project root */ MEMORY { FLASH : ORIGIN = 0x08000000, LENGTH = 512K RAM : ORIGIN = 0x20000000, LENGTH = 128K }

### src/main.rs — Blink LD2 on PA5

#![no_std] #![no_main] use panic_halt as _; use cortex_m_rt::entry; use stm32f4xx_hal::{ gpio::GpioExt, pac, prelude::*, }; #[entry] fn main() -> ! { let dp = pac::Peripherals::take().unwrap(); let gpioa = dp.GPIOA.split(); // PA5 = LD2 on Nucleo-F446RE let mut led = gpioa.pa5.into_push_pull_output(); // Configure systick-based delay let rcc = dp.RCC.constrain(); let clocks = rcc.cfgr.sysclk(168.MHz()).freeze(); let mut delay = dp.TIM1.delay_ms(&clocks); loop { led.set_high(); delay.delay_ms(500u32); led.set_low(); delay.delay_ms(500u32); } }

### Build & Flash via probe-rs

cargo build --release # Flash (ST-Link must be attached via USB, usbipd-bound in WSL): cargo run --release # Or manually: probe-rs download \ target/thumbv7em-none-eabihf/release/stm32f446_blink \ --chip STM32F446RETx

## 3.4  UART Logging with defmt + RTT

defmt is Espressif's and the embedded-rs community's preferred logging framework for no_std. RTT (Real-Time Transfer) sends log data over the SWD debug channel — no extra UART wires needed.

# Cargo.toml additions: defmt = "0.3" defmt-rtt = "0.4" panic-probe = { version = "0.3", features = ["print-defmt"] }

// src/main.rs changes: use defmt::info; use defmt_rtt as _; // transport use panic_probe as _; // replace panic-halt // In your loop: info!("LED toggled, count = {}", count);

# View defmt logs in terminal: cargo run --release # probe-rs automatically prints defmt logs # Or separately: probe-rs run --chip STM32F446RETx \ target/thumbv7em-none-eabihf/release/stm32f446_blink

## 3.5  GPIO Input — User Button PC13

use stm32f4xx_hal::gpio::{Input, PullUp}; let gpioc = dp.GPIOC.split(); // B1 user button on PC13 — active-LOW (pressed = low) let button = gpioc.pc13.into_pull_up_input(); loop { if button.is_low() { led.set_high(); info!("Button pressed!"); } else { led.set_low(); } delay.delay_ms(10u32); }

## 3.6  SPI — External Sensor Interface

use stm32f4xx_hal::spi::{Mode, Phase, Polarity, Spi}; let gpiob = dp.GPIOB.split(); let sck = gpiob.pb3.into_alternate(); let miso = gpiob.pb4.into_alternate(); let mosi = gpiob.pb5.into_alternate(); let cs = gpiob.pb6.into_push_pull_output(); let mode = Mode { polarity: Polarity::IdleLow, phase: Phase::CaptureOnFirstTransition }; let mut spi = Spi::new( dp.SPI1, (sck, miso, mosi), mode, 1.MHz(), &clocks, ); // Transfer 1 byte: let mut buf = [0x3Fu8]; // e.g. read WHO_AM_I register cs.set_low(); spi.transfer_in_place(&mut buf).unwrap(); cs.set_high(); info!("WHO_AM_I = 0x{:02X}", buf[0]);

## 3.7  Embassy Async on STM32F446

Embassy provides an async executor and async HAL drivers. It enables clean, zero-cost cooperative multitasking without an RTOS — ideal for sensor fusion and communication stacks.

# Cargo.toml for Embassy on STM32F446: embassy-executor = { version = "0.5", features = ["arch-cortex-m", "executor-thread"] } embassy-stm32 = { version = "0.1", features = ["stm32f446re", "time-driver-tim2", "defmt"] } embassy-time = { version = "0.3", features = ["defmt"] } static-cell = "2"

#![no_std] #![no_main] use defmt::info; use defmt_rtt as _; use panic_probe as _; use embassy_executor::Spawner; use embassy_stm32::{gpio::{Level, Output, Speed}, time::Hertz, Config}; use embassy_time::Timer; #[embassy_executor::main] async fn main(_spawner: Spawner) { let mut config = Config::default(); config.rcc.sys_ck = Some(Hertz(168_000_000)); let p = embassy_stm32::init(config); let mut led = Output::new(p.PA5, Level::Low, Speed::Low); let mut count: u32 = 0; loop { led.set_high(); Timer::after_millis(500).await; led.set_low(); Timer::after_millis(500).await; count += 1; info!("Blink #{}", count); } }

## 3.8  Multi-Task Embassy Example — LED + UART

use embassy_executor::Spawner; use embassy_stm32::usart::{Config as UartConfig, Uart}; use embassy_time::Timer; use static_cell::StaticCell; #[embassy_executor::task] async fn blink_task(mut led: Output<'static, PA5>) { loop { led.toggle(); Timer::after_millis(250).await; } } #[embassy_executor::task] async fn uart_task(mut uart: Uart<'static, USART2, DMA1_CH6, DMA1_CH5>) { let mut buf = [0u8; 64]; loop { let n = uart.read_until_idle(&mut buf).await.unwrap(); uart.write_all(&buf[..n]).await.unwrap(); // echo back } } #[embassy_executor::main] async fn main(spawner: Spawner) { let p = embassy_stm32::init(Default::default()); let led = Output::new(p.PA5, Level::Low, Speed::Low); let uart = Uart::new(p.USART2, p.PA3, p.PA2, Irqs, p.DMA1_CH6, p.DMA1_CH5, UartConfig::default()).unwrap(); spawner.spawn(blink_task(led)).unwrap(); spawner.spawn(uart_task(uart)).unwrap(); }

# PART 4 — DEBUGGING & TOOLING

## 4.1  GDB Debugging — STM32 via OpenOCD

# Terminal 1 — start OpenOCD openocd -f interface/stlink.cfg -f target/stm32f4x.cfg # Terminal 2 — connect GDB arm-none-eabi-gdb target/thumbv7em-none-eabihf/release/stm32f446_blink (gdb) target remote localhost:3333 (gdb) monitor reset halt (gdb) load (gdb) break main (gdb) continue # Or use probe-rs (simpler): probe-rs gdb --chip STM32F446RETx

## 4.2  ESP32-S3 Serial Monitor & Logging

# espflash built-in monitor (115200 baud default): espflash monitor --port /dev/ttyUSB0 # Or use screen / minicom: screen /dev/ttyUSB0 115200 minicom -D /dev/ttyUSB0 -b 115200 # Exit screen: Ctrl-A then K # Exit minicom: Ctrl-A then Q

## 4.3  VS Code + rust-analyzer Setup

Install these VS Code extensions inside WSL (via the Remote - WSL extension):

- rust-analyzer — Rust language server (IntelliSense, goto-def, inline errors)

- Even Better TOML — Cargo.toml syntax support

- probe-rs-debugger — SWD-based debug in VS Code for STM32

- cortex-debug — GDB-based debug (alternative to probe-rs-debugger)

# .vscode/settings.json for ESP32-S3 (xtensa target): { "rust-analyzer.cargo.target": "xtensa-esp32s3-none-elf", "rust-analyzer.cargo.features": ["esp32s3"], "rust-analyzer.checkOnSave.allTargets": false, "rust-analyzer.rustfmt.rangeFormatting.enable": true } # .vscode/settings.json for STM32: { "rust-analyzer.cargo.target": "thumbv7em-none-eabihf" }

## 4.4  probe-rs Chip ID & Flash Probe

# List connected probes: probe-rs list # Identify chip attached to probe: probe-rs info --chip auto # Erase chip flash: probe-rs erase --chip STM32F446RETx # Read memory (address 0x08000000, 256 bytes): probe-rs read --chip STM32F446RETx 32 0x08000000 256

# PART 5 — COMPARISON & TROUBLESHOOTING

## 5.1  ESP32-S3 vs STM32F446 — Rust Perspective

| Aspect | ESP32-S3 (esp-hal) vs STM32F446 (embassy/hal) |
| --- | --- |
| Rust target | xtensa-esp32s3-none-elf vs thumbv7em-none-eabihf |
| Toolchain | nightly (mandatory) vs stable (embassy needs nightly) |
| Logging | esp-println/UART vs defmt/RTT (over SWD) |
| Flashing | espflash (USB-C direct) vs probe-rs (ST-Link SWD) |
| HAL maturity | esp-hal 0.18 (active) vs stm32f4xx-hal 0.21 (mature) |
| Async support | embassy + Xtensa executor vs embassy-stm32 (excellent) |
| Wi-Fi/BLE | Native (esp-wifi crate) vs External module needed |
| Binary size | ~200 KB typical vs ~20-80 KB typical |

## 5.2  Common Errors & Fixes

### ESP32-S3: 'xtensa target not found'

# The xtensa target requires the espressif Rust fork: rustup toolchain install esp rustup override set esp # Or use espup (recommended): cargo install espup espup install source ~/export-esp.sh # adds Xtensa linker to PATH # Add to ~/.bashrc for persistence: echo 'source ~/export-esp.sh' >> ~/.bashrc

### ESP32-S3: USB device not found in WSL

# In PowerShell (Admin): usbipd list usbipd bind --busid <BUSID> usbipd attach --wsl --busid <BUSID> # In WSL: verify ls /dev/ttyUSB* dmesg ¦ tail -20 # look for 'cp210x' or 'ch341'

### STM32: probe-rs 'No probes found'

# Check udev rules for ST-Link: cat /etc/udev/rules.d/99-probe-rs.rules # Install probe-rs udev rules: probe-rs complete install # installs udev rules sudo udevadm control --reload-rules sudo udevadm trigger # Check USB device is attached in WSL: lsusb ¦ grep -i 'st'

### STM32: 'error: linker script not found (link.x)'

# Make sure cortex-m-rt is in Cargo.toml (it provides link.x) # Also ensure memory.x is in the project root, not src/ ls memory.x # should exist # Linker flag in .cargo/config.toml: rustflags = ["-C", "link-arg=-Tlink.x"]

### ESP32-S3: Flash too large (build fails)

# Check your partition table — default is 2 MB app # Use release profile with size optimizations: [profile.release] opt-level = "s" # optimize for size lto = "thin" codegen-units = 1 # Check binary size: cargo size --release -- -A

# PART 6 — QUICK REFERENCE CHEAT SHEET

## 6.1  Cargo & Toolchain Commands

# ESP32-S3 workflow: source ~/export-esp.sh # activate Xtensa toolchain cargo build --release # compile cargo run --release # flash + monitor (via espflash runner) espflash monitor --port /dev/ttyUSB0 # serial only # STM32 workflow: cargo build --release # compile cargo run --release # flash + RTT logs (via probe-rs runner) probe-rs run --chip STM32F446RETx <elf> # manual flash arm-none-eabi-size <elf> # check binary size # Universal: cargo clippy -- -D warnings # lint cargo fmt # format cargo tree # dependency tree cargo update # update Cargo.lock

## 6.2  usbipd Quick Reference (PowerShell)

usbipd list # list all USB devices usbipd bind --busid <ID> # share device with WSL (once) usbipd attach --wsl --busid <ID> # attach to WSL session usbipd detach --busid <ID> # detach from WSL usbipd unbind --busid <ID> # stop sharing

## 6.3  Key Crate Versions (as of 2025)

| Crate | Version / Notes |
| --- | --- |
| esp-hal | 0.18.x — ESP32-S3 bare-metal HAL |
| esp-wifi | 0.6.x — Wi-Fi + BLE for ESP32 |
| bleps | 0.1.x — BLE host stack for ESP32 |
| stm32f4xx-hal | 0.21.x — STM32F4 HAL |
| embassy-stm32 | 0.1.x — async HAL for STM32 |
| embassy-executor | 0.5.x — async executor |
| probe-rs | 0.24.x — flashing + debugging |
| defmt | 0.3.x — efficient embedded logging |
| cortex-m-rt | 0.7.x — Cortex-M runtime |

Tutorial prepared for Roy Roychowdhury — Rust Embedded Engineering | ESP32-S3 + STM32F446RE on WSL2