C++ Programming on STM32F446RE

Bare Metal Development with STM32CubeMX

Including DMA and FreeRTOS Integration

A Comprehensive Step-by-Step Tutorial

Table of Contents

1. Introduction to C++ on Embedded Systems

2. Prerequisites and Development Environment

3. STM32CubeMX Configuration for C++

4. Basic C++ Features in Bare Metal

5. Object-Oriented Programming on STM32

6. Templates and Generic Programming

7. Memory Management and RAII

8. Advanced C++ Techniques

9. Practical Examples and Projects

10. Best Practices and Common Pitfalls

11. Practice Exercises

12. DMA Programming with C++

13. FreeRTOS Integration with C++

14. Exercise Solutions

# 1. Introduction to C++ on Embedded Systems

C++ brings powerful programming paradigms to embedded systems while maintaining the low-level control necessary for bare metal development. This tutorial focuses on using modern C++ (C++11/14/17) features with the STM32F446RE microcontroller in a bare metal environment—no HAL, no RTOS initially, just direct hardware manipulation with C++ elegance. Later chapters introduce DMA and FreeRTOS integration.

## 1.1 Why C++ for Embedded Systems?

- Zero-cost abstractions: C++ features compile down to efficient machine code

- Type safety: Catch errors at compile time instead of runtime

- Code reusability: Templates and classes enable modular, reusable code

- RAII (Resource Acquisition Is Initialization): Automatic resource management

- Namespace support: Better organization and no name collisions

## 1.2 STM32F446RE Overview

Key specifications:

- ARM Cortex-M4F core running at up to 180 MHz

- 512 KB Flash memory, 128 KB SRAM

- Floating Point Unit (FPU) for hardware acceleration

- Rich peripheral set: UART, SPI, I2C, ADC, DAC, timers, DMA

- 2 DMA controllers with 16 streams

# 12. DMA Programming with C++

Direct Memory Access (DMA) enables peripherals to transfer data to/from memory without CPU intervention, freeing the processor for other tasks. The STM32F446RE has two DMA controllers with 16 streams total, supporting memory-to-memory, peripheral-to-memory, and memory-to-peripheral transfers.

## 12.1 DMA Basics and Configuration

Key DMA Concepts:

- Streams: DMA1 and DMA2 each have 8 streams (0-7)

- Channels: Each stream has 8 channels for different peripherals

- Priority: Low, Medium, High, Very High

- Transfer Modes: Normal (single) or Circular (continuous)

- Data Sizes: 8-bit, 16-bit, or 32-bit

## 12.2 DMA Base Class

A type-safe DMA class with RAII:

// DMA.hpp

#ifndef DMA_HPP

#define DMA_HPP

#include "stm32f446xx.h"

#include <cstdint>

#include <cstddef>

class DMA {

public:

enum class Direction {

PERIPHERAL_TO_MEMORY = 0,

MEMORY_TO_PERIPHERAL = 1,

MEMORY_TO_MEMORY = 2

};

enum class Priority {

LOW = 0, MEDIUM = 1, HIGH = 2, VERY_HIGH = 3

};

enum class DataSize {

BYTE = 0, HALFWORD = 1, WORD = 2

};

DMA(DMA_Stream_TypeDef* stream, uint32_t channel)

: m_stream(stream), m_channel(channel) {

enableClock();

}

void configure(Direction dir, Priority prio, DataSize mSize,

DataSize pSize, bool memIncrement, bool periIncrement,

bool circular) {

// Disable stream before configuration

m_stream->CR &= ~DMA_SxCR_EN;

while(m_stream->CR & DMA_SxCR_EN);

uint32_t cr = 0;

// Set channel

cr |= (m_channel << DMA_SxCR_CHSEL_Pos);

// Set priority

cr |= (static_cast<uint32_t>(prio) << DMA_SxCR_PL_Pos);

// Set data sizes

cr |= (static_cast<uint32_t>(mSize) << DMA_SxCR_MSIZE_Pos);

cr |= (static_cast<uint32_t>(pSize) << DMA_SxCR_PSIZE_Pos);

// Set increments

if(memIncrement) cr |= DMA_SxCR_MINC;

if(periIncrement) cr |= DMA_SxCR_PINC;

// Set direction

cr |= (static_cast<uint32_t>(dir) << DMA_SxCR_DIR_Pos);

// Set circular mode

if(circular) cr |= DMA_SxCR_CIRC;

// Enable transfer complete interrupt

cr |= DMA_SxCR_TCIE;

m_stream->CR = cr;

}

void setAddresses(volatile void* peripheral, void* memory, size_t count) {

m_stream->PAR = reinterpret_cast<uint32_t>(peripheral);

m_stream->M0AR = reinterpret_cast<uint32_t>(memory);

m_stream->NDTR = count;

}

void enable() {

m_stream->CR |= DMA_SxCR_EN;

}

void disable() {

m_stream->CR &= ~DMA_SxCR_EN;

while(m_stream->CR & DMA_SxCR_EN);

}

bool isTransferComplete() const {

// Check appropriate TCIF flag based on stream number

uint32_t streamNum = getStreamNumber();

DMA_TypeDef* dma = getDMAController();

if(streamNum < 4) {

return (dma->LISR & (DMA_LISR_TCIF0 << (streamNum * 6))) != 0;

} else {

return (dma->HISR & (DMA_HISR_TCIF4 << ((streamNum - 4) * 6))) != 0;

}

}

void clearTransferCompleteFlag() {

uint32_t streamNum = getStreamNumber();

DMA_TypeDef* dma = getDMAController();

if(streamNum < 4) {

dma->LIFCR = (DMA_LIFCR_CTCIF0 << (streamNum * 6));

} else {

dma->HIFCR = (DMA_HIFCR_CTCIF4 << ((streamNum - 4) * 6));

}

}

private:

DMA_Stream_TypeDef* m_stream;

uint32_t m_channel;

void enableClock() {

if(m_stream >= DMA1_Stream0 && m_stream <= DMA1_Stream7) {

RCC->AHB1ENR |= RCC_AHB1ENR_DMA1EN;

} else if(m_stream >= DMA2_Stream0 && m_stream <= DMA2_Stream7) {

RCC->AHB1ENR |= RCC_AHB1ENR_DMA2EN;

}

}

uint32_t getStreamNumber() const {

if(m_stream >= DMA1_Stream0 && m_stream <= DMA1_Stream7) {

return (reinterpret_cast<uint32_t>(m_stream) -

reinterpret_cast<uint32_t>(DMA1_Stream0)) / 0x18;

} else {

return (reinterpret_cast<uint32_t>(m_stream) -

reinterpret_cast<uint32_t>(DMA2_Stream0)) / 0x18;

}

}

DMA_TypeDef* getDMAController() const {

if(m_stream >= DMA1_Stream0 && m_stream <= DMA1_Stream7) {

return DMA1;

} else {

return DMA2;

}

}

};

#endif

## 12.3 DMA with UART Example

Using DMA for non-blocking UART transmission:

// UART_DMA.hpp

class UART_DMA {

public:

UART_DMA(USART_TypeDef* uart, DMA_Stream_TypeDef* txStream,

uint32_t txChannel, uint32_t baudRate)

: m_uart(uart), m_dma(txStream, txChannel) {

initializeUART(baudRate);

configureDMA();

}

void transmit(const char* data, size_t length) {

// Wait if previous transfer is ongoing

while(m_dma.isTransferComplete() == false);

m_dma.disable();

m_dma.setAddresses(&m_uart->DR, const_cast<char*>(data), length);

m_dma.clearTransferCompleteFlag();

m_dma.enable();

}

bool isBusy() const {

return !m_dma.isTransferComplete();

}

private:

USART_TypeDef* m_uart;

DMA m_dma;

void initializeUART(uint32_t baudRate) {

if(m_uart == USART2) {

RCC->APB1ENR |= RCC_APB1ENR_USART2EN;

}

m_uart->BRR = (45000000 + baudRate/2) / baudRate;

m_uart->CR3 = USART_CR3_DMAT;  // Enable DMA for TX

m_uart->CR1 = USART_CR1_TE | USART_CR1_UE;

}

void configureDMA() {

m_dma.configure(

DMA::Direction::MEMORY_TO_PERIPHERAL,

DMA::Priority::MEDIUM,

DMA::DataSize::BYTE,

DMA::DataSize::BYTE,

true,   // Memory increment

false,  // Peripheral no increment

false   // Normal mode (not circular)

);

}

};

// Usage

UART_DMA uart(USART2, DMA1_Stream6, 4, 115200);

uart.transmit("Hello DMA!", 10);

while(uart.isBusy());  // Or do other work

## 12.4 ADC with DMA for Continuous Sampling

template<size_t NUM_CHANNELS>

class ADC_DMA {

public:

ADC_DMA(ADC_TypeDef* adc, DMA_Stream_TypeDef* stream, uint32_t channel)

: m_adc(adc), m_dma(stream, channel) {

initializeADC();

configureDMA();

}

void start() {

m_dma.setAddresses(&m_adc->DR, m_buffer, NUM_CHANNELS);

m_dma.enable();

m_adc->CR2 |= ADC_CR2_SWSTART;  // Start conversion

}

uint16_t getValue(size_t channel) const {

if(channel < NUM_CHANNELS) {

return m_buffer[channel];

}

return 0;

}

float getVoltage(size_t channel, float vref = 3.3f) const {

return (getValue(channel) * vref) / 4095.0f;

}

private:

ADC_TypeDef* m_adc;

DMA m_dma;

volatile uint16_t m_buffer[NUM_CHANNELS];

void initializeADC() {

RCC->APB2ENR |= RCC_APB2ENR_ADC1EN;

m_adc->CR1 = ADC_CR1_SCAN;  // Scan mode for multiple channels

m_adc->CR2 = ADC_CR2_DMA | ADC_CR2_DDS | ADC_CR2_CONT;

m_adc->SQR1 = ((NUM_CHANNELS - 1) << 20);  // Number of conversions

m_adc->CR2 |= ADC_CR2_ADON;  // Enable ADC

}

void configureDMA() {

m_dma.configure(

DMA::Direction::PERIPHERAL_TO_MEMORY,

DMA::Priority::HIGH,

DMA::DataSize::HALFWORD,

DMA::DataSize::HALFWORD,

true,   // Memory increment

false,  // Peripheral no increment

true    // Circular mode

);

}

};

# 13. FreeRTOS Integration with C++

FreeRTOS is a real-time operating system kernel for embedded devices. Integrating FreeRTOS with C++ enables object-oriented task management, RAII for synchronization primitives, and type-safe inter-task communication.

## 13.1 Setting Up FreeRTOS with CubeMX

Configuration Steps:

- In CubeMX, enable FreeRTOS from Middleware panel

- Configure heap size (typically 8-16 KB for STM32F446RE)

- Set tick rate (default 1000 Hz = 1ms tick)

- Choose CMSIS_V2 API or native FreeRTOS API

- Generate code and modify for C++ compatibility

## 13.2 Task Wrapper Class

A C++ wrapper for FreeRTOS tasks using CRTP (Curiously Recurring Template Pattern):

// Task.hpp

#ifndef TASK_HPP

#define TASK_HPP

#include "FreeRTOS.h"

#include "task.h"

#include <cstdint>

template<typename T>

class Task {

public:

Task(const char* name, uint32_t stackSize, UBaseType_t priority)

: m_handle(nullptr) {

xTaskCreate(taskFunction, name, stackSize, this, priority, &m_handle);

}

virtual ~Task() {

if(m_handle != nullptr) {

vTaskDelete(m_handle);

}

}

void suspend() { vTaskSuspend(m_handle); }

void resume() { vTaskResume(m_handle); }

UBaseType_t getPriority() const { return uxTaskPriorityGet(m_handle); }

void setPriority(UBaseType_t priority) { vTaskPrioritySet(m_handle, priority); }

protected:

// Derived class must implement this

virtual void run() = 0;

static void delay(uint32_t ms) {

vTaskDelay(pdMS_TO_TICKS(ms));

}

private:

TaskHandle_t m_handle;

static void taskFunction(void* param) {

T* task = static_cast<T*>(param);

task->run();

vTaskDelete(nullptr);  // Task should never return

}

};

#endif

Example task implementation:

class BlinkTask : public Task<BlinkTask> {

public:

BlinkTask(GPIO& led)

: Task("Blink", 128, 1), m_led(led) {}

protected:

void run() override {

while(true) {

m_led.toggle();

delay(500);

}

}

private:

GPIO& m_led;

};

## 13.3 Queue Wrapper

Type-safe queue wrapper using templates:

template<typename T, size_t LENGTH>

class Queue {

public:

Queue() {

m_handle = xQueueCreate(LENGTH, sizeof(T));

}

~Queue() {

if(m_handle != nullptr) {

vQueueDelete(m_handle);

}

}

bool send(const T& item, uint32_t timeout = portMAX_DELAY) {

return xQueueSend(m_handle, &item, timeout) == pdTRUE;

}

bool receive(T& item, uint32_t timeout = portMAX_DELAY) {

return xQueueReceive(m_handle, &item, timeout) == pdTRUE;

}

bool sendFromISR(const T& item) {

BaseType_t higherPriorityTaskWoken = pdFALSE;

bool result = xQueueSendFromISR(m_handle, &item,

&higherPriorityTaskWoken) == pdTRUE;

portYIELD_FROM_ISR(higherPriorityTaskWoken);

return result;

}

size_t available() const {

return uxQueueMessagesWaiting(m_handle);

}

bool isEmpty() const { return available() == 0; }

bool isFull() const { return available() == LENGTH; }

private:

QueueHandle_t m_handle;

// Prevent copying

Queue(const Queue&) = delete;

Queue& operator=(const Queue&) = delete;

};

## 13.4 Mutex and Semaphore Wrappers

// Mutex with RAII lock guard

class Mutex {

public:

Mutex() { m_handle = xSemaphoreCreateMutex(); }

~Mutex() { if(m_handle) vSemaphoreDelete(m_handle); }

bool lock(uint32_t timeout = portMAX_DELAY) {

return xSemaphoreTake(m_handle, timeout) == pdTRUE;

}

void unlock() { xSemaphoreGive(m_handle); }

class LockGuard {

public:

LockGuard(Mutex& mutex) : m_mutex(mutex) {

m_mutex.lock();

}

~LockGuard() { m_mutex.unlock(); }

private:

Mutex& m_mutex;

};

private:

SemaphoreHandle_t m_handle;

};

// Binary Semaphore

class BinarySemaphore {

public:

BinarySemaphore() { m_handle = xSemaphoreCreateBinary(); }

~BinarySemaphore() { if(m_handle) vSemaphoreDelete(m_handle); }

bool take(uint32_t timeout = portMAX_DELAY) {

return xSemaphoreTake(m_handle, timeout) == pdTRUE;

}

void give() { xSemaphoreGive(m_handle); }

void giveFromISR() {

BaseType_t higherPriorityTaskWoken = pdFALSE;

xSemaphoreGiveFromISR(m_handle, &higherPriorityTaskWoken);

portYIELD_FROM_ISR(higherPriorityTaskWoken);

}

private:

SemaphoreHandle_t m_handle;

};

## 13.5 Complete Producer-Consumer Example

struct SensorData {

uint32_t timestamp;

float temperature;

float humidity;

};

Queue<SensorData, 10> dataQueue;

class SensorTask : public Task<SensorTask> {

public:

SensorTask() : Task("Sensor", 256, 2) {}

protected:

void run() override {

while(true) {

SensorData data;

data.timestamp = xTaskGetTickCount();

data.temperature = readTemperature();

data.humidity = readHumidity();

dataQueue.send(data);

delay(1000);

}

}

float readTemperature() { return 25.5f; }

float readHumidity() { return 60.0f; }

};

class ProcessTask : public Task<ProcessTask> {

public:

ProcessTask() : Task("Process", 256, 1) {}

protected:

void run() override {

SensorData data;

while(true) {

if(dataQueue.receive(data)) {

processData(data);

}

}

}

void processData(const SensorData& data) {

// Process the data

}

};

# 14. Exercise Solutions

## 14.1 Solution: LED Blinker Class

// LEDBlinker.hpp

#ifndef LED_BLINKER_HPP

#define LED_BLINKER_HPP

#include "GPIO.hpp"

#include <cstdint>

class LEDBlinker {

public:

LEDBlinker(GPIO_TypeDef* port, uint8_t pin, uint32_t period_ms)

: m_gpio(port, pin), m_period(period_ms), m_lastToggle(0) {

m_gpio.setMode(GPIO::Mode::OUTPUT);

m_gpio.setPull(GPIO::Pull::NONE);

initializeSysTick();

}

void setPeriod(uint32_t period_ms) {

m_period = period_ms;

}

void update() {

uint32_t currentTime = getTicks();

if(currentTime - m_lastToggle >= m_period) {

m_gpio.toggle();

m_lastToggle = currentTime;

}

}

void on() { m_gpio.set(); }

void off() { m_gpio.clear(); }

static uint32_t getTicks() {

return s_tickCount;

}

static void incrementTick() {

s_tickCount++;

}

private:

GPIO m_gpio;

uint32_t m_period;

uint32_t m_lastToggle;

static volatile uint32_t s_tickCount;

void initializeSysTick() {

SysTick->LOAD = (180000000 / 1000) - 1;  // 1ms tick at 180MHz

SysTick->VAL = 0;

SysTick->CTRL = SysTick_CTRL_CLKSOURCE_Msk |

SysTick_CTRL_TICKINT_Msk |

SysTick_CTRL_ENABLE_Msk;

}

};

// Initialize static member

volatile uint32_t LEDBlinker::s_tickCount = 0;

// SysTick ISR

extern "C" void SysTick_Handler(void) {

LEDBlinker::incrementTick();

}

#endif

// Usage

LEDBlinker led(GPIOA, 5, 500);  // Blink every 500ms

while(1) {

led.update();

}

## 14.2 Solution: ADC Manager Template

Complete ADC manager with DMA for multi-channel sampling (shown in Chapter 12.4).

## 14.3 Solution: Protocol State Machine

// ProtocolParser.hpp

class ProtocolParser {

public:

enum class State {

IDLE, SYNC, RECEIVE_HEADER, RECEIVE_DATA, CHECKSUM

};

struct Packet {

uint8_t id;

uint8_t length;

uint8_t data[256];

uint16_t crc;

};

ProtocolParser() : m_state(State::IDLE), m_dataIndex(0) {}

bool processByte(uint8_t byte, Packet& outPacket) {

switch(m_state) {

case State::IDLE:

if(byte == 0xAA) {  // Sync byte

m_state = State::SYNC;

}

break;

case State::SYNC:

if(byte == 0x55) {  // Second sync byte

m_state = State::RECEIVE_HEADER;

m_dataIndex = 0;

} else {

m_state = State::IDLE;

}

break;

case State::RECEIVE_HEADER:

if(m_dataIndex == 0) {

m_currentPacket.id = byte;

m_dataIndex++;

} else {

m_currentPacket.length = byte;

m_dataIndex = 0;

m_state = State::RECEIVE_DATA;

}

break;

case State::RECEIVE_DATA:

m_currentPacket.data[m_dataIndex++] = byte;

if(m_dataIndex >= m_currentPacket.length) {

m_dataIndex = 0;

m_state = State::CHECKSUM;

}

break;

case State::CHECKSUM:

if(m_dataIndex == 0) {

m_currentPacket.crc = byte << 8;

m_dataIndex++;

} else {

m_currentPacket.crc |= byte;

uint16_t calcCRC = calculateCRC();

m_state = State::IDLE;

m_dataIndex = 0;

if(calcCRC == m_currentPacket.crc) {

outPacket = m_currentPacket;

return true;

}

}

break;

}

return false;

}

private:

State m_state;

Packet m_currentPacket;

size_t m_dataIndex;

uint16_t calculateCRC() {

uint16_t crc = 0xFFFF;

crc = updateCRC(crc, m_currentPacket.id);

crc = updateCRC(crc, m_currentPacket.length);

for(size_t i = 0; i < m_currentPacket.length; i++) {

crc = updateCRC(crc, m_currentPacket.data[i]);

}

return crc;

}

uint16_t updateCRC(uint16_t crc, uint8_t data) {

crc ^= data;

for(int i = 0; i < 8; i++) {

if(crc & 0x0001) {

crc = (crc >> 1) ^ 0xA001;

} else {

crc = crc >> 1;

}

}

return crc;

}

};

Note: Solutions for exercises 4 and 5 follow similar patterns using OOP principles, templates, and the classes developed throughout this tutorial.

# Conclusion

This comprehensive tutorial has covered C++ programming on STM32F446RE from basic concepts to advanced techniques including DMA and FreeRTOS integration. You now have the tools to write elegant, efficient embedded firmware that leverages the power of modern C++.

Key Takeaways:

- C++ provides zero-cost abstractions perfect for embedded development

- Classes and templates enable clean, reusable peripheral drivers

- DMA eliminates CPU overhead for data transfers

- FreeRTOS with C++ wrappers provides elegant multitasking

- RAII automates resource management and prevents leaks

- Type safety catches errors at compile time

Continue practicing and building your own projects. The combination of C++ elegance and bare metal control gives you the best of both worlds!