# STM32F4 Bare Metal C++ Tutorial

Understanding Hardware-Level Programming

# What is Bare Metal Programming?

Bare metal programming means writing code that runs directly on hardware without an operating system. Your code has complete control of the processor, memory, and peripherals.

Key Characteristics:

- No OS overhead - every CPU cycle is yours

- Direct hardware access via memory-mapped registers

- Interrupts are YOUR responsibility

- Complete control but also complete responsibility

- Smallest possible code footprint

Why Learn Bare Metal?

- Understand HOW hardware actually works

- Debug embedded systems effectively

- Write efficient, predictable code

- Foundation for RTOS understanding

- Required for bootloaders and low-level drivers

# STM32F4 Memory Architecture

The ARM Cortex-M4 uses memory-mapped I/O. Peripherals are accessed like memory addresses.

## Memory Map Overview

0x0000 0000 - 0x0007 FFFF : Flash (512 KB)

0x2000 0000 - 0x2001 FFFF : SRAM (128 KB)

0x4000 0000 - 0x4007 FFFF : Peripherals (APB1)

0x4001 0000 - 0x4001 5FFF : Peripherals (APB2)

0x4002 0000 - 0x4002 3FFF : Peripherals (AHB1)

0xE000 0000 - 0xE00F FFFF : Cortex-M4 Core Peripherals

Each peripheral has its own address block containing registers. For example, GPIOD starts at 0x40020C00.

## How Registers Work

A register is just a memory location that controls hardware. Writing to a register configures or commands the peripheral.

// Example: GPIOD registers

GPIOD_MODER   @ 0x40020C00  // Pin mode (input/output)

GPIOD_ODR     @ 0x40020C14  // Output data

GPIOD_IDR     @ 0x40020C10  // Input data

GPIOD_BSRR    @ 0x40020C18  // Bit set/reset

// To turn on LED on PD12:

*(volatile uint32_t*)0x40020C18 = (1 << 12);  // Set bit 12

# Understanding Interrupts

Interrupts are hardware events that pause your code and execute a handler function.

## How Interrupts Work

1. Event Occurs:

Hardware detects event (UART byte received, timer overflow, GPIO edge)

2. CPU Saves Context:

Automatically saves registers (R0-R3, R12, LR, PC, xPSR) to stack

3. Vector Table Lookup:

CPU reads interrupt vector table to find handler address

4. Execute Handler:

Jumps to your ISR (Interrupt Service Routine)

5. Return:

CPU restores context and returns to interrupted code

## Interrupt Safety

Code that can be interrupted must be careful with shared data:

// WRONG - Race condition!

volatile uint32_t counter = 0;

void main_loop() {

counter++;  // Can be interrupted mid-operation!

}

void TIMER_IRQHandler() {

counter++;  // Both modify counter - RACE!

}

// CORRECT - Use atomic operations

void main_loop() {

__disable_irq();

counter++;

__enable_irq();

}

# The Critical Role of Volatile

The volatile keyword tells the compiler "this variable can change unexpectedly."

## Why Volatile Matters

// WITHOUT volatile - BROKEN!

uint32_t flag = 0;

void wait_for_interrupt() {

while (flag == 0) {

// Compiler sees: "flag never changes in this loop"

// Optimizes to: while(1) {} - INFINITE LOOP!

}

}

void IRQHandler() {

flag = 1;  // ISR sets flag, but code never sees it!

}

// WITH volatile - CORRECT!

volatile uint32_t flag = 0;

void wait_for_interrupt() {

while (flag == 0) {

// Compiler must read flag every iteration

// Sees change from interrupt

}

}

Use volatile for:

- Hardware registers (always!)

- Variables modified by interrupts

- Variables shared between threads/interrupts

- Memory-mapped I/O

# Clock Configuration

STM32F4 has a complex clock tree. Multiple clocks can be derived from different sources.

## Clock Sources

- HSI: High-Speed Internal (16 MHz RC oscillator)

- HSE: High-Speed External (4-26 MHz crystal)

- LSI: Low-Speed Internal (32 kHz for watchdog)

- LSE: Low-Speed External (32.768 kHz for RTC)

- PLL: Phase-Locked Loop (multiply clock frequency)

## Typical Configuration

// STM32F407 @ 168 MHz

HSE = 8 MHz (external crystal)

PLL Input = HSE / M = 8 MHz / 8 = 1 MHz

PLL VCO = 1 MHz * N = 1 MHz * 336 = 336 MHz

SYSCLK = 336 MHz / P = 336 MHz / 2 = 168 MHz

AHB = SYSCLK / 1 = 168 MHz

APB1 = AHB / 4 = 42 MHz

APB2 = AHB / 2 = 84 MHz

Why this matters: Peripheral clocks affect baud rates, timer frequencies, and I2C/SPI speeds.

# Exercise Overview

We will create 4 exercises demonstrating bare metal C++ on STM32F4:

Exercise 1: GPIO and UART Driver

Direct register access, interrupt-driven UART reception, circular buffers.

Exercise 2: Timer and PWM

Hardware timer configuration, PWM generation, precise timing.

Exercise 3: ADC and DMA

Analog-to-digital conversion, DMA for zero-CPU data transfer.

Exercise 4: I2C Sensor Driver

Bit-banging vs hardware I2C, sensor initialization, data reading.

# Exercise 1: UART Driver (Bare Metal)

We will create a UART driver that works directly with hardware registers.

## UART Hardware Overview

Key Registers:

- USART_CR1: Control register (enable TX/RX, interrupts)

- USART_BRR: Baud rate register

- USART_SR: Status register (flags: TXE, RXNE, etc.)

- USART_DR: Data register (read RX, write TX)

## Bare Metal UART Initialization

void uart_init() {

// 1. Enable clock to USART2 (on APB1)

RCC->APB1ENR |= RCC_APB1ENR_USART2EN;

// 2. Enable clock to GPIOA (for PA2/PA3)

RCC->AHB1ENR |= RCC_AHB1ENR_GPIOAEN;

// 3. Configure PA2/PA3 as alternate function

GPIOA->MODER |= GPIO_MODER_MODER2_1;  // PA2 = AF

GPIOA->MODER |= GPIO_MODER_MODER3_1;  // PA3 = AF

// 4. Select AF7 (USART2) for PA2/PA3

GPIOA->AFR[0] |= (7 << 8);   // PA2

GPIOA->AFR[0] |= (7 << 12);  // PA3

// 5. Calculate and set baud rate

// BRR = fclk / (16 * baud)

// For 42 MHz APB1, 115200 baud:

// BRR = 42000000 / (16 * 115200) = 22.786

USART2->BRR = 0x016C;  // 22.75 in fixed point

// 6. Enable USART, TX, RX

USART2->CR1 = USART_CR1_UE | USART_CR1_TE | USART_CR1_RE;

}

## Complete Bare Metal UART Class

class BareMetalUart {

public:

explicit BareMetalUart(USART_TypeDef* uart) : uart_(uart) {

initHardware();

}

void transmitByte(uint8_t byte) {

// Wait for TX buffer empty

while (!(uart_->SR & USART_SR_TXE));

// Write data

uart_->DR = byte;

}

bool rxAvailable() const {

return (uart_->SR & USART_SR_RXNE);

}

uint8_t readByte() {

// Wait for data

while (!rxAvailable());

// Read clears RXNE flag

return uart_->DR & 0xFF;

}

private:

USART_TypeDef* uart_;

void initHardware() {

// See initialization code above

}

};

# Exercise 2: Hardware Timer (Bare Metal)

## Timer Hardware Concepts

STM32 timers are incredibly versatile. They can generate PWM, measure frequency, trigger ADC, and more.

Key Timer Components:

- Prescaler: Divides input clock

- Counter: Counts up or down

- Auto-Reload Register (ARR): Counter resets at this value

- Compare Registers (CCR): Generate events when counter matches

## Timer Math Example

// Goal: 1 kHz interrupt (1 ms period)

// Timer clock = 84 MHz (from APB2)

// Prescaler divides clock by (PSC + 1)

PSC = 84 - 1  // 84 MHz / 84 = 1 MHz tick

// ARR sets period in ticks

ARR = 1000 - 1  // 1 MHz / 1000 = 1 kHz

// Result: Timer generates interrupt every 1 ms

## Bare Metal Timer Class

class HardwareTimer {

public:

explicit HardwareTimer(TIM_TypeDef* timer) : timer_(timer) {}

void initPeriodic(uint32_t frequency_hz) {

// Enable timer clock

if (timer_ == TIM2) {

RCC->APB1ENR |= RCC_APB1ENR_TIM2EN;

}

// Calculate prescaler and period

// Assuming 84 MHz timer clock

uint32_t ticks_per_second = 84000000;

uint32_t prescaler = 84 - 1;  // 1 MHz tick

uint32_t period = (ticks_per_second / (prescaler + 1)) /

frequency_hz - 1;

timer_->PSC = prescaler;

timer_->ARR = period;

// Enable update interrupt

timer_->DIER |= TIM_DIER_UIE;

// Enable NVIC interrupt

if (timer_ == TIM2) {

NVIC_EnableIRQ(TIM2_IRQn);

}

// Start timer

timer_->CR1 |= TIM_CR1_CEN;

}

void clearInterruptFlag() {

timer_->SR &= ~TIM_SR_UIF;

}

private:

TIM_TypeDef* timer_;

};

# Key Bare Metal Concepts Summary

## 1. Memory-Mapped I/O

Peripherals = Memory addresses. Writing to an address controls hardware.

## 2. Volatile is Critical

Always use volatile for hardware registers and interrupt-shared variables.

## 3. Clock Management

Every peripheral needs its clock enabled. Check which bus (AHB, APB1, APB2).

## 4. Interrupt Safety

Disable interrupts when modifying shared data. Keep ISRs short and fast.

## 5. Register Bit Manipulation

- Set bit: REG |= (1 << bit)

- Clear bit: REG &= ~(1 << bit)

- Toggle bit: REG ^= (1 << bit)

- Read bit: (REG & (1 << bit))

## 6. Polling vs Interrupts

Polling: Check status in loop

Simple but wastes CPU. Use for fast operations.

Interrupts: Hardware notifies CPU

Efficient but more complex. Use for slow operations.

Now you understand the hardware!