# STM32F4 C++ Practice Exercises

# Introduction

This document contains 4 complete STM32F4 C++ exercises with production-ready code.

Prerequisites:

- STM32CubeIDE or similar toolchain

- STM32F4 Discovery board

- STM32 HAL library

- C++17 compiler with -fno-exceptions -fno-rtti

# Exercise 1: UART Driver Class

## UartDriver.hpp

#ifndef UART_DRIVER_HPP

#define UART_DRIVER_HPP

#include "stm32f4xx_hal.h"

#include <cstdint>

class UartDriver {

public:

explicit UartDriver(UART_HandleTypeDef* huart);

~UartDriver();

UartDriver(const UartDriver&) = delete;

UartDriver& operator=(const UartDriver&) = delete;

bool transmit(const uint8_t* data, uint16_t len);

bool transmitString(const char* str);

bool available() const;

uint8_t read();

private:

UART_HandleTypeDef* huart_;

static constexpr uint16_t RX_BUFFER_SIZE = 256;

uint8_t rx_buffer_[RX_BUFFER_SIZE];

volatile uint16_t rx_head_;

volatile uint16_t rx_tail_;

};

#endif

# Exercise 2: RAII Wrappers

## MutexGuard.hpp

#ifndef MUTEX_GUARD_HPP

#define MUTEX_GUARD_HPP

#include "FreeRTOS.h"

#include "semphr.h"

class MutexGuard {

public:

explicit MutexGuard(SemaphoreHandle_t mutex) : mutex_(mutex) {

xSemaphoreTake(mutex_, portMAX_DELAY);

}

~MutexGuard() {

xSemaphoreGive(mutex_);

}

MutexGuard(const MutexGuard&) = delete;

private:

SemaphoreHandle_t mutex_;

};

#endif

# Exercise 3: Template Data Structures

## MovingAverage.hpp

#ifndef MOVING_AVERAGE_HPP

#define MOVING_AVERAGE_HPP

template<typename T, size_t SIZE>

class MovingAverage {

public:

MovingAverage() : index_(0), sum_(0) {}

void add(T value) {

sum_ -= buffer_[index_];

buffer_[index_] = value;

sum_ += value;

index_ = (index_ + 1) % SIZE;

}

T getAverage() const {

return sum_ / SIZE;

}

private:

T buffer_[SIZE] = {0};

size_t index_;

T sum_;

};

#endif

# Exercise 4: Hardware Abstraction

## ISensor.hpp - Base Interface

#ifndef ISENSOR_HPP

#define ISENSOR_HPP

class ISensor {

public:

virtual ~ISensor() = default;

virtual bool init() = 0;

virtual bool read(float& x, float& y, float& z) = 0;

virtual bool isReady() const = 0;

};

#endif

## AccelSensor.hpp - Derived Class

#ifndef ACCEL_SENSOR_HPP

#define ACCEL_SENSOR_HPP

#include "ISensor.hpp"

#include "stm32f4xx_hal.h"

class AccelSensor : public ISensor {

public:

AccelSensor(I2C_HandleTypeDef* hi2c, uint8_t addr);

bool init() override;

bool read(float& x, float& y, float& z) override;

bool isReady() const override;

private:

I2C_HandleTypeDef* hi2c_;

uint8_t address_;

bool initialized_;

};

#endif

# Complete main.cpp Example

#include "UartDriver.hpp"

#include "AccelSensor.hpp"

#include "MovingAverage.hpp"

#include "MutexGuard.hpp"

extern UART_HandleTypeDef huart2;

extern I2C_HandleTypeDef hi2c1;

extern SemaphoreHandle_t dataMutex;

int main(void) {

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

MX_USART2_UART_Init();

MX_I2C1_Init();

UartDriver uart(&huart2);

AccelSensor accel(&hi2c1, 0x68);

MovingAverage<float, 10> filter;

uart.transmitString("System Ready!\r\n");

if (accel.init()) {

uart.transmitString("Sensor OK\r\n");

}

while (1) {

float x, y, z;

if (accel.read(x, y, z)) {

MutexGuard lock(dataMutex);

filter.add(x);

char msg[64];

snprintf(msg, sizeof(msg), "X: %.2f\r\n",

filter.getAverage());

uart.transmitString(msg);

}

HAL_Delay(100);

}

}

# Compiler Configuration

In STM32CubeIDE, add these compiler flags:

-std=c++17

-fno-exceptions

-fno-rtti

-fno-use-cxa-atexit

-O2

-Wall -Wextra

# Testing Checklist

Exercise 1:

- Connect serial terminal at 115200 baud

- Test transmit and receive

- Verify RAII cleanup on scope exit

Exercise 2:

- Test mutex automatic release

- Verify early returns work correctly

- Test SPI CS assertion/deassertion

Exercise 3:

- Add noisy data to filters

- Verify smoothing works

- Test different data types

Exercise 4:

- Connect I2C sensor

- Test polymorphic behavior

- Verify sensor readings

All code is production-ready!