STM32 DMA Tutorial

Direct Memory Access

Theory, Architecture & Complete Examples

# TABLE OF CONTENTS

Part 1: DMA Fundamentals

1.1 What is DMA?

1.2 Why Use DMA?

1.3 DMA vs CPU Transfer

1.4 When to Use DMA

Part 2: STM32 DMA Architecture

2.1 DMA Controllers

2.2 Channels and Streams

2.3 Transfer Modes

2.4 Priority Levels

2.5 FIFO Mode

Part 3: DMA Configuration

3.1 Configuration Steps

3.2 Register Overview

3.3 Transfer Parameters

3.4 Interrupt Configuration

Part 4: Practical Examples

4.1 Memory-to-Memory Transfer

4.2 ADC with DMA

4.3 UART TX with DMA

4.4 UART RX with DMA

4.5 SPI with DMA

4.6 Double Buffering

Part 5: Advanced Topics

5.1 Circular Mode

5.2 DMA Bursts

5.3 Performance Optimization

5.4 Error Handling

Part 6: Best Practices & Troubleshooting

# PART 1: DMA FUNDAMENTALS

## 1.1 What is DMA?

Definition:

Direct Memory Access (DMA) is a hardware mechanism that allows peripherals to transfer data directly to/from memory WITHOUT CPU involvement.

Key Concept:

DMA controller performs data transfers independently while CPU does other work.

Analogy:

Imagine you need to move boxes from warehouse A to warehouse B:

WITHOUT DMA (CPU does everything):

• You personally carry each box

• You cannot do anything else while moving boxes

• Very inefficient use of your time

WITH DMA (dedicated mover):

• You hire a moving company (DMA controller)

• They move boxes while you do other work

• They notify you when finished (interrupt)

• Much more efficient!

## 1.2 Why Use DMA?

### Reason 1: CPU Efficiency

CPU freed to do other tasks while data transfers happen in background

### Reason 2: Higher Throughput

DMA can transfer data faster than CPU because:

• No instruction fetch/decode overhead

• Optimized for bulk transfers

• Can use burst mode

### Reason 3: Lower Power

CPU can enter low-power mode during transfers

Wake only when transfer complete

### Reason 4: Deterministic Timing

Transfer happens at fixed rate, not affected by CPU load

Important for real-time applications

### Reason 5: Simplified Code

No need to manually copy data in loops

Hardware handles transfer automatically

## 1.3 DMA vs CPU Transfer - Quantitative Comparison

Scenario: Transfer 1000 bytes from UART to memory

### CPU-Based Transfer:

void USART_IRQHandler() {

if (USART->SR & USART_SR_RXNE) {

buffer[index++] = USART->DR;  // Manual copy

}

}

Analysis:

• ISR called 1000 times (once per byte)

• Each ISR: ~50 cycles overhead + 10 cycles work

• Total: 60,000 cycles

• At 180 MHz: 333 microseconds

• CPU busy entire time

### DMA-Based Transfer:

// Setup DMA once

DMA_Stream->M0AR = (uint32_t)buffer;

DMA_Stream->PAR = (uint32_t)&USART->DR;

DMA_Stream->NDTR = 1000;

DMA_Stream->CR |= DMA_SxCR_EN;

// Transfer happens automatically

// CPU does other work

void DMA_IRQHandler() {  // Called ONCE when done

// Transfer complete!

}

Analysis:

• Setup: ~100 cycles

• Transfer: Hardware handles (CPU free!)

• ISR called once: ~50 cycles

• Total CPU cycles: 150 cycles

• CPU utilization: 150 vs 60,000 = 400x improvement!

Result: DMA saves 59,850 CPU cycles = 332 microseconds of CPU time!

## 1.4 When to Use DMA

### Use DMA When:

✓ Transferring blocks of data (> 10 bytes typically)

✓ Continuous data streams (ADC, audio, video)

✓ High-speed communication (SPI, UART at high baud)

✓ CPU needs to do other critical work

✓ Deterministic timing required

✓ Low power consumption important

### Do NOT Use DMA When:

✗ Transferring single bytes sporadically

✗ Data needs processing during transfer

✗ Simple, low-frequency transfers

✗ Setup overhead > transfer time

### Real-World Applications:

• ADC: Sample multiple channels continuously

• UART: Send/receive large buffers

• SPI: Transfer to SD card, display, flash memory

• I2C: Read sensor arrays

• Audio: Stream from DAC/ADC

• Memory: Copy large arrays

# PART 2: STM32 DMA ARCHITECTURE

## 2.1 DMA Controllers

STM32F4 has TWO DMA Controllers:

### DMA1 (8 Streams)

Connected to APB1 peripherals:

• SPI2, SPI3

• I2C1, I2C2, I2C3

• USART2, USART3, UART4, UART5

• TIM2, TIM3, TIM4, TIM5, TIM6, TIM7

• DAC

### DMA2 (8 Streams)

Connected to APB2 peripherals and AHB:

• SPI1, SPI4, SPI5, SPI6

• USART1, USART6

• ADC1, ADC2, ADC3

• SDIO

• TIM1, TIM8

• Memory-to-Memory transfers

Important: Only DMA2 supports Memory-to-Memory transfers!

## 2.2 Channels and Streams

Each DMA Controller has 8 STREAMS:

Stream = Independent DMA transfer unit

Each stream can be active simultaneously

Each Stream has 8 CHANNELS:

Channel = Multiplexer selecting peripheral

Only ONE channel active per stream at a time

Example: DMA2 Stream 0

Channel 0: SPI1_RX

Channel 1: SPI4_RX

Channel 2: SPI5_RX

Channel 3: I2C1_RX

Channel 4: SPI6_RX

Channel 5: I2C1_RX

Channel 6: TIM1_TRIG

Channel 7: -

You select one channel to use Stream 0 for that peripheral.

Stream Selection Table:

Reference Manual has table showing which stream/channel for each peripheral.

Example: ADC1 can use DMA2 Stream 0 or Stream 4 (both Channel 0)

## 2.3 Transfer Modes

### 1. Normal Mode

Transfer happens once, then stops

Use for: One-time transfers, sending buffers

DMA transfers NDTR items

NDTR counts down to 0

Transfer complete interrupt fires

DMA stops

### 2. Circular Mode

Transfer repeats continuously

When NDTR reaches 0, it reloads and continues

Use for: Continuous ADC sampling, audio streaming

DMA transfers NDTR items

NDTR reaches 0 → Transfer complete interrupt

NDTR reloads automatically

Transfer continues indefinitely

### 3. Double Buffer Mode

Two memory buffers alternate

While DMA fills buffer 1, CPU processes buffer 0

Then swap: DMA fills buffer 0, CPU processes buffer 1

Use for: Real-time data processing

## 2.4 Priority Levels

When multiple streams request bus access, priority determines order.

Four Priority Levels:

0: Low

1: Medium

2: High

3: Very High

If same priority: Lower stream number wins

Example: Stream 0 (High) beats Stream 5 (High)

Guidelines:

• High priority: Time-critical (ADC, audio)

• Medium priority: Normal communication (UART, SPI)

• Low priority: Background tasks (memory copies)

## 2.5 FIFO Mode

Direct Mode (No FIFO):

Each peripheral request triggers one transfer

Simple but can cause bus contention

FIFO Mode (with 4-word buffer):

DMA accumulates data in FIFO before transferring

Allows burst transfers (more efficient)

Reduces bus usage

FIFO Threshold Levels:

1/4 full:  1 word  (4 bytes)

1/2 full:  2 words (8 bytes)

3/4 full:  3 words (12 bytes)

Full:      4 words (16 bytes)

When threshold reached, DMA performs burst transfer.

When to use FIFO:

✓ High-speed transfers

✓ Memory-to-Memory

✓ When burst mode beneficial

# PART 3: DMA CONFIGURATION

## 3.1 Configuration Steps

Step 1: Enable DMA Clock

RCC->AHB1ENR |= RCC_AHB1ENR_DMA1EN;  // or DMA2EN

Step 2: Disable Stream (if already enabled)

DMA1_Stream0->CR &= ~DMA_SxCR_EN;

while(DMA1_Stream0->CR & DMA_SxCR_EN);  // Wait

Step 3: Configure Stream

DMA1_Stream0->CR = 0;  // Clear all

DMA1_Stream0->CR |= (channel << 25);  // Channel

DMA1_Stream0->CR |= (priority << 16); // Priority

DMA1_Stream0->CR |= DMA_SxCR_MINC;    // Mem increment

// ... more configuration

Step 4: Set Addresses

DMA1_Stream0->PAR = (uint32_t)&peripheral->DR;

DMA1_Stream0->M0AR = (uint32_t)buffer;

Step 5: Set Number of Data

DMA1_Stream0->NDTR = buffer_size;

Step 6: Configure Interrupts (optional)

DMA1_Stream0->CR |= DMA_SxCR_TCIE;  // Transfer complete

NVIC_EnableIRQ(DMA1_Stream0_IRQn);

Step 7: Enable Stream

DMA1_Stream0->CR |= DMA_SxCR_EN;

Step 8: Enable Peripheral DMA Request

USART2->CR3 |= USART_CR3_DMAT;  // For UART TX

USART2->CR3 |= USART_CR3_DMAR;  // For UART RX

## 3.2 Key Registers

### DMA_SxCR (Control Register)

Bits 25-27: CHSEL (Channel selection)

Bits 16-17: PL (Priority level)

Bits 13-14: MSIZE (Memory data size)

Bits 11-12: PSIZE (Peripheral data size)

Bit 10: MINC (Memory increment)

Bit 9:  PINC (Peripheral increment)

Bit 8:  CIRC (Circular mode)

Bits 6-7:  DIR (Direction)

Bit 4:  TCIE (Transfer complete interrupt)

Bit 0:  EN (Enable)

### DMA_SxNDTR (Number of Data Register)

Bits 0-15: NDT (Number of data to transfer)

Counts down during transfer

Reload in circular mode

### DMA_SxPAR (Peripheral Address Register)

32-bit address of peripheral data register

Example: &USART2->DR, &ADC1->DR

### DMA_SxM0AR (Memory Address Register)

32-bit address of memory buffer

Example: &myBuffer[0]

## 3.3 Transfer Parameters

### Direction (DIR bits):

00: Peripheral-to-Memory (e.g., ADC → Buffer)

01: Memory-to-Peripheral (e.g., Buffer → UART)

10: Memory-to-Memory (DMA2 only!)

### Data Size (MSIZE, PSIZE):

00: Byte (8-bit)

01: Half-word (16-bit)

10: Word (32-bit)

Important: Sizes must match or be compatible!

Example: ADC is 16-bit → Use half-word

### Increment Mode (MINC, PINC):

MINC = 1: Memory address increments after each transfer

PINC = 1: Peripheral address increments

Typical:

• MINC = 1 (buffer advances)

• PINC = 0 (peripheral register stays same)

## 3.4 Interrupt Configuration

Available Interrupts:

### Transfer Complete (TCIE)

Fires when NDTR reaches 0

Most commonly used

DMA_SxCR |= DMA_SxCR_TCIE;

### Half Transfer (HTIE)

Fires when NDTR reaches half

Useful for double buffering

DMA_SxCR |= DMA_SxCR_HTIE;

### Transfer Error (TEIE)

Fires on bus error or configuration error

DMA_SxCR |= DMA_SxCR_TEIE;

### Direct Mode Error (DMEIE)

Fires on FIFO error

DMA_SxCR |= DMA_SxCR_DMEIE;

Interrupt Handler:

void DMA1_Stream0_IRQHandler() {

if (DMA1->LISR & DMA_LISR_TCIF0) {

DMA1->LIFCR = DMA_LIFCR_CTCIF0;  // Clear flag

// Transfer complete!

}

}

# PART 4: PRACTICAL EXAMPLES

## 4.1 Example 1: Memory-to-Memory Transfer

Use Case: Copy array to another array

uint32_t source[1000];

uint32_t destination[1000];

void dma_mem_to_mem() {

// Enable DMA2 clock (only DMA2 does mem-to-mem)

RCC->AHB1ENR |= RCC_AHB1ENR_DMA2EN;

// Disable stream

DMA2_Stream0->CR = 0;

// Configure

DMA2_Stream0->CR =

(0 << 25) |           // Channel 0

(3 << 16) |           // Very high priority

(2 << 13) |           // MSIZE = 32-bit

(2 << 11) |           // PSIZE = 32-bit

DMA_SxCR_MINC |       // Memory increment

DMA_SxCR_PINC |       // Peripheral increment

(2 << 6) |            // DIR = Memory-to-Memory

DMA_SxCR_TCIE;        // Transfer complete IRQ

// Set addresses

DMA2_Stream0->PAR = (uint32_t)source;

DMA2_Stream0->M0AR = (uint32_t)destination;

// Number of items (not bytes!)

DMA2_Stream0->NDTR = 1000;

// Enable interrupt

NVIC_EnableIRQ(DMA2_Stream0_IRQn);

// Start transfer

DMA2_Stream0->CR |= DMA_SxCR_EN;

}

void DMA2_Stream0_IRQHandler() {

if (DMA2->LISR & DMA_LISR_TCIF0) {

DMA2->LIFCR = DMA_LIFCR_CTCIF0;

printf("Transfer complete!\n");

}

}

## 4.2 Example 2: ADC with DMA (Continuous)

Use Case: Read 3 ADC channels continuously

uint16_t adcBuffer[3];  // Store ADC results

void adc_dma_init() {

// Enable clocks

RCC->AHB1ENR |= RCC_AHB1ENR_DMA2EN;

RCC->APB2ENR |= RCC_APB2ENR_ADC1EN;

RCC->AHB1ENR |= RCC_AHB1ENR_GPIOAEN;

// Configure GPIO (PA0, PA1, PA4 as analog)

GPIOA->MODER |= (3<<0) | (3<<2) | (3<<8);

// Configure ADC

ADC1->CR2 = 0;

ADC1->SQR1 = (3-1) << 20;  // 3 conversions

ADC1->SQR3 = (0<<0) | (1<<5) | (4<<10);  // Ch 0,1,4

ADC1->CR2 = ADC_CR2_ADON |  // Enable

ADC_CR2_DMA |   // DMA mode

ADC_CR2_DDS |   // DMA continuous

ADC_CR2_CONT;   // Continuous conversion

// Configure DMA2 Stream 0 (ADC1)

DMA2_Stream0->CR = 0;

DMA2_Stream0->CR =

(0 << 25) |           // Channel 0 (ADC1)

(2 << 16) |           // High priority

(1 << 13) |           // MSIZE = 16-bit

(1 << 11) |           // PSIZE = 16-bit

DMA_SxCR_MINC |       // Memory increment

DMA_SxCR_CIRC |       // Circular mode

(0 << 6) |            // Peripheral-to-Memory

DMA_SxCR_TCIE;        // Transfer complete IRQ

DMA2_Stream0->PAR = (uint32_t)&ADC1->DR;

DMA2_Stream0->M0AR = (uint32_t)adcBuffer;

DMA2_Stream0->NDTR = 3;

NVIC_EnableIRQ(DMA2_Stream0_IRQn);

// Start DMA

DMA2_Stream0->CR |= DMA_SxCR_EN;

// Start ADC

ADC1->CR2 |= ADC_CR2_SWSTART;

}

void DMA2_Stream0_IRQHandler() {

if (DMA2->LISR & DMA_LISR_TCIF0) {

DMA2->LIFCR = DMA_LIFCR_CTCIF0;

// All 3 channels converted!

float ch0 = adcBuffer[0] * 3.3f / 4096.0f;

float ch1 = adcBuffer[1] * 3.3f / 4096.0f;

float ch2 = adcBuffer[2] * 3.3f / 4096.0f;

// Process data...

}

}

Result: ADC samples continuously, DMA updates buffer, CPU notified when ready!

## 4.3 Example 3: UART TX with DMA

Use Case: Send large buffer over UART

char txBuffer[256] = "Hello from DMA UART!\r\n";

void uart_dma_tx_init() {

// Enable clocks

RCC->AHB1ENR |= RCC_AHB1ENR_DMA1EN;

RCC->APB1ENR |= RCC_APB1ENR_USART2EN;

RCC->AHB1ENR |= RCC_AHB1ENR_GPIOAEN;

// Configure GPIO (PA2 = TX)

GPIOA->MODER &= ~(3 << 4);

GPIOA->MODER |= (2 << 4);  // Alternate function

GPIOA->AFR[0] &= ~(0xF << 8);

GPIOA->AFR[0] |= (7 << 8);  // AF7 = USART2

// Configure UART

USART2->BRR = 45000000 / 115200;

USART2->CR3 = USART_CR3_DMAT;  // Enable TX DMA

USART2->CR1 = USART_CR1_UE | USART_CR1_TE;

// Configure DMA1 Stream 6 (USART2_TX)

DMA1_Stream6->CR = 0;

DMA1_Stream6->CR =

(4 << 25) |           // Channel 4 (USART2_TX)

(1 << 16) |           // Medium priority

(0 << 13) |           // MSIZE = 8-bit

(0 << 11) |           // PSIZE = 8-bit

DMA_SxCR_MINC |       // Memory increment

(1 << 6) |            // Memory-to-Peripheral

DMA_SxCR_TCIE;        // Transfer complete IRQ

DMA1_Stream6->PAR = (uint32_t)&USART2->DR;

DMA1_Stream6->M0AR = (uint32_t)txBuffer;

DMA1_Stream6->NDTR = 23;  // String length

NVIC_EnableIRQ(DMA1_Stream6_IRQn);

// Start transfer

DMA1_Stream6->CR |= DMA_SxCR_EN;

}

void DMA1_Stream6_IRQHandler() {

if (DMA1->HISR & DMA_HISR_TCIF6) {

DMA1->HIFCR = DMA_HIFCR_CTCIF6;

printf("TX complete!\n");

}

}

## 4.4 Example 4: UART RX with DMA (Circular)

Use Case: Receive continuous data stream

#define RX_BUFFER_SIZE 256

uint8_t rxBuffer[RX_BUFFER_SIZE];

void uart_dma_rx_init() {

// Clock and GPIO setup (similar to TX)

// ...

// Configure UART RX

USART2->CR3 = USART_CR3_DMAR;  // Enable RX DMA

USART2->CR1 = USART_CR1_UE | USART_CR1_RE;

// Configure DMA1 Stream 5 (USART2_RX)

DMA1_Stream5->CR = 0;

DMA1_Stream5->CR =

(4 << 25) |           // Channel 4 (USART2_RX)

(1 << 16) |           // Medium priority

(0 << 13) |           // MSIZE = 8-bit

(0 << 11) |           // PSIZE = 8-bit

DMA_SxCR_MINC |       // Memory increment

DMA_SxCR_CIRC |       // Circular mode!

(0 << 6) |            // Peripheral-to-Memory

DMA_SxCR_TCIE;        // Transfer complete IRQ

DMA1_Stream5->PAR = (uint32_t)&USART2->DR;

DMA1_Stream5->M0AR = (uint32_t)rxBuffer;

DMA1_Stream5->NDTR = RX_BUFFER_SIZE;

NVIC_EnableIRQ(DMA1_Stream5_IRQn);

// Start receiving

DMA1_Stream5->CR |= DMA_SxCR_EN;

}

void DMA1_Stream5_IRQHandler() {

if (DMA1->HISR & DMA_HISR_TCIF5) {

DMA1->HIFCR = DMA_HIFCR_CTCIF5;

// Buffer full! Process data

// NDTR auto-reloads, continues receiving

}

}

Reading Data During Reception:

// Check how much data received

uint16_t bytesRemaining = DMA1_Stream5->NDTR;

uint16_t bytesReceived = RX_BUFFER_SIZE - bytesRemaining;

## 4.5 Example 5: SPI with DMA (TX and RX)

Use Case: Transfer large data to SPI device

uint8_t txData[100];  // Data to send

uint8_t rxData[100];  // Data received

void spi_dma_init() {

// Enable clocks

RCC->AHB1ENR |= RCC_AHB1ENR_DMA2EN;

RCC->APB2ENR |= RCC_APB2ENR_SPI1EN;

// Configure SPI1

SPI1->CR1 = SPI_CR1_MSTR |   // Master

SPI_CR1_SSM |    // Software SS

SPI_CR1_SSI |    // Internal SS high

(3 << 3);        // BR = /16

// Enable SPI TX and RX DMA

SPI1->CR2 = SPI_CR2_TXDMAEN | SPI_CR2_RXDMAEN;

// Configure DMA2 Stream 3 (SPI1_TX)

DMA2_Stream3->CR = 0;

DMA2_Stream3->CR =

(3 << 25) |           // Channel 3 (SPI1_TX)

(1 << 16) |           // Medium priority

DMA_SxCR_MINC |       // Memory increment

(1 << 6);             // Memory-to-Peripheral

DMA2_Stream3->PAR = (uint32_t)&SPI1->DR;

DMA2_Stream3->M0AR = (uint32_t)txData;

DMA2_Stream3->NDTR = 100;

// Configure DMA2 Stream 0 (SPI1_RX)

DMA2_Stream0->CR = 0;

DMA2_Stream0->CR =

(3 << 25) |           // Channel 3 (SPI1_RX)

(1 << 16) |           // Medium priority

DMA_SxCR_MINC |       // Memory increment

(0 << 6) |            // Peripheral-to-Memory

DMA_SxCR_TCIE;        // Transfer complete IRQ

DMA2_Stream0->PAR = (uint32_t)&SPI1->DR;

DMA2_Stream0->M0AR = (uint32_t)rxData;

DMA2_Stream0->NDTR = 100;

NVIC_EnableIRQ(DMA2_Stream0_IRQn);

// Start RX first, then TX

DMA2_Stream0->CR |= DMA_SxCR_EN;

DMA2_Stream3->CR |= DMA_SxCR_EN;

// Enable SPI

SPI1->CR1 |= SPI_CR1_SPE;

}

## 4.6 Example 6: Double Buffering

Use Case: Process data while next batch arrives

uint16_t buffer0[512];

uint16_t buffer1[512];

volatile bool useBuffer0 = true;

void double_buffer_adc_init() {

// ADC and DMA setup...

// Enable double buffer mode

DMA2_Stream0->CR |= DMA_SxCR_DBM;

// Set both memory addresses

DMA2_Stream0->M0AR = (uint32_t)buffer0;

DMA2_Stream0->M1AR = (uint32_t)buffer1;

// Enable half and full transfer interrupts

DMA2_Stream0->CR |= DMA_SxCR_HTIE | DMA_SxCR_TCIE;

// Start transfer

DMA2_Stream0->CR |= DMA_SxCR_EN;

ADC1->CR2 |= ADC_CR2_SWSTART;

}

void DMA2_Stream0_IRQHandler() {

if (DMA2->LISR & DMA_LISR_HTIF0) {

DMA2->LIFCR = DMA_LIFCR_CHTIF0;

// Half transfer: Process inactive buffer

if (DMA2_Stream0->CR & DMA_SxCR_CT) {

// Currently using M1, process M0

processData(buffer0, 512);

} else {

// Currently using M0, process M1

processData(buffer1, 512);

}

}

if (DMA2->LISR & DMA_LISR_TCIF0) {

DMA2->LIFCR = DMA_LIFCR_CTCIF0;

// Full transfer: Process other buffer

if (DMA2_Stream0->CR & DMA_SxCR_CT) {

processData(buffer1, 512);

} else {

processData(buffer0, 512);

}

}

}

# PART 5: ADVANCED TOPICS

## 5.1 Circular Mode Details

How It Works:

1. DMA transfers NDTR items

2. NDTR reaches 0 → Transfer Complete interrupt

3. NDTR automatically reloads to original value

4. Transfer continues without stopping

Use Cases:

• Continuous ADC sampling

• Audio streaming (I2S)

• Continuous UART reception

• Periodic data logging

Important Notes:

• Must manually stop DMA when done

• Be careful of buffer overrun

• Track position using NDTR register

## 5.2 Burst Mode

What is Burst Mode?

DMA transfers multiple data items in one bus transaction

More efficient use of bus bandwidth

Burst Sizes:

MBURST/PBURST bits:

00: Single transfer

01: 4-beat burst (INCR4)

10: 8-beat burst (INCR8)

11: 16-beat burst (INCR16)

When to Use:

✓ High-speed memory-to-memory

✓ Large block transfers

✓ When FIFO enabled

Configuration:

DMA2_Stream0->CR |= (1 << 23);  // MBURST = 4-beat

DMA2_Stream0->CR |= (1 << 21);  // PBURST = 4-beat

## 5.3 Performance Optimization

### Tip 1: Use Appropriate Data Width

Match peripheral data size

ADC: 16-bit → MSIZE = PSIZE = 01 (half-word)

UART: 8-bit → MSIZE = PSIZE = 00 (byte)

### Tip 2: Enable FIFO for Large Transfers

DMA2_Stream0->FCR |= DMA_SxFCR_DMDIS;  // Enable FIFO

DMA2_Stream0->FCR |= (3 << 0);  // Threshold: Full

### Tip 3: Set Appropriate Priority

Time-critical: High or Very High

Background: Low or Medium

### Tip 4: Align Memory Buffers

// 4-byte aligned for 32-bit transfers

__attribute__((aligned(4))) uint32_t buffer[1000];

### Tip 5: Use Direct Mode for Small Transfers

FIFO overhead not worth it for < 4 words

## 5.4 Error Handling

### Transfer Error Interrupt

if (DMA1->LISR & DMA_LISR_TEIF0) {

DMA1->LIFCR = DMA_LIFCR_CTEIF0;

// Handle error:

// - Check addresses (valid?)

// - Check sizes (match?)

// - Check bus access

}

### FIFO Error

if (DMA1->LISR & DMA_LISR_FEIF0) {

DMA1->LIFCR = DMA_LIFCR_CFEIF0;

// FIFO overrun or underrun

// Adjust threshold or disable FIFO

}

### Direct Mode Error

if (DMA1->LISR & DMA_LISR_DMEIF0) {

DMA1->LIFCR = DMA_LIFCR_CDMEIF0;

// Peripheral sent data but FIFO full

}

# PART 6: BEST PRACTICES

## 6.1 Configuration Checklist

✓ Enable DMA and peripheral clocks

✓ Disable stream before configuration

✓ Set correct channel for peripheral

✓ Configure data sizes (MSIZE, PSIZE)

✓ Set increment modes (MINC, PINC)

✓ Set correct direction (DIR)

✓ Configure addresses (PAR, M0AR)

✓ Set transfer count (NDTR)

✓ Enable peripheral DMA request

✓ Enable stream last

## 6.2 Common Mistakes

Mistake 1: Wrong Stream/Channel

Check reference manual table!

Example: ADC1 must use DMA2 Stream 0 or 4, Channel 0

Mistake 2: Data Size Mismatch

// BAD: ADC is 16-bit but configured as 8-bit

DMA->CR |= (0 << 13);  // WRONG!

// GOOD: Match ADC size

DMA->CR |= (1 << 13);  // 16-bit

Mistake 3: Forgetting Peripheral DMA Enable

// UART TX: Must enable DMAT

USART2->CR3 |= USART_CR3_DMAT;

// ADC: Must enable DMA and DDS

ADC1->CR2 |= ADC_CR2_DMA | ADC_CR2_DDS;

Mistake 4: Not Clearing Interrupt Flags

// MUST clear in ISR!

DMA1->LIFCR = DMA_LIFCR_CTCIF0;

Mistake 5: Buffer Alignment

32-bit transfers need 4-byte aligned buffers

## 6.3 Debugging Tips

### DMA Not Transferring?

1. Check if stream enabled (EN bit set)

2. Verify clock enabled

3. Check peripheral DMA request enabled

4. Verify addresses are valid

5. Check NDTR is non-zero

### Transfer Incomplete?

1. Check NDTR value - shows remaining items

2. Verify data sizes match

3. Check for transfer errors

### Interrupt Not Firing?

1. Verify interrupt enabled in DMA_CR

2. Check NVIC enabled

3. Check correct IRQ handler name

4. Verify flags being cleared

# SUMMARY

DMA Benefits:

• Frees CPU for other tasks

• Higher throughput than CPU transfer

• Lower power consumption

• Deterministic timing

• Simplified code

Key Concepts:

• DMA1 and DMA2 controllers

• 8 streams per controller

• 8 channels per stream

• Normal, Circular, Double Buffer modes

• Priority levels (0-3)

• FIFO mode for burst transfers

Configuration Steps:

1. Enable clocks

2. Disable stream

3. Configure stream (channel, priority, sizes)

4. Set addresses

5. Set count

6. Enable interrupts

7. Enable stream

8. Enable peripheral DMA

Common Uses:

• ADC continuous sampling

• UART large transfers

• SPI flash read/write

• Audio streaming

• Memory copies

Master DMA for efficient STM32 programming!