STM32 Interrupts - Theory & Architecture

From CPU Architecture to NVIC Implementation

# PART 1: INTERRUPT FUNDAMENTALS

## 1.1 What is an Interrupt?

Formal Definition:

An interrupt is an asynchronous signal that causes the processor to:

• Save its current execution context (PC, registers, flags)

• Jump to a predetermined address (ISR/Interrupt Vector)

• Execute interrupt service routine

• Restore context and resume interrupted execution

## 1.2 Interrupt Mechanism

Normal CPU Execution:

Fetch → Decode → Execute → Write

With Interrupt:

Fetch → [Check IRQ?] → Decode → Execute

↓ (if IRQ pending)

1. Save PC & Registers

2. Load ISR address

3. Execute ISR

4. Restore context

5. Resume execution

## 1.3 Interrupt Types

1. Hardware Interrupts (External)

• Generated by peripheral devices

• Examples: Timer overflow, UART byte received, GPIO pin change

• Asynchronous to CPU execution

• Most common in embedded systems

2. Software Interrupts

• Generated by executing special instruction (SWI, INT)

• Synchronous to CPU execution

• Used for system calls, debugging

3. Exceptions (Faults)

• Generated by CPU itself

• Examples: Division by zero, memory access fault

• Synchronous to instruction causing it

# PART 2: WHY INTERRUPTS?

## 2.1 The Polling Problem

Example: Checking for UART Data

while(1) {

if (UART_has_data()) {

process_data();

}

// CPU wastes cycles checking!

}

Analysis:

• UART receives 1 byte every 1ms (at 9600 baud)

• CPU runs at 180 MHz (180 million instructions/second)

• Between bytes: 180,000 instructions wasted checking!

• CPU Utilization: ~99.999% wasted on polling

• Power: CPU cannot sleep, high power consumption

## 2.2 The Interrupt Solution

void UART_ISR() {

process_data();

}

int main() {

while(1) {

sleep();  // Low-power mode

}

}

Analysis:

• CPU sleeps until UART interrupt

• Wakes up, processes byte (~100 instructions)

• Returns to sleep

• CPU Utilization: ~0.0005% active

• Power: Dramatically reduced (10-100x less)

## 2.3 Quantitative Comparison

Scenario: Monitor button press (every 5 seconds)

POLLING:

CPU: 180 MHz

Polling Rate: 1000 Hz

Instructions per check: 10

CPU cycles/sec: 10,000

Current: ~50 mA (active)

INTERRUPTS:

Button frequency: 0.2 Hz

ISR execution: ~5 µs

CPU cycles/sec: 180

Current: ~10 µA (sleep)

POWER SAVINGS: 5000x improvement!

Battery life: Days → Years

# PART 3: INTERRUPT LATENCY

## 3.1 Definition

Interrupt latency = Time from interrupt signal to first ISR instruction

## 3.2 Latency Components (Cortex-M)

1. Interrupt Detection: 1-2 cycles

2. Pipeline Flush: 0-12 cycles

3. Context Save: 8-12 cycles

4. Vector Fetch: 2 cycles

5. ISR Entry: 2-4 cycles

TOTAL: 13-32 clock cycles

At 180 MHz: 72-178 nanoseconds

## 3.3 Response Time vs Latency

LATENCY: Hardware delay (fixed)

RESPONSE TIME: Latency + time until event processed

Response time affected by:

• Current interrupt executing

• Interrupt disabled (critical section)

• ISR code before handling event

# PART 4: INTERRUPT OVERHEAD

## 4.1 Cost of Using Interrupts

Each interrupt has cost:

Entry/Exit Overhead: ~25-40 cycles

• Context save: 12 cycles

• ISR execution: variable

• Context restore: 12 cycles

• Return: 2 cycles

## 4.2 Overhead Analysis

Example 1: Very Short ISR (BAD)

ISR does 5 instructions:

Overhead: 40 cycles

Work: 5 cycles

Percentage: 88% overhead

Result: Consider polling instead

Example 2: Reasonable ISR (GOOD)

ISR does 100 instructions:

Overhead: 40 cycles

Work: 100 cycles

Percentage: 28% overhead

Result: Interrupt justified

# PART 5: STM32 NVIC ARCHITECTURE

## 5.1 NVIC Features

• Up to 240 interrupt channels

• 16 programmable priority levels (0-15)

• Nested interrupts (preemption)

• Automatic context save/restore

• Tail-chaining optimization

• Late arrival optimization

## 5.2 Priority System

CRITICAL: Priority 0 = HIGHEST, 15 = LOWEST

Two-Level Priority:

1. PREEMPT PRIORITY - Can interrupt lower

2. SUB-PRIORITY - Determines order when pending

## 5.3 Priority Grouping

Group 0: 0.4 → 1 preempt, 16 sub

Group 2: 2.2 → 4 preempt, 4 sub (balanced)

Group 4: 4.0 → 16 preempt, 1 sub (common)

## 5.4 Preemption Example

t=0: Main running

t=1: ISR_D (priority 15) starts

t=2: ISR_B (priority 3) pending

t=3: ISR_B PREEMPTS ISR_D

t=4: ISR_B completes

t=5: ISR_D resumes

t=6: ISR_D completes

t=7: Main resumes

## 5.5 Optimizations

Tail-Chaining:

Skip context restore/save between ISRs

Result: 4x faster back-to-back execution

Late Arrival:

Abort low-priority ISR if high-priority arrives

Saves ~12 cycles

# PART 6: PRIORITY ASSIGNMENT

### Priority 0-3: CRITICAL

• System tick timer

• Safety sensors

• Real-time control

### Priority 4-7: HIGH

• User input

• Fast communication

• Motor control

### Priority 8-11: MEDIUM

• UART

• Sensor readings

• Display updates

### Priority 12-15: LOW

• Status LEDs

• Logging

• Housekeeping

Example System:

TIM6 (1ms):   Priority 0

EXTI (Button): Priority 4

USART2:        Priority 8

TIM7 (LED):    Priority 12

# PART 7: INTERRUPTS vs POLLING

## Use Polling When:

✓ Very high frequency (> 10 kHz)

✓ Predictable, regular events

✓ Simple systems

✓ Minimal latency required

## Use Interrupts When:

✓ Sporadic events (< 1 kHz)

✓ Unpredictable timing

✓ Low power needed

✓ Multiple event sources

✓ CPU does other work

## Hybrid Approach:

Use both!

• Interrupt triggers when ready

• Polling reads burst of data

Example: UART IDLE interrupt + FIFO polling

# SUMMARY

Key Concepts:

• Interrupts = asynchronous event handling

• Power savings: 1000x-5000x vs polling

• Latency: 72-178 ns on STM32F4

• NVIC: 16 priorities with preemption

• Priority 0 = highest, 15 = lowest

• Tail-chaining + late arrival optimize

Design Guidelines:

• Assign priorities by criticality

• Keep ISRs short (< 10 µs)

• Use interrupts for sporadic events

• Use polling for high-frequency

• Consider power requirements

Common Applications:

• Button/sensor (GPIO EXTI)

• Periodic tasks (Timers)

• Communication (UART, SPI, I2C)

• DMA completion

• ADC conversion complete

Master interrupts for efficient embedded systems!