SMART SHOE WEARABLE PLATFORM

4-Layer Carrier PCB

Design Reference Document

─────────────────────────────────────────────────

| Document No. | MADEPLUS-PCB-001-RevA |
| --- | --- |
| Board Dimensions | 55 mm × 40 mm × 1.6 mm |
| Layer Count | 4 (L1 Signal / L2 GND / L3 Power / L4 Signal) |
| Modules | PowerBoost 500C + ESP32-S3 Zero + GY-86 |
| Date | March 5, 2026 |
| Author | Ratneswar Roychowdhury — MADEPLUS INC |
| Status | Released for Fabrication |

MADEPLUS INC  |  Parole, Maryland  |  Confidential

# 1.  Project Overview

This document covers the complete design specification for the 4-Layer Carrier PCB developed as part of the Smart Shoe Gait Analysis Platform. The carrier PCB integrates three off-the-shelf modules — the Adafruit PowerBoost 500C, the Waveshare ESP32-S3 Zero, and the GY-86 10-DOF IMU — through a dual-sided plug-in socket architecture. All inter-module connections, power distribution, and passive components are pre-routed on the carrier, requiring no hand wiring after module installation.

## 1.1  System Purpose

The platform enables real-time gait analysis and dead-reckoning navigation for a smart shoe application. The ESP32-S3 executes sensor fusion algorithms (complementary filter + ZUPT-based dead reckoning) on data from the GY-86 inertial measurement unit and transmits processed stride metrics over BLE to a companion Flutter mobile application. The PowerBoost 500C provides regulated 5V from a single-cell LiPo battery with onboard USB charging.

## 1.2  Module Summary

| Module | Part | Location | Interface | Supply |
| --- | --- | --- | --- | --- |
| Microcontroller | Waveshare ESP32-S3 Zero | TOP — L1 | USB-C, BLE, I²C | 5V → onboard LDO → 3V3 |
| IMU | GY-86 (MPU6050 + HMC5883L + MS5611) | TOP — L1 | I²C (0x68/0x1E/0x77) | 3V3 from ESP32 |
| Power | Adafruit PowerBoost 500C | BOTTOM — L4 | Micro-USB (charge) | 3.7V LiPo → 5V boost |
| Battery | YTKavq 3.7V 300mAh JST PH 2.0mm | External (edge conn.) | JST PH 2.0mm | 3.7V LiPo |

# 2.  Board Specifications

## 2.1  Physical

| Parameter | Value |
| --- | --- |
| Board Length (X) | 55 mm |
| Board Width (Y) | 40 mm |
| Board Thickness | 1.6 mm ± 10% |
| Base Material | FR4, Tg150 |
| Corner Radius | 2.0 mm |
| Mounting Holes | 4× M2 (2.2 mm dia) at corners, no copper |
| Weight (bare PCB, est.) | ~12 g |

## 2.2  Layer Stack-Up

| Layer | Function | Copper Weight | Thickness | Notes |
| --- | --- | --- | --- | --- |
| L1 — F.Cu | Signal Top | 1 oz (35 µm) | 35 µm | ESP32-S3 + GY-86 sockets, I²C traces, 3V3 dist. |
| Prepreg 7628 | — | — | 0.20 mm | Dielectric between L1 and L2 |
| L2 — In1.Cu | GND Plane | 1 oz (35 µm) | 35 µm | Solid copper pour, full board extent |
| Core FR4 | — | — | 0.80 mm | Structural core |
| L3 — In2.Cu | Power Plane | 1 oz (35 µm) | 35 µm | 5V island (left) + 3V3 island (right) |
| Prepreg 7628 | — | — | 0.20 mm | Dielectric between L3 and L4 |
| L4 — B.Cu | Signal Bottom | 1 oz (35 µm) | 35 µm | PowerBoost socket, EN/LBO, BAT/USB traces |
| Total |  |  | 1.6 mm |  |

## 2.3  Design Rules

| Rule | Value |
| --- | --- |
| Minimum trace width (signal) | 0.20 mm |
| Minimum trace width (5V/GND) | 1.60 mm |
| Minimum trace width (3V3) | 0.80 mm |
| Minimum trace width (I²C) | 0.35 mm |
| Minimum clearance (trace–trace) | 0.20 mm |
| Minimum via drill | 0.30 mm |
| Via annular ring | 0.75 mm outer pad |
| Minimum hole (TH socket) | 0.80 mm |
| Copper to edge clearance | 0.30 mm |

## 2.4  Surface Finish & Solder Mask

| Item | Specification |
| --- | --- |
| Surface finish | ENIG (Electroless Nickel Immersion Gold) |
| Solder mask — top | Green LPI, both layers |
| Solder mask — bottom | Green LPI |
| Silkscreen — top | White (component references, orientation marks) |
| Silkscreen — bottom | White (module zone outlines, connector polarity) |
| Solder mask expansion | +0.10 mm (0.05 mm per side) |

# 3.  Connector Specifications

All three external connectors are accessible from the board edges. The PCB includes precision edge slot cutouts machined into the board outline so that USB connectors protrude flush with the carrier edge while their parent modules sit flat on their respective sockets.

## 3.1  USB-C — ESP32-S3 Zero (Top Edge)

| Parameter | Detail |
| --- | --- |
| Type | USB Type-C receptacle |
| Location | Top edge of carrier PCB |
| PCB slot size | 9 mm wide × 3.5 mm deep |
| Purpose | Firmware upload (esptool), serial debug (UART0), USB OTG |
| Connected to | Waveshare ESP32-S3 Zero onboard USB-C port |
| Notes | Not for power input to the system. No VBUS routing on carrier. |

## 3.2  Micro-USB — PowerBoost 500C (Bottom Edge)

| Parameter | Detail |
| --- | --- |
| Type | USB Micro-B receptacle |
| Location | Bottom edge of carrier PCB |
| PCB slot size | 6 mm wide × 3.0 mm deep |
| Purpose | LiPo battery charging only (MCP73831, 500 mA max) |
| Connected to | PowerBoost 500C USB pin |
| Notes | This is the CHARGER input. NOT the 5V system output port. |

⚠  Charging Caution

Only connect the Micro-USB port to a standard 5V USB charger or power bank. Never connect a voltage above 5.25V. The MCP73831 charger IC does not have overvoltage input protection.

## 3.3  JST PH 2.0mm — LiPo Battery (Right Edge)

| Parameter | Detail |
| --- | --- |
| Type | JST PH 2-pin, 2.0 mm pitch |
| Location | Right edge of carrier PCB |
| PCB slot size | 4 mm wide × 3.5 mm deep |
| Pin 1 (Red wire) | + (positive) |
| Pin 2 (Black wire) | – (negative/GND) |
| Compatible battery | YTKavq 3.7V 300mAh 481645 (or any 3.7V LiPo with PH 2.0mm) |
| Battery size | 4.8 × 16 × 45 mm |
| Notes | Polarity is marked on bottom silkscreen. Verify before inserting. |

⚠  Polarity Warning

The JST PH 2.0mm connector is NOT keyed for polarity. Always verify RED wire goes to pin 1 (+) before connecting battery. Reversed polarity will permanently damage the PowerBoost 500C and potentially cause battery fire.

# 4.  Net List & Signal Connections

## 4.1  Power Nets

| Net Name | From | To | Layer(s) | Trace Width | Notes |
| --- | --- | --- | --- | --- | --- |
| +5V | PB 500C 5V pin | Via → L3 pour → ESP32 5V | L4 + L3 + L1 | 1.6 mm | 1A capable |
| +3V3 | ESP32 3V3 pin | Via → L3 pour → GY-86 VCC | L1 + L3 | 0.8 mm | Via ferrite L1 |
| GND | All modules GND | L2 solid plane (all vias) | L2 | Plane | Star topology |
| VBAT | JST J1 Pin 1 | PB BAT pin | L4 | 1.0 mm | 3.7V LiPo |

## 4.2  Signal Nets

| Net Name | From | To | Layer | Width | Notes |
| --- | --- | --- | --- | --- | --- |
| I2C_SDA | ESP32 GPIO8 | GY-86 SDA | L1 | 0.35 mm | Pull-up R3 4.7kΩ to 3V3 |
| I2C_SCL | ESP32 GPIO9 | GY-86 SCL | L1 | 0.35 mm | Pull-up R4 4.7kΩ to 3V3 |
| MAG_DRDY | GY-86 DRDY | ESP32 GPIO4 | L1 | 0.25 mm | HMC5883L data ready interrupt |
| LBO_DIV | PB LBO pin | ESP32 GPIO5 | L4 → L1 via | 0.25 mm | Via R1/R2 divider. MANDATORY |
| PWR_EN | PB EN pin | SW1 → GND | L4 | 0.25 mm | Open = boost ON |
| USB_CHGR | PB USB pin | µUSB connector | L4 | 0.35 mm | 5V charger input path |

## 4.3  Via Map

The following vias connect between layers for power delivery. All vias are PTH (Plated Through-Hole), drill 0.30 mm, annular ring 0.75 mm.

| Via Ref | Net | From Layer | To Layer | X (mm) | Y (mm) |
| --- | --- | --- | --- | --- | --- |
| VIA1 | +5V | L4 | L3 | 25.0 | 39.5 |
| VIA2 | +5V | L3 | L1 | 27.0 | 39.5 |
| VIA3 | GND | L4 | L2 | 29.0 | 39.5 |
| VIA4 | GND | L2 | L1 | 29.0 | 39.5 |
| VIA5 | +3V3 | L1 | L3 | 47.0 | 38.0 |
| VIA6 | +3V3 | L3 | L1 (GY86) | 49.0 | 38.0 |
| VIA7 | GND | L1 | L2 | 51.0 | 38.0 |

# 5.  Bill of Materials (BOM)

| Ref | Description | Value / P/N | Package | Qty | Layer | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| R1 | LBO voltage divider upper | 100kΩ 1% | 0603 SMD | 1 | L4 Bot | MANDATORY — see §6.1 |
| R2 | LBO voltage divider lower | 150kΩ 1% | 0603 SMD | 1 | L4 Bot | MANDATORY — see §6.1 |
| R3 | I²C SDA pull-up | 4.7kΩ | 0603 SMD | 1 | L1 Top | Check GY-86 for existing |
| R4 | I²C SCL pull-up | 4.7kΩ | 0603 SMD | 1 | L1 Top | Check GY-86 for existing |
| C1 | 5V bulk decoupling | 100µF / 10V | D8 Electrolytic | 1 | L4 Bot | Near PB 5V output pad |
| C2 | 3V3 bulk decoupling | 10µF / 6.3V | 0805 SMD | 1 | L1 Top | Near ESP32 3V3 via |
| C3 | HF decoupling 3V3 | 100nF X7R | 0402 SMD | 1 | L1 Top | Near ESP32 3V3 pin |
| C4 | HF decoupling GY-86 VCC | 100nF X7R | 0402 SMD | 1 | L1 Top | As close to GY-86 as possible |
| C5 | HF decoupling GY-86 GND | 100nF X7R | 0402 SMD | 1 | L1 Top | As close to GY-86 as possible |
| L1 | Ferrite bead — 3V3 filter | 600Ω @ 100MHz | 0603 SMD | 1 | L1 Top | Blocks PB switching noise |
| SW1 | Power enable switch | SPDT Slide | Edge SMD | 1 | L4 Bot | Controls PB EN pin |
| J1 | LiPo battery connector | JST PH 2-pin 2.0mm | SMD/THT Edge | 1 | L4 Bot | Right edge, polarity marked |
| SK1 | ESP32-S3 Zero socket | 2×9 pin 2.54mm female | THT | 1 | L1 Top | 9+9 = 18 pins total |
| SK2 | GY-86 socket | 1×6 pin 2.54mm female | THT | 1 | L1 Top | 6 pins |
| SK3 | PowerBoost 500C socket | 2×7 pin 2.54mm female | THT | 1 | L4 Bot | 7+7 = 14 pins total |
| MH1–MH4 | Mounting holes | M2 standoff | 2.2mm dia | 4 | Both | No copper — NPTH |

# 6.  Critical Design Notes

## 6.1  LBO Voltage Divider — MANDATORY

The PowerBoost 500C LBO (Low Battery Output) pin is internally pulled up to the BAT rail, which swings between 3.0V (discharged) and 4.2V (fully charged). The ESP32-S3 GPIO absolute maximum input voltage is 3.6V. Without the R1/R2 divider, a fully charged battery (4.2V) will exceed this limit and permanently damage the GPIO pin.

Divider values: R1 = 100kΩ (upper, LBO to junction), R2 = 150kΩ (lower, junction to GND).

Voltage at GPIO5 = VBAT × R2/(R1+R2) = 4.2V × 150k/250k = 2.52V. Safe for 3.3V GPIO.

⚠  Do NOT omit R1 and R2

Connecting LBO directly to any ESP32-S3 GPIO will destroy the pin. These resistors must be populated before powering the board.

## 6.2  I²C Pull-up Resistors

The GY-86 module may already carry 4.7kΩ pull-up resistors on its SDA and SCL lines. Before soldering R3 and R4 on the carrier, inspect the GY-86 PCB. If onboard pull-ups are present, use 10kΩ for R3/R4 instead — two 4.7kΩ resistors in parallel create an effective 2.35kΩ which causes signal integrity problems at 400kHz I²C Fast Mode. Use the table below:

| GY-86 onboard pull-ups | Recommended R3/R4 on carrier | Effective pull-up | Max I²C speed |
| --- | --- | --- | --- |
| None (bare module) | 4.7kΩ | 4.7kΩ | 400kHz Fast Mode |
| 4.7kΩ present | 10kΩ | 3.2kΩ | 400kHz Fast Mode |
| 10kΩ present | 10kΩ | 5.0kΩ | 100kHz Standard Mode |

## 6.3  RF Antenna Keepout

The Waveshare ESP32-S3 Zero uses an integrated ceramic antenna located at the right end of the module (facing the GY-86 zone on the carrier). A 5mm keepout from all copper pours, ground planes, and metal components is required around this antenna. The L2 GND plane has an anti-pad clearance cut under the antenna footprint.

ℹ  Antenna Orientation

The ESP32-S3 Zero must be installed with the antenna end pointing toward the right edge of the carrier PCB, away from the main ground pour area of L2.

## 6.4  GY-86 EMI Isolation

The TPS61090 boost converter inside the PowerBoost 500C switches at approximately 700kHz. Magnetic field coupling from the inductor can corrupt the HMC5883L magnetometer readings. The 4-layer stack mitigates this through:

- L2 GND plane providing a continuous magnetic shielding layer between L1 (GY-86) and L4 (PowerBoost)

- Ferrite bead L1 (600Ω @ 100MHz) on the 3V3 power feed to GY-86 — blocks conducted switching noise

- Physical separation: GY-86 placed ≥20mm from the PowerBoost inductor

- C4/C5 decoupling caps placed within 1mm of GY-86 VCC and GND pads

## 6.5  Power Plane Isolation

The L3 power plane is divided into two isolated copper islands separated by a 1mm gap at X=32mm from left edge. The left island carries +5V (accessed by vias from the PowerBoost 5V output on L4 and the ESP32-S3 5V input on L1). The right island carries +3V3 (accessed by vias from the ESP32-S3 3V3 output on L1 and the GY-86 VCC on L1). There is no DC connection between the two islands — the ESP32-S3 onboard AMS1117 LDO creates the regulated 3V3.

# 7.  Firmware Pin Assignment

## 7.1  I²C Bus — GY-86

| ESP32-S3 GPIO | Signal | GY-86 Pin | Notes |
| --- | --- | --- | --- |
| GPIO8 | I2C_SDA | SDA | 4.7kΩ pull-up to 3V3 (R3) |
| GPIO9 | I2C_SCL | SCL | 4.7kΩ pull-up to 3V3 (R4) |
| GPIO4 | MAG_DRDY | DRDY | HMC5883L data-ready interrupt, active high |
| 3V3 | Power | VCC | Via ferrite bead L1 |
| GND | Ground | GND | L2 GND plane |

## 7.2  I²C Device Addresses

| Device | Default I²C Address | Alt Address (AD0=1) | Notes |
| --- | --- | --- | --- |
| MPU6050 (Accel+Gyro) | 0x68 | 0x69 | AD0 pin on GY-86 = GND by default |
| HMC5883L (Magnetometer) | 0x1E | Fixed | No address select pin |
| MS5611 (Barometer) | 0x77 | 0x76 | CSB pin determines address |

## 7.3  Power Management GPIO

| GPIO | Net | Direction | Function | Logic |
| --- | --- | --- | --- | --- |
| GPIO5 | LBO_DIV | Input | Low-battery alert | LOW when VBAT < 3.2V |
| GPIO via SW1 | PWR_EN | HW Switch | System power enable | Open = ON, Closed to GND = OFF |

## 7.4  Firmware I²C Initialization (Zephyr RTOS)

/* I2C bus — GY-86 Carrier PCB */
#define I2C_NODE  DT_NODELABEL(i2c0)
#define SDA_PIN   8   /* GPIO8 */
#define SCL_PIN   9   /* GPIO9 */
#define DRDY_PIN  4   /* GPIO4 — HMC5883L */
#define LBO_PIN   5   /* GPIO5 — Low Battery */

/* Device I2C addresses */
#define MPU6050_ADDR   0x68
#define HMC5883L_ADDR  0x1E
#define MS5611_ADDR    0x77

# 8.  Gerber File Reference

The following RS-274X Gerber files and Excellon drill file are provided for PCB fabrication. All files follow the naming convention expected by JLCPCB and PCBWay.

| File Name | Layer / Function | Description |
| --- | --- | --- |
| SmartShoe_Carrier-F_Cu.gtl | L1 — Top Copper | ESP32-S3 + GY-86 sockets, signal traces, via pads |
| SmartShoe_Carrier-In1_Cu.g2 | L2 — GND Plane | Full-board solid copper GND flood |
| SmartShoe_Carrier-In2_Cu.g3 | L3 — Power Plane | 5V copper island + 3V3 copper island |
| SmartShoe_Carrier-B_Cu.gbl | L4 — Bottom Copper | PowerBoost socket, battery/USB traces, passives |
| SmartShoe_Carrier-F_SilkS.gto | Top Silkscreen | Component refs, zone outlines, pin 1 markers |
| SmartShoe_Carrier-B_SilkS.gbo | Bottom Silkscreen | Module zone, connector polarity marks |
| SmartShoe_Carrier-F_Mask.gts | Top Solder Mask | Pad openings for top-side components |
| SmartShoe_Carrier-B_Mask.gbs | Bottom Solder Mask | Pad openings for bottom-side components |
| SmartShoe_Carrier-Edge_Cuts.gm1 | Board Outline | PCB perimeter + USB-C / µUSB / JST edge slots |
| SmartShoe_Carrier-NPTH.drl | Drill File (Excellon) | Vias (T1 0.30mm) + TH (T2 0.80mm) + M2 (T3 2.20mm) |
| FABRICATION_NOTES.txt | Fab Notes | Complete stackup, JLCPCB order settings checklist |

## 8.1  JLCPCB Order Settings

| Setting | Value |
| --- | --- |
| Base Material | FR4 |
| Layers | 4 |
| Dimensions | 55 × 40 mm |
| PCB Thickness | 1.6 mm |
| PCB Color | Green |
| Silkscreen Color | White |
| Surface Finish | ENIG (Electroless Nickel Immersion Gold) |
| Copper Weight (Outer) | 1 oz |
| Copper Weight (Inner) | 0.5 oz (JLCPCB default for 4-layer) |
| Via Covering | Tented |
| Remove Order Number | Yes (specify 'Remove' in notes) |
| Estimated Cost (5 pcs) | ~$15–20 USD |

ℹ  Upload Instructions

Zip all 11 files (10 Gerbers + 1 drill) into a single archive named 'SmartShoe_Carrier_RevA_Gerbers.zip'. Upload this single zip to JLCPCB / PCBWay. Do not rename individual files — the layer mapping is automatic.

# 9.  Assembly Instructions

## 9.1  SMD Component Assembly (Top Side, L1)

Reflow solder the following SMD components on the top face before installing socket headers:

- R3, R4: 4.7kΩ 0603 I²C pull-ups — place near GY-86 socket

- L1: Ferrite bead 0603 — on 3V3 trace between via and GY-86 socket

- C2: 10µF 0805 — near ESP32-S3 3V3 via

- C3, C4, C5: 100nF 0402 — as close to module pads as possible

## 9.2  SMD Component Assembly (Bottom Side, L4)

- R1: 100kΩ 0603 — upper resistor of LBO divider

- R2: 150kΩ 0603 — lower resistor of LBO divider

- C1: 100µF D8 electrolytic — near PowerBoost 5V output pad. Observe polarity (+).

- SW1: SPDT slide switch — edge-mount at board edge

- J1: JST PH 2-pin 2.0mm — right edge slot. Verify pin 1 = + (marked on silkscreen)

## 9.3  Through-Hole Socket Assembly

Solder female socket headers in the following order (use a flat surface to ensure vertical alignment):

- Solder SK3 (PowerBoost 500C, 2×7) on the bottom face (L4). Use a header alignment jig or plug the PowerBoost module in temporarily to align.Step 1:

- Solder SK1 (ESP32-S3 Zero, 2×9) on the top face (L1). Ensure USB-C end of module aligns with the edge slot cutout.Step 2:

- Solder SK2 (GY-86, 1×6) on the top face (L1). Orient with VCC pin at pin 1 (marked with dot on silkscreen).Step 3:

## 9.4  Module Installation

- Insert PowerBoost 500C into SK3 (bottom face). USB Micro-B port must seat into the edge slot cutout.1.

- Insert ESP32-S3 Zero into SK1 (top face). USB-C port seats into top edge slot.2.

- Insert GY-86 into SK2 (top face). Verify VCC and GND orientation.3.

- Connect LiPo battery to J1 at right edge. RED = pin 1 (+), BLACK = pin 2 (–).4.

## 9.5  Power-On Sequence

- Verify SW1 is in the OPEN position (boost converter enabled).1.

- Connect LiPo battery to J1.2.

- Measure 5V across carrier test points (5V via pad and GND via pad on top face).3.

- Measure 3V3 on GY-86 VCC pad.4.

- Connect USB-C cable. Flash firmware using esptool or PlatformIO.5.

- Verify I²C bus: run i2c_scan. Expected devices: 0x68, 0x1E, 0x77.6.

# 10.  Revision History

| Rev | Date | Author | Description |
| --- | --- | --- | --- |
| A | Mar 5, 2026 | R. Roychowdhury | Initial release — 4-layer dual-sided design, all connectors on board edges |

END OF DOCUMENT

MADEPLUS INC  ·  Smart Shoe Platform  ·  Patent Pending