SMART SHOE WEARABLE PLATFORM

4-Layer Carrier PCB

Design Reference Document  —  Revision A

| Document No. | MADEPLUS-PCB-001-RevA |
| --- | --- |
| Board Dimensions | 60 mm × 40 mm × 1.6 mm |
| Layer Count | 4 (L1 Signal / L2 GND / L3 Power / L4 Signal) |
| Modules | PowerBoost 500C + ESP32-S3 Zero + GY-86 IMU |
| USB-C Port | TOP edge — native (ESP32 module, no PCB slot) |
| Micro-USB Port | LEFT edge — native (PowerBoost module, no PCB slot) |
| Battery | 3.7V 300mAh LiPo via JST PH 2.0mm at RIGHT edge |
| Surface Finish | ENIG — Green solder mask — White silkscreen |
| Date | March 5, 2026 |
| Author | Ratneswar Roychowdhury — MADEPLUS INC |
| Status | Released for Fabrication |

MADEPLUS INC  |  Parole, Maryland  |  Confidential

# 1.  Project Overview

This document covers the complete design specification for the 4-Layer Carrier PCB developed as part of the Smart Shoe Gait Analysis Platform. The carrier PCB integrates three modules — the Adafruit PowerBoost 500C, the Waveshare ESP32-S3 Zero, and the GY-86 10-DOF IMU — through a dual-sided socket architecture. All inter-module connections, power distribution, and passive components are pre-routed on the carrier, requiring no hand wiring after module installation.

A key design goal is natural USB connector access: the ESP32-S3 Zero socket is oriented portrait so its USB-C port reaches the top PCB edge natively, and the PowerBoost 500C socket is oriented landscape so its Micro-USB port reaches the left PCB edge natively. No edge slot cutouts are required in the PCB outline.

## 1.1  Module Summary

| Module | Part | Location | Interface | Supply |
| --- | --- | --- | --- | --- |
| Microcontroller | Waveshare ESP32-S3 Zero | TOP — L1 | USB-C, BLE, I²C | 5V → AMS1117 LDO → 3.3V |
| IMU | GY-86 (MPU6050 + HMC5883L + MS5611) | TOP — L1 | I²C 0x68 / 0x1E / 0x77 | 3.3V from ESP32 |
| Power | Adafruit PowerBoost 500C | BOTTOM — L4 | Micro-USB (charger) | 3.7V LiPo → 5.2V boost |
| Battery | YTKavq 3.7V 300mAh JST PH 2.0mm | External (JST J1) | JST PH 2.0mm | 3.7V LiPo |

# 2.  PCB Layout

## 2.1  Top Face (L1)  —  ESP32-S3 Zero & GY-86

The top face carries the ESP32-S3 Zero in portrait orientation (USB-C at top edge), the GY-86 IMU module, I²C signal traces, 3.3V power traces, and all top-side passive components.

Figure 1 — PCB Top Face (L1): ESP32-S3 Zero socket (portrait, USB-C at top edge) · GY-86 IMU · I²C traces · 3V3 distribution

## 2.2  Bottom Face (L4)  —  PowerBoost 500C

The bottom face carries the PowerBoost 500C in landscape orientation (Micro-USB at left edge), the LBO voltage divider, power enable switch, battery connector, and bulk decoupling capacitors.

Figure 2 — PCB Bottom Face (L4): PowerBoost 500C socket (landscape, µUSB at left edge) · JST battery · LBO divider · SW1

# 3.  Layer Stack-Up

The board uses a standard 4-layer FR4 stack. L2 provides a continuous GND plane that shields the IMU on L1 from the switching converter on L4. L3 carries 5V and 3.3V power islands with a 1 mm isolation gap between them.

Figure 3 — 4-Layer Stack-Up: 1.6 mm FR4  ·  1 oz copper all layers  ·  ENIG finish

| Layer | Function | Cu Weight | Thickness | Description |
| --- | --- | --- | --- | --- |
| L1 — F.Cu | Signal Top | 1 oz | 35 µm | ESP32-S3 Zero + GY-86 sockets · I²C · 3V3 distribution |
| Prepreg 7628 | Dielectric | — | 0.20 mm | Between L1 and L2 |
| L2 — In1.Cu | GND Plane | 1 oz | 35 µm | Solid copper flood · full board extent · EMI shield |
| FR4 Core | Substrate | — | 0.80 mm | Structural core |
| L3 — In2.Cu | Power Plane | 1 oz | 35 µm | 5V island (left) + 3V3 island (right) · 1 mm gap |
| Prepreg 7628 | Dielectric | — | 0.20 mm | Between L3 and L4 |
| L4 — B.Cu | Signal Bottom | 1 oz | 35 µm | PowerBoost 500C socket · LBO divider · JST traces |
| Total |  |  | 1.60 mm |  |

# 4.  System Connections

The block diagram below shows all inter-module connections including power, I²C, status signals, and USB paths. The LiPo battery connects to the PowerBoost 500C which boosts to 5.2V; the ESP32-S3 onboard AMS1117 LDO regulates this to 3.3V for the GY-86 IMU.

Figure 4 — System Connection Diagram: Power, I²C, USB, LBO signal, and battery paths

## 4.1  Power Net Routing

| Net | From | To | Layers | Trace Width |
| --- | --- | --- | --- | --- |
| +5V | PowerBoost 500C 5V pin (L4) | Via → L3 island → ESP32 5V (L1) | L4 + L3 + L1 | 1.6 mm |
| +3V3 | ESP32-S3 3V3 pin (L1) | Via → L3 island → GY-86 VCC (L1) | L1 + L3 | 0.8 mm |
| GND | All module GND pins | L2 solid copper plane | L2 | Plane |
| VBAT | JST J1 pin 1 (+) | PowerBoost BAT pin (L4) | L4 | 1.0 mm |

## 4.2  Signal Net Routing

| Net | From | To | Layer | Width | Notes |
| --- | --- | --- | --- | --- | --- |
| I2C_SDA | ESP32 GPIO8 | GY-86 SDA | L1 | 0.35 mm | R3 4.7kΩ pull-up to 3V3 |
| I2C_SCL | ESP32 GPIO9 | GY-86 SCL | L1 | 0.35 mm | R4 4.7kΩ pull-up to 3V3 |
| MAG_DRDY | GY-86 DRDY | ESP32 GPIO4 | L1 | 0.25 mm | HMC5883L data-ready interrupt |
| LBO_DIV | PowerBoost LBO | ESP32 GPIO5 | L4→via→L1 | 0.25 mm | Via R1/R2 divider — MANDATORY |
| PWR_EN | PowerBoost EN | SW1 → GND | L4 | 0.25 mm | Open = boost ON |

# 5.  Connector Specifications

All three external connectors are accessible from the board edges. The two USB connectors are on the modules themselves — no PCB edge slots are machined. Correct module socket orientation ensures natural edge accessibility.

Figure 5 — Connector Detail: USB-C (top edge) · Micro-USB (left edge) · JST PH 2.0mm (right edge) with pin assignments

| Connector | Type | Location | Access Method | Purpose |
| --- | --- | --- | --- | --- |
| USB-C | USB Type-C (on ESP32 module) | TOP edge | Module overhang — no PCB slot | Firmware flash / debug / OTG |
| Micro-USB | USB Micro-B (on PB module) | LEFT edge | Module flush at edge — no slot | LiPo charging only (500mA) |
| JST J1 | JST PH 2-pin 2.0mm (carrier) | RIGHT edge | SMD receptacle on carrier PCB | 3.7V LiPo battery |

⚠  JST PH 2.0mm Polarity

The JST PH 2.0mm connector is NOT keyed for polarity. Always verify RED wire = Pin 1 (+) and BLACK wire = Pin 2 (−) before inserting the battery. Reversed polarity permanently damages the PowerBoost 500C and may cause battery fire.

# 6.  Passive Components

The carrier PCB includes 11 passive components serving decoupling, filtering, protection, and switching functions. All SMD components are 0402/0603/0805 standard packages for reflow assembly.

Figure 6 — Passive Components Reference: values, packages, placement layers, and functional notes

## 6.1  Bill of Materials

| Ref | Description | Value | Package | Qty | Layer | Critical Notes |
| --- | --- | --- | --- | --- | --- | --- |
| R1 | LBO divider upper | 100kΩ 1% | 0603 SMD | 1 | L4 Bot | MANDATORY — see §7.1 |
| R2 | LBO divider lower | 150kΩ 1% | 0603 SMD | 1 | L4 Bot | MANDATORY — see §7.1 |
| R3 | I²C SDA pull-up | 4.7kΩ | 0603 SMD | 1 | L1 Top | Check GY-86 onboard pull-ups |
| R4 | I²C SCL pull-up | 4.7kΩ | 0603 SMD | 1 | L1 Top | See §7.2 for value selection |
| C1 | 5V bulk decoupling | 100µF 10V | D8 Electrolytic | 1 | L4 Bot | Observe polarity (+) |
| C2 | 3V3 bulk decoupling | 10µF 6.3V | 0805 SMD | 1 | L1 Top | Near ESP32 3V3 via |
| C3 | 3V3 HF decoupling | 100nF X7R | 0402 SMD | 1 | L1 Top | Near ESP32 3V3 supply pin |
| C4 | GY-86 VCC decouple | 100nF X7R | 0402 SMD | 1 | L1 Top | Within 1mm of GY-86 VCC |
| C5 | GY-86 GND decouple | 100nF X7R | 0402 SMD | 1 | L1 Top | Within 1mm of GY-86 GND |
| L1 | 3V3 EMI ferrite | 600Ω@100MHz | 0603 SMD | 1 | L1 Top | Blocks 700kHz switching noise |
| SW1 | Power enable switch | SPDT Slide | Edge SMD | 1 | L4 Bot | Open = Boost ON |
| J1 | Battery connector | JST PH 2.0mm | 2-pin SMD | 1 | L4 Bot | Right edge · polarity marked |

# 7.  Critical Design Notes

## 7.1  LBO Voltage Divider  —  MANDATORY

The PowerBoost 500C LBO pin is internally pulled to VBAT, which ranges from 3.0V (discharged) to 4.2V (fully charged). The ESP32-S3 GPIO absolute maximum input is 3.6V. Without R1/R2, a full battery exceeds this limit and permanently damages the GPIO.

Divider math: V_GPIO5 = VBAT × R2 / (R1 + R2) = 4.2V × 150k / 250k = 2.52V  ✓  (safe for 3.3V logic)

⚠  Do NOT omit R1 and R2

Connecting LBO directly to ESP32-S3 GPIO5 without the divider will destroy the pin on first full charge. These resistors must be populated before any power-on.

## 7.2  I²C Pull-up Selection

The GY-86 may already carry pull-up resistors on SDA and SCL. Inspect the module before soldering R3/R4:

| GY-86 onboard pull-ups | R3/R4 on carrier | Effective pull-up | Max I²C speed |
| --- | --- | --- | --- |
| None | 4.7kΩ | 4.7kΩ | 400kHz Fast Mode |
| 4.7kΩ | 10kΩ | 3.2kΩ | 400kHz Fast Mode |
| 10kΩ | 10kΩ | 5.0kΩ | 100kHz Standard Mode |

## 7.3  ESP32-S3 RF Antenna Keepout

The ESP32-S3 Zero ceramic antenna is on the right end of the module. A 5mm copper-free keepout is maintained around this area in the L2 GND plane. The GY-86 is placed ≥20mm from the PowerBoost switching inductor.

## 7.4  Module Orientation

ℹ  Assembly Critical

ESP32-S3 Zero: install PORTRAIT with USB-C end toward TOP board edge. PowerBoost 500C: install LANDSCAPE with Micro-USB end toward LEFT board edge. Wrong orientation misaligns USB ports with the board edge. Silkscreen arrows indicate correct direction.

| Module | Socket Orientation | Pin Rows | USB Port Faces |
| --- | --- | --- | --- |
| ESP32-S3 Zero | Portrait (long axis vertical) | Left col + Right col (9 pads each, Y-axis) | TOP edge |
| PowerBoost 500C | Landscape (long axis horizontal) | Top row + Bottom row (7 pads each, X-axis) | LEFT edge |
| GY-86 | Standard (standard horizontal) | Single row 6 pads (X-axis) | N/A |

# 8.  Firmware Pin Reference

| GPIO | Net | Direction | Function | Notes |
| --- | --- | --- | --- | --- |
| GPIO8 | I2C_SDA | I/O | I²C data → GY-86 SDA | 4.7kΩ pull-up R3 |
| GPIO9 | I2C_SCL | Out | I²C clock → GY-86 SCL | 4.7kΩ pull-up R4 |
| GPIO4 | MAG_DRDY | In | HMC5883L data-ready interrupt | Active high |
| GPIO5 | LBO_DIV | In | Low battery alert (LBO÷divider) | LOW when VBAT < 3.2V |

## 8.1  I²C Device Addresses

| Device | I²C Address | Function |
| --- | --- | --- |
| MPU6050 | 0x68 | 3-axis accelerometer + 3-axis gyroscope |
| HMC5883L | 0x1E | 3-axis magnetometer (DRDY → GPIO4) |
| MS5611 | 0x77 | Barometric pressure + temperature |

/* Zephyr RTOS — GY-86 I²C configuration */
#define SDA_PIN     8   // GPIO8
#define SCL_PIN     9   // GPIO9
#define DRDY_PIN    4   // HMC5883L ready
#define LBO_PIN     5   // Low battery (via R1/R2 divider)

#define MPU6050_ADDR   0x68
#define HMC5883L_ADDR  0x1E
#define MS5611_ADDR    0x77

# 9.  Assembly Instructions

## 9.1  SMD Reflow  —  Top Face (L1)

- R3, R4 (4.7kΩ 0603) — near GY-86 SDA/SCL socket pads

- L1 (ferrite bead 0603) — on 3V3 trace between via and GY-86 VCC

- C2 (10µF 0805) — near ESP32-S3 Zero 3V3 via

- C3, C4, C5 (100nF 0402 each) — as close as possible to respective module pads

## 9.2  SMD Reflow  —  Bottom Face (L4)

- R1 (100kΩ 0603) and R2 (150kΩ 0603) — LBO divider. MANDATORY before any power-on.

- C1 (100µF D8 electrolytic) — near PowerBoost 5V pad. Observe polarity: (+) marked on silkscreen.

- SW1 (SPDT slide switch) — at SW1 footprint, bottom edge area

- J1 (JST PH 2-pin 2.0mm) — right edge. Pin 1 (+) toward top; verify silkscreen marking.

## 9.3  Socket Headers

- Solder SK3 (PowerBoost 500C, 2×7 female header) on bottom face. 7 pins per row at 2.54mm pitch along X-axis.

- Solder SK1 (ESP32-S3 Zero, 2×9 female header) on top face. 9 pins per row at 2.54mm pitch along Y-axis.

- Solder SK2 (GY-86, 1×6 female header) on top face. VCC at pin 1 (left end).

## 9.4  Module Installation

- Insert PowerBoost 500C into SK3 (bottom face). Micro-USB end must face LEFT PCB edge.

- Insert ESP32-S3 Zero into SK1 (top face). USB-C end must face TOP PCB edge. Module will overhang top edge ~3mm.

- Insert GY-86 into SK2 (top face). VCC at pin 1. Module body ≥20mm from PowerBoost inductor below.

- Connect LiPo battery to J1. RED = Pin 1 (+). BLACK = Pin 2 (−). Verify polarity before inserting.

## 9.5  Power-On Verification

- Confirm SW1 is OPEN (boost enabled).

- Connect battery to J1. Measure +5V across top-face 5V and GND via pads.

- Measure +3.3V at GY-86 VCC socket pad.

- Connect USB-C cable to top edge — verify ESP32-S3 Zero enumerates on host PC.

- Connect Micro-USB to left edge — verify charging LED lights on PowerBoost module.

- Flash firmware. Run I²C scan: expect devices at 0x68, 0x1E, 0x77.

# 10.  Gerber File Reference

| File | Layer / Content |
| --- | --- |
| SmartShoe_Carrier_RevA-F_Cu.gtl | L1 — Top copper: ESP32-S3 (portrait) + GY-86 sockets, traces |
| SmartShoe_Carrier_RevA-In1_Cu.g2 | L2 — GND plane: full solid pour |
| SmartShoe_Carrier_RevA-In2_Cu.g3 | L3 — Power plane: 5V island (left) + 3V3 island (right) |
| SmartShoe_Carrier_RevA-B_Cu.gbl | L4 — Bottom copper: PowerBoost (landscape) + passives |
| SmartShoe_Carrier_RevA-F_SilkS.gto | Top silkscreen: zone outlines, orientation arrows, pin 1 markers |
| SmartShoe_Carrier_RevA-B_SilkS.gbo | Bottom silkscreen: module zones, µUSB arrow, JST polarity |
| SmartShoe_Carrier_RevA-F_Mask.gts | Top solder mask: pad openings |
| SmartShoe_Carrier_RevA-B_Mask.gbs | Bottom solder mask: pad openings |
| SmartShoe_Carrier_RevA-Edge_Cuts.gm1 | Board outline: 60×40mm rounded rectangle — NO EDGE SLOTS |
| SmartShoe_Carrier_RevA-NPTH.drl | Drill file: vias (T1 0.30mm) · TH pads (T2 0.80mm) · M2 (T3 2.20mm) |
| FABRICATION_NOTES.txt | JLCPCB order settings checklist |

ℹ  Submitting to Fab

Zip all 11 files into SmartShoe_Carrier_RevA_Gerbers.zip and upload to JLCPCB or PCBWay. Order settings: 4 layers · 60×40mm · 1.6mm FR4 · ENIG · Green mask · White silk · 1oz outer copper.

# 11.  Design Rules & Board Specifications

| Parameter | Value |
| --- | --- |
| Board dimensions | 60 × 40 mm |
| Board thickness | 1.6 mm ± 10% |
| Material | FR4, Tg150 |
| Corner radius | 2.0 mm |
| Edge slots | NONE |
| Mounting holes | 4× M2 2.2mm dia NPTH at corners |
| Surface finish | ENIG (Electroless Nickel / Immersion Gold) |
| Solder mask | Green LPI both sides |
| Silkscreen | White both sides |
| Min trace width (signal) | 0.20 mm |
| Min trace width (+5V) | 1.60 mm |
| Min trace width (+3V3) | 0.80 mm |
| Min trace width (I²C) | 0.35 mm |
| Min clearance | 0.20 mm |
| Via drill | 0.30 mm |
| Via annular ring | 0.75 mm outer pad |
| Socket TH drill | 0.80 mm |

END OF DOCUMENT

MADEPLUS INC  ·  Smart Shoe Wearable Platform  ·  Patent Pending