MADEPLUS SMART SHOE

Zephyr RTOS Gait Analysis Platform

ESP32-S3 • GY-86 IMU • Zephyr RTOS 4.x • BLE 5.0

200 Hz Sensor Fusion  •  ZUPT Dead Reckoning  •  Stride Analytics

MADEPLUS INC

Version 1.0  —  March 2026  —  Confidential

# Table of Contents

# 1. Project Overview

The MADEPLUS Smart Shoe is a foot-mounted wearable gait analysis platform built on Zephyr RTOS running on the ESP32-S3 microcontroller. It integrates the GY-86 10-DOF sensor module to deliver real-time biomechanical data over Bluetooth Low Energy (BLE) to a companion Flutter mobile application.

The system is designed for clinical gait assessment, sports performance monitoring, and rehabilitation tracking. It captures 200 Hz inertial data, applies sensor fusion and zero-velocity update (ZUPT) algorithms, and derives gait metrics including step count, cadence, stride length, and altitude change — all streamed wirelessly at 10 Hz.

## 1.1 Key Capabilities

| Capability | Description |
| --- | --- |
| 200 Hz Sampling | MPU-6050 configured at 200 Hz with 20 Hz DLPF for high-fidelity motion capture |
| Sensor Fusion | Complementary filter merging accelerometer and gyroscope for stable roll/pitch angles |
| ZUPT Algorithm | Zero Velocity Update detects stance phase via gyro magnitude and accel variance window |
| Stride Length | Swing-phase double integration with ZUPT reset produces per-step horizontal displacement |
| Cadence | Heel-strike timestamps averaged over 8 steps for accurate steps-per-minute calculation |
| Altitude Tracking | MS5611 barometer measures per-step altitude change for uphill/downhill detection |
| Tilt-Compensated Heading | HMC5883L magnetometer with roll/pitch correction for accurate yaw angle |
| BLE Streaming | 11 individual GATT characteristics at 10 Hz for direct Flutter app compatibility |
| Runtime Commands | BLE write characteristic accepts live threshold tuning and algorithm reset |
| Gyro Calibration | 2-second startup calibration removes bias offset for all three gyro axes |

# 2. Hardware

## 2.1 Bill of Materials

| Component | Part Number | Notes |
| --- | --- | --- |
| Microcontroller | ESP32-S3-DevKitC-1 | Dual-core Xtensa LX7, 240 MHz, Wi-Fi + BLE 5.0 |
| IMU + Baro + Mag Module | GY-86 10DOF | Includes MPU-6050, MS5611, HMC5883L on one PCB |
| MPU-6050 | on GY-86 | 6-axis IMU: 3-axis accel, 3-axis gyro, I2C addr 0x68 |
| MS5611 | on GY-86 | Barometric pressure sensor, I2C addr 0x77 |
| HMC5883L | on GY-86 | 3-axis magnetometer, I2C addr 0x1E (via bypass) |
| Power Supply | 3.3V LDO | GY-86 VCC to 3.3V rail of ESP32-S3 |

## 2.2 Wiring Diagram

Connect the GY-86 module to the ESP32-S3-DevKitC as follows:

| GY-86 Pin | Signal | ESP32-S3 GPIO | Notes |
| --- | --- | --- | --- |
| VCC | Power | 3.3V | Do NOT connect to 5V |
| GND | Ground | GND | Common ground |
| SDA | I2C Data | GPIO 8 | Zephyr overlay: I2C0_SDA_GPIO8 |
| SCL | I2C Clock | GPIO 9 | Zephyr overlay: I2C0_SCL_GPIO9 |

NOTE: The GY-86 module has onboard 3.3K pull-up resistors on SDA and SCL. No external pull-ups are required. The MPU-6050 I2C bypass mode exposes the HMC5883L directly on the main I2C bus.

## 2.3 I2C Bus Architecture

The GY-86 module uses a hierarchical I2C topology:

- ESP32-S3 acts as I2C master on GPIO8/GPIO9 at 400 kHz

- MPU-6050 (0x68) is always accessible on the main bus

- MS5611 (0x77) is always accessible on the main bus

- HMC5883L (0x1E) sits on the MPU-6050 auxiliary I2C bus — accessible only after enabling bypass mode via register 0x37

The firmware enables I2C bypass mode in mpu_enable_bypass() at startup by writing 0x02 to MPU register 0x37, which connects the auxiliary bus directly to the main bus.

# 3. Software Environment

## 3.1 Development Environment

| Component | Version / Details |
| --- | --- |
| Operating System | Ubuntu 24.04 LTS (WSL2 on Windows 11) |
| Zephyr RTOS | v4.3.99 (west managed) |
| Zephyr SDK | v0.16.8 |
| West | v1.5.0 |
| Board Target | esp32s3_devkitc/esp32s3/procpu |
| Python venv | ~/zephyr-venv |
| Espressif HAL | hal_espressif (with BLE blobs fetched) |
| USB Passthrough | usbipd-win (WSL2 USB device sharing) |
| IDE | Any editor + West CLI |

## 3.2 Project Structure

Project location: ~/zephyrproject/my_86/

| File | Purpose |
| --- | --- |
| src/main.c | All firmware: sensor drivers, gait algorithm, BLE GATT server |
| prj.conf | Kconfig: enables I2C, BLE, FPU, float printf support |
| CMakeLists.txt | Standard Zephyr CMake build file |
| boards/esp32s3_devkitc_esp32s3_procpu.overlay | Device tree: I2C pinctrl, console redirect to USB JTAG |

## 3.3 prj.conf Configuration

Complete Kconfig configuration:

CONFIG_PRINTK=y

CONFIG_BOOT_BANNER=y

CONFIG_MAIN_STACK_SIZE=8192

CONFIG_I2C=y

CONFIG_FPU=y

CONFIG_CBPRINTF_FP_SUPPORT=y

CONFIG_MULTITHREADING=y

CONFIG_BT=y

CONFIG_BT_PERIPHERAL=y

CONFIG_BT_DEVICE_NAME="MADEPLUS SMART SHOE"

CONFIG_BT_DEVICE_NAME_MAX=32

CONFIG_BT_MAX_CONN=1

CONFIG_BT_GAP_AUTO_UPDATE_CONN_PARAMS=y

CONFIG_BT_BUF_ACL_TX_SIZE=251

CONFIG_BT_L2CAP_TX_MTU=247

CONFIG_BT_WAIT_NOP=y

CONFIG_BT_GATT_NOTIFY_MULTIPLE=n

## 3.4 Device Tree Overlay

File: boards/esp32s3_devkitc_esp32s3_procpu.overlay

#include <zephyr/dt-bindings/pinctrl/esp32s3-pinctrl.h>

/ {

chosen {

zephyr,console = &usb_serial;

zephyr,flash = &flash0;

};

};

&usb_serial { status = "okay"; };

&flash0 { reg = <0x0 DT_SIZE_M(4)>; };

&pinctrl {

i2c0_default: i2c0_default {

group1 {

pinmux = <I2C0_SDA_GPIO8>, <I2C0_SCL_GPIO9>;

bias-pull-up; drive-open-drain; output-high;

};

};

};

&i2c0 {

status = "okay";

clock-frequency = <I2C_BITRATE_FAST>;

pinctrl-0 = <&i2c0_default>;

pinctrl-names = "default";

};

# 4. Firmware Architecture

## 4.1 Thread Model

The firmware uses two threads plus the main thread for clean separation of timing-critical sensor work from BLE communication:

| Thread | Priority | Rate | Responsibility |
| --- | --- | --- | --- |
| sensor_200hz | 5 (high) | 200 Hz | Read MPU-6050, run filter, ZUPT, step detect, stride integration |
| main (BLE notify) | Default | 10 Hz | Snapshot shared data, send BLE notifications, print 1 Hz serial log |
| BLE stack (internal) | Zephyr managed | Event-driven | Handle connections, GATT writes, advertising |

Thread synchronization uses a K_MUTEX (gait_mutex) protecting the shared gait_data struct. The sensor thread locks briefly to write, the main thread locks briefly to snapshot before BLE notify.

## 4.2 Sensor Pipeline

Each 200 Hz tick executes the following pipeline in order:

- Read 14 bytes from MPU-6050 in a single I2C burst (accel + temp + gyro)

- Subtract calibrated gyro bias from all three gyro axes

- Apply 20 Hz first-order IIR low-pass filter to accel and gyro (alpha = 0.468)

- Complementary filter: fuse gyro integration and accel-derived tilt for roll and pitch

- Read HMC5883L magnetometer, apply tilt compensation, update yaw with 0.95/0.05 smoothing

- Compute gyro magnitude and accel magnitude variance over 20-sample (100 ms) rolling window

- Evaluate ZUPT stance conditions; require 12 consecutive samples (60 ms) to confirm stance

- Detect state transitions: SWING-to-STANCE triggers heel-strike event; STANCE-to-SWING starts swing

- During swing: remove gravity vector, rotate linear acceleration to world frame, double-integrate for velocity and displacement

- Update shared gait_data struct under mutex lock

## 4.3 MPU-6050 Configuration

| Register | Value | Effect |
| --- | --- | --- |
| PWR_MGMT_1 (0x6B) | 0x00 | Wake from sleep mode |
| CONFIG (0x1A) | 0x04 | DLPF_CFG=4: ~20 Hz bandwidth on gyro and accel |
| GYRO_CONFIG (0x1B) | 0x08 | Full scale: ±500 deg/s, sensitivity 65.5 LSB/deg/s |
| ACCEL_CONFIG (0x1C) | 0x08 | Full scale: ±4g, sensitivity 8192 LSB/g |
| SMPLRT_DIV (0x19) | 0x04 | Sample rate: 1 kHz / (1+4) = 200 Hz |
| USER_CTRL (0x6A) | 0x00 | Disable I2C master mode |
| INT_PIN_CFG (0x37) | 0x02 | Enable I2C bypass to expose HMC5883L |

# 5. Gait Algorithm

## 5.1 Complementary Filter

Tilt estimation fuses the gyroscope (short-term accuracy) with the accelerometer (long-term stability):

roll_gyro  = roll  + gx_filtered * DT

pitch_gyro = pitch + gy_filtered * DT

roll_acc   = atan2(ay, az) * RAD2DEG

pitch_acc  = atan2(-ax, sqrt(ay^2+az^2)) * RAD2DEG

roll       = 0.98 * roll_gyro  + 0.02 * roll_acc

pitch      = 0.98 * pitch_gyro + 0.02 * pitch_acc

The 0.98/0.02 (CF_ALPHA) weighting means gyro dominates at short time scales while the accelerometer corrects long-term drift. DT = 0.005 s at 200 Hz.

## 5.2 ZUPT Stance Detection

Stance phase (foot flat on ground) is detected using two simultaneous conditions:

| Condition | Threshold | Rationale |
| --- | --- | --- |
| Gyro magnitude ¦omega¦ | < 25 deg/s (tunable) | Foot is not rotating during stance flat phase |
| Accel variance var(a) | < 0.03 (m/s^2)^2 (tunable) | Foot is not accelerating; low vibration = stance |
| Hold time | >= 12 samples (60 ms) | Prevent false stance from momentary stillness |

The accel variance is computed over a rolling 20-sample (100 ms) window using an online algorithm that avoids recomputing the full window each tick.

## 5.3 Swing Integration and Stride Length

When the state transitions from STANCE to SWING (toe-off), the system begins dead-reckoning integration:

- Remove gravity from body-frame acceleration using current roll and pitch angles

- Rotate the linear acceleration vector from body frame to world frame using rotation matrix Ry(pitch) x Rx(roll), with yaw set to zero (indoor mag not reliable for heading-based integration)

- Integrate world-frame acceleration to velocity: v += a * dt

- Integrate velocity to position: p += v * dt

- At heel-strike (SWING-to-STANCE), compute stride length as horizontal magnitude: sqrt(px^2 + py^2)

- Apply ZUPT correction: zero all velocity and reset displacement accumulator

ZUPT is essential because double integration of IMU data accumulates errors rapidly. Resetting velocity to zero at each stance phase prevents unbounded drift and produces accurate per-step stride estimates.

## 5.4 Cadence Calculation

Cadence in steps per minute is computed from the 8 most recent heel-strike timestamps:

cadence_spm = 60000.0 * (N-1) / (t_newest - t_oldest)   [ms timestamps]

Using a sliding window of 8 heel-strikes provides a smooth cadence that adapts quickly to pace changes while averaging out timing jitter from the stance detection algorithm.

## 5.5 Altitude Tracking

The MS5611 barometer is read at each heel-strike event. Per-step altitude change is calculated as:

alt_m = 44330 * (1 - (P / P0)^0.190295)    [International Standard Atmosphere]

dH    = alt_m - prev_step_alt_m

P0 is captured at startup as the reference sea-level pressure. Positive dH indicates uphill, negative dH indicates downhill. The barometer is read on-demand at heel-strike rather than continuously to avoid blocking the 200 Hz sensor loop.

## 5.6 Algorithm Tuning Parameters

| Parameter | Default | Command | Effect |
| --- | --- | --- | --- |
| TH_GYRO_DPS | 25.0 deg/s | gyro_th=20 | Lower = more sensitive stance detection, may false-trigger |
| TH_VAR_A | 0.03 (m/s^2)^2 | var_th=0.02 | Lower = stricter stance; raise if ZUPT misses steps |
| k_gain | 0.50 | k=0.45 | Model-based stride scale factor; calibrate on known distance |
| notify_hz | 10 Hz | notify_hz=5 | BLE notification rate; reduce to save power |
| debug | 0 (off) | debug=1 | Enable verbose per-sample serial output |

# 6. BLE Protocol

## 6.1 Device Identity

| Property | Value |
| --- | --- |
| Advertised Name | MADEPLUS SMART SHOE |
| Role | BLE Peripheral (GATT Server) |
| Service UUID | 0000feed-0000-1000-8000-00805f9b34fb |
| Advertising Power | Maximum (ESP_PWR_LVL_P9) |
| Notify Rate | 10 Hz (configurable via command char) |
| Data Format | IEEE 754 float (4 bytes, little-endian) |
| Steps Format | uint32 (4 bytes, little-endian) |
| ASCII Format | UTF-8 string, variable length |

## 6.2 GATT Characteristics

| Name | UUID | Properties | Data |
| --- | --- | --- | --- |
| Roll | 0000a001-... | READ, NOTIFY | float deg |
| Pitch | 0000a002-... | READ, NOTIFY | float deg |
| Yaw | 0000a003-... | READ, NOTIFY | float deg |
| Temperature | 0000b001-... | READ, NOTIFY | float degC |
| Steps | 0000c001-... | READ, NOTIFY | uint32 |
| Cadence | 0000c002-... | READ, NOTIFY | float spm |
| Stride Length | 0000c003-... | READ, NOTIFY | float m |
| Alt Delta | 0000b002-... | READ, NOTIFY | float m |
| Command | 0000d001-... | WRITE | UTF-8 string |
| ASCII Line | 0000a0ee-... | READ, NOTIFY | UTF-8 string |

All UUIDs use the base suffix: -0000-1000-8000-00805f9b34fb. The ASCII line characteristic sends a human-readable summary string identical to the Arduino firmware format for Flutter app compatibility.

## 6.3 ASCII Line Format

The ASCII characteristic sends a formatted string at each notify interval:

st=STANCE S=42 cad=112.3 Lzupt=0.721 dH=-0.050 R=-12.4 P=-35.3 Y=127.4 T=28.91

| Field | Unit | Description |
| --- | --- | --- |
| st | STANCE/SWING | Current gait phase |
| S | steps | Total cumulative step count |
| cad | steps/min | Cadence from 8-step rolling average |
| Lzupt | meters | ZUPT-integrated stride length of last step |
| dH | meters | Altitude change from previous step (+ = uphill) |
| R | degrees | Roll angle from complementary filter |
| P | degrees | Pitch angle from complementary filter |
| Y | degrees | Yaw / heading from tilt-compensated magnetometer |
| T | deg C | Temperature from MPU-6050 die sensor |

## 6.4 BLE Command Reference

Write UTF-8 strings to the Command characteristic (0000d001-...) to tune the algorithm at runtime:

| Command | Effect |
| --- | --- |
| reset | Zero step count, cadence, stride length, altitude delta |
| notify_hz=10 | Set BLE notify rate (1-50 Hz). Default: 10 |
| gyro_th=20 | Set ZUPT gyro threshold in deg/s. Default: 25 |
| var_th=0.02 | Set ZUPT accel variance threshold. Default: 0.03 |
| k=0.45 | Set stride length model scale factor. Default: 0.50 |
| debug=1 | Enable verbose serial output. debug=0 to disable |

# 7. Build, Flash and Monitor

## 7.1 Prerequisites

- Zephyr SDK v0.16.8 installed at ~/zephyr-sdk-0.16.8

- West 1.5.0 in Python venv: ~/zephyr-venv

- Espressif BLE blobs fetched: west blobs fetch hal_espressif

- usbipd-win installed on Windows host

## 7.2 USB Passthrough (WSL2)

Run in Windows PowerShell (Administrator) each time the board is plugged in:

usbipd list                        # find BUSID (e.g. 2-13)

usbipd bind --busid 2-13           # bind once

usbipd attach --wsl --busid 2-13  # attach to WSL2

In WSL2, the device appears as /dev/ttyACM0. Verify:

ls /dev/ttyACM0

## 7.3 Build Commands

source ~/zephyr-venv/bin/activate

cd ~/zephyrproject

west build -p always -b esp32s3_devkitc/esp32s3/procpu my_86

## 7.4 Flash Commands

west flash --esp-device=/dev/ttyACM0

## 7.5 Monitor

west espressif monitor --port /dev/ttyACM0

Exit monitor: press Ctrl+T then X, or kill from another terminal with:

pkill -f idf_monitor

## 7.6 Expected Boot Output

[BOOT] MADEPLUS SMART SHOE -- Zephyr ZUPT Gait

[MPU] INT_PIN_CFG=0x02 (BYPASS_EN)

[MS5611] OK

[BLE] Initialized

[BLE] Advertising as 'MADEPLUS SMART SHOE'

[CAL] Hold still 2 s for gyro bias...

[CAL] g_bias dps=[-1.290, -0.122, 0.213]

[CAL] seed roll=-11.8 pitch=-35.3

[MS5611] p0=1035.42 mbar

[SENSOR] Running at 200 Hz

[TX] st=STANCE S=0 cad=0.0 Lzupt=0.000 dH=0.000 ...

BLE advertising is delayed by 2 seconds after boot to allow the USB Serial JTAG peripheral to settle. The device will not appear in BLE scans until this delay completes. This prevents the USB/BLE radio conflict known to affect ESP32-S3.

# 8. Testing and Validation

## 8.1 I2C Bus Scan

At startup the firmware verifies I2C bypass mode. To manually confirm all sensors are detected, use the scan code (available in initial test builds):

Found device at 0x1E   <- HMC5883L (after bypass enabled)

Found device at 0x68   <- MPU-6050

Found device at 0x77   <- MS5611

## 8.2 Static Validation

| Test | Expected | Pass Criterion |
| --- | --- | --- |
| Accel magnitude at rest | ~1.00 g | sqrt(ax^2+ay^2+az^2) within 0.05 g of 1.0 |
| Gyro at rest (after cal) | ~0.00 deg/s | All axes < 0.5 deg/s when board is still |
| IMU temperature | 20-35 degC | Stable reading within ambient +5 degC |
| Baro temperature | Within 2 degC of IMU | MS5611 and MPU temp agree within 3 degC |
| ZUPT at rest | STANCE | ASCII line shows st=STANCE within 1 second |
| Altitude at startup | 0.0 m | Alt resets to 0 after p0 reference captured |
| BLE advertising | Visible in nRF Connect | Device appears as MADEPLUS SMART SHOE |

## 8.3 Dynamic Validation (Walking Test)

- Attach sensor to shoe with SDA/SCL cable secured to prevent disconnection

- Boot device, wait for calibration to complete (watch for [SENSOR] Running at 200 Hz)

- Connect nRF Connect or Flutter app, subscribe to all notify characteristics

- Walk 20 steps at normal pace; verify step count increments at each heel-strike

- Walk a measured 10 m course; record Lzupt stride values; expected average 0.65-0.85 m

- Walk upstairs; verify dH positive values appear at each step

- Stand still 3 seconds; verify st=STANCE, gyro values near zero

## 8.4 Known Limitations

- Stride length accuracy degrades at speeds below 0.5 m/s or above 3.0 m/s

- ZUPT fails on soft surfaces (carpet, grass) where foot contact vibration raises accel variance

- Magnetometer (yaw) is unreliable near metal structures, reinforced concrete, or electronic devices

- MS5611 altitude resolution is approximately 0.1 m; small steps may not register dH change

- Double-integration errors accumulate during long swing phases (running stride > 400 ms)

# 9. Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| No BLE advertising when USB connected | USB JTAG / BLE radio conflict | Normal; 2s delay in firmware resolves it. Unplug USB for clean BLE test. |
| MPU-6050 not found (I2C scan) | Wiring error or wrong I2C pins | Verify SDA=GPIO8, SCL=GPIO9. Check 3.3V not 5V. |
| HMC5883L not found after bypass | Bypass mode failed | Check MPU USER_CTRL=0x00 before INT_PIN_CFG=0x02 |
| Roll/Pitch drift at rest | Gyro calibration failed or ALPHA too high | Ensure board is still during 2s cal. Verify ALPHA_STILL triggers (gyro < 2 deg/s) |
| ZUPT never triggers (always SWING) | Thresholds too tight | Raise gyro_th or var_th via BLE command |
| Steps not counting | ZUPT too loose or too tight | Watch ASCII line for st= transitions. Tune thresholds. |
| Build fails: BLE blob missing | BLE blobs not fetched | Run: west blobs fetch hal_espressif |
| BT_LE_ADV_CONN undeclared | Zephyr 4.x API change | Use BT_LE_ADV_CONN_FAST_1 in all adv_start calls |
| Flash SHA-256 mismatch warning | Old flash remnant | Normal warning; firmware boots correctly after it |
| Altitude shows -46 m at startup | Using 1013.25 mbar reference instead of local | Firmware captures p0 from first MS5611 reading as reference |

# 10. Development Roadmap

## 10.1 Completed

- Zephyr RTOS environment setup on ESP32-S3 with WSL2

- USB Serial JTAG console redirect (single cable operation)

- GY-86 I2C integration: MPU-6050, MS5611, HMC5883L

- 200 Hz sensor loop with 20 Hz IIR low-pass filter

- Gyro bias calibration (2-second startup)

- Complementary filter for roll, pitch, yaw

- ZUPT stance detection with accel variance window

- Swing integration for stride length

- Cadence from 8-step rolling heel-strike average

- Per-step altitude change from MS5611

- BLE GATT server with 10 characteristics matching Flutter app UUIDs

- Runtime algorithm tuning via BLE command characteristic

- USB/BLE coexistence fix (2s startup delay)

## 10.2 Planned

- Flutter mobile app: real-time display of all 10 gait metrics

- SD card logging for offline gait session recording

- Heel-toe pressure sensors (FSR) for ground contact force

- Kalman filter replacement for complementary filter

- Machine learning gait phase classifier (edge inference)

- Power optimization: 200 Hz only during activity, 1 Hz idle

- OTA firmware update via BLE

- Multi-device support (left shoe + right shoe synchronized)

- Clinical report generation from session data

## 10.3 Research Areas

- ZUPT-aided INS velocity accuracy vs. GPS ground truth

- Stride length calibration across walking speeds and shoe types

- Barometric altitude sensitivity for stair vs. ramp detection

- Magnetometer hard-iron calibration for heading accuracy in indoor environments

# Appendix A: Key Register Map

| Device | Register | Address (Hex) | Purpose |
| --- | --- | --- | --- |
| MPU-6050 | PWR_MGMT_1 | 0x6B | Wake from sleep (write 0x00) |
| MPU-6050 | CONFIG | 0x1A | DLPF bandwidth (0x04 = ~20 Hz) |
| MPU-6050 | GYRO_CONFIG | 0x1B | Gyro range (0x08 = +-500 dps) |
| MPU-6050 | ACCEL_CONFIG | 0x1C | Accel range (0x08 = +-4g) |
| MPU-6050 | SMPLRT_DIV | 0x19 | Sample rate divider (0x04 = 200 Hz) |
| MPU-6050 | USER_CTRL | 0x6A | I2C master control (0x00 = disable) |
| MPU-6050 | INT_PIN_CFG | 0x37 | Bypass enable (0x02 = on) |
| MPU-6050 | ACCEL_XOUT_H | 0x3B | Start of 14-byte burst read |
| HMC5883L | CONFIG_A | 0x00 | 8 samples avg, 15 Hz (0x70) |
| HMC5883L | CONFIG_B | 0x01 | Gain +-1.3 Gauss (0x20) |
| HMC5883L | MODE | 0x02 | Continuous measurement (0x00) |
| MS5611 | RESET | 0x1E | Reset command (send once at init) |
| MS5611 | PROM_READ_BASE | 0xA2 | Start of 6 calibration coefficients |
| MS5611 | CONVERT_D1 | 0x48 | Start pressure conversion OSR=4096 |
| MS5611 | CONVERT_D2 | 0x58 | Start temperature conversion OSR=4096 |
| MS5611 | ADC_READ | 0x00 | Read conversion result (3 bytes) |

End of Document

MADEPLUS INC  —  Confidential  —  March 2026