Zephyr RTOS on ESP32

Complete Tutorial: Setup, Configuration & Coding

Embedded Firmware Engineering Reference

Platform: ESP32 / ESP32-S3  |  Host: Ubuntu 22.04 / Windows WSL2

Zephyr Version: v3.6.x+

# 1. Introduction to Zephyr RTOS

Zephyr is a small, scalable, real-time operating system (RTOS) designed for resource-constrained and embedded systems. It is an open-source project under the Linux Foundation and supports a wide range of architectures including ARM, RISC-V, x86, and Xtensa — the core architecture of the ESP32.

## 1.1 Why Zephyr on ESP32?

ESP32 is traditionally programmed using Espressif's ESP-IDF or Arduino frameworks. Zephyr provides an alternative with key advantages for professional embedded development:

- Standards-based POSIX-like threading and synchronization primitives

- Built-in Bluetooth LE (BLE), Wi-Fi, IEEE 802.15.4 networking stacks

- Device tree-based hardware abstraction — same code, multiple boards

- Comprehensive power management framework

- Native west build system and CMake integration

- FDA and safety-critical ready — supports functional safety profiles

- First-class sensor subsystem — IMU, barometric, optical sensors

## 1.2 ESP32 Family Support in Zephyr

| Board | CPU | Status | Zephyr Board ID |
| --- | --- | --- | --- |
| ESP32-DevKitC | Xtensa LX6 Dual-Core | Stable | esp32_devkitc_wroom |
| ESP32-S3-DevKitC | Xtensa LX7 Dual-Core | Stable | esp32s3_devkitc |
| ESP32-C3-DevKitM | RISC-V 32-bit | Stable | esp32c3_devkitm |
| ESP32-S2-Saola | Xtensa LX7 Single | Supported | esp32s2_saola |

# 2. Environment Setup

## 2.1 Host System Requirements

- Ubuntu 20.04 / 22.04 (recommended) or Windows 10/11 with WSL2

- Minimum 8 GB RAM, 20 GB free disk space

- Python 3.8+ and pip3

- Git 2.28+

- CMake 3.20+

## 2.2 Install System Dependencies (Ubuntu)

Open a terminal and install all required packages:

sudo apt update && sudo apt upgrade -y

sudo apt install -y git cmake ninja-build gperf \

ccache dfu-util device-tree-compiler wget \

python3-dev python3-pip python3-setuptools \

python3-tk python3-wheel xz-utils file \

make gcc libsdl2-dev libmagic1

## 2.3 Install West (Zephyr's Meta-Tool)

West is the Zephyr project's multi-repository management and build tool. Install it with pip:

pip3 install --user -U west

echo 'export PATH=~/.local/bin:$PATH' >> ~/.bashrc

source ~/.bashrc

west --version  # Verify: West version x.y.z

## 2.4 Get the Zephyr SDK and Source

- Initialize the Zephyr workspace:

mkdir ~/zephyrproject && cd ~/zephyrproject

west init

west update  # Downloads all required modules (~2-3 GB)

west zephyr-export

- Install Python requirements:

pip3 install --user -r ~/zephyrproject/zephyr/scripts/requirements.txt

- Download and install the Zephyr SDK:

cd ~

wget https://github.com/zephyrproject-rtos/sdk-ng/releases/download/v0.16.8/zephyr-sdk-0.16.8_linux-x86_64.tar.xz

tar -xvf zephyr-sdk-0.16.8_linux-x86_64.tar.xz

cd zephyr-sdk-0.16.8

./setup.sh

## 2.5 Install Espressif HAL (ESP32 Support)

Zephyr uses the Espressif HAL for low-level ESP32 peripheral access. This is automatically fetched by west update, but ensure the toolchain is configured:

# Set Zephyr environment variables

echo 'export ZEPHYR_BASE=~/zephyrproject/zephyr' >> ~/.bashrc

echo 'export ZEPHYR_SDK_INSTALL_DIR=~/zephyr-sdk-0.16.8' >> ~/.bashrc

source ~/.bashrc

# Verify ESP32 toolchain is present

ls $ZEPHYR_SDK_INSTALL_DIR/xtensa-espressif_esp32_zephyr-elf/

💡 Note on ESP32 Flash Tool Zephyr uses esptool.py for flashing ESP32 targets. Install it with: pip3 install esptool

# 3. Project Structure & Build System

## 3.1 Zephyr Application Structure

A minimal Zephyr application for ESP32 has this directory structure:

my_app/

├── CMakeLists.txt       # Build configuration

├── prj.conf             # Kconfig: feature enable/disable

├── boards/

│   └── esp32_devkitc_wroom.conf  # Board-specific overrides

├── dts/

│   └── esp32_devkitc_wroom.overlay  # Device tree overlay

└── src/

└── main.c           # Application entry point

## 3.2 CMakeLists.txt

cmake_minimum_required(VERSION 3.20.0)

find_package(Zephyr REQUIRED HINTS $ENV{ZEPHYR_BASE})

project(my_esp32_app)

target_sources(app PRIVATE src/main.c)

## 3.3 prj.conf (Kconfig)

The prj.conf file enables or disables kernel features using Kconfig symbols. Every peripheral, protocol stack, and subsystem is selectable:

# Core kernel settings

CONFIG_MAIN_STACK_SIZE=4096

CONFIG_HEAP_MEM_POOL_SIZE=8192

# GPIO

CONFIG_GPIO=y

# UART / Serial

CONFIG_UART_CONSOLE=y

CONFIG_SERIAL=y

# I2C

CONFIG_I2C=y

# SPI

CONFIG_SPI=y

# Logging

CONFIG_LOG=y

CONFIG_LOG_DEFAULT_LEVEL=3   # 3=INFO

# Bluetooth LE

CONFIG_BT=y

CONFIG_BT_PERIPHERAL=y

CONFIG_BT_DEVICE_NAME="Zephyr_ESP32"

# Wi-Fi

CONFIG_WIFI=y

CONFIG_ESP32_WIFI=y

## 3.4 Building for ESP32

cd ~/zephyrproject

# Build for ESP32-DevKitC (classic ESP32)

west build -b esp32_devkitc_wroom -- -DBOARD_FLASH_RUNNER=esp32 ./my_app

# Build for ESP32-S3

west build -b esp32s3_devkitc ./my_app

# Flash the firmware

west flash --esp-device=/dev/ttyUSB0

# Monitor serial output

west espressif monitor

# 4. Core Concepts: Threads, Semaphores & Queues

## 4.1 Creating Threads

Zephyr uses cooperative and preemptive threading. Threads are created statically (recommended) or dynamically. Below is a complete example with two tasks:

#include <zephyr/kernel.h>

#include <zephyr/logging/log.h>

LOG_MODULE_REGISTER(main, LOG_LEVEL_INF);

/* Stack sizes for each thread */

#define SENSOR_STACK_SIZE  2048

#define PROC_STACK_SIZE    2048

#define THREAD_PRIORITY    5

/* Static thread stacks */

K_THREAD_STACK_DEFINE(sensor_stack, SENSOR_STACK_SIZE);

K_THREAD_STACK_DEFINE(proc_stack,   PROC_STACK_SIZE);

struct k_thread sensor_thread_data;

struct k_thread proc_thread_data;

/* Thread entry functions */

void sensor_task(void *p1, void *p2, void *p3)

{

while (1) {

LOG_INF("Sensor task: reading data...");

k_msleep(100);  /* 100 ms sampling period */

}

}

void processing_task(void *p1, void *p2, void *p3)

{

while (1) {

LOG_INF("Processing task: running algorithm");

k_msleep(500);

}

}

int main(void)

{

LOG_INF("ESP32 Zephyr RTOS Starting...");

k_thread_create(&sensor_thread_data, sensor_stack,

K_THREAD_STACK_SIZEOF(sensor_stack),

sensor_task, NULL, NULL, NULL,

THREAD_PRIORITY, 0, K_NO_WAIT);

k_thread_create(&proc_thread_data, proc_stack,

K_THREAD_STACK_SIZEOF(proc_stack),

processing_task, NULL, NULL, NULL,

THREAD_PRIORITY + 1, 0, K_NO_WAIT);

return 0;

}

## 4.2 Semaphores

/* Declare a semaphore */

K_SEM_DEFINE(data_ready_sem, 0, 1);

void producer_task(void *p1, void *p2, void *p3)

{

while (1) {

/* Simulate data acquisition */

k_msleep(50);

k_sem_give(&data_ready_sem);  /* Signal consumer */

}

}

void consumer_task(void *p1, void *p2, void *p3)

{

while (1) {

/* Block until data is ready */

k_sem_take(&data_ready_sem, K_FOREVER);

LOG_INF("Processing new data sample");

}

}

## 4.3 Message Queues

Message queues are ideal for passing sensor data between threads safely:

struct sensor_data {

int16_t accel_x;

int16_t accel_y;

int16_t accel_z;

uint32_t timestamp_ms;

};

K_MSGQ_DEFINE(sensor_msgq, sizeof(struct sensor_data), 16, 4);

void imu_producer(void *p1, void *p2, void *p3)

{

struct sensor_data sample;

while (1) {

sample.accel_x = read_accel_x();

sample.accel_y = read_accel_y();

sample.accel_z = read_accel_z();

sample.timestamp_ms = k_uptime_get_32();

k_msgq_put(&sensor_msgq, &sample, K_NO_WAIT);

k_msleep(10);

}

}

void data_consumer(void *p1, void *p2, void *p3)

{

struct sensor_data rx;

while (1) {

k_msgq_get(&sensor_msgq, &rx, K_FOREVER);

LOG_INF("Accel: X=%d Y=%d Z=%d @%ums",

rx.accel_x, rx.accel_y, rx.accel_z,

rx.timestamp_ms);

}

}

# 5. GPIO — LED Blink Example

The classic blink example, Zephyr-style, using Device Tree macros:

#include <zephyr/kernel.h>

#include <zephyr/drivers/gpio.h>

#include <zephyr/logging/log.h>

LOG_MODULE_REGISTER(blink, LOG_LEVEL_INF);

/* Get LED0 node from device tree */

#define LED0_NODE DT_ALIAS(led0)

static const struct gpio_dt_spec led =

GPIO_DT_SPEC_GET(LED0_NODE, gpios);

int main(void)

{

int ret;

if (!gpio_is_ready_dt(&led)) {

LOG_ERR("LED GPIO device not ready!");

return -ENODEV;

}

ret = gpio_pin_configure_dt(&led, GPIO_OUTPUT_ACTIVE);

if (ret < 0) {

LOG_ERR("GPIO configure failed: %d", ret);

return ret;

}

LOG_INF("Starting LED blink on ESP32...");

while (1) {

ret = gpio_pin_toggle_dt(&led);

if (ret < 0) {

return ret;

}

k_msleep(500);

}

}

## 5.1 Device Tree Overlay for LED

If no alias 'led0' exists on your board, add a .overlay file:

/* boards/esp32_devkitc_wroom.overlay */

/ {

aliases {

led0 = &led_gpio2;

};

leds {

compatible = "gpio-leds";

led_gpio2: led_0 {

gpios = <&gpio0 2 GPIO_ACTIVE_HIGH>;

label = "LED GPIO2";

};

};

};

# 6. I2C — Reading an IMU Sensor

This example demonstrates reading from an ICM-42688-P IMU over I2C — a common pattern in biomedical wearables:

## 6.1 Kconfig (prj.conf)

CONFIG_I2C=y

CONFIG_SENSOR=y

CONFIG_LOG=y

## 6.2 I2C Read/Write Code

#include <zephyr/kernel.h>

#include <zephyr/drivers/i2c.h>

#include <zephyr/logging/log.h>

LOG_MODULE_REGISTER(imu, LOG_LEVEL_DBG);

#define ICM42688_I2C_ADDR  0x68

#define REG_WHO_AM_I       0x75

#define REG_ACCEL_DATA_X1  0x1F

#define I2C_NODE DT_NODELABEL(i2c0)

static const struct device *i2c_dev;

static int icm42688_read_reg(uint8_t reg, uint8_t *data, size_t len)

{

return i2c_burst_read(i2c_dev, ICM42688_I2C_ADDR, reg, data, len);

}

static int icm42688_write_reg(uint8_t reg, uint8_t val)

{

uint8_t buf[2] = { reg, val };

return i2c_write(i2c_dev, buf, sizeof(buf), ICM42688_I2C_ADDR);

}

int main(void)

{

uint8_t who_am_i;

int16_t accel_raw[3];

uint8_t raw[6];

i2c_dev = DEVICE_DT_GET(I2C_NODE);

if (!device_is_ready(i2c_dev)) {

LOG_ERR("I2C device not ready");

return -ENODEV;

}

/* Read WHO_AM_I register */

icm42688_read_reg(REG_WHO_AM_I, &who_am_i, 1);

LOG_INF("ICM-42688 WHO_AM_I: 0x%02X", who_am_i);

/* Power on accel + gyro */

icm42688_write_reg(0x4E, 0x0F);  /* PWR_MGMT0 */

k_msleep(10);

while (1) {

icm42688_read_reg(REG_ACCEL_DATA_X1, raw, 6);

accel_raw[0] = (int16_t)((raw[0] << 8) | raw[1]);

accel_raw[1] = (int16_t)((raw[2] << 8) | raw[3]);

accel_raw[2] = (int16_t)((raw[4] << 8) | raw[5]);

/* Scale: 16g range -> LSB = 2048 counts/g */

LOG_INF("Accel: X=%.2f Y=%.2f Z=%.2f g",

accel_raw[0]/2048.0f,

accel_raw[1]/2048.0f,

accel_raw[2]/2048.0f);

k_msleep(100);

}

}

# 7. Bluetooth LE (BLE) on ESP32

## 7.1 Kconfig for BLE

CONFIG_BT=y

CONFIG_BT_PERIPHERAL=y

CONFIG_BT_DEVICE_NAME="ESP32_Zephyr_BLE"

CONFIG_BT_DEVICE_APPEARANCE=0

CONFIG_BT_MAX_CONN=1

CONFIG_BT_GAP_AUTO_UPDATE_CONN_PARAMS=y

## 7.2 BLE Peripheral with Custom GATT Service

This example creates a BLE peripheral with a custom sensor data characteristic — the pattern used in wearable gait analysis and biomedical sensor devices:

#include <zephyr/kernel.h>

#include <zephyr/bluetooth/bluetooth.h>

#include <zephyr/bluetooth/hci.h>

#include <zephyr/bluetooth/conn.h>

#include <zephyr/bluetooth/uuid.h>

#include <zephyr/bluetooth/gatt.h>

#include <zephyr/logging/log.h>

LOG_MODULE_REGISTER(ble_sensor, LOG_LEVEL_INF);

/* Custom Service UUID: 12345678-1234-5678-1234-56789ABCDEF0 */

#define BT_UUID_SENSOR_SVC_VAL \

BT_UUID_128_ENCODE(0x12345678, 0x1234, 0x5678, 0x1234, 0x56789ABCDEF0)

/* Sensor Data Characteristic UUID */

#define BT_UUID_SENSOR_DATA_VAL \

BT_UUID_128_ENCODE(0x12345678, 0x1234, 0x5678, 0x1234, 0x56789ABCDEF1)

#define BT_UUID_SENSOR_SVC  BT_UUID_DECLARE_128(BT_UUID_SENSOR_SVC_VAL)

#define BT_UUID_SENSOR_DATA BT_UUID_DECLARE_128(BT_UUID_SENSOR_DATA_VAL)

static uint8_t  sensor_value[6] = {0};

static bool     notify_enabled = false;

/* CCCD (Client Characteristic Config Descriptor) callback */

static void sensor_ccc_changed(const struct bt_gatt_attr *attr,

uint16_t value)

{

notify_enabled = (value == BT_GATT_CCC_NOTIFY);

LOG_INF("Notifications %s", notify_enabled ? "enabled" : "disabled");

}

/* Read callback */

static ssize_t read_sensor(struct bt_conn *conn,

const struct bt_gatt_attr *attr,

void *buf, uint16_t len, uint16_t offset)

{

return bt_gatt_attr_read(conn, attr, buf, len, offset,

sensor_value, sizeof(sensor_value));

}

/* GATT Service definition */

BT_GATT_SERVICE_DEFINE(sensor_svc,

BT_GATT_PRIMARY_SERVICE(BT_UUID_SENSOR_SVC),

BT_GATT_CHARACTERISTIC(BT_UUID_SENSOR_DATA,

BT_GATT_CHRC_READ | BT_GATT_CHRC_NOTIFY,

BT_GATT_PERM_READ,

read_sensor, NULL, sensor_value),

BT_GATT_CCC(sensor_ccc_changed,

BT_GATT_PERM_READ | BT_GATT_PERM_WRITE),

);

/* Advertising data */

static const struct bt_data ad[] = {

BT_DATA_BYTES(BT_DATA_FLAGS, (BT_LE_AD_GENERAL | BT_LE_AD_NO_BREDR)),

BT_DATA(BT_DATA_NAME_COMPLETE, CONFIG_BT_DEVICE_NAME,

sizeof(CONFIG_BT_DEVICE_NAME) - 1),

};

static void connected(struct bt_conn *conn, uint8_t err)

{

LOG_INF(err ? "Connection failed (err %d)" : "BLE Connected!", err);

}

static void disconnected(struct bt_conn *conn, uint8_t reason)

{

LOG_INF("BLE Disconnected (reason %d)", reason);

}

BT_CONN_CB_DEFINE(conn_callbacks) = {

.connected    = connected,

.disconnected = disconnected,

};

int main(void)

{

int err = bt_enable(NULL);

if (err) {

LOG_ERR("Bluetooth init failed: %d", err);

return err;

}

LOG_INF("Bluetooth initialized");

err = bt_le_adv_start(BT_LE_ADV_CONN, ad, ARRAY_SIZE(ad), NULL, 0);

if (err) {

LOG_ERR("Advertising start failed: %d", err);

return err;

}

LOG_INF("BLE advertising as '%s'", CONFIG_BT_DEVICE_NAME);

uint16_t counter = 0;

while (1) {

/* Update sensor value with simulated data */

sensor_value[0] = (counter >> 8) & 0xFF;

sensor_value[1] = counter & 0xFF;

counter++;

/* Send BLE notification if subscribed */

if (notify_enabled) {

bt_gatt_notify(NULL, &sensor_svc.attrs[1],

sensor_value, sizeof(sensor_value));

}

k_msleep(100);

}

}

# 8. Power Management

## 8.1 Enabling Power Management

# prj.conf

CONFIG_PM=y

CONFIG_PM_DEVICE=y

CONFIG_PM_DEVICE_RUNTIME=y

CONFIG_ESP32_LIGHT_SLEEP=y

## 8.2 Setting Power States

#include <zephyr/pm/pm.h>

#include <zephyr/pm/device.h>

/* Request light sleep from a thread */

void power_saving_task(void *p1, void *p2, void *p3)

{

while (1) {

/* Do work... */

LOG_INF("Active: processing...");

k_msleep(10);

/* Enter light sleep for 990 ms */

/* Zephyr PM automatically manages sleep when

no threads are active — configure tickless kernel */

k_msleep(990);

}

}

# Enable tickless kernel for automatic power savings

CONFIG_TICKLESS_KERNEL=y

## 8.3 Runtime Power Management for Peripherals

/* Suspend an I2C device when not in use */

const struct device *i2c = DEVICE_DT_GET(DT_NODELABEL(i2c0));

/* Power down peripheral */

pm_device_action_run(i2c, PM_DEVICE_ACTION_SUSPEND);

/* Power up peripheral */

pm_device_action_run(i2c, PM_DEVICE_ACTION_RESUME);

# 9. Logging and Debugging

## 9.1 Zephyr Logging System

#include <zephyr/logging/log.h>

/* Register a module with a name */

LOG_MODULE_REGISTER(my_module, LOG_LEVEL_DBG);

/* Log at different levels */

LOG_ERR("Critical error: code %d", err);

LOG_WRN("Warning: value out of range");

LOG_INF("Info: device initialized");

LOG_DBG("Debug: raw = 0x%04X", raw_val);

## 9.2 Runtime Filter via Shell

# prj.conf - enable shell for runtime log control

CONFIG_SHELL=y

CONFIG_LOG_BACKEND_UART=y

CONFIG_LOG_RUNTIME_FILTERING=y

# In serial terminal:

uart:~$ log enable dbg my_module

uart:~$ log disable inf my_module

## 9.3 Assert and Error Handling

#include <zephyr/sys/__assert.h>

/* Hard assert — fatal error if condition false */

__ASSERT(ptr != NULL, "Pointer must not be NULL");

/* Soft assert — logs warning but continues */

__ASSERT_NO_MSG(len > 0);

# 10. Quick Reference

## Kernel API Cheat Sheet

| API | Description |
| --- | --- |
| k_msleep(ms) | Sleep for milliseconds, yielding CPU |
| k_uptime_get() | Get system uptime in ms (64-bit) |
| k_uptime_get_32() | Get system uptime in ms (32-bit) |
| k_sem_give(&sem) | Signal (give) a semaphore |
| k_sem_take(&sem, timeout) | Wait (take) a semaphore |
| k_mutex_lock(&mtx, timeout) | Lock a mutex |
| k_mutex_unlock(&mtx) | Unlock a mutex |
| k_msgq_put(&q, &data, timeout) | Enqueue message |
| k_msgq_get(&q, &data, timeout) | Dequeue message |
| k_thread_create() | Create a new thread |
| k_thread_suspend(&tid) | Suspend thread execution |
| k_thread_resume(&tid) | Resume suspended thread |
| K_SECONDS(s) | Convert seconds to kernel ticks |
| K_MSEC(ms) | Convert ms to kernel ticks |
| K_FOREVER | Wait indefinitely (timeout constant) |
| K_NO_WAIT | Return immediately if not ready |

## West Build Commands

west build -b esp32_devkitc_wroom ./app    # Build

west build -t menuconfig                    # GUI config

west build -t guiconfig                     # Qt GUI config

west flash --esp-device=/dev/ttyUSB0        # Flash

west espressif monitor                       # Serial monitor

west build -t pristine                       # Clean build

west update                                  # Update modules

Zephyr RTOS ESP32 Tutorial  |  Embedded Firmware Reference  |  Generated 2026