# Zephyr RTOS Quick Start

For FreeRTOS Developers

# Introduction

If you already know FreeRTOS, learning Zephyr is straightforward - the core RTOS concepts are identical. This tutorial focuses on what's different and shows you the Zephyr equivalents of FreeRTOS patterns you already know.

Key Differences from FreeRTOS:

- Device Tree for hardware configuration (vs. C header files)

- Kconfig for build configuration (like Linux kernel)

- More standardized HAL across platforms

- Built-in device driver model

- Native support for multiple architectures

- Better security features (MPU, stack guards, etc.)

# 1. FreeRTOS vs Zephyr Terminology

Here's a quick translation table:

FreeRTOS              Zephyr

--------              ------

Task                  Thread

Queue                 Message Queue / FIFO

Semaphore             Semaphore (same)

Mutex                 Mutex (same)

Timer                 Timer

Event Group           Event

xTaskCreate()         k_thread_create()

vTaskDelay()          k_sleep() / k_msleep()

xQueueSend()          k_msgq_put() / k_fifo_put()

xSemaphoreTake()      k_sem_take()

# 2. Thread Creation Comparison

## FreeRTOS Way (what you know)

void sensor_task(void *param) {

while(1) {

readSensor();

vTaskDelay(pdMS_TO_TICKS(100));

}

}

xTaskCreate(sensor_task, "Sensor", 2048, NULL, 5, NULL);

## Zephyr Way (very similar)

void sensor_thread(void *p1, void *p2, void *p3) {

while(1) {

readSensor();

k_msleep(100);  // Simpler!

}

}

K_THREAD_DEFINE(sensor_tid, 2048, sensor_thread,

NULL, NULL, NULL, 5, 0, 0);

## Or Dynamic Thread Creation

K_THREAD_STACK_DEFINE(sensor_stack, 2048);

struct k_thread sensor_thread_data;

k_thread_create(&sensor_thread_data, sensor_stack,

K_THREAD_STACK_SIZEOF(sensor_stack),

sensor_thread, NULL, NULL, NULL,

5, 0, K_NO_WAIT);

# 3. Synchronization Primitives

## Semaphores

FreeRTOS:

SemaphoreHandle_t sem = xSemaphoreCreateBinary();

xSemaphoreGive(sem);

xSemaphoreTake(sem, portMAX_DELAY);

Zephyr:

K_SEM_DEFINE(sem, 0, 1);  // Static definition

k_sem_give(&sem);

k_sem_take(&sem, K_FOREVER);

## Mutexes

FreeRTOS:

SemaphoreHandle_t mutex = xSemaphoreCreateMutex();

xSemaphoreTake(mutex, portMAX_DELAY);

xSemaphoreGive(mutex);

Zephyr:

K_MUTEX_DEFINE(mutex);

k_mutex_lock(&mutex, K_FOREVER);

k_mutex_unlock(&mutex);

## Message Queues

FreeRTOS:

QueueHandle_t queue = xQueueCreate(10, sizeof(uint32_t));

uint32_t value = 42;

xQueueSend(queue, &value, portMAX_DELAY);

xQueueReceive(queue, &value, portMAX_DELAY);

Zephyr:

K_MSGQ_DEFINE(queue, sizeof(uint32_t), 10, 4);

uint32_t value = 42;

k_msgq_put(&queue, &value, K_FOREVER);

k_msgq_get(&queue, &value, K_FOREVER);

# 4. Device Tree - The Big Difference

Device Tree is Zephyr's hardware description system. Instead of C header files with register addresses, you use a declarative .dts file.

## Example Device Tree (app.overlay)

&i2c0 {

status = "okay";

imu@68 {

compatible = "invensense,icm42688";

reg = <0x68>;

status = "okay";

};

};

## Accessing Device Tree in Code

#define IMU_NODE DT_NODELABEL(imu)

const struct device *imu_dev = DEVICE_DT_GET(IMU_NODE);

if (!device_is_ready(imu_dev)) {

printk("IMU device not ready\n");

return;

}

# 5. Sensor API - High Level Abstraction

Zephyr has a standardized sensor API that works across all sensors:

#include <zephyr/drivers/sensor.h>

const struct device *imu = DEVICE_DT_GET(DT_NODELABEL(imu));

// Fetch all sensor data

sensor_sample_fetch(imu);

// Get accelerometer data

struct sensor_value accel[3];

sensor_channel_get(imu, SENSOR_CHAN_ACCEL_XYZ, accel);

// Get gyroscope data

struct sensor_value gyro[3];

sensor_channel_get(imu, SENSOR_CHAN_GYRO_XYZ, gyro);

printk("Accel: x=%d.%d y=%d.%d z=%d.%d\n",

accel[0].val1, accel[0].val2,

accel[1].val1, accel[1].val2,

accel[2].val1, accel[2].val2);

# 6. GPIO in Zephyr

#include <zephyr/drivers/gpio.h>

// Get LED from device tree

#define LED0_NODE DT_ALIAS(led0)

static const struct gpio_dt_spec led = GPIO_DT_SPEC_GET(LED0_NODE, gpios);

void main(void) {

gpio_pin_configure_dt(&led, GPIO_OUTPUT_ACTIVE);

while(1) {

gpio_pin_toggle_dt(&led);

k_msleep(1000);

}

}

# 7. Build System (Kconfig)

Instead of FreeRTOSConfig.h, Zephyr uses Kconfig files:

# prj.conf

CONFIG_SERIAL=y

CONFIG_UART_CONSOLE=y

CONFIG_I2C=y

CONFIG_SENSOR=y

CONFIG_ICM42688=y

# Stack sizes

CONFIG_MAIN_STACK_SIZE=2048

# Heap size

CONFIG_HEAP_MEM_POOL_SIZE=4096

# 8. Complete Example - Sensor Reading Thread

#include <zephyr/kernel.h>

#include <zephyr/drivers/sensor.h>

#define IMU_NODE DT_NODELABEL(imu)

K_SEM_DEFINE(data_ready, 0, 1);

void sensor_thread(void *p1, void *p2, void *p3) {

const struct device *imu = DEVICE_DT_GET(IMU_NODE);

if (!device_is_ready(imu)) {

printk("IMU not ready!\n");

return;

}

while(1) {

k_sem_take(&data_ready, K_FOREVER);

sensor_sample_fetch(imu);

struct sensor_value accel[3];

sensor_channel_get(imu, SENSOR_CHAN_ACCEL_XYZ, accel);

printk("Accel: x=%d.%d m/s²\n", accel[0].val1, accel[0].val2);

}

}

K_THREAD_DEFINE(sensor_tid, 2048, sensor_thread,

NULL, NULL, NULL, 5, 0, 0);

# 9. Key Advantages for Interview Discussion

- Device Tree makes hardware configuration declarative and reviewable

- Standardized HAL works across ESP32, iMX8, NXP, STM32, etc.

- Built-in security features (MPU, stack guards, thread isolation)

- Better driver model - easier to swap hardware

- Kconfig provides compile-time optimization

- Active community and growing ecosystem

# 10. Interview Talking Points

When asked about Zephyr:

"I have strong FreeRTOS experience, and I understand Zephyr uses very similar RTOS concepts - threads instead of tasks, message queues, semaphores, and mutexes work almost identically. The main differences I'm aware of are:"

- Device Tree for hardware configuration

- Kconfig for build system

- More standardized HAL across platforms

- Built-in device driver model

"The core skills transfer directly - I already understand priority-based scheduling, inter-task communication, synchronization primitives, and real-time constraints. Learning Zephyr is mostly about learning new APIs and the Device Tree syntax, which I can pick up quickly with your team's coding standards and examples."

# Learning Resources

- Official Docs: docs.zephyrproject.org

- Device Tree Guide: devicetree.org

- Zephyr samples directory: zephyrproject-rtos/zephyr/samples

- Nordic Semiconductor Developer Academy (free courses)

- YouTube: Zephyr Developer Summit talks

# Practice Exercises

- Install Zephyr and build the "Hello World" sample

- Create a blinking LED program using Device Tree

- Port one of your FreeRTOS tasks to a Zephyr thread

- Read a sensor using the Zephyr sensor API

- Implement inter-thread communication with message queues

Final Note:

Your FreeRTOS skills translate almost 1:1 to Zephyr. Focus on understanding Device Tree and the standardized driver model - these are the main conceptual differences. The RTOS fundamentals you already know remain the same.