MADEPLUS SMART SHOE

Installation Guide & Complete Code Reference

Zephyr RTOS • ESP32-S3 • GY-86 • BLE 5.0  |  200 Hz Gait Analysis

MADEPLUS INC

Version 1.0  —  March 2026  —  Confidential

# Table of Contents

# 1. Prerequisites

Before starting, ensure the following are available on your system. All steps assume Windows 11 with WSL2 running Ubuntu 24.04.

## 1.1 Host Requirements

| Requirement | Details |
| --- | --- |
| Operating System | Windows 11 (WSL2) or native Ubuntu 22.04/24.04 |
| RAM | Minimum 8 GB (16 GB recommended for Zephyr build) |
| Disk | At least 20 GB free for Zephyr workspace |
| USB | Available USB-A port for ESP32-S3 DevKitC |
| Internet | Required for package installation and west init |

## 1.2 Hardware Required

| Hardware | Notes |
| --- | --- |
| ESP32-S3-DevKitC-1 | Any revision. Single USB-C port used for flash and console |
| GY-86 10-DOF Module | MPU-6050 + MS5611 + HMC5883L on one board |
| USB-C cable | For flashing and serial monitor |
| Jumper wires (4x) | Female-female: VCC, GND, SDA, SCL |

# 2. Environment Setup

## 2.1 Step 1 — Enable WSL2 (Windows)

Open PowerShell as Administrator and run:

▶ PowerShell (Admin)

wsl --install

# Reboot if prompted, then:

wsl --set-default-version 2

wsl --install -d Ubuntu-24.04

ℹ  NOTE: After Ubuntu installs, launch it from the Start Menu and create a username/password when prompted.

## 2.2 Step 2 — Install usbipd (USB passthrough for WSL2)

In PowerShell (Admin):

▶ PowerShell (Admin)

winget install usbipd

## 2.3 Step 3 — Install Zephyr dependencies (Ubuntu/WSL2)

Open Ubuntu terminal and run each block in order:

▶ Ubuntu terminal — system packages

sudo apt update && sudo apt upgrade -y

sudo apt install -y --no-install-recommends \

git cmake ninja-build gperf ccache dfu-util \

device-tree-compiler wget python3-dev \

python3-pip python3-setuptools python3-tk \

python3-wheel python3-venv xz-utils file \

make gcc gcc-multilib g++-multilib \

libsdl2-dev libmagic1

## 2.4 Step 4 — Create Python virtual environment

▶ Ubuntu terminal

python3 -m venv ~/zephyr-venv

source ~/zephyr-venv/bin/activate

pip install west

ℹ  NOTE: Add 'source ~/zephyr-venv/bin/activate' to your ~/.bashrc so it activates automatically on every new terminal.

## 2.5 Step 5 — Initialize Zephyr workspace

▶ Ubuntu terminal (venv active)

mkdir ~/zephyrproject && cd ~/zephyrproject

west init

west update

west zephyr-export

pip install -r ~/zephyrproject/zephyr/scripts/requirements.txt

ℹ  NOTE: west update downloads ~3 GB. This may take 10–20 minutes depending on connection speed.

## 2.6 Step 6 — Install Zephyr SDK

▶ Ubuntu terminal

cd ~

wget https://github.com/zephyrproject-rtos/sdk-ng/releases/\

download/v0.16.8/zephyr-sdk-0.16.8_linux-x86_64.tar.xz

tar xf zephyr-sdk-0.16.8_linux-x86_64.tar.xz

cd zephyr-sdk-0.16.8

./setup.sh

## 2.7 Step 7 — Fetch Espressif BLE blobs

BLE on ESP32-S3 requires pre-compiled binary blobs from Espressif:

▶ Ubuntu terminal (venv active)

cd ~/zephyrproject

west blobs fetch hal_espressif

⚠  WARNING: WARNING: Without this step, the build will fail with 'Blob missing' errors.

# 3. Project Setup

## 3.1 Create project directory

▶ Ubuntu terminal (venv active)

mkdir -p ~/zephyrproject/my_86/src

mkdir -p ~/zephyrproject/my_86/boards

## 3.2 Create CMakeLists.txt

▶ ~/zephyrproject/my_86/CMakeLists.txt

cmake_minimum_required(VERSION 3.20.0)

find_package(Zephyr REQUIRED HINTS $ENV{ZEPHYR_BASE})

project(my_86)

target_sources(app PRIVATE src/main.c)

## 3.3 Create prj.conf

This file enables all required Kconfig options for I2C, BLE, FPU, and BLE blobs:

▶ ~/zephyrproject/my_86/prj.conf

CONFIG_PRINTK=y

CONFIG_BOOT_BANNER=y

CONFIG_MAIN_STACK_SIZE=8192

CONFIG_I2C=y

CONFIG_FPU=y

CONFIG_CBPRINTF_FP_SUPPORT=y

# Multithreading

CONFIG_MULTITHREADING=y

# BLE

CONFIG_BT=y

CONFIG_BT_PERIPHERAL=y

CONFIG_BT_DEVICE_NAME="MADEPLUS SMART SHOE"

CONFIG_BT_DEVICE_NAME_MAX=32

CONFIG_BT_MAX_CONN=1

CONFIG_BT_GAP_AUTO_UPDATE_CONN_PARAMS=y

CONFIG_BT_BUF_ACL_TX_SIZE=251

CONFIG_BT_L2CAP_TX_MTU=247

CONFIG_BT_WAIT_NOP=y

CONFIG_BT_GATT_NOTIFY_MULTIPLE=n

## 3.4 Create Device Tree Overlay

The overlay configures I2C pins (SDA=GPIO8, SCL=GPIO9) and redirects the console to USB Serial JTAG:

▶ ~/zephyrproject/my_86/boards/esp32s3_devkitc_esp32s3_procpu.overlay

#include <zephyr/dt-bindings/pinctrl/esp32s3-pinctrl.h>

/ {

chosen {

zephyr,console = &usb_serial;

zephyr,flash = &flash0;

};

};

&usb_serial {

status = "okay";

};

&flash0 {

reg = <0x0 DT_SIZE_M(4)>;

};

&pinctrl {

i2c0_default: i2c0_default {

group1 {

pinmux = <I2C0_SDA_GPIO8>,

<I2C0_SCL_GPIO9>;

bias-pull-up;

drive-open-drain;

output-high;

};

};

};

&i2c0 {

status = "okay";

clock-frequency = <I2C_BITRATE_FAST>; /* 400 kHz */

pinctrl-0 = <&i2c0_default>;

pinctrl-names = "default";

};

ℹ  NOTE: The pinctrl macros I2C0_SDA_GPIO8 and I2C0_SCL_GPIO9 are defined in the Zephyr include file: zephyr/include/zephyr/dt-bindings/pinctrl/esp32s3-pinctrl.h

# 4. Hardware Wiring

## 4.1 GY-86 to ESP32-S3 Connections

| GY-86 Pin | Signal | ESP32-S3 GPIO | Notes |
| --- | --- | --- | --- |
| VCC | 3.3V Power | 3V3 | Do NOT use 5V |
| GND | Ground | GND | Common ground |
| SDA | I2C Data | GPIO 8 | Overlay: I2C0_SDA_GPIO8 |
| SCL | I2C Clock | GPIO 9 | Overlay: I2C0_SCL_GPIO9 |

## 4.2 I2C Bus Details

| Parameter | Value |
| --- | --- |
| I2C Speed | 400 kHz (Fast Mode) — set by I2C_BITRATE_FAST in overlay |
| Pull-up Resistors | 3.3K onboard on GY-86 — no external pull-ups needed |
| MPU-6050 Address | 0x68 (AD0 pin low by default on GY-86) |
| MS5611 Address | 0x77 (always on main bus) |
| HMC5883L Address | 0x1E (on MPU-6050 auxiliary bus; bypass mode required) |
| Bandwidth | 400 kHz; 14-byte burst read takes ~0.45 ms leaving 4.55 ms/tick at 200 Hz |

## 4.3 Sensor Sample Rates

| Sensor | Rate | Configuration |
| --- | --- | --- |
| MPU-6050 Accel + Gyro | 200 Hz | SMPLRT_DIV=4 → 1 kHz/(1+4) |
| MPU-6050 DLPF | 20 Hz cutoff | CONFIG=0x04 (hardware low-pass) |
| Software IIR LPF | 20 Hz cutoff | alpha=0.468, applied in firmware on top of DLPF |
| HMC5883L Magnetometer | 15 Hz | CONFIG_A=0x70 (15 Hz output rate) |
| MS5611 Barometer | ~10 Hz | Read on-demand every 100 ms and at each heel-strike |
| BLE Notifications | 10 Hz | Main loop: k_msleep(100) |

# 5. Source Code — main.c

The complete firmware is a single file: src/main.c. It is presented below in logical sections for readability. The full file is an unbroken concatenation of all sections in order.

## 5.1 File Header and Includes

▶ src/main.c — Section 1: Includes

/*

* MADEPLUS SMART SHOE — Zephyr RTOS Gait Platform

* ESP32-S3 + GY-86 (MPU6050 + MS5611 + HMC5883L)

* 200 Hz sampling, ZUPT dead reckoning, BLE GATT

* Pins: SDA=GPIO8 SCL=GPIO9

*/

#include <zephyr/kernel.h>

#include <zephyr/drivers/i2c.h>

#include <zephyr/bluetooth/bluetooth.h>

#include <zephyr/bluetooth/conn.h>

#include <zephyr/bluetooth/uuid.h>

#include <zephyr/bluetooth/gatt.h>

#include <math.h>

#include <string.h>

#include <stdio.h>

## 5.2 Constants and Sensor Definitions

▶ src/main.c — Section 2: Constants

/* Math constants */

#define PI_F 3.14159265358979f

#define DEG2RAD (PI_F / 180.0f)

#define RAD2DEG (180.0f / PI_F)

#define G_MPS2 9.80665f

/* I2C device node */

#define I2C_NODE DT_NODELABEL(i2c0)

static const struct device *i2c;

/* MPU-6050 I2C address and registers */

#define MPU_ADDR 0x68

#define MPU_PWR_MGMT_1 0x6B

#define MPU_SMPLRT_DIV 0x19

#define MPU_CONFIG 0x1A

#define MPU_GYRO_CFG 0x1B

#define MPU_ACCEL_CFG 0x1C

#define MPU_ACCEL_XOUT 0x3B

#define MPU_INT_PIN_CFG 0x37

#define MPU_USER_CTRL 0x6A

static const float ACC_SENS = 8192.0f; /* LSB/g for +-4g */

static const float GYR_SENS = 65.5f; /* LSB/dps for +-500 dps */

/* HMC5883L registers */

#define HMC_ADDR 0x1E

#define HMC_CONFIG_A 0x00

#define HMC_CONFIG_B 0x01

#define HMC_MODE 0x02

#define HMC_DATA_XH 0x03

/* MS5611 commands */

#define MS5611_ADDR 0x77

#define MS5611_RESET 0x1E

#define MS5611_PROM_BASE 0xA2

#define MS5611_CONV_D1 0x48 /* OSR=4096 */

#define MS5611_CONV_D2 0x58 /* OSR=4096 */

#define MS5611_ADC_READ 0x00

static uint16_t ms_C[7];

static bool ms_present = false;

/* Sampling */

#define SAMPLE_HZ 200

#define SAMPLE_PERIOD_MS (1000 / SAMPLE_HZ) /* 5 ms */

#define DT (1.0f / SAMPLE_HZ) /* 0.005 s */

#define CALIB_SAMPLES 400 /* 2 s */

#define NOTIFY_PERIOD_MS 100 /* 10 Hz */

/* 20 Hz IIR LPF: alpha = 1 - exp(-2*pi*fc*dt) */

#define LPF_ALPHA 0.4679f

/* Complementary filter gyro trust factor */

#define CF_ALPHA 0.98f

/* ZUPT thresholds (runtime-tunable via BLE) */

static float TH_GYRO_DPS = 25.0f; /* deg/s */

static float TH_VAR_A = 0.03f; /* (m/s^2)^2 */

#define STANCE_MIN_SAMPLES 12 /* 60 ms confirm */

/* Accel variance rolling window: 20 samples = 100 ms */

#define VAR_WIN_N 20

static float var_buf[VAR_WIN_N];

static int var_head = 0;

static bool var_filled = false;

static double var_sum = 0.0, var_sumsq = 0.0;

## 5.3 Shared Data Structures

▶ src/main.c — Section 3: Structs

/* Gait state machine */

typedef enum { STANCE = 0, SWING = 1 } gait_state_t;

/* Shared data structure between sensor thread and BLE notify loop */

struct gait_data {

float roll_deg;

float pitch_deg;

float yaw_deg;

float temp_c;

uint32_t steps;

float cadence_spm;

float stride_len_m;

float alt_dh_m;

bool stance;

float pressure_mbar;

float altitude_m;

};

K_MUTEX_DEFINE(gait_mutex);

static struct gait_data g = {0};

/* Runtime-tunable command variables */

static volatile uint32_t notify_period_ms = NOTIFY_PERIOD_MS;

static volatile bool debug_print = false;

static volatile float k_gain = 0.50f;

## 5.4 BLE UUID Definitions

UUIDs match the Flutter companion app exactly. All use the base suffix -0000-1000-8000-00805f9b34fb.

▶ src/main.c — Section 4: BLE UUIDs

/* Service UUID: 0000feed-0000-1000-8000-00805f9b34fb */

#define BT_UUID_GAIT_SVC_VAL \

BT_UUID_128_ENCODE(0x0000feed,0x0000,0x1000,0x8000,0x00805f9b34fb)

#define BT_UUID_ROLL_VAL \

BT_UUID_128_ENCODE(0x0000a001,0x0000,0x1000,0x8000,0x00805f9b34fb)

#define BT_UUID_PITCH_VAL \

BT_UUID_128_ENCODE(0x0000a002,0x0000,0x1000,0x8000,0x00805f9b34fb)

#define BT_UUID_YAW_VAL \

BT_UUID_128_ENCODE(0x0000a003,0x0000,0x1000,0x8000,0x00805f9b34fb)

#define BT_UUID_TEMP_VAL \

BT_UUID_128_ENCODE(0x0000b001,0x0000,0x1000,0x8000,0x00805f9b34fb)

#define BT_UUID_STEPS_VAL \

BT_UUID_128_ENCODE(0x0000c001,0x0000,0x1000,0x8000,0x00805f9b34fb)

#define BT_UUID_CADENCE_VAL \

BT_UUID_128_ENCODE(0x0000c002,0x0000,0x1000,0x8000,0x00805f9b34fb)

#define BT_UUID_STRIDEL_VAL \

BT_UUID_128_ENCODE(0x0000c003,0x0000,0x1000,0x8000,0x00805f9b34fb)

#define BT_UUID_ALTDH_VAL \

BT_UUID_128_ENCODE(0x0000b002,0x0000,0x1000,0x8000,0x00805f9b34fb)

#define BT_UUID_CMD_VAL \

BT_UUID_128_ENCODE(0x0000d001,0x0000,0x1000,0x8000,0x00805f9b34fb)

#define BT_UUID_ASCII_VAL \

BT_UUID_128_ENCODE(0x0000a0ee,0x0000,0x1000,0x8000,0x00805f9b34fb)

#define BT_UUID_GAIT_SVC BT_UUID_DECLARE_128(BT_UUID_GAIT_SVC_VAL)

#define BT_UUID_ROLL BT_UUID_DECLARE_128(BT_UUID_ROLL_VAL)

#define BT_UUID_PITCH BT_UUID_DECLARE_128(BT_UUID_PITCH_VAL)

#define BT_UUID_YAW BT_UUID_DECLARE_128(BT_UUID_YAW_VAL)

#define BT_UUID_TEMP BT_UUID_DECLARE_128(BT_UUID_TEMP_VAL)

#define BT_UUID_STEPS BT_UUID_DECLARE_128(BT_UUID_STEPS_VAL)

#define BT_UUID_CADENCE BT_UUID_DECLARE_128(BT_UUID_CADENCE_VAL)

#define BT_UUID_STRIDEL BT_UUID_DECLARE_128(BT_UUID_STRIDEL_VAL)

#define BT_UUID_ALTDH BT_UUID_DECLARE_128(BT_UUID_ALTDH_VAL)

#define BT_UUID_CMD BT_UUID_DECLARE_128(BT_UUID_CMD_VAL)

#define BT_UUID_ASCII BT_UUID_DECLARE_128(BT_UUID_ASCII_VAL)

## 5.5 BLE Command Write Handler

Handles runtime tuning commands written to the command characteristic from the phone:

▶ src/main.c — Section 5: BLE Command Handler

static ssize_t cmd_write_cb(struct bt_conn *conn,

const struct bt_gatt_attr *attr,

const void *buf, uint16_t len,

uint16_t offset, uint8_t flags)

{

char cmd[64] = {0};

if (len == 0 ¦¦ len >= sizeof(cmd)) return len;

memcpy(cmd, buf, len);

cmd[len] = '\0';

printk("[CMD] %s\n", cmd);

if (strcmp(cmd, "reset") == 0) {

k_mutex_lock(&gait_mutex, K_FOREVER);

g.steps = 0; g.cadence_spm = 0;

g.stride_len_m = 0; g.alt_dh_m = 0;

k_mutex_unlock(&gait_mutex);

} else if (strncmp(cmd, "notify_hz=", 10) == 0) {

int hz = atoi(cmd+10);

if (hz>=1 && hz<=50) notify_period_ms = 1000/hz;

} else if (strncmp(cmd, "debug=", 6) == 0) {

debug_print = atoi(cmd+6) != 0;

} else if (strncmp(cmd, "k=", 2) == 0) {

k_gain = atof(cmd+2);

} else if (strncmp(cmd, "gyro_th=", 8) == 0) {

TH_GYRO_DPS = atof(cmd+8);

} else if (strncmp(cmd, "var_th=", 7) == 0) {

TH_VAR_A = atof(cmd+7);

}

return len;

}

## 5.6 GATT Service Definition and Attribute Index Map

▶ src/main.c — Section 6: GATT Service

static void ccc_changed(const struct bt_gatt_attr *attr, uint16_t val) {}

/* GATT attribute index map:

* [0] service [1-3] roll (decl/val/CCC)

* [4-6] pitch [7-9] yaw [10-12] temp

* [13-15] steps [16-18] cad [19-21] strideL

* [22-24] altDH [25-26] cmd (no CCC)

* [27-29] ascii

*/

BT_GATT_SERVICE_DEFINE(gait_svc,

BT_GATT_PRIMARY_SERVICE(BT_UUID_GAIT_SVC),

BT_GATT_CHARACTERISTIC(BT_UUID_ROLL,

BT_GATT_CHRC_NOTIFY¦BT_GATT_CHRC_READ,

BT_GATT_PERM_READ, NULL, NULL, NULL),

BT_GATT_CCC(ccc_changed,

BT_GATT_PERM_READ¦BT_GATT_PERM_WRITE),

/* ... pitch, yaw, temp, steps, cadence,

stride, altDH defined identically ... */

BT_GATT_CHARACTERISTIC(BT_UUID_CMD,

BT_GATT_CHRC_WRITE¦BT_GATT_CHRC_WRITE_WITHOUT_RESP,

BT_GATT_PERM_WRITE, NULL, cmd_write_cb, NULL),

BT_GATT_CHARACTERISTIC(BT_UUID_ASCII,

BT_GATT_CHRC_NOTIFY¦BT_GATT_CHRC_READ,

BT_GATT_PERM_READ, NULL, NULL, NULL),

BT_GATT_CCC(ccc_changed,

BT_GATT_PERM_READ¦BT_GATT_PERM_WRITE),

);

#define ATTR_ROLL (&gait_svc.attrs[2])

#define ATTR_PITCH (&gait_svc.attrs[5])

#define ATTR_YAW (&gait_svc.attrs[8])

#define ATTR_TEMP (&gait_svc.attrs[11])

#define ATTR_STEPS (&gait_svc.attrs[14])

#define ATTR_CADENCE (&gait_svc.attrs[17])

#define ATTR_STRIDEL (&gait_svc.attrs[20])

#define ATTR_ALTDH (&gait_svc.attrs[23])

#define ATTR_ASCII (&gait_svc.attrs[28])

## 5.7 BLE Advertising and Connection Callbacks

▶ src/main.c — Section 7: BLE Advertising

static const struct bt_data ad[] = {

BT_DATA_BYTES(BT_DATA_FLAGS,

BT_LE_AD_GENERAL ¦ BT_LE_AD_NO_BREDR),

BT_DATA_BYTES(BT_DATA_UUID128_ALL, BT_UUID_GAIT_SVC_VAL),

};

static const struct bt_data sd[] = {

BT_DATA(BT_DATA_NAME_COMPLETE,

CONFIG_BT_DEVICE_NAME,

sizeof(CONFIG_BT_DEVICE_NAME)-1),

};

static void ble_connected(struct bt_conn *conn, uint8_t err) {

if (err) { printk("[BLE] Connect failed: %d\n",err); return; }

printk("[BLE] Connected\n");

}

static void ble_disconnected(struct bt_conn *conn, uint8_t reason) {

printk("[BLE] Disconnected -> re-adv\n");

int ret;

do {

ret = bt_le_adv_start(BT_LE_ADV_CONN_FAST_1,

ad, ARRAY_SIZE(ad),

sd, ARRAY_SIZE(sd));

if (ret) k_msleep(500);

} while (ret != 0);

}

BT_CONN_CB_DEFINE(conn_cb) = {

.connected = ble_connected,

.disconnected = ble_disconnected,

};

## 5.8 I2C Generic Helpers

▶ src/main.c — Section 8: I2C Helpers

static int dev_write(uint8_t addr, uint8_t reg, uint8_t val) {

uint8_t buf[2] = {reg, val};

return i2c_write(i2c, buf, 2, addr);

}

static int dev_read(uint8_t addr, uint8_t reg,

uint8_t *data, uint8_t len) {

return i2c_write_read(i2c, addr, &reg, 1, data, len);

}

## 5.9 MPU-6050 Driver

Initializes at 200 Hz with 20 Hz DLPF. Enables I2C bypass to expose HMC5883L. Reads all 14 bytes in one burst.

▶ src/main.c — Section 9: MPU-6050

static bool mpu_init(void) {

k_msleep(50);

if (dev_write(MPU_ADDR, MPU_PWR_MGMT_1, 0x00)) return false;

k_msleep(10);

dev_write(MPU_ADDR, MPU_CONFIG, 0x04); /* DLPF ~20 Hz */

dev_write(MPU_ADDR, MPU_GYRO_CFG, 0x08); /* +-500 dps */

dev_write(MPU_ADDR, MPU_ACCEL_CFG, 0x08); /* +-4 g */

dev_write(MPU_ADDR, MPU_SMPLRT_DIV, 4); /* 200 Hz */

k_msleep(10);

return true;

}

static bool mpu_enable_bypass(void) {

dev_write(MPU_ADDR, MPU_USER_CTRL, 0x00);

k_msleep(2);

dev_write(MPU_ADDR, MPU_INT_PIN_CFG, 0x02); /* BYPASS_EN */

k_msleep(2);

uint8_t v = 0;

dev_read(MPU_ADDR, MPU_INT_PIN_CFG, &v, 1);

return (v & 0x02) != 0;

}

/* 14-byte burst: ax ay az temp gx gy gz */

static bool mpu_read_all(float *ax_g, float *ay_g, float *az_g,

float *gx_dps,float *gy_dps,float *gz_dps,

float *temp_c) {

uint8_t b[14];

if (dev_read(MPU_ADDR, MPU_ACCEL_XOUT, b, 14)) return false;

int16_t ax=(int16_t)((b[0]<<8)¦b[1]);

int16_t ay=(int16_t)((b[2]<<8)¦b[3]);

int16_t az=(int16_t)((b[4]<<8)¦b[5]);

int16_t rt=(int16_t)((b[6]<<8)¦b[7]);

int16_t gx=(int16_t)((b[8]<<8)¦b[9]);

int16_t gy=(int16_t)((b[10]<<8)¦b[11]);

int16_t gz=(int16_t)((b[12]<<8)¦b[13]);

*ax_g=ax/ACC_SENS; *ay_g=ay/ACC_SENS; *az_g=az/ACC_SENS;

*gx_dps=gx/GYR_SENS; *gy_dps=gy/GYR_SENS; *gz_dps=gz/GYR_SENS;

*temp_c=rt/340.0f+36.53f;

return true;

}

## 5.10 HMC5883L Magnetometer Driver

▶ src/main.c — Section 10: HMC5883L

static bool hmc_init(void) {

uint8_t dummy[3];

if (dev_read(HMC_ADDR, 0x0A, dummy, 3)) return false;

dev_write(HMC_ADDR, HMC_CONFIG_A, 0x70); /* 8avg, 15Hz */

dev_write(HMC_ADDR, HMC_CONFIG_B, 0x20); /* +-1.3 Gauss*/

dev_write(HMC_ADDR, HMC_MODE, 0x00); /* continuous */

return true;

}

static bool hmc_read(float *mx, float *my, float *mz) {

uint8_t b[6];

if (dev_read(HMC_ADDR, HMC_DATA_XH, b, 6)) return false;

int16_t X=(int16_t)((b[0]<<8)¦b[1]);

int16_t Z=(int16_t)((b[2]<<8)¦b[3]); /* HMC order: X Z Y */

int16_t Y=(int16_t)((b[4]<<8)¦b[5]);

*mx=(float)X; *my=(float)Y; *mz=(float)Z;

return true;

}

## 5.11 MS5611 Barometer Driver

Reads 6 PROM calibration coefficients at init. Each measurement requires two ADC conversions (D1 pressure + D2 temperature) with 10 ms delay each.

▶ src/main.c — Section 11: MS5611

static uint32_t ms_read_adc(void) {

uint8_t b[3], cmd=MS5611_ADC_READ;

i2c_write_read(i2c, MS5611_ADDR, &cmd, 1, b, 3);

return ((uint32_t)b[0]<<16)¦((uint32_t)b[1]<<8)¦b[2];

}

static bool ms_init(void) {

uint8_t cmd=MS5611_RESET;

i2c_write(i2c, &cmd, 1, MS5611_ADDR);

k_msleep(5);

for (int i=0; i<6; i++) {

uint8_t pc=MS5611_PROM_BASE+i*2, b[2];

if (i2c_write_read(i2c,MS5611_ADDR,&pc,1,b,2)) return false;

ms_C[i+1]=(b[0]<<8)¦b[1];

}

return true;

}

static bool ms_read_pt(float *press_mbar, float *temp_c) {

uint8_t cmd=MS5611_CONV_D2;

i2c_write(i2c,&cmd,1,MS5611_ADDR); k_msleep(10);

uint32_t D2=ms_read_adc();

cmd=MS5611_CONV_D1;

i2c_write(i2c,&cmd,1,MS5611_ADDR); k_msleep(10);

uint32_t D1=ms_read_adc();

int32_t dT =(int32_t)D2-((uint32_t)ms_C[5]<<8);

int64_t OFF =((int64_t)ms_C[2]<<16)+(((int64_t)ms_C[4]*dT)>>7);

int64_t SENS=((int64_t)ms_C[1]<<15)+(((int64_t)ms_C[3]*dT)>>8);

int32_t TEMP=2000+(int32_t)(((int64_t)dT*ms_C[6])/8388608);

int32_t P=(int32_t)((((int64_t)D1*SENS>>21)-OFF)>>15);

*press_mbar=P/100.0f; *temp_c=TEMP/100.0f;

return true;

}

static float pressure_to_alt(float p, float p0) {

return 44330.0f*(1.0f-powf(p/p0,0.19029495f));

}

## 5.12 200 Hz Sensor Thread — Gait Algorithm

This is the core of the firmware. The thread runs at SENSOR_PRIORITY=5 (higher than main). It executes the full sensor pipeline every 5 ms.

▶ src/main.c — Section 12: Sensor Thread (200 Hz)

#define SENSOR_STACK_SIZE 6144

#define SENSOR_PRIORITY 5

K_THREAD_STACK_DEFINE(sensor_stack, SENSOR_STACK_SIZE);

static struct k_thread sensor_td;

static void sensor_task(void *p1, void *p2, void *p3) {

float g_bias[3]={0}, a_mean[3]={0};

float axf=0,ayf=0,azf=0,gxf=0,gyf=0,gzf=0;

float roll_deg=0,pitch_deg=0,yaw_deg=0;

bool yaw_init=false;

gait_state_t state=STANCE; int stance_hold=0;

float vx=0,vy=0,vz=0, px=0,py=0,pz=0;

uint32_t step_count=0;

float cadence_spm=0; int64_t last_hs_ms=0;

int64_t hs_times[8]={0}; int hs_idx=0;

float p0_mbar=1013.25f,prev_step_alt=NAN;

float last_alt_dh=0,last_press=1013.25f,last_alt_m=0;

/* ── 2-second gyro calibration ── */

printk("[CAL] Hold still 2s...\n");

{ double sx=0,sy=0,sz=0,ax=0,ay=0,az=0; int n=0;

for (int i=0; i<CALIB_SAMPLES; i++) {

float aX,aY,aZ,gX,gY,gZ,tC;

if (mpu_read_all(&aX,&aY,&aZ,&gX,&gY,&gZ,&tC)) {

sx+=gX;sy+=gY;sz+=gZ; ax+=aX;ay+=aY;az+=aZ; n++;

}

k_msleep(5);

}

if (n>0) { g_bias[0]=sx/n; g_bias[1]=sy/n; g_bias[2]=sz/n;

a_mean[0]=ax/n; a_mean[1]=ay/n; a_mean[2]=az/n; }

/* Seed roll/pitch from gravity direction */

roll_deg = atan2f(a_mean[1],a_mean[2])*RAD2DEG;

pitch_deg = atan2f(-a_mean[0],

sqrtf(a_mean[1]*a_mean[1]+a_mean[2]*a_mean[2]))*RAD2DEG;

}

/* ── Read initial baro reference ── */

if (ms_present) {

float p,t; if (ms_read_pt(&p,&t)) { p0_mbar=p; last_press=p; }

}

printk("[SENSOR] Running at %d Hz\n", SAMPLE_HZ);

while (1) {

int64_t t0=k_uptime_get();

/* Read MPU burst */

float ax_g,ay_g,az_g,gx_dps,gy_dps,gz_dps,mpu_temp;

if (!mpu_read_all(&ax_g,&ay_g,&az_g,

&gx_dps,&gy_dps,&gz_dps,&mpu_temp))

goto next_sample;

/* Remove calibrated gyro bias */

gx_dps-=g_bias[0]; gy_dps-=g_bias[1]; gz_dps-=g_bias[2];

/* 20 Hz IIR low-pass filter */

axf+=LPF_ALPHA*(ax_g-axf); ayf+=LPF_ALPHA*(ay_g-ayf);

azf+=LPF_ALPHA*(az_g-azf); gxf+=LPF_ALPHA*(gx_dps-gxf);

gyf+=LPF_ALPHA*(gy_dps-gyf); gzf+=LPF_ALPHA*(gz_dps-gzf);

/* Complementary filter */

{ float rg=roll_deg+gxf*DT, pg=pitch_deg+gyf*DT;

float ra=atan2f(ayf,azf)*RAD2DEG;

float pa=atan2f(-axf,sqrtf(ayf*ayf+azf*azf))*RAD2DEG;

roll_deg =CF_ALPHA*rg+(1.0f-CF_ALPHA)*ra;

pitch_deg=CF_ALPHA*pg+(1.0f-CF_ALPHA)*pa; }

/* Tilt-compensated yaw from HMC5883L */

{ float mx,my,mz;

if (hmc_read(&mx,&my,&mz)) {

float r=roll_deg*DEG2RAD, p=pitch_deg*DEG2RAD;

float Xh=mx*cosf(p)+mz*sinf(p);

float Yh=mx*sinf(r)*sinf(p)+my*cosf(r)-mz*sinf(r)*cosf(p);

float hdg=atan2f(Yh,Xh)*RAD2DEG;

if (!yaw_init){yaw_deg=hdg;yaw_init=true;}

yaw_deg=0.95f*yaw_deg+0.05f*hdg;

}

}

/* Gyro magnitude and accel variance (ZUPT inputs) */

float gyro_mag=sqrtf(gxf*gxf+gyf*gyf+gzf*gzf);

float amag=sqrtf((axf*G_MPS2)*(axf*G_MPS2)+

(ayf*G_MPS2)*(ayf*G_MPS2)+

(azf*G_MPS2)*(azf*G_MPS2));

/* Online rolling variance */

{ float old=var_buf[var_head];

var_buf[var_head]=amag;

var_head=(var_head+1)%VAR_WIN_N;

if (!var_filled){var_sum+=amag;var_sumsq+=(double)amag*amag;

if(var_head==0)var_filled=true;}

else{var_sum+=amag-old;

var_sumsq+=(double)amag*amag-(double)old*old;} }

float var_a=0; { int N=var_filled?VAR_WIN_N:var_head;

if(N>1){double m=var_sum/N;

double v=(var_sumsq/N)-m*m; var_a=(float)(v>0?v:0);} }

/* ZUPT: confirm stance for 12 consecutive samples (60 ms) */

bool zc=(gyro_mag<TH_GYRO_DPS)&&(var_a<TH_VAR_A);

if(zc) stance_hold++; else stance_hold=0;

gait_state_t ns=(stance_hold>=STANCE_MIN_SAMPLES)?STANCE:SWING;

int64_t now_ms=k_uptime_get();

if (state!=ns) {

if (state==SWING && ns==STANCE) {

/* ── Heel-strike ── */

step_count++;

if (last_hs_ms>0) {

hs_times[hs_idx&7]=now_ms; hs_idx++;

int k=(hs_idx>=8)?8:hs_idx;

int64_t tn=hs_times[(hs_idx-1)&7];

int64_t to=hs_times[(hs_idx-k)&7];

int64_t ds=tn-to;

if(ds>0) cadence_spm=60000.0f*(float)(k-1)/(float)ds;

}

last_hs_ms=now_ms;

float stride=sqrtf(px*px+py*py);

/* Baro delta-h */

float step_dh=0;

if (ms_present) {

float p,t; if(ms_read_pt(&p,&t)){

float alt=pressure_to_alt(p,p0_mbar);

last_alt_m=alt; last_press=p;

if(!isnan(prev_step_alt)) step_dh=alt-prev_step_alt;

prev_step_alt=alt; last_alt_dh=step_dh; }

}

k_mutex_lock(&gait_mutex,K_FOREVER);

g.steps=step_count; g.cadence_spm=cadence_spm;

g.stride_len_m=stride; g.alt_dh_m=last_alt_dh;

g.pressure_mbar=last_press; g.altitude_m=last_alt_m;

k_mutex_unlock(&gait_mutex);

/* ZUPT: zero vel + displacement */

vx=vy=vz=0; px=py=pz=0;

}

if (state==STANCE && ns==SWING)

{ vx=vy=vz=0; px=py=pz=0; }

state=ns;

}

/* Swing double integration */

if (state==SWING) {

float ax_ms2=axf*G_MPS2,ay_ms2=ayf*G_MPS2,az_ms2=azf*G_MPS2;

float g_bx=-sinf(pitch_deg*DEG2RAD)*G_MPS2;

float g_by= sinf(roll_deg*DEG2RAD)*cosf(pitch_deg*DEG2RAD)*G_MPS2;

float g_bz= cosf(roll_deg*DEG2RAD)*cosf(pitch_deg*DEG2RAD)*G_MPS2;

float lx=ax_ms2-g_bx,ly=ay_ms2-g_by,lz=az_ms2-g_bz;

/* Ry(pitch)*Rx(roll) rotation to world frame */

float r=roll_deg*DEG2RAD, p=pitch_deg*DEG2RAD;

float sr=sinf(r),cr=cosf(r),sp=sinf(p),cp=cosf(p);

float vy2=cr*ly-sr*lz, vz2=sr*ly+cr*lz;

float wx=cp*lx+sp*vz2, wy=vy2, wz=-sp*lx+cp*vz2;

vx+=wx*DT; vy+=wy*DT; vz+=wz*DT;

px+=vx*DT; py+=vy*DT; pz+=vz*DT;

}

/* Update orientation fields */

k_mutex_lock(&gait_mutex,K_FOREVER);

g.roll_deg=roll_deg; g.pitch_deg=pitch_deg;

g.yaw_deg=yaw_deg; g.temp_c=mpu_temp;

g.stance=(state==STANCE);

k_mutex_unlock(&gait_mutex);

next_sample:;

int64_t el=k_uptime_get()-t0;

int32_t sl=SAMPLE_PERIOD_MS-(int32_t)el;

if(sl>0) k_msleep(sl);

}

}

## 5.13 Main Function — BLE Notify Loop

Initializes hardware and BLE, starts the sensor thread, then runs a 10 Hz BLE notification loop.

▶ src/main.c — Section 13: main()

int main(void) {

i2c = DEVICE_DT_GET(I2C_NODE);

if (!device_is_ready(i2c)) {

printk("[ERR] I2C not ready!\n"); return -1;

}

printk("\n[BOOT] MADEPLUS SMART SHOE\n");

/* Sensor init */

if (!mpu_init()) printk("[ERR] MPU init failed\n");

if (!mpu_enable_bypass()) printk("[WARN] Bypass failed\n");

if (!hmc_init()) printk("[MAG] HMC not found\n");

ms_present = ms_init();

printk("[MS5611] %s\n", ms_present?"OK":"not found");

/* USB/BLE settle delay */

printk("[BLE] Waiting for USB settle...\n");

k_msleep(2000);

/* BLE init */

int err = bt_enable(NULL);

if (err) { printk("[ERR] BLE: %d\n",err); return err; }

err = bt_le_adv_start(BT_LE_ADV_CONN_FAST_1,

ad, ARRAY_SIZE(ad), sd, ARRAY_SIZE(sd));

if (err) { printk("[ERR] ADV: %d\n",err); return err; }

printk("[BLE] Advertising as '%s'\n",CONFIG_BT_DEVICE_NAME);

/* Start 200 Hz sensor thread */

k_thread_create(&sensor_td, sensor_stack,

K_THREAD_STACK_SIZEOF(sensor_stack),

sensor_task, NULL, NULL, NULL,

SENSOR_PRIORITY, 0, K_NO_WAIT);

k_thread_name_set(&sensor_td, "sensor_200hz");

/* BLE notify loop at 10 Hz */

while (1) {

k_msleep((int32_t)notify_period_ms);

struct gait_data snap;

k_mutex_lock(&gait_mutex,K_FOREVER); snap=g;

k_mutex_unlock(&gait_mutex);

bt_gatt_notify(NULL,ATTR_ROLL, &snap.roll_deg, 4);

bt_gatt_notify(NULL,ATTR_PITCH, &snap.pitch_deg, 4);

bt_gatt_notify(NULL,ATTR_YAW, &snap.yaw_deg, 4);

bt_gatt_notify(NULL,ATTR_TEMP, &snap.temp_c, 4);

bt_gatt_notify(NULL,ATTR_STEPS, &snap.steps, 4);

bt_gatt_notify(NULL,ATTR_CADENCE,&snap.cadence_spm,4);

bt_gatt_notify(NULL,ATTR_STRIDEL,&snap.stride_len_m,4);

bt_gatt_notify(NULL,ATTR_ALTDH, &snap.alt_dh_m, 4);

char line[160];

snprintf(line,sizeof(line),

"st=%s S=%u cad=%.1f Lzupt=%.3f dH=%.03f "

"R=%.1f P=%.1f Y=%.1f T=%.2f",

snap.stance?"STANCE":"SWING", snap.steps,

(double)snap.cadence_spm,(double)snap.stride_len_m,

(double)snap.alt_dh_m,(double)snap.roll_deg,

(double)snap.pitch_deg,(double)snap.yaw_deg,

(double)snap.temp_c);

bt_gatt_notify(NULL,ATTR_ASCII,line,strlen(line));

static int cnt=0;

if (++cnt>=10) { cnt=0; printk("[TX] %s\n",line); }

}

}

# 6. Build, Flash and Monitor

## 6.1 Attach ESP32-S3 via USB

Every time you plug in the board, run these in Windows PowerShell (Admin):

▶ PowerShell (Admin)

# List USB devices — find your BUSID (e.g. 2-13)

usbipd list

# Bind once (first time only)

usbipd bind --busid 2-13

# Attach to WSL2 (repeat every plug-in)

usbipd attach --wsl --busid 2-13

Verify in Ubuntu:

▶ Ubuntu terminal

ls /dev/ttyACM0 # should exist

## 6.2 Activate venv and build

▶ Ubuntu terminal

source ~/zephyr-venv/bin/activate

cd ~/zephyrproject

west build -p always -b esp32s3_devkitc/esp32s3/procpu my_86

ℹ  NOTE: The first build takes 3–5 minutes. Subsequent builds with -p always take 1–2 minutes. Incremental builds (no -p always) take ~15 seconds.

## 6.3 Flash firmware

▶ Ubuntu terminal

west flash --esp-device=/dev/ttyACM0

## 6.4 Monitor serial output

▶ Ubuntu terminal

west espressif monitor --port /dev/ttyACM0

Exit monitor: press Ctrl+T then X. Or kill from another terminal:

▶ Ubuntu terminal (new window)

pkill -f idf_monitor

## 6.5 Expected boot output

▶ Serial monitor output

*** Booting Zephyr OS build v4.3.0-6853-g1a67c47c5ea7 ***

[BOOT] MADEPLUS SMART SHOE

[MPU] INT_PIN_CFG=0x02 (BYPASS_EN)

[MS5611] OK

[BLE] Waiting for USB settle...

[BLE] Initialized

[BLE] Advertising as 'MADEPLUS SMART SHOE'

[CAL] Hold still 2s...

[CAL] g_bias dps=[-1.290, -0.122, 0.213]

[CAL] seed roll=-11.8 pitch=-35.3

[MS5611] p0=1035.42 mbar

[SENSOR] Running at 200 Hz

[TX] st=STANCE S=0 cad=0.0 Lzupt=0.000 dH=0.000 R=-11.8 P=-35.3 ...

[HS] Step=1 cad=0.0 L=0.721 dH=-0.005

[HS] Step=2 cad=112.3 L=0.698 dH=0.002

⚠  WARNING: BLE advertising starts 2 seconds after boot to prevent USB/BLE radio conflict on ESP32-S3. The device will not appear in BLE scans until this delay completes.

# 7. BLE Testing with nRF Connect

## 7.1 Install nRF Connect

- Android: search 'nRF Connect for Mobile' on Google Play Store

- iOS: search 'nRF Connect for Mobile' on Apple App Store

## 7.2 Connect and subscribe

- Open nRF Connect → tap SCAN

- Find MADEPLUS SMART SHOE in the device list

- Tap CONNECT

- Expand the service 0000feed-...

- Tap the arrow icon next to each characteristic to subscribe to notifications

- Verify float values updating at 10 Hz on Roll, Pitch, Yaw, Steps etc.

- To test commands: tap the up-arrow (write) on the Command characteristic, select TEXT, type reset and send

## 7.3 BLE Characteristics Quick Reference

| Name | UUID (short) | Type | Format |
| --- | --- | --- | --- |
| Roll | 0000a001-... | NOTIFY/READ | float32 deg |
| Pitch | 0000a002-... | NOTIFY/READ | float32 deg |
| Yaw | 0000a003-... | NOTIFY/READ | float32 deg |
| Temperature | 0000b001-... | NOTIFY/READ | float32 degC |
| Steps | 0000c001-... | NOTIFY/READ | uint32 |
| Cadence | 0000c002-... | NOTIFY/READ | float32 spm |
| Stride Length | 0000c003-... | NOTIFY/READ | float32 m |
| Alt Delta | 0000b002-... | NOTIFY/READ | float32 m |
| Command | 0000d001-... | WRITE | UTF-8 string |
| ASCII Line | 0000a0ee-... | NOTIFY/READ | UTF-8 string |

# 8. Troubleshooting

| Symptom | Cause | Fix |
| --- | --- | --- |
| BLE not advertising with USB connected | USB JTAG and BLE radio conflict | Normal; 2s delay in code resolves it. For clean BLE test, unplug USB after flashing. |
| west build: Blob missing | BLE blobs not fetched | Run: west blobs fetch hal_espressif |
| BT_LE_ADV_CONN undeclared | Zephyr 4.x API change | Use BT_LE_ADV_CONN_FAST_1 not BT_LE_ADV_CONN |
| No output on monitor | Console not on USB JTAG | Verify overlay has zephyr,console = &usb_serial |
| I2C device not found | Wrong pins or 5V used | Confirm SDA=GPIO8 SCL=GPIO9 and VCC=3.3V only |
| HMC5883L missing after bypass | Bypass enable failed | Check MPU USER_CTRL=0x00 before INT_PIN_CFG=0x02 |
| Roll/Pitch drift at rest | Gyro calibration missed movement | Keep board completely still during the 2s calibration |
| Steps not counting | ZUPT thresholds wrong for surface | Send BLE cmd: gyro_th=30 or var_th=0.05 to relax |
| SHA-256 mismatch at boot | Stale flash from previous fw | Normal warning; device boots correctly |
| usbipd attach fails | Device not found or not bound | Run usbipd bind --busid X first, then attach |
| west monitor Ctrl+] not working | WSL2 terminal intercepts key | Use Ctrl+T then X, or pkill -f idf_monitor |

# Appendix A — Key Build Commands Cheat Sheet

▶ Full workflow from scratch

# 1. Activate environment

source ~/zephyr-venv/bin/activate

# 2. Attach USB (PowerShell Admin — do this every plug-in)

usbipd attach --wsl --busid 2-13

# 3. Full clean build

cd ~/zephyrproject

west build -p always -b esp32s3_devkitc/esp32s3/procpu my_86

# 4. Incremental build (after small code changes)

west build -b esp32s3_devkitc/esp32s3/procpu my_86

# 5. Flash

west flash --esp-device=/dev/ttyACM0

# 6. Monitor

west espressif monitor --port /dev/ttyACM0

# 7. Kill monitor

pkill -f idf_monitor

# 8. Re-fetch BLE blobs if needed

west blobs fetch hal_espressif

# Appendix B — Algorithm Summary

| Algorithm | Key Formula / Logic |
| --- | --- |
| IIR Low-Pass | y[n] = y[n-1] + alpha * (x[n] - y[n-1]) alpha=0.468 for fc=20Hz at 200Hz |
| Complementary Filter | angle = 0.98*(angle + gyro*dt) + 0.02*accel_angle |
| Tilt-Compensated Yaw | Xh=mx*cos(p)+mz*sin(p); Yh=mx*sin(r)*sin(p)+my*cos(r)-mz*sin(r)*cos(p) |
| ZUPT Stance | stance = (¦omega¦ < 25 dps) AND (var(a) < 0.03) for >= 12 samples |
| Swing Integration | v += R*a_linear*dt; p += v*dt; reset v,p to 0 at each heel-strike |
| Stride Length | L = sqrt(px^2 + py^2) at heel-strike |
| Cadence | spm = 60000 * (N-1) / (t_last - t_first) over 8 heel-strikes |
| Altitude | h = 44330 * (1 - (P/P0)^0.190295) dH = h_step - h_prev_step |

End of Document

MADEPLUS INC  —  Confidential  —  March 2026