ZEPHYR RTOS

Hands-On Mastery Tutorial

| ESP32-S3 Xtensa LX7 Dual-Core | STM32F446RE ARM Cortex-M4 @ 180MHz |
| --- | --- |

10 Progressive Exercises  •  Minute Details  •  WSL2 Ready

From GPIO Blink to BLE GATT & DMA — Both Boards, Side by Side

# 0.  WSL2 Environment Setup & Verification

Before writing a single line of Zephyr code, verify your WSL2 toolchain is properly configured. Subtle misconfigurations here cause mysterious build failures later.

## 0.1  SDK & West Tool Check

$ bash

# Check West version (must be >= 1.2 for Zephyr 3.x) west --version # Check SDK location — ZEPHYR_SDK_INSTALL_DIR must be set echo $ZEPHYR_SDK_INSTALL_DIR # Confirm ESP32-S3 and ARM toolchains exist ls $ZEPHYR_SDK_INSTALL_DIR/xtensa-espressif_esp32s3_zephyr-elf/ ls $ZEPHYR_SDK_INSTALL_DIR/arm-zephyr-eabi/ # Verify Zephyr base echo $ZEPHYR_BASE west topdir

## 0.2  Board Names Used in This Tutorial

| Board | Zephyr Board Name |
| --- | --- |
| ESP32-S3-DevKitC-1 | esp32s3_devkitc |
| Nucleo-F446RE | nucleo_f446re |
| ESP32-S3 generic | esp32s3/esp32s3 |
| Custom ESP32-S3 | esp32s3_devkitc/esp32s3/procpu |

⚠ Always use "west flash" for ESP32-S3 — it wraps esptool.py. For STM32 Nucleo, "west flash" uses OpenOCD via ST-Link. Confirm USB devices pass-through in WSL2: usbipd-win is required on Windows host.

## 0.3  WSL2 USB Passthrough (Windows Host)

$ PowerShell + WSL2

# On Windows PowerShell (Admin) — attach ST-Link or CP2102 usbipd list usbipd bind --busid <BUSID> usbipd attach --wsl --busid <BUSID> # Inside WSL2 — confirm device appeared lsusb ls /dev/ttyUSB* /dev/ttyACM* # Add yourself to dialout group (one-time) sudo usermod -aG dialout $USER

# Exercise 01 — GPIO Blink (The Real "Hello World")

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

Blinking an LED in Zephyr teaches the Device Tree (DT) abstraction, the GPIO API, and how Zephyr's build system selects board-specific hardware. Do NOT skip this — every subsequent exercise builds on these concepts.

## 1.1  Project Structure

Project Tree

zephyr_blink/ ├── CMakeLists.txt ├── prj.conf └── src/ └── main.c

## 1.2  CMakeLists.txt

CMakeLists.txt

cmake_minimum_required(VERSION 3.20.0) find_package(Zephyr REQUIRED HINTS $ENV{ZEPHYR_BASE}) project(blink) target_sources(app PRIVATE src/main.c)

## 1.3  prj.conf — Minimal Config

prj.conf

# Nothing extra needed for GPIO blink # Zephyr enables GPIO driver automatically when # board DTS has GPIO nodes. CONFIG_GPIO=y

## 1.4  Source Code — Side by Side

| // main.c (ESP32-S3) #include <zephyr/kernel.h> #include <zephyr/drivers/gpio.h> /* DT alias from board DTS */ #define LED0_NODE DT_ALIAS(led0) static const struct gpio_dt_spec led = GPIO_DT_SPEC_GET(LED0_NODE, gpios); int main(void) { if (!gpio_is_ready_dt(&led)) { return -ENODEV; } gpio_pin_configure_dt(&led, GPIO_OUTPUT_ACTIVE); while (1) { gpio_pin_toggle_dt(&led); k_msleep(500); } return 0; } | // main.c (STM32F446RE) #include <zephyr/kernel.h> #include <zephyr/drivers/gpio.h> /* Nucleo green LED = PA5 via alias */ #define LED0_NODE DT_ALIAS(led0) static const struct gpio_dt_spec led = GPIO_DT_SPEC_GET(LED0_NODE, gpios); int main(void) { if (!gpio_is_ready_dt(&led)) { return -ENODEV; } gpio_pin_configure_dt(&led, GPIO_OUTPUT_ACTIVE); while (1) { gpio_pin_toggle_dt(&led); k_msleep(500); } return 0; } /* Code is IDENTICAL — DTS does the work */ |
| --- | --- |

## 1.5  Build & Flash

| # ESP32-S3 west build -b esp32s3_devkitc \ -- -DBOARD_FLASH_RUNNER=esp32 west flash --runner esp32 west espressif monitor | # STM32F446RE (Nucleo) west build -b nucleo_f446re west flash # ST-Link auto-detected # Serial via /dev/ttyACM0 |
| --- | --- |

## 1.6  Minute Details & Common Pitfalls

| GPIO_DT_SPEC_GET vs GET_BY_IDX | Use GET when DTS has single pin; GET_BY_IDX(node, gpios, 0) for arrays |
| --- | --- |
| gpio_is_ready_dt() | ALWAYS check — returns false if clock/power not enabled |
| GPIO_OUTPUT_ACTIVE | Drives to the "active" logic level from DTS (handles active-low LEDs transparently) |
| GPIO_ACTIVE_LOW in DTS | If DTS has GPIO_ACTIVE_LOW flag, pin is LOW when "active". Code needs no change |
| k_msleep() vs k_busy_wait() | k_msleep releases CPU; k_busy_wait is a spin-loop — use msleep unless sub-ms timing needed |
| ESP32-S3 GPIO38+ | GPIO 35-39 on ESP32-S3 are input-only on some package variants — check your schematic |

# Exercise 02 — UART Console & Shell

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

UART is your primary debug window. Zephyr's shell subsystem gives you a live interactive console — invaluable for testing without re-flashing.

## 2.1  prj.conf — Enable Shell

prj.conf

CONFIG_UART_CONSOLE=y CONFIG_SHELL=y CONFIG_SHELL_BACKEND_SERIAL=y CONFIG_SHELL_PROMPT_UART="zephyr:~$ " CONFIG_LOG=y CONFIG_LOG_BACKEND_UART=y CONFIG_LOG_DEFAULT_LEVEL=3 # 0=off 1=err 2=warn 3=inf 4=dbg

## 2.2  Using printk vs LOG_*

src/main.c

#include <zephyr/kernel.h> #include <zephyr/logging/log.h> LOG_MODULE_REGISTER(myapp, LOG_LEVEL_DBG); int main(void) { /* printk: always compiled in, no module filter */ printk("Boot\n"); /* LOG_*: filtered at compile+runtime per module */ LOG_INF("System started, uptime %u ms", k_uptime_get_32()); LOG_WRN("Low memory warning"); LOG_ERR("This is an error"); LOG_DBG("Debug: cycle count %u", k_cycle_get_32()); return 0; }

## 2.3  Custom Shell Command

src/shell_cmds.c

#include <zephyr/shell/shell.h> static int cmd_hello(const struct shell *sh, size_t argc, char **argv) { shell_print(sh, "Hello, %s!", argc > 1 ? argv[1] : "World"); return 0; } /* Register as: "myapp hello [name]" */ SHELL_CMD_REGISTER(hello, NULL, "Print greeting", cmd_hello);

## 2.4  Board-Specific UART Notes

| # ESP32-S3 UART config # Default console = UART0 (GPIO43/44) # In prj.conf for ESP32-S3 with USB-OTG: CONFIG_USB_DEVICE_STACK=y CONFIG_USB_CDC_ACM=y CONFIG_UART_LINE_CTRL=y # Then connect to /dev/ttyACM0 in WSL2 | # STM32F446RE UART config # Nucleo routes UART2 to ST-Link VCP # PA2=TX, PA3=RX → /dev/ttyACM0 # No extra config needed — board.cmake # already sets CONFIG_UART_CONSOLE=y # Baud: 115200 (Zephyr default) minicom -D /dev/ttyACM0 -b 115200 |
| --- | --- |

⚠ On ESP32-S3 devkit, USB-JTAG/Serial port (GPIO19/20) also works as console. Set CONFIG_ESP_CONSOLE_UART_NUM=0 to keep UART0 vs USB-CDC choice consistent.

# Exercise 03 — Threads, Semaphores & Mutexes

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

This is the heart of Zephyr. Understanding threads, scheduling, and synchronization primitives separates real RTOS developers from bare-metal coders.

## 3.1  Thread Fundamentals

src/main.c — Threads + Semaphore + Mutex

#include <zephyr/kernel.h> #include <zephyr/logging/log.h> LOG_MODULE_REGISTER(threads_ex, LOG_LEVEL_INF); /* Static stack allocation — PREFERRED over dynamic */ #define STACK_SZ 1024 #define PRIORITY 5 K_THREAD_STACK_DEFINE(sensor_stack, STACK_SZ); K_THREAD_STACK_DEFINE(blink_stack, STACK_SZ); struct k_thread sensor_tid, blink_tid; /* ── Semaphore: signal between threads ────────────── */ K_SEM_DEFINE(data_ready_sem, 0, 1); /* args: name, initial_count, limit */ /* ── Mutex: protect shared resource ──────────────── */ K_MUTEX_DEFINE(uart_mutex); static uint32_t shared_value; /* Sensor thread — produces data */ void sensor_thread(void *p1, void *p2, void *p3) { ARG_UNUSED(p1); ARG_UNUSED(p2); ARG_UNUSED(p3); while (1) { /* Simulate sensor read */ shared_value = k_cycle_get_32() & 0xFFFF; LOG_INF("Sensor: value=%u", shared_value); k_sem_give(&data_ready_sem); /* signal */ k_msleep(1000); } } /* Processing thread — consumes data */ void process_thread(void *p1, void *p2, void *p3) { ARG_UNUSED(p1); ARG_UNUSED(p2); ARG_UNUSED(p3); while (1) { /* Block until sensor signals — no CPU spin */ k_sem_take(&data_ready_sem, K_FOREVER); k_mutex_lock(&uart_mutex, K_FOREVER); LOG_INF("Process: got value=%u", shared_value); k_mutex_unlock(&uart_mutex); } } int main(void) { k_thread_create(&sensor_tid, sensor_stack, K_THREAD_STACK_SIZEOF(sensor_stack), sensor_thread, NULL, NULL, NULL, PRIORITY, 0, K_NO_WAIT); k_thread_name_set(&sensor_tid, "sensor"); k_thread_create(&blink_tid, blink_stack, K_THREAD_STACK_SIZEOF(blink_stack), process_thread, NULL, NULL, NULL, PRIORITY + 1, 0, K_NO_WAIT); k_thread_name_set(&blink_tid, "process"); return 0; /* main() becomes idle thread in Zephyr */ }

## 3.2  Priority & Scheduling Rules

| Priority 0 | Highest priority (pre-empts everything) |
| --- | --- |
| Negative priorities | Meta-IRQ threads — only for specific use cases |
| CONFIG_NUM_PREEMPT_PRIORITIES | Default = 15; set in prj.conf to change range |
| k_yield() | Voluntarily give up CPU slice to threads of same priority |
| K_FOREVER | Block indefinitely — safe for semaphore/mutex takes |
| K_NO_WAIT | Return immediately if resource unavailable (returns -EBUSY) |
| K_MSEC(n) | Timeout value: wait up to n milliseconds |

## 3.3  Minute Details — Stack Overflows

prj.conf + debug tip

# prj.conf — Enable stack sentinel (detects overflow) CONFIG_THREAD_STACK_INFO=y CONFIG_STACK_SENTINEL=y CONFIG_THREAD_ANALYZER=y # In code — print thread stats to find correct stack size #include <zephyr/debug/thread_analyzer.h> thread_analyzer_print(); /* call from main after threads start */ /* Output shows: name ¦ unused stack bytes ¦ used ¦ size */

⚠ Rule of thumb: Start with 2048 bytes per thread. Run thread_analyzer after your workload, then reduce to (peak_used + 20% margin). Never go below 512 bytes.

# Exercise 04 — Timers, Work Queues & Delayed Work

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

Timers and work queues are the preferred way to schedule periodic actions without blocking threads — the Zephyr equivalent of FreeRTOS software timers and deferred interrupt handlers.

## 4.1  Kernel Timers

src/main.c — k_timer

#include <zephyr/kernel.h> #include <zephyr/logging/log.h> LOG_MODULE_REGISTER(timers_ex, LOG_LEVEL_INF); /* Timer callback runs in SYSTEM WORK QUEUE context */ void heartbeat_handler(struct k_timer *t) { /* KEEP SHORT — no blocking calls here */ LOG_INF("Heartbeat tick @ %lld ms", k_uptime_get()); } void stop_handler(struct k_timer *t) { LOG_WRN("Timer expired and stopped"); } K_TIMER_DEFINE(heartbeat_timer, heartbeat_handler, /* expiry fn */ stop_handler); /* stop fn */ int main(void) { /* Start: fire after 500ms, repeat every 1000ms */ k_timer_start(&heartbeat_timer, K_MSEC(500), K_MSEC(1000)); k_sleep(K_SECONDS(5)); k_timer_stop(&heartbeat_timer); /* Query remaining time */ uint32_t rem = k_timer_remaining_get( &heartbeat_timer); LOG_INF("Remaining: %u ms", rem); return 0; }

## 4.2  Work Queue (Deferred ISR Pattern)

src/main.c — Work Queue

#include <zephyr/kernel.h> #include <zephyr/drivers/gpio.h> /* Work item for deferred processing */ struct k_work process_work; struct k_work_delayable periodic_work; static void process_handler(struct k_work *work) { /* This runs in system work queue thread */ /* Safe to use LOG, I2C, SPI here */ LOG_INF("Work queue: processing event"); } static void periodic_handler(struct k_work *work) { LOG_INF("Periodic: %lld ms", k_uptime_get()); /* Re-schedule itself */ k_work_reschedule( CONTAINER_OF(work, struct k_work_delayable, work), K_MSEC(500)); } /* GPIO ISR — called in ISR context, CANNOT block */ static void button_isr(const struct device *dev, struct gpio_callback *cb, uint32_t pins) { /* Submit work — don't do anything heavy here */ k_work_submit(&process_work); } int main(void) { k_work_init(&process_work, process_handler); k_work_init_delayable(&periodic_work, periodic_handler); /* Start periodic work after 1s, then self-reschedules */ k_work_reschedule(&periodic_work, K_SECONDS(1)); return 0; }

## 4.3  Custom Work Queue (Separate Thread)

Custom Work Queue

/* Create a dedicated work queue thread — isolate from sys WQ */ K_THREAD_STACK_DEFINE(my_wq_stack, 2048); struct k_work_q my_work_q; void init_custom_wq(void) { k_work_queue_init(&my_work_q); k_work_queue_start(&my_work_q, my_wq_stack, K_THREAD_STACK_SIZEOF(my_wq_stack), 5, /* priority */ NULL); /* no config */ } /* Submit to custom queue, not system queue */ k_work_submit_to_queue(&my_work_q, &process_work);

⚠ Never call k_sleep(), k_sem_take(K_FOREVER) or any blocking API from a timer callback. Timer callbacks run in ISR context. Use k_work_submit() to defer to a thread.

# Exercise 05 — GPIO Interrupts & Callbacks

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

GPIO interrupts in Zephyr use a callback registration model. Understanding the ISR vs thread boundary is critical for reliable, race-condition-free code.

## 5.1  Button Interrupt with Debounce

src/main.c — GPIO IRQ + Debounce

#include <zephyr/kernel.h> #include <zephyr/drivers/gpio.h> #include <zephyr/logging/log.h> LOG_MODULE_REGISTER(gpio_irq, LOG_LEVEL_INF); #define BTN_NODE DT_ALIAS(sw0) static const struct gpio_dt_spec btn = GPIO_DT_SPEC_GET(BTN_NODE, gpios); static struct gpio_callback btn_cb; static struct k_work_delayable debounce_work; /* Debounce handler — runs in work queue thread */ static void debounce_handler(struct k_work *work) { /* Re-read pin to confirm stable */ int val = gpio_pin_get_dt(&btn); if (val == 0) { LOG_INF("Button PRESSED (debounced)"); } } /* ISR — ultra-fast, no logs, no blocking */ static void btn_isr(const struct device *dev, struct gpio_callback *cb, uint32_t pins) { /* Schedule debounce check 20ms later */ k_work_reschedule(&debounce_work, K_MSEC(20)); } int main(void) { if (!gpio_is_ready_dt(&btn)) { return -1; } gpio_pin_configure_dt(&btn, GPIO_INPUT); gpio_pin_interrupt_configure_dt( &btn, GPIO_INT_EDGE_TO_ACTIVE); /* falling edge */ gpio_init_callback(&btn_cb, btn_isr, BIT(btn.pin)); gpio_add_callback(btn.port, &btn_cb); k_work_init_delayable(&debounce_work, debounce_handler); LOG_INF("Waiting for button..."); while (1) { k_msleep(1000); } return 0; }

## 5.2  Interrupt Trigger Options

| GPIO_INT_EDGE_RISING | Trigger on 0→1 transition |
| --- | --- |
| GPIO_INT_EDGE_FALLING | Trigger on 1→0 transition |
| GPIO_INT_EDGE_BOTH | Both edges — good for encoder quadrature |
| GPIO_INT_EDGE_TO_ACTIVE | Edge to "active" level — respects GPIO_ACTIVE_LOW flag |
| GPIO_INT_LEVEL_HIGH | Level-sensitive HIGH (held, re-triggers) |
| GPIO_INT_LEVEL_LOW | Level-sensitive LOW |

## 5.3  Board-Specific Details

| // ESP32-S3 NOTE /* ESP32-S3: All GPIOs support interrupts * EXCEPT GPIO 35-39 (input only, some silicon) * * For esp32s3_devkitc: BOOT button = GPIO0 * DTS alias sw0 → gpio0 pin 0 * * Internal pull-up/down: * GPIO_PULL_UP or GPIO_PULL_DOWN in DTS * OR at runtime: * gpio_pin_configure_dt(&btn, * GPIO_INPUT ¦ GPIO_PULL_UP); */ | // STM32F446RE NOTE /* STM32 uses EXTI lines: GPIO PA0-PA15 share * EXTI0-EXTI15. Only ONE port per EXTI line: * PA0 and PB0 CANNOT both be interrupts! * * Nucleo B1 button = PC13 → EXTI13 * DTS alias sw0 → gpioc pin 13 * * STM32 has no internal pull management * in gpio_pin_configure_dt for ACTIVE_LOW: * Add GPIO_PULL_UP to configure flags */ |
| --- | --- |

⚠ On STM32, EXTI lines are shared between ports. PA5 and PB5 cannot both generate interrupts simultaneously — only one can be configured per EXTI line number.

# Exercise 06 — I2C Sensor Reading (GY-86 / IMU)

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

I2C in Zephyr uses the sensor driver abstraction for supported sensors, or raw i2c_write/read for custom/unsupported devices. This exercise covers both paths — directly relevant to your GY-86 IMU work.

## 6.1  prj.conf — I2C + Sensor

prj.conf

CONFIG_I2C=y CONFIG_SENSOR=y CONFIG_MPU6050=y # for GY-86 MPU6050 accel/gyro CONFIG_HMC5883L=y # for GY-86 compass (if using driver) CONFIG_LOG=y CONFIG_LOG_DEFAULT_LEVEL=3

## 6.2  Device Tree Overlay (both boards)

| // boards/esp32s3_devkitc.overlay /* I2C0 on ESP32-S3: SDA=GPIO8, SCL=GPIO9 * Adjust pins for your wiring */ &i2c0 { status = "okay"; sda-pin = <8>; scl-pin = <9>; clock-frequency = <I2C_BITRATE_FAST>; /* 400kHz */ mpu6050@68 { compatible = "invensense,mpu6050"; reg = <0x68>; /* AD0 low = 0x68 */ label = "MPU6050"; }; }; | // boards/nucleo_f446re.overlay /* I2C1 on Nucleo: PB8=SCL, PB9=SDA */ &i2c1 { status = "okay"; /* STM32 uses pinctrl for pin assignment */ pinctrl-0 = <&i2c1_scl_pb8 &i2c1_sda_pb9>; pinctrl-names = "default"; clock-frequency = <I2C_BITRATE_FAST>; mpu6050@68 { compatible = "invensense,mpu6050"; reg = <0x68>; label = "MPU6050"; }; }; |
| --- | --- |

## 6.3  Using Sensor API (High Level)

src/main.c — Sensor API

#include <zephyr/kernel.h> #include <zephyr/device.h> #include <zephyr/drivers/sensor.h> #include <zephyr/logging/log.h> LOG_MODULE_REGISTER(imu, LOG_LEVEL_INF); int main(void) { const struct device *mpu = DEVICE_DT_GET_ANY(invensense_mpu6050); if (!device_is_ready(mpu)) { LOG_ERR("MPU6050 not ready"); return -ENODEV; } struct sensor_value ax, ay, az; struct sensor_value gx, gy, gz; while (1) { /* Trigger measurement */ sensor_sample_fetch(mpu); sensor_channel_get(mpu, SENSOR_CHAN_ACCEL_X, &ax); sensor_channel_get(mpu, SENSOR_CHAN_ACCEL_Y, &ay); sensor_channel_get(mpu, SENSOR_CHAN_ACCEL_Z, &az); sensor_channel_get(mpu, SENSOR_CHAN_GYRO_X, &gx); /* sensor_value: val1 = integer part, * val2 = fractional (millionths) */ LOG_INF("Accel: %d.%06d, %d.%06d, %d.%06d m/s2", ax.val1, abs(ax.val2), ay.val1, abs(ay.val2), az.val1, abs(az.val2)); k_msleep(50); /* 20 Hz */ } return 0; }

## 6.4  Raw I2C Transactions (When No Driver Exists)

Raw I2C Transactions

#include <zephyr/drivers/i2c.h> #define I2C_NODE DT_NODELABEL(i2c0) const struct device *i2c_dev = DEVICE_DT_GET(I2C_NODE); /* Write single register */ void i2c_write_reg(uint8_t dev_addr, uint8_t reg, uint8_t val) { uint8_t buf[2] = { reg, val }; i2c_write(i2c_dev, buf, 2, dev_addr); } /* Read burst: write reg address, then read n bytes */ void i2c_read_regs(uint8_t dev_addr, uint8_t reg, uint8_t *out, size_t len) { /* Combined write+read in one transaction */ struct i2c_msg msgs[2] = { { .buf = &reg, .len = 1, .flags = I2C_MSG_WRITE }, { .buf = out, .len = len, .flags = I2C_MSG_READ ¦ I2C_MSG_STOP }, }; i2c_transfer(i2c_dev, msgs, 2, dev_addr); }

⚠ sensor_value uses fixed-point: actual_value = val1 + val2 / 1,000,000. To convert to float: sensor_value_to_double(&sv). Avoid float math on Cortex-M4 without FPU flag — enable CONFIG_FPU=y for STM32F446RE.

# Exercise 07 — SPI & ADC

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

## 7.1  SPI Configuration

| // ESP32-S3 SPI overlay /* SPI2 on ESP32-S3 */ &spi2 { status = "okay"; #address-cells = <1>; #size-cells = <0>; cs-gpios = <&gpio0 10 GPIO_ACTIVE_LOW>; /* MOSI=11, MISO=13, CLK=12 */ mosi-pin = <11>; miso-pin = <13>; sclk-pin = <12>; mydevice@0 { compatible = "vnd,spi-device"; reg = <0>; spi-max-frequency = <1000000>; }; }; | // STM32F446RE SPI overlay /* SPI1 on Nucleo: PA5=CLK, * PA6=MISO, PA7=MOSI, PA4=CS */ &spi1 { status = "okay"; pinctrl-0 = <&spi1_sck_pa5 &spi1_miso_pa6 &spi1_mosi_pa7>; pinctrl-names = "default"; cs-gpios = <&gpioa 4 GPIO_ACTIVE_LOW>; mydevice@0 { compatible = "vnd,spi-device"; reg = <0>; spi-max-frequency = <1000000>; }; }; |
| --- | --- |

## 7.2  SPI Read/Write

SPI Full-Duplex Transfer

#include <zephyr/drivers/spi.h> static const struct spi_dt_spec spidev = SPI_DT_SPEC_GET(DT_NODELABEL(mydevice), SPI_OP_MODE_MASTER ¦ SPI_WORD_SET(8) ¦ SPI_TRANSFER_MSB, 0); int spi_transfer(uint8_t *tx, uint8_t *rx, size_t len) { struct spi_buf tx_buf = { .buf = tx, .len = len }; struct spi_buf rx_buf = { .buf = rx, .len = len }; struct spi_buf_set tx_set = { .buffers = &tx_buf, .count = 1 }; struct spi_buf_set rx_set = { .buffers = &rx_buf, .count = 1 }; return spi_transceive_dt(&spidev, &tx_set, &rx_set); }

## 7.3  ADC Sampling

ADC Sampling — STM32F446RE / ESP32-S3

#include <zephyr/drivers/adc.h> /* prj.conf: CONFIG_ADC=y */ #define ADC_NODE DT_NODELABEL(adc1) #define ADC_CHANNEL 0 #define ADC_GAIN ADC_GAIN_1 #define ADC_REF ADC_REF_INTERNAL #define ADC_RES 12 /* bits */ const struct device *adc_dev = DEVICE_DT_GET(ADC_NODE); struct adc_channel_cfg ch_cfg = { .gain = ADC_GAIN, .reference = ADC_REF, .acquisition_time = ADC_ACQ_TIME_DEFAULT, .channel_id = ADC_CHANNEL, }; int16_t raw; struct adc_sequence seq = { .channels = BIT(ADC_CHANNEL), .buffer = &raw, .buffer_size = sizeof(raw), .resolution = ADC_RES, }; int read_adc_mv(void) { adc_channel_setup(adc_dev, &ch_cfg); adc_read(adc_dev, &seq); int32_t mv = raw; /* Convert to mV using Zephyr helper */ adc_raw_to_millivolts( adc_ref_internal(adc_dev), ADC_GAIN, ADC_RES, &mv); return (int)mv; }

| ESP32-S3 ADC | 12-bit SAR, up to 20 channels; GPIO1-10 = ADC1, GPIO11-20 = ADC2. WiFi/BLE interferes with ADC2 |
| --- | --- |
| STM32F446RE ADC | ADC1/2/3 up to 16ch each; 12-bit; PA0 = ADC1_IN0. Enable VDD filter caps |
| adc_raw_to_millivolts() | Converts raw counts to mV accounting for gain and reference voltage |
| DMA ADC (STM32) | Use CONFIG_ADC_STM32_DMA=y for continuous DMA transfer without CPU polling |

# Exercise 08 — DMA Transfers (STM32) & PSRAM (ESP32-S3)

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

This exercise dives into memory and data movement: DMA on STM32F446RE for zero-CPU transfers, and PSRAM/SPIRAM usage on ESP32-S3 for large buffers.

## 8.1  STM32 DMA — UART with DMA

prj.conf (STM32)

# prj.conf CONFIG_UART_ASYNC_API=y CONFIG_DMA=y CONFIG_UART_STM32_DMA=y

DMA UART — STM32F446RE

#include <zephyr/drivers/uart.h> #define UART_NODE DT_NODELABEL(usart2) const struct device *uart = DEVICE_DT_GET(UART_NODE); static uint8_t rx_buf[256]; static uint8_t tx_buf[] = "Hello via DMA!\r\n"; /* Async callback for DMA completion */ static void uart_cb(const struct device *dev, struct uart_event *evt, void *user_data) { switch (evt->type) { case UART_TX_DONE: LOG_INF("DMA TX done"); break; case UART_RX_RDY: LOG_INF("DMA RX: %d bytes", evt->data.rx.len); /* Process evt->data.rx.buf + offset */ break; case UART_RX_STOPPED: uart_rx_enable(dev, rx_buf, sizeof(rx_buf), 100 * USEC_PER_MSEC); break; default: break; } } int main(void) { uart_callback_set(uart, uart_cb, NULL); uart_rx_enable(uart, rx_buf, sizeof(rx_buf), 100 * USEC_PER_MSEC); uart_tx(uart, tx_buf, sizeof(tx_buf), SYS_FOREVER_MS); while (1) { k_msleep(1000); } return 0; }

## 8.2  ESP32-S3 PSRAM (SPIRAM) Usage

prj.conf (ESP32-S3)

# prj.conf — Enable PSRAM on ESP32-S3 CONFIG_ESP_SPIRAM=y CONFIG_SPIRAM_MODE_OCT=y # OPI mode (Octal) CONFIG_SPIRAM_SPEED_80M=y CONFIG_HEAP_MEM_POOL_SIZE=0 # Use SPIRAM for heap

PSRAM Allocation — ESP32-S3

#include <zephyr/kernel.h> #include <stdlib.h> /* malloc uses SPIRAM heap */ /* Large buffer — goes to PSRAM automatically * when CONFIG_ESP_SPIRAM=y */ uint8_t *large_buf; int main(void) { /* Allocate 100KB — impossible in internal SRAM */ large_buf = k_malloc(100 * 1024); if (!large_buf) { LOG_ERR("PSRAM alloc failed"); return -ENOMEM; } LOG_INF("PSRAM buffer @ %p", large_buf); /* Use normally */ memset(large_buf, 0, 100 * 1024); k_free(large_buf); return 0; }

| STM32F446RE DMA streams | Each peripheral has assigned DMA stream+channel; check RM Table 27/28 for UART2 = DMA1 Stream 6, Ch 4 |
| --- | --- |
| DMA burst size | Set burst for cache-line aligned transfers; avoid unaligned DMA addresses on STM32 AHB bus |
| ESP32-S3 PSRAM note | 8MB OPI PSRAM on devkit; cache-coherent at 80MHz; slower than internal SRAM (~40ns vs ~10ns) |
| esp32s3 internal SRAM | 512KB internal SRAM + 16KB RTC SRAM; place ISR handlers in IRAM_ATTR for speed |

# Exercise 09 — BLE GATT Server (ESP32-S3)  &  USB CDC (Both)

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

BLE GATT on ESP32-S3 using Zephyr's Bluetooth stack. This exercise connects directly to your smart shoe gait platform work. STM32 section covers USB CDC-ACM as an alternative serial transport.

## 9.1  prj.conf — BLE

prj.conf (ESP32-S3 BLE)

# ESP32-S3: BLE GATT Server CONFIG_BT=y CONFIG_BT_PERIPHERAL=y CONFIG_BT_DEVICE_NAME="SmartShoe" CONFIG_BT_DEVICE_APPEARANCE=832 /* Running shoe category */ CONFIG_BT_GATT_DYNAMIC_DB=y CONFIG_BT_MAX_CONN=1 CONFIG_BT_GAP_AUTO_UPDATE_CONN_PARAMS=y CONFIG_BT_LOG_LEVEL_INF=y

## 9.2  Custom GATT Service

BLE GATT Service — ESP32-S3

#include <zephyr/bluetooth/bluetooth.h> #include <zephyr/bluetooth/gatt.h> #include <zephyr/bluetooth/uuid.h> /* Custom Service UUID */ #define SHOE_SERVICE_UUID \ BT_UUID_DECLARE_128(BT_UUID_128_ENCODE( \ 0x12345678, 0x1234, 0x5678, \ 0x1234, 0x56789abcdef0)) /* Gait Data Characteristic UUID */ #define GAIT_CHAR_UUID \ BT_UUID_DECLARE_128(BT_UUID_128_ENCODE( \ 0x12345679, 0x1234, 0x5678, \ 0x1234, 0x56789abcdef0)) static uint8_t gait_data[20]; static bool notify_enabled; /* Notification CCC changed callback */ static void ccc_cfg_changed(const struct bt_gatt_attr *attr, uint16_t value) { notify_enabled = (value == BT_GATT_CCC_NOTIFY); LOG_INF("Notify: %s", notify_enabled ? "ON" : "OFF"); } /* Read callback */ static ssize_t read_gait(struct bt_conn *conn, const struct bt_gatt_attr *attr, void *buf, uint16_t len, uint16_t offset) { return bt_gatt_attr_read(conn, attr, buf, len, offset, gait_data, sizeof(gait_data)); } /* Service definition — macro expands to static vars */ BT_GATT_SERVICE_DEFINE(shoe_svc, BT_GATT_PRIMARY_SERVICE(SHOE_SERVICE_UUID), BT_GATT_CHARACTERISTIC(GAIT_CHAR_UUID, BT_GATT_CHRC_READ ¦ BT_GATT_CHRC_NOTIFY, BT_GATT_PERM_READ, read_gait, NULL, gait_data), BT_GATT_CCC(ccc_cfg_changed, BT_GATT_PERM_READ ¦ BT_GATT_PERM_WRITE), ); /* Send notification */ void send_gait_notify(uint8_t *data, size_t len) { if (!notify_enabled) return; memcpy(gait_data, data, MIN(len, sizeof(gait_data))); bt_gatt_notify(NULL, &shoe_svc.attrs[1], /* char attr */ gait_data, sizeof(gait_data)); }

## 9.3  BLE Advertising & Connection

BLE Advertising

static const struct bt_data ad[] = { BT_DATA_BYTES(BT_DATA_FLAGS, BT_LE_AD_GENERAL ¦ BT_LE_AD_NO_BREDR), BT_DATA(BT_DATA_NAME_COMPLETE, CONFIG_BT_DEVICE_NAME, sizeof(CONFIG_BT_DEVICE_NAME) - 1), }; static struct bt_conn_cb conn_cb = { .connected = on_connected, .disconnected = on_disconnected, .le_param_updated = on_param_updated, }; void on_connected(struct bt_conn *conn, uint8_t err) { if (!err) { LOG_INF("Connected: RSSI watch thread here"); } } int main(void) { bt_conn_cb_register(&conn_cb); bt_enable(NULL); /* synchronous init */ bt_le_adv_start( BT_LE_ADV_CONN_NAME, ad, ARRAY_SIZE(ad), NULL, 0); while (1) { k_msleep(1000); } return 0; }

## 9.4  USB CDC-ACM (Both Boards — Virtual COM Port)

prj.conf — USB CDC

# prj.conf CONFIG_USB_DEVICE_STACK=y CONFIG_USB_CDC_ACM=y CONFIG_UART_LINE_CTRL=y CONFIG_LOG=y CONFIG_LOG_BACKEND_USB_CDC_ACM=y

USB CDC DTR Wait

/* Wait for DTR (host opened port) before printing */ const struct device *cdc = DEVICE_DT_GET_ONE(zephyr_cdc_acm_uart); uint32_t dtr = 0; usb_enable(NULL); while (!dtr) { uart_line_ctrl_get(cdc, UART_LINE_CTRL_DTR, &dtr); k_msleep(100); } printk("USB CDC ready!\n");

# Exercise 10 — Power Management & Production Patterns

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

Power management is what separates prototype firmware from production-grade IoT devices. Zephyr's PM subsystem abstracts sleep states across both platforms.

## 10.1  prj.conf — Power Management

| # ESP32-S3 PM config CONFIG_PM=y CONFIG_PM_DEVICE=y CONFIG_POWEROFF=y # Light sleep (RAM retained) CONFIG_PM_S2RAM=y # Deep sleep with RTC wakeup CONFIG_ESP_SLEEP_ENABLE_RTC_TIMER=y CONFIG_ESP32_LIGHT_SLEEP_UART_ALLOW=y | # STM32F446RE PM config CONFIG_PM=y CONFIG_PM_DEVICE=y CONFIG_PM_DEVICE_RUNTIME=y # STM32 sleep states: # PM_STATE_RUNTIME_IDLE = WFI # PM_STATE_SUSPEND_TO_IDLE = SLEEP # PM_STATE_STANDBY = STANDBY # PM_STATE_SOFT_OFF = SHUTDOWN |
| --- | --- |

## 10.2  Device Runtime PM

Device Runtime PM

#include <zephyr/pm/pm.h> #include <zephyr/pm/device.h> #include <zephyr/pm/device_runtime.h> /* Suspend a peripheral when not in use */ void use_sensor(const struct device *sensor) { /* Wake up device — increments usage count */ pm_device_runtime_get(sensor); /* ... use sensor ... */ sensor_sample_fetch(sensor); /* Release — driver can now power-gate */ pm_device_runtime_put(sensor); }

## 10.3  System Sleep with Timed Wakeup

System Sleep

#include <zephyr/pm/pm.h> /* Force system into sleep for 5 seconds */ void deep_sleep_5s(void) { /* Create wakeup constraint — prevents sleep * if other threads are still busy */ struct pm_state_info info = { .state = PM_STATE_SOFT_OFF, .substate_id = 0, .min_residency_us = 5000000, /* 5s */ }; LOG_INF("Entering deep sleep..."); /* RTC/timer wakeup handled by board driver */ pm_state_force(0u, &info); /* Does not return for SOFT_OFF */ }

## 10.4  Production Checklist

| CONFIG_RESET_ON_FATAL_ERROR=n | Prevent silent reboots in production — log and halt instead |
| --- | --- |
| CONFIG_WATCHDOG=y | Enable WDT; configure k_wdt_feed() in each main thread |
| CONFIG_STACK_SENTINEL=y | Keep in production for stack overflow detection |
| CONFIG_ASSERT=n | Disable asserts in release builds for performance |
| CONFIG_BOOT_BANNER=n | Suppress boot banner to reduce UART noise in production |
| CONFIG_LOG_DEFAULT_LEVEL=1 | Set to ERR only in production; use LOG_INF for development |
| CONFIG_REBOOT=y | Enable sys_reboot() for controlled restart after errors |
| CONFIG_HWINFO=y | Access chip UID for device identity / provisioning |

⚠ Enable CONFIG_SHELL in development builds only. It adds ~20KB of flash. For production, use a lightweight command parser over UART or strip the shell entirely.

## 10.5  Flash & RAM Optimization

Memory Analysis

# Check memory usage after build: west build ... && \ arm-zephyr-eabi-size build/zephyr/zephyr.elf # Or use Zephyr's built-in: west build -t ram_report west build -t rom_report # Outputs per-symbol breakdown — find memory hogs # Reduce flash: disable unused subsystems CONFIG_PRINTK=n # saves ~2KB CONFIG_CBPRINTF_FP_SUPPORT=n # no float formatting CONFIG_ASSERT=n

# Quick Reference — Commands, Kconfig & Zephyr APIs

## West Commands Cheat Sheet

| west build -b <board> | Build for target board |
| --- | --- |
| west build -b <board> -p always | Pristine build (clean) |
| west flash | Flash using board default runner |
| west flash --runner jlink | Force JLink runner |
| west espressif monitor | Serial monitor (ESP32) |
| west build -t menuconfig | Interactive Kconfig GUI |
| west build -t guiconfig | Graphical Kconfig (requires tkinter) |
| west build -t hardenconfig | Apply security hardening configs |
| west build -t ram_report | Per-symbol RAM breakdown |
| west build -t rom_report | Per-symbol Flash breakdown |
| west debugserver | Start GDB server (OpenOCD/JLink) |
| west attach | Attach GDB to running target |

## Critical Kconfig Options by Category

| CONFIG_GPIO=y | GPIO subsystem — almost always needed |
| --- | --- |
| CONFIG_I2C=y | I2C bus driver |
| CONFIG_SPI=y | SPI bus driver |
| CONFIG_UART_CONSOLE=y | Enable UART console (usually default) |
| CONFIG_BT=y + BT_PERIPHERAL=y | BLE stack + peripheral role |
| CONFIG_SENSOR=y | Sensor abstraction layer |
| CONFIG_DMA=y | DMA subsystem |
| CONFIG_USB_DEVICE_STACK=y | USB device (CDC, HID, etc.) |
| CONFIG_SHELL=y | Interactive shell over UART |
| CONFIG_LOG=y | Logging subsystem |
| CONFIG_THREAD_ANALYZER=y | Thread stack analysis tool |
| CONFIG_PM=y | Power management |
| CONFIG_WATCHDOG=y | Hardware watchdog |
| CONFIG_FPU=y | Float hardware (STM32F4/ESP32-S3 have FPU) |

## Key Kernel API Summary

| k_msleep(ms) | Sleep thread for ms milliseconds |
| --- | --- |
| k_uptime_get() | Uptime in ms since boot (int64) |
| k_cycle_get_32() | CPU cycle counter (fast, for profiling) |
| k_sem_give / k_sem_take | Counting semaphore operations |
| k_mutex_lock / k_mutex_unlock | Mutex lock/unlock |
| k_work_submit(&work) | Submit work to system work queue |
| k_work_reschedule(&dwork, K_MSEC(n)) | Schedule delayed work |
| k_timer_start / k_timer_stop | Software timer control |
| K_THREAD_DEFINE() | Static thread creation macro |
| K_SEM_DEFINE(name, init, max) | Static semaphore definition |
| K_MUTEX_DEFINE(name) | Static mutex definition |
| DEVICE_DT_GET(node) | Get device handle from DT node |
| device_is_ready(dev) | Check if device initialized OK |