ZEPHYR RTOS

Hands-On Mastery Tutorial

| ESP32-S3 Xtensa LX7 Dual-Core | STM32F446RE ARM Cortex-M4 @ 180MHz |
| --- | --- |

10 Progressive Exercises  •  Minute Details  •  WSL2 Ready

From GPIO Blink to BLE GATT & DMA — Both Boards, Side by Side

# 0.  WSL2 Environment Setup & Verification

Before writing a single line of Zephyr code, verify your WSL2 toolchain is properly configured. Subtle misconfigurations here cause mysterious build failures later.

## 0.1  SDK & West Tool Check

$ bash

# Check West version (must be >= 1.2 for Zephyr 3.x) west --version # Check SDK location — ZEPHYR_SDK_INSTALL_DIR must be set echo $ZEPHYR_SDK_INSTALL_DIR # Confirm ESP32-S3 and ARM toolchains exist ls $ZEPHYR_SDK_INSTALL_DIR/xtensa-espressif_esp32s3_zephyr-elf/ ls $ZEPHYR_SDK_INSTALL_DIR/arm-zephyr-eabi/ # Verify Zephyr base echo $ZEPHYR_BASE west topdir

## 0.2  Board Names Used in This Tutorial

| Board | Zephyr Board Name |
| --- | --- |
| ESP32-S3-DevKitC-1 | esp32s3_devkitc |
| Nucleo-F446RE | nucleo_f446re |
| ESP32-S3 generic | esp32s3/esp32s3 |
| Custom ESP32-S3 | esp32s3_devkitc/esp32s3/procpu |

⚠ Always use "west flash" for ESP32-S3 — it wraps esptool.py. For STM32 Nucleo, "west flash" uses OpenOCD via ST-Link. Confirm USB devices pass-through in WSL2: usbipd-win is required on Windows host.

## 0.3  WSL2 USB Passthrough (Windows Host)

$ PowerShell + WSL2

# On Windows PowerShell (Admin) — attach ST-Link or CP2102 usbipd list usbipd bind --busid <BUSID> usbipd attach --wsl --busid <BUSID> # Inside WSL2 — confirm device appeared lsusb ls /dev/ttyUSB* /dev/ttyACM* # Add yourself to dialout group (one-time) sudo usermod -aG dialout $USER

# Theory Chapter 1 — Zephyr RTOS Architecture

Before writing a single line of application code, you must understand what Zephyr actually IS — not just as an RTOS, but as a full operating system framework with distinct architectural layers. This understanding prevents the most common mistakes new Zephyr developers make.

## 1.1  The Five Architectural Layers

Zephyr is not a monolithic kernel. It is a layered system where each layer has a clearly defined responsibility:

Zephyr Layer Stack

┌─────────────────────────────────────────────────────────┐ │ APPLICATION CODE (your main.c) │ ├─────────────────────────────────────────────────────────┤ │ SUBSYSTEMS (BLE, USB, Shell, FS, Net) │ ├─────────────────────────────────────────────────────────┤ │ DEVICE DRIVERS (GPIO, I2C, SPI, UART, ADC) │ ├─────────────────────────────────────────────────────────┤ │ ZEPHYR KERNEL (threads, IPC, timers, PM) │ ├─────────────────────────────────────────────────────────┤ │ HARDWARE ABSTRACTION LAYER (HAL) │ ├───────────────────────────┬─────────────────────────────┤ │ ESP32-S3 (Xtensa LX7) │ STM32F446RE (Cortex-M4) │ └───────────────────────────┴─────────────────────────────┘

| Application Layer | Your code. Uses only public Zephyr APIs. Never calls HAL directly. |
| --- | --- |
| Subsystems | Protocol stacks and frameworks: Bluetooth, LwIP, FAT-FS, Shell, Logging. Each is an optional module compiled only if CONFIG_* enabled. |
| Device Drivers | Implements the Zephyr driver model (struct device + driver_api). Provides uniform API: gpio_pin_set(), i2c_write(), etc. Board-agnostic. |
| Zephyr Kernel | Core RTOS primitives: scheduler, memory slabs, semaphores, mutexes, mailboxes, pipes, timers. Implemented in kernel/*.c |
| HAL | Thin wrappers around vendor SDK (ESP-IDF hal, STM32 HAL/LL). Zephyr drivers call HAL; application NEVER calls HAL directly. |

## 1.2  The Zephyr Kernel — How It Actually Works

The Zephyr kernel uses a tickless scheduler by default (CONFIG_TICKLESS_KERNEL=y). This is a critical design decision that affects power consumption and timer resolution.

Kernel Internals

TICKLESS SCHEDULER (default in Zephyr): ───────────────────────────────────────────────────────── Traditional RTOS: Hardware timer fires every 1ms (tick) Kernel runs at each tick → checks all timeouts CPU wakes 1000x/second minimum → wastes power Zephyr Tickless: Kernel programs timer for NEXT event only CPU stays in sleep until something needs attention → Naturally enables power management → Timer resolution = hardware timer resolution (not 1ms) SCHEDULER TYPES in Zephyr: ───────────────────────────────────────────────────────── CONFIG_SCHED_SIMPLE Simple linked list — O(n) but tiny footprint CONFIG_SCHED_DUMB Always picks highest-priority ready thread CONFIG_SCHED_SCALABLE Red-black tree — O(log n) — best for many threads CONFIG_SCHED_MULTIQ Multiple queues per priority — best throughput

## 1.3  Thread Lifecycle State Machine

Thread State Machine

k_thread_create() │ ▼ ┌─────────┐ pre-empted ┌─────────┐ │ READY │◄────────────────────│ RUNNING │ │ │────────────────────►│ │ └─────────┘ scheduled/CPU └─────────┘ │ │ │ k_sem_take │ k_msleep() │ k_mutex_lock│ k_mbox_get() │ ▼ │ ┌─────────┐ │ timeout/signal │ WAITING │ └────────────────────────►│ (Pend) │ └─────────┘ │ k_thread_abort() ▼ ┌─────────┐ │ DEAD │ └─────────┘

A thread in WAITING state consumes ZERO CPU cycles. The scheduler will not select it until the wait condition is satisfied. This is the fundamental mechanism that makes RTOS-based firmware power-efficient compared to bare-metal polling loops.

## 1.4  Memory Architecture in Zephyr

Memory Layout

Flash (ROM) layout — zephyr.elf sections: ────────────────────────────────────────────────────────── ┌──────────────────────────────┐ ← Flash start │ Interrupt Vector Table │ ├──────────────────────────────┤ │ .text (code) │ ← All compiled functions ├──────────────────────────────┤ │ .rodata (read-only data) │ ← const strings, lookup tables ├──────────────────────────────┤ │ .data init values │ ← Initial values of global vars └──────────────────────────────┘ SRAM layout at runtime: ────────────────────────────────────────────────────────── ┌──────────────────────────────┐ ← SRAM start │ .data (initialized globals)│ ← Copied from Flash at boot ├──────────────────────────────┤ │ .bss (zero-init globals) │ ← Zeroed at boot by z_bss_zero() ├──────────────────────────────┤ │ Kernel objects │ ← k_thread, k_sem, etc. ├──────────────────────────────┤ │ Heap (k_heap / k_malloc) │ ← CONFIG_HEAP_MEM_POOL_SIZE ├──────────────────────────────┤ │ Thread stacks │ ← K_THREAD_STACK_DEFINE() └──────────────────────────────┘ ← SRAM end

| K_THREAD_STACK_DEFINE | Places stack in special .noinit section with proper alignment (required for MPU) |
| --- | --- |
| .noinit section | Not zeroed at boot — used for crash log retention across resets |
| IRAM_ATTR (ESP32-S3) | Forces function into IRAM (fast internal RAM) — critical for ISR handlers on ESP32 |
| CONFIG_MPU (STM32) | Memory Protection Unit: prevents stack overflow from corrupting other memory regions |
| CONFIG_USERSPACE | Optional: full user/kernel space separation with syscall gates — not common on MCUs |

## 1.5  The init Infrastructure — How Drivers Start

Zephyr uses a priority-ordered initialization system. When you call DEVICE_DT_GET() and use device_is_ready(), the device was already initialized automatically — you did not call any init function. Here is how that works:

Driver Init System

/* Zephyr driver registration macro (inside driver source): */ DEVICE_DT_DEFINE(node_id, /* DT node */ my_driver_init, /* init function, called at boot */ PM_DEVICE_NULL, &my_data, /* private data */ &my_cfg, /* config (const, in Flash) */ POST_KERNEL, /* init level */ 50, /* init priority within level */ &my_api); /* driver API struct */ /* Boot initialization ORDER: * PRE_KERNEL_1 → Very early hardware (clocks, pin mux) * PRE_KERNEL_2 → Interrupt controller, memory * POST_KERNEL → All normal drivers (GPIO, I2C, SPI, etc.) * APPLICATION → App-level init, Bluetooth stack, LwIP * * Your main() runs AFTER all APPLICATION-level inits. */

⚠ This is why you never need to call "gpio_driver_init()" or "i2c_bus_init()". Zephyr boots, runs all DEVICE_DT_DEFINE init functions in priority order, then calls your main(). device_is_ready() returns false if the init function returned an error.

# Theory Chapter 2 — Device Tree & Board System

Device Tree (DT) is the single most important concept to master in Zephyr. It is the mechanism by which hardware description is completely separated from driver code. Confusion about DT is the #1 source of bugs in new Zephyr projects.

## 2.1  Why Device Tree Exists

DT Motivation

WITHOUT Device Tree (old approach — brittle): #ifdef BOARD_NUCLEO_F446RE #define LED_PIN GPIO_PIN_5 #define LED_PORT GPIOA #elif BOARD_ESP32S3 #define LED_PIN GPIO_NUM_48 #endif HAL_GPIO_WritePin(LED_PORT, LED_PIN, GPIO_PIN_SET); WITH Device Tree (Zephyr approach — board-agnostic): /* In DTS file for each board: */ leds { led0: led_0 { gpios = <&gpio0 48 GPIO_ACTIVE_HIGH>; }; }; /* In your main.c — IDENTICAL for all boards: */ static const struct gpio_dt_spec led = GPIO_DT_SPEC_GET(DT_ALIAS(led0), gpios); gpio_pin_configure_dt(&led, GPIO_OUTPUT_ACTIVE); gpio_pin_set_dt(&led, 1);

## 2.2  DTS File Hierarchy

DTS File Hierarchy

For ESP32-S3: zephyr/boards/xtensa/esp32s3_devkitc/ esp32s3_devkitc.dts ← board-level: LEDs, buttons, connectors esp32s3_devkitc_defconfig ← default Kconfig for this board zephyr/dts/xtensa/espressif/ esp32s3.dtsi ← SoC-level: all peripherals, memory map esp32s3-pinctrl.dtsi ← Pin mux options For STM32F446RE: zephyr/boards/arm/nucleo_f446re/ nucleo_f446re.dts zephyr/dts/arm/st/f4/ stm32f446.dtsi ← SoC-level stm32f4.dtsi ← Family-level YOUR overlay (highest priority — overrides everything): <your_app>/boards/esp32s3_devkitc.overlay <your_app>/boards/nucleo_f446re.overlay

## 2.3  DTS Syntax — The Full Picture

DTS Syntax Reference

/* A DTS node has this structure: */ node_label: node_name@unit_address { compatible = "vendor,driver-binding"; /* links to C driver */ reg = <base_address size>; /* register/address */ interrupts = <irq_num flags>; status = "okay"; /* "okay" = enabled */ #address-cells = <1>; #size-cells = <1>; }; /* Property types: * <1 2 3> = array of uint32 cells * "string" = string * [aa bb cc] = byte array * &node_ref = phandle (reference to another node) * <&gpio0 5 GPIO_ACTIVE_HIGH> = phandle + specifiers */ /* Aliases and chosen — special nodes: */ / { aliases { led0 = &led_0; /* DT_ALIAS(led0) resolves to &led_0 */ sw0 = &button_0; }; chosen { zephyr,console = &usart2; /* which UART = console */ zephyr,shell-uart = &usart2; zephyr,sram = &sram0; zephyr,flash = &flash0; }; };

## 2.4  DT Macros — What They Expand To

DT Macro Expansions

/* The DTS for Nucleo F446RE LED: */ leds { led0: led_0 { gpios = <&gpioa 5 GPIO_ACTIVE_HIGH>; label = "Green LED"; }; }; /* In C, GPIO_DT_SPEC_GET expands at COMPILE TIME to: */ static const struct gpio_dt_spec led = { .port = DEVICE_DT_GET(DT_NODELABEL(gpioa)), .pin = 5, .dt_flags = GPIO_ACTIVE_HIGH, }; /* Zero runtime cost — pure compile-time resolution */ /* Commonly used DT macros: */ DT_ALIAS(led0) /* node id of alias "led0" */ DT_NODELABEL(gpioa) /* node id by label "gpioa" */ DT_INST(0, st_stm32_gpio) /* first instance of compatible */ DT_PROP(node, compatible) /* property value at compile time */ DT_REG_ADDR(node) /* register base address */ DT_IRQ(node, irq) /* interrupt number */ DT_NODE_HAS_STATUS(node, okay) /* is node enabled? */

## 2.5  Pinctrl — STM32 Pin Multiplexing

STM32 Pinctrl vs ESP32

/* STM32 pinctrl: each pin-function combo is pre-defined */ /* I2C1 on PB8(SCL), PB9(SDA) */ &i2c1 { pinctrl-0 = <&i2c1_scl_pb8 &i2c1_sda_pb9>; pinctrl-names = "default"; status = "okay"; }; /* Internally defined as: */ i2c1_scl_pb8: i2c1_scl_pb8 { pinmux = <STM32_PINMUX('B', 8, AF4)>; /* PB8, Alt Func 4 */ bias-pull-up; drive-open-drain; }; /* ESP32-S3: no pinctrl — use direct pin properties */ &i2c0 { sda-pin = <8>; scl-pin = <9>; status = "okay"; };

⚠ On STM32, if a peripheral uses DMA, the DMA channel assignment is ALSO done in DTS. Look for "dmas" property in the overlay. Getting the DMA stream/channel wrong is a very common STM32 Zephyr bug.

# Theory Chapter 3 — Kconfig, CMake & the Build Pipeline

Understanding how Zephyr's build system processes your project from source to binary is essential for debugging cryptic build errors and for optimizing firmware size.

## 3.1  The Build Pipeline Step by Step

Build Pipeline

west build -b nucleo_f446re │ ├─ 1. CMake configure phase │ Reads CMakeLists.txt, finds Zephyr package │ Processes all DTS files → generates devicetree_generated.h │ Processes Kconfig → generates autoconf.h │ ├─ 2. Kconfig resolution │ Reads Kconfig files in zephyr/, drivers/, subsys/ │ Applies: board defconfig → prj.conf (your config) │ Output: build/zephyr/.config (complete merged config) │ build/zephyr/include/generated/autoconf.h │ ├─ 3. DTS compilation │ DTC merges: SoC.dtsi + board.dts + overlay │ gen_defines.py generates: devicetree_generated.h │ All DT_* macros resolve at THIS step │ ├─ 4. C compilation │ GCC compiles all .c files with -DCONFIG_* and DT headers │ Each CONFIG_* maps to a #define in autoconf.h │ ├─ 5. Linking │ Linker script generated from DTS memory regions │ Final ELF: build/zephyr/zephyr.elf │ └─ 6. Post-build zephyr.bin / zephyr.hex for flashing zephyr.map — full symbol→address map

## 3.2  Kconfig Dependency Resolution

Kconfig Syntax

# Kconfig mini-language: config BT_GATT_DYNAMIC_DB bool "Dynamic GATT database" depends on BT # BT must be y for this to appear select BT_GATT # automatically enables BT_GATT default n # DEPENDENCY CHAIN EXAMPLE: # To get BLE GATT notifications: CONFIG_BT=y # base stack CONFIG_BT_PERIPHERAL=y # peripheral role CONFIG_BT_GATT_DYNAMIC_DB=y # dynamic services # The above auto-selects: BT_GATT, BT_HCI, BT_CRYPTO, MBEDTLS # COMMON MISTAKE: enabling a feature without its parent # CONFIG_BT_MESH without CONFIG_BT=y → CONFIG NOT FOUND error

## 3.3  Configuration Layer Override Order

Config Override Order

Priority (LOW → HIGH — higher value wins): 1. Zephyr default Kconfig values 2. SoC-level defconfig 3. Board defconfig (e.g., nucleo_f446re_defconfig) 4. Shield defconfig (if applicable) 5. prj.conf ← YOUR main configuration 6. Extra conf files via -DEXTRA_CONF_FILE=debug.conf Example: Board sets CONFIG_UART_CONSOLE=y. Your prj.conf sets CONFIG_UART_CONSOLE=n → YOUR value wins. Multiple conf files (stacked builds): west build -- -DEXTRA_CONF_FILE="debug.conf;ble.conf" All merged left-to-right; last write wins. Inspect final merged config: cat build/zephyr/.config ¦ grep CONFIG_BT

## 3.4  west.yml & Module Pinning

west.yml Pinning

# west.yml — controls which Zephyr version and HALs are used manifest: projects: - name: zephyr url: https://github.com/zephyrproject-rtos/zephyr revision: v3.6.0 # ALWAYS pin a specific version! - name: hal_espressif url: https://github.com/zephyrproject-rtos/hal_espressif revision: v5.1.2-zephyr-r0 path: modules/hal/espressif - name: hal_stm32 url: https://github.com/zephyrproject-rtos/hal_stm32 revision: 76ef5f0

⚠ Always pin revision= in west.yml for production projects. The "main" branch can introduce breaking DTS or API changes at any time. Use "west update" to pull new revisions intentionally, not accidentally.

# Theory Chapter 4 — RTOS Scheduling, IPC & Memory

Zephyr's scheduler is a fully preemptive, priority-based scheduler with optional time-slicing. Understanding the scheduling model at a deep level is what allows you to design firmware that is both responsive AND deterministic.

## 4.1  Preemptive vs Cooperative Scheduling

Scheduling Models

PREEMPTIVE (default in Zephyr for all normal threads): Thread A (priority 3) is running. Thread B (priority 1 = higher) becomes ready. Scheduler IMMEDIATELY switches to B — A is pre-empted mid-execution. A resumes exactly where it left off when B blocks again. Key implication: Data shared between threads MUST be protected with mutex or semaphore — even a simple counter update is not atomic on most architectures (read-modify-write = 3 separate bus cycles). COOPERATIVE (CONFIG_COOPERATIVE_SCHEDULING): Thread runs until it VOLUNTARILY yields or blocks. No context switch can interrupt it mid-execution. Simpler reasoning about shared data, but risks starvation. TIME-SLICING (CONFIG_TIMESLICING=y): Threads at the SAME priority get round-robin CPU slices. CONFIG_TIMESLICE_SIZE defines slice size in ms. Enable when multiple equal-priority threads do CPU-bound work. REAL-TIME GUARANTEE: Zephyr guarantees the highest-priority READY thread always runs. This is the definition of hard real-time scheduling.

## 4.2  IPC Primitive Selection Guide

| k_sem (Semaphore) | Signal between threads or ISR→thread. "Event happened N times". ISR-safe. Producer does not wait for consumer. |
| --- | --- |
| k_mutex (Mutex) | Exclusive resource access. Supports priority inheritance. NEVER use in ISR. Only locking thread can unlock. |
| k_spinlock | Protects data accessed from ISR context. Disables IRQs. Keep critical section to microseconds only. |
| k_msgq (Message Queue) | FIFO of fixed-size messages. Thread or ISR can enqueue. Consumer blocks when empty. Best for: sensor readings, events. |
| k_fifo | FIFO of variable-size memory blocks (pointer-passing). Zero-copy. Best for: network packets, audio buffers. |
| k_pipe | Byte-stream between threads. Like a UNIX pipe. Best for: streaming ADC data to processing thread. |
| k_poll | Wait on MULTIPLE objects at once (sem + msgq + fifo). Like select() for RTOS. Avoids one-thread-per-event design. |

## 4.3  Priority Inversion & Inheritance

Priority Inversion

PRIORITY INVERSION PROBLEM: Thread H (priority 1) waits for mutex held by Thread L (priority 9). Thread M (priority 5) pre-empts Thread L. Now Thread H is blocked by Thread M indirectly — INVERSION! Thread H misses its real-time deadline. Historical example: Mars Pathfinder 1997 system resets were caused by priority inversion between a high-priority bus management task and a low-priority meteorological data collection thread. ZEPHYR SOLUTION — Automatic Priority Inheritance via k_mutex: 1. Thread L holds mutex, Thread H blocks on it. 2. Zephyr TEMPORARILY raises Thread L's priority to match H (p=1). 3. Thread L finishes quickly, unlocks mutex, priority drops to p=9. 4. Thread H immediately gets the mutex and runs. k_mutex → implements priority inheritance ✅ k_sem → does NOT ❌ k_spinlock → does NOT (IRQ-level, no scheduling)

## 4.4  Stack Sizing & MPU Guard

Stack & MPU

WHY STATIC STACKS: K_THREAD_STACK_DEFINE places stack in .noinit section. Aligned to MPU region boundary (power-of-2, min 32 bytes). Optional MPU guard page for immediate overflow detection. No heap fragmentation — deterministic from first boot. STACK SENTINEL MECHANISM (CONFIG_STACK_SENTINEL=y): Magic value written at bottom of stack at thread creation. Checked on every context switch. If overwritten → k_oops() with thread name → system halt. Better than silent corruption but detection is deferred. MPU STACK GUARD (CONFIG_MPU_STACK_GUARD=y — Cortex-M4): Hardware MPU marks bottom-of-stack as No Access. Stack overflow → immediate HardFault → instant detection. Preferred over sentinel when MPU is available. SIZING METHODOLOGY: 1. Start with 2048 bytes 2. Enable CONFIG_THREAD_ANALYZER=y 3. Call thread_analyzer_print() after full workload 4. Target: (peak_used + 20% margin) 5. Threads calling LOG/BLE: add 512–1024 bytes extra

## 4.5  ISR Hard Rules

ISR Rules

ALLOWED in ISR context: │ FORBIDDEN in ISR context: ──────────────────────────────┼─────────────────────────────── k_sem_give() ✅ │ k_msleep() ❌ k_work_submit() ✅ │ k_sem_take(K_FOREVER) ❌ k_msgq_put(K_NO_WAIT) ✅ │ k_mutex_lock() ❌ gpio_pin_get() ✅ │ malloc() / k_malloc() ❌ k_spin_lock() ✅ │ LOG_INF() ❌ printk() ✅ │ i2c_write() ❌ CANONICAL ISR PATTERN: ISR → k_work_submit(&my_work) my_work_handler() runs in work queue thread — can do anything. ARM CORTEX-M PRIORITY NOTE: Lower number = HIGHER priority (opposite of Zephyr thread priority!). IRQ priority 0 = highest hardware priority. Zephyr ISRs: use IRQ_CONNECT(irq, priority, handler, arg, flags). Never set IRQ priority to 0 — Zephyr needs that for itself.

# Theory Chapter 5 — GPIO, UART & Signal Integrity

Understanding the hardware behind GPIO and UART prevents a whole class of bugs that cannot be solved by looking at code alone. Physical layer issues require physical layer understanding.

## 5.1  GPIO Hardware Modes — Complete Picture

GPIO Hardware Modes

GPIO OUTPUT MODES: Push-Pull (GPIO_OUTPUT): Actively drives HIGH (+VCC) or LOW (GND). Fast edges. Cannot be wire-ORed. Default for LEDs, SPI CS, digital signals. STM32: OTYPER bit = 0. ESP32-S3: default output mode. Open-Drain (GPIO_OPEN_DRAIN): Drives LOW actively; HIGH is float (requires external pull-up). Multiple devices can share one wire → wired-AND topology. MANDATORY for I2C (enables clock stretching). STM32: OTYPER bit = 1. GPIO INPUT MODES: Floating (no pull): NEVER leave digital inputs floating → capacitive noise → false IRQs. Pull-Up (GPIO_PULL_UP): Internal resistor to VCC (~40kΩ–50kΩ typical). Use for: active-low buttons, I2C (prefer external for speed). Pull-Down (GPIO_PULL_DOWN): Internal resistor to GND. Use for: signals that idle LOW. SCHMITT TRIGGER (all STM32/ESP32 digital GPIO): Rising threshold ≠ Falling threshold. STM32F446: Vth+ ≈ 1.7V, Vth- ≈ 0.8V @ 3.3V supply. Prevents noise-induced extra edges on slow signals. Slow RC signals can still cause double-triggering if edge crosses hysteresis band multiple times.

## 5.2  GPIO Electrical Limits

| STM32F446RE per-pin current | Max ±8mA per GPIO pin; ±120mA total across all VDD/VSS pins combined. |
| --- | --- |
| ESP32-S3 per-pin current | Max 40mA for GPIO0-21; 20mA for GPIO33-48. GPIO 35-39: INPUT ONLY. |
| LED series resistor | (VCC - Vf_LED) / I_max. At 3.3V with 2V Vf and 10mA: R = 130Ω minimum. |
| 5V signals on 3.3V GPIO | Instant damage. Use voltage divider (R1=10kΩ, R2=20kΩ) or TXB0104 level shifter. |
| STM32 output slew rate | Configurable per pin in pinctrl: low/medium/high/very-high. Slow slew reduces EMI. |

## 5.3  UART Frame & Baud Rate Theory

UART Theory

UART FRAME STRUCTURE (8N1 — most common): ┌──┬────────────────────┬───┐ │St│ D0 D1 D2 D3 D4 D5 D6 D7 │Sp│ └──┴────────────────────┴───┘ Start + 8 data bits (LSB first) + 1 stop bit = 10 bit-times/byte. At 115200 baud: 1 bit = 8.68µs. 1 byte = 86.8µs → 11,520 B/sec max. BAUD RATE ERROR: UART has NO clock line — both sides must independently generate baud. Derived from peripheral clock ÷ prescaler register. STM32F446RE USART2 on APB1 (45MHz): Prescaler = 45,000,000 / 115,200 = 390.625 → rounds to 391 Actual baud = 45,000,000 / 391 = 115,090 baud → 0.096% error ✅ Maximum allowed error: ±2.5% for reliable reception. FLOW CONTROL (RTS/CTS): Prevents UART buffer overflow during burst traffic. Enable: CONFIG_UART_STM32_HW_FLOW_CTRL=y Required for: >460800 baud, BLE HCI UART, GPS modules.

## 5.4  Interrupt Latency Analysis

Interrupt Latency

WORST-CASE INTERRUPT LATENCY (Cortex-M4 @ 180MHz): Hardware pipeline flush: ~12 cycles ISR vector fetch + branch: ~12 cycles Context save (8 registers HW): ~12 cycles Zephyr ISR wrapper overhead: ~20–40 cycles TOTAL to first ISR instruction: ~56–76 cycles ≈ 0.4µs CRITICAL SECTION IMPACT: k_spin_lock() disables interrupts on local CPU. Each µs of spinlock held adds 1µs to worst-case IRQ latency. Rule: Never hold spinlock for more than 2–3µs. ESP32-S3 IRAM_ATTR FOR ISR LATENCY: ESP32-S3 Flash access goes through cache (6-cycle miss penalty). ISR handler in Flash: +6 cycles on first execution (cache miss). ISR handler in IRAM: zero cache miss, deterministic latency. Always use IRAM_ATTR (or Z_ISR_DECLARE attribute) for ISR functions.

# Theory Chapter 6 — I2C, SPI & Serial Bus Protocols

I2C and SPI are the two dominant sensor interfaces in embedded systems. Understanding their electrical and protocol properties is essential for debugging communication failures — and for knowing WHEN to use which bus.

## 6.1  I2C Electrical Model & Pull-Up Selection

I2C Pull-up Theory

I2C ELECTRICAL MODEL: Both SDA and SCL are OPEN-DRAIN with external pull-up resistors. Any device can pull a line LOW. No device actively drives HIGH. The pull-up resistor pulls the line HIGH when nobody pulls it LOW. This architecture allows multi-master and clock-stretching. PULL-UP RESISTOR SELECTION: ┌────────────┬─────────────┬─────────────────────────────┐ │ Speed │ Typical Rp │ Notes │ ├────────────┼─────────────┼─────────────────────────────┤ │ 100kHz std │ 4.7kΩ–10kΩ │ Long cables OK │ │ 400kHz fast│ 2.2kΩ–4.7kΩ │ Short cables <0.5m │ │ 1MHz fast+ │ 1kΩ–2kΩ │ PCB trace only │ └────────────┴─────────────┴─────────────────────────────┘ Too high: slow rise time → glitches at high speed. Too low: excess GPIO driver current → possible damage. GY-86 module: 4.7kΩ pull-ups onboard at 3.3V. Rise time = 2.2 × Rp × C_bus. At 400kHz: max rise = 300ns. 4.7kΩ × 20pF = 94ns × 2.2 = 207ns → borderline at 400kHz. Add 2.2kΩ external pull-up in parallel for reliable 400kHz.

## 6.2  I2C Transaction Timing & ACK/NACK

I2C Protocol Detail

I2C TRANSACTION SEQUENCE: START: SDA↓ while SCL=H (unique condition — not a data bit) ADDRESS FRAME: [A6 A5 A4 A3 A2 A1 A0][R/W][ACK] DATA BYTES: [D7..D0][ACK] repeated for each byte STOP: SDA↑ while SCL=H MPU6050 addresses: 0x68 (AD0 pin LOW) or 0x69 (AD0 pin HIGH) ACK/NACK MEANING: After address byte: ACK (SDA pulled LOW by slave) = device present, ready NACK (SDA stays HIGH) = device absent or busy After write data byte: ACK = slave accepted the byte NACK = slave rejected it (e.g., register address invalid) After last read byte: Master sends NACK to signal "I'm done reading" REPEATED START (critical for sensors): Write register address → ReSTART (no STOP) → Read data. A full STOP between write and read can reset some sensors' internal byte pointer. Zephyr i2c_transfer() with two msgs automatically generates Repeated START. CLOCK STRETCHING: Slave holds SCL LOW to pause master while preparing data. Required support by all I2C-compliant master hardware. Bit-bang I2C often lacks this → intermittent sensor failures.

## 6.3  SPI Protocol — Clock Modes & Timing

SPI Theory

SPI vs I2C — DECISION GUIDE: I2C: 2 wires, shared bus with addressing, up to 3.4MHz, open-drain. Best for: sensors, EEPROMs, RTCs, low-speed peripherals. SPI: 4 wires, dedicated CS per device, up to 80MHz, push-pull. Best for: ADCs, displays, flash memory, SD cards. SPI CLOCK MODES (CPOL × CPHA — most misunderstood topic): CPOL: clock idle polarity (0=LOW idle, 1=HIGH idle) CPHA: data sampling edge (0=first edge, 1=second edge) Mode 0 (CPOL=0,CPHA=0): idle LOW, sample rising ← MOST COMMON Mode 1 (CPOL=0,CPHA=1): idle LOW, sample falling Mode 2 (CPOL=1,CPHA=0): idle HIGH, sample falling Mode 3 (CPOL=1,CPHA=1): idle HIGH, sample rising In Zephyr SPI_OP flags: Mode 0: neither SPI_MODE_CPOL nor SPI_MODE_CPHA set (default) Mode 3: both SPI_MODE_CPOL and SPI_MODE_CPHA set WRONG MODE produces garbage data with NO ERROR REPORTED. Always check device datasheet timing diagram: "SCK idle state" and "data latched on rising/falling edge". SPI FRAME FORMAT: SPI_TRANSFER_MSB: most-significant bit first (default, most devices) SPI_TRANSFER_LSB: least-significant bit first (some displays, DACs) Word width: SPI_WORD_SET(8) for byte transfers, SPI_WORD_SET(16) etc.

## 6.4  Bus Speed vs Signal Integrity

| I2C wire length | 100kHz: up to 3m. 400kHz: under 0.5m. Parasitic capacitance dominates at high speed. |
| --- | --- |
| SPI trace length | On PCB: safe up to 50MHz with matched lengths. Off-board: limit to <10MHz without differential signaling. |
| Ground return path | Every signal must have a nearby ground return trace. Distant GND = loop antenna = EMI + noise pickup. |
| Decoupling capacitors | 100nF ceramic directly adjacent to each IC VCC pin. Absorbs high-frequency supply glitches from switching. |
| I2C buffer/repeater | For cables >1m: use PCA9617A or similar. Regenerates signal, allows longer runs. |

# Theory Chapter 7 — DMA Architecture & Memory Buses

DMA (Direct Memory Access) is misunderstood by most embedded engineers. The real value is understanding WHEN it helps, when it hurts, and how to configure it correctly on both platforms.

## 7.1  Why DMA Matters for Firmware Timing

DMA Value Proposition

WITHOUT DMA — CPU-driven SPI transfer: For each byte: CPU writes SPI_DR → polls SPI_SR.TXE → reads SPI_DR CPU is BLOCKED for entire transfer. 1000 bytes at 1MHz SPI = 1ms of CPU time wasted. WITH DMA — parallel operation: CPU sets up DMA (source ptr, dest ptr, count) in ~10 cycles. DMA engine transfers data via AHB/APB bus autonomously. CPU is FREE — runs other threads, processes, or sleeps. DMA fires interrupt when done → CPU resumes. 1000 bytes at 1MHz SPI = 1ms, CPU used only ~20 cycles! WHEN NOT TO USE DMA: Transfers < 4 bytes: setup overhead > transfer time. Very short periodic transfers (< 1ms period): overhead dominates. Non-contiguous source/dest buffers without scatter-gather support. Cortex-A with D-cache: requires explicit cache flush before DMA. On Cortex-M (no D-cache): DMA is always safe and beneficial.

## 7.2  STM32F446RE DMA Architecture

STM32 DMA

STM32F446RE: 2 DMA controllers, each with 8 streams × multiple channels. Each stream = dedicated hardware transfer engine. Channel = which peripheral event triggers the stream. PARTIAL MAPPING TABLE (from STM32F446 Reference Manual Table 27/28): ┌──────────┬────────┬─────────┬────────────────────────┐ │ DMA │ Stream │ Channel │ Peripheral │ ├──────────┼────────┼─────────┼────────────────────────┤ │ DMA1 │ 5 │ CH4 │ USART2_RX │ │ DMA1 │ 6 │ CH4 │ USART2_TX │ │ DMA1 │ 0 │ CH1 │ I2C1_RX │ │ DMA1 │ 7 │ CH1 │ I2C1_TX │ │ DMA2 │ 2 │ CH3 │ SPI1_RX │ │ DMA2 │ 3 │ CH3 │ SPI1_TX │ │ DMA2 │ 0 │ CH0 │ ADC1 │ └──────────┴────────┴─────────┴────────────────────────┘ CONFLICT RULE: Two peripherals cannot share the same DMA stream. Check stream assignment before enabling DMA on multiple peripherals. DMA TRANSFER MODES: Normal: One-shot transfer of N bytes, fires completion IRQ. Circular: Auto-restart — fills ring buffer continuously. Perfect for continuous ADC sampling with double-buffer. Double-buffer: Alternate between buf A and buf B. Process buf A while DMA fills buf B → zero dead time.

## 7.3  ESP32-S3 GDMA & Memory Considerations

ESP32-S3 DMA & Memory

ESP32-S3 MEMORY HIERARCHY: Internal SRAM: 512KB (split across SRAM0/1/2 domains) Internal IRAM: 384KB (instruction RAM — fast, no cache miss) PSRAM: Up to 8MB OctalSPI SRAM (~40ns access latency) Flash: Up to 16MB OctalSPI NOR Flash (~60ns via XIP) ROM: 384KB (bootloader, crypto primitives) ESP32-S3 GDMA (General DMA — 10 channels): ALL peripherals (SPI, I2C, UART, ADC) share GDMA channels. DMA can access: internal SRAM + PSRAM. DMA CANNOT access Flash (XIP does not support DMA). PSRAM + DMA: Works but latency is higher (SPI-bus-connected). Avoid PSRAM for time-critical DMA buffers. Place DMA buffers in internal SRAM: static uint8_t dma_buf[256] DRAM_ATTR; CACHE COHERENCY NOTE: ESP32-S3 SRAM is NOT cached → DMA and CPU always see same data. No cache flush needed before DMA transfer (unlike Cortex-A7/A53). Flash XIP cache is separate and not affected by SRAM DMA.

# Theory Chapter 8 — Bluetooth LE Stack Architecture

BLE is the most complex subsystem in Zephyr. Debugging BLE issues without understanding the protocol stack layers is nearly impossible. This chapter maps the BLE standard to Zephyr's implementation.

## 8.1  BLE Protocol Stack Layers

BLE Stack Layers

BLE LAYER STACK: ┌──────────────────────────────────────────────────────┐ │ APPLICATION (your GATT services — Exercise 09) │ ├──────────────────────────────────────────────────────┤ │ GATT Generic Attribute Profile │ ← services, chars ├──────────────────────────────────────────────────────┤ │ ATT Attribute Protocol │ ← Read/Write/Notify PDUs ├──────────────────────────────────────────────────────┤ │ SMP Security Manager │ ← Pairing, Bonding ├──────────────────────────────────────────────────────┤ │ L2CAP Logical Link Control │ ← Channels, fragmentation ├──────────────────────────────────────────────────────┤ │ HCI Host↔Controller Interface │ ← Command/Event ├──────────────────────────────────────────────────────┤ │ LL Link Layer │ ← Advertising, connections ├──────────────────────────────────────────────────────┤ │ PHY Physical Layer (2.4GHz, 1Mbps / 2Mbps) │ └──────────────────────────────────────────────────────┘ On ESP32-S3 with Zephyr: HOST layers (GATT → HCI): Zephyr Bluetooth Host (open source C) HCI → PHY: Espressif binary BLE controller blob They communicate via virtual HCI over shared memory ring buffers.

## 8.2  GAP — Advertising, Scanning & Connection

GAP Theory

ADVERTISING: Device broadcasts ADV packets on channels 37, 38, 39 every ADV_INTERVAL. Channels 37/38/39 avoid 2.4GHz WiFi channels 1/6/11. ADV_INTERVAL range: 20ms (fast/high-power) to 10.24s (slow/low-power). Connectable advertising (BT_LE_ADV_CONN_NAME): Any scanner can initiate connection. Non-connectable advertising (BT_LE_ADV_NCONN): Beacon mode — broadcasts sensor data without accepting connections. CONNECTION PARAMETERS (battery life critical): Connection Interval: Time between connection events (7.5ms – 4s). Slave Latency: Peripheral may skip N events if no data pending. Saves power: skip=9 → only wake 1/10 events. Supervision Timeout: Disconnect if no contact within this period. Fast profile: Interval=7.5ms, Latency=0, Timeout=2s → 25mA Slow profile: Interval=500ms, Latency=3, Timeout=6s → 0.5mA iOS/Android central has final say on connection parameters. Your peripheral REQUESTS params; central may modify them.

## 8.3  GATT Services, Characteristics & Notify vs Indicate

GATT Theory

GATT HIERARCHY: Profile └─ Service (UUID = 16-bit standard or 128-bit custom) ├─ Characteristic (UUID + properties + value) │ ├─ Value (the actual data: 1–512 bytes) │ ├─ CCCD (Client Config Descriptor, UUID=0x2902) │ │ Client writes 0x0001 → enable notifications │ │ Client writes 0x0002 → enable indications │ └─ User Description Descriptor (human-readable) └─ Another Characteristic ATT OPERATIONS: Read: Client requests current value → server responds. Write: Client sends new value → server ACKs. Write No Rsp: Client sends value, no ACK → faster but unreliable. Notify: Server PUSHES data → client does NOT ACK. Indicate: Server PUSHES data → client MUST ACK. NOTIFICATIONS vs INDICATIONS: Notification: Fire-and-forget. Can be dropped if buffers full. Throughput: ~2kbps–128kbps depending on interval+MTU. Use for: IMU streaming, heart rate, audio. Indication: Guaranteed delivery. Must wait for ACK before next. Throughput: ~half of notification. Use for: configuration changes, critical alerts. ATT MTU (Maximum Transmission Unit): Default: 23 bytes → max 20 bytes payload per packet. After MTU exchange: up to 517 bytes → 514 bytes payload. Enable large MTU: CONFIG_BT_L2CAP_TX_MTU=247

## 8.4  BLE + WiFi Coexistence on ESP32-S3

BLE Coexistence & Throughput

ESP32-S3: single 2.4GHz radio shared between WiFi and BLE. Hardware coexistence arbitration unit manages time-sharing. ENABLE COEXISTENCE: CONFIG_ESP32_WIFI_BT_COEXIST=y (required when using both) Without this: WiFi TX jams BLE RX → packet loss. IMPACT ON BLE: With WiFi active: BLE connection intervals <100ms → increased latency and occasional missed connection events. Solution: Use connection interval ≥ 100ms when WiFi is active. ADC2 CONFLICT: ADC2 shares the radio domain on ESP32-S3. ADC2 readings erratic when WiFi TX is active. Use ADC1 channels (GPIO1–GPIO10) for reliable ADC+WiFi. BLE THROUGHPUT CALCULATION: Interval=7.5ms, 1 packet, default MTU (20 bytes payload): Throughput = 20 × (1/0.0075) ≈ 2,667 B/s ≈ 21 kbps With 2M PHY + MTU=247 (244 payload) + interval=15ms: Throughput ≈ 244 × (1/0.015) ≈ 16,267 B/s ≈ 130 kbps peak

# Theory Chapter 9 — Power Management Theory & Sleep States

Power management in embedded firmware is not a feature you add at the end — it is an architectural decision that affects every aspect of thread design, peripheral usage, and interrupt handling.

## 9.1  Power Consumption Physics

Power Physics

DYNAMIC POWER: P_dynamic = α × C_load × V_DD² × f_clk α = activity factor (fraction of gates switching per cycle) C = total load capacitance of all switching signals V = supply voltage f = clock frequency KEY INSIGHT: Power scales with V² — halving VDD saves 75% dynamic power. Power scales linearly with frequency — halving clock saves 50%. Combined: reducing VDD from 3.3V→1.8V AND halving frequency → saves ~86%. STATIC (LEAKAGE) POWER: P_static = I_leak × V_DD Sub-threshold leakage of transistors at rest. Dominates in deep sleep. Increases significantly with temperature. STM32F446RE: ~2µA in STANDBY. ESP32-S3: ~7µA in deep sleep (RTC only).

## 9.2  Zephyr Power States

| PM_STATE_ACTIVE | All clocks running. Highest power. Normal operation. |
| --- | --- |
| PM_STATE_RUNTIME_IDLE | CPU executes WFI instruction. Clocks still running. Wakes on any IRQ in ~1 cycle. STM32 WFI: ~30mA → ~5mA. |
| PM_STATE_SUSPEND_TO_IDLE | CPU + most peripherals clock-gated. SRAM retained. Wakes from RTC, GPIO. STM32 STOP mode / ESP32 light sleep. ~200µA. |
| PM_STATE_STANDBY | Deeper: peripheral state lost (except RTC backup). SRAM may be lost. STM32 STANDBY: ~2µA. |
| PM_STATE_SOFT_OFF | Everything off except RTC/wakeup logic. Full reboot on wakeup. ESP32 deep sleep: ~7µA (ULP running). |

## 9.3  Tickless Kernel + PM Integration

PM Integration

HOW ZEPHYR DECIDES WHEN TO SLEEP: 1. All runnable threads block (waiting for sem/timer/msgq). 2. Scheduler finds nothing to run → calls PM subsystem. 3. PM queries: "time until next scheduled wakeup?" 4. If time > min_residency_us of a sleep state → enter that state. 5. Hardware timer reprogrammed for wakeup → CPU enters sleep. 6. IRQ or timer fires → CPU wakes → scheduler resumes. min_residency_us: minimum sleep duration where power saved > cost of entering + exiting sleep. Set per state in board DTS: zephyr,pm-states { compatible = "zephyr,power-state"; power-state-name = "suspend-to-idle"; min-residency-us = <500>; exit-latency-us = <200>; }; DEVICE PM (peripheral suspend on system sleep): Devices registered with pm_device_action_cb get: PM_DEVICE_ACTION_SUSPEND → save state, power down hardware PM_DEVICE_ACTION_RESUME → restore state, reinitialize hardware Called automatically in reverse init order on sleep, forward on wake.

## 9.4  Practical Power Budget Example

Power Budget Example

Smart shoe sensor node — 3.7V LiPo 300mAh battery target: Operation mode │ Current │ Duration │ Energy/cycle ─────────────────────────┼─────────┼───────────┼───────────── BLE advertising TX │ 25 mA │ 1 ms │ 0.007 µAh I2C sensor read (IMU) │ 15 mA │ 5 ms │ 0.021 µAh MCU active processing │ 20 mA │ 10 ms │ 0.056 µAh BLE connected event │ 5 mA │ 100 ms │ 0.139 µAh Light sleep (rest) │ 0.3 mA │ 884 ms │ 0.074 µAh ─────────────────────────┼─────────┼───────────┼───────────── Per 1-second cycle total │ │ 1000 ms │ ~0.30 µAh Daily: 0.30 µAh × 86,400 = 25.9 mAh/day Life: 300 mAh / 25.9 mAh/day ≈ 11.6 days To reach 30 days: reduce active window from 116ms → 45ms per cycle.

# Theory Chapter 10 — Boot Sequence, Linker & Reset Handlers

What happens between power-on and the first line of main()? This is the least-documented part of embedded firmware and the source of the most baffling bugs: corrupted variables, wrong stack pointer, peripheral init order failures.

## 10.1  Cortex-M4 Boot Sequence (STM32F446RE)

Cortex-M4 Boot

1. POWER ON: CPU reads BOOT0 pin. BOOT0=0 → boot from Flash at 0x08000000 (normal operation). BOOT0=1 → ST system memory bootloader (for DFU/UART programming). 2. CPU reads MSP from Flash[0x00000000]: This becomes the Main Stack Pointer (interrupt/reset stack). Zephyr linker places: _interrupt_stack + CONFIG_ISR_STACK_SIZE here. 3. CPU reads reset vector from Flash[0x00000004]: Branches to __reset (Zephyr assembly startup in arch/arm/core/). 4. __reset assembly: → Set VTOR (Vector Table Offset Register) to Flash start. → Copy .data from Flash (LMA) to SRAM (VMA). → Zero .bss section in SRAM. → Call z_arm_prep_c() → z_cstart(). 5. z_cstart() — Kernel bootstrap (kernel/init.c): → Initialize kernel scheduler data structures. → Run all SYS_INIT() hooks in order: PRE_KERNEL_1 → PRE_KERNEL_2 → POST_KERNEL → APPLICATION → Create main thread, idle thread. → Transfer CPU to scheduler → scheduler picks main thread. 6. YOUR main() RUNS.

## 10.2  Xtensa LX7 Boot Sequence (ESP32-S3)

ESP32-S3 Boot Sequence

1. ROM bootloader (hardcoded in ROM at 0x40000000): Initializes basic clocks, GPIO strapping. Reads 2nd stage bootloader from SPI Flash offset 0x1000. 2. 2nd stage bootloader (ESP-IDF based, included in Zephyr image): Sets up flash SPI interface and clock. Reads partition table at Flash offset 0x8000. Loads Zephyr app partition. Sets up MMU for flash-mapped code execution (XIP). Jumps to Zephyr entry point. 3. Zephyr entry (__start in arch/xtensa/core/startup.S): Initialize L1 cache, watchdog, system clocks. Copy .data from Flash → DRAM. Zero .bss in DRAM. If CONFIG_ESP_SPIRAM=y: initialize PSRAM controller. Call z_cstart() → same as Cortex-M from here. 4. DUAL CORE NOTE: PRO CPU (CPU0) runs Zephyr application normally. APP CPU (CPU1) stays in reset unless CONFIG_SMP=y. With CONFIG_SMP=y: Zephyr SMP scheduler distributes threads. ESP32-S3 SMP support in Zephyr is experimental as of v3.6.

## 10.3  Linker Sections & Code Placement

Linker & Section Theory

STANDARD ELF SECTIONS AND THEIR ROLE: .text All compiled code (in Flash/ROM, execute-in-place) .rodata const data, string literals (in Flash) .data Initialized globals (initialized copy in Flash, live in SRAM) .bss Zero-initialized globals (only in SRAM, zeroed at boot) .noinit NOT initialized at boot — survives reset for crash logs Zephyr-specific linker sections: _k_timer_area All K_TIMER_DEFINE() timers _k_thread_area All K_THREAD_DEFINE() threads init_PRE_KERNEL_1_start/end SYS_INIT table for boot level 1 PLACING CODE IN SPECIFIC MEMORY: /* ESP32-S3: Force ISR into IRAM (avoids cache miss latency) */ static void IRAM_ATTR my_fast_isr(const struct device *d, struct gpio_callback *cb, uint32_t pins) { ... } /* STM32: Place hot path into CCMRAM (64KB zero-wait-state) */ /* CCMRAM is CPU-only — NO DMA to/from CCMRAM! */ CONFIG_ARCH_ZEPHYR_CODE_DATA_RELOCATION=y zephyr_code_relocate(FILES src/hot_path.c LOCATION SRAM) /* ESP32-S3: Large const table in PSRAM */ static const uint8_t big_lut[65536] __attribute__((section(".ext_ram.bss")));

## 10.4  Hardware Watchdog Theory & Usage

Watchdog Theory

WATCHDOG PURPOSE: Hardware counter that resets MCU if firmware fails to "feed" it. Catches: deadlocks, infinite loops, stack corruption, hung ISRs. STM32 WATCHDOG TYPES: IWDG (Independent WDT): Clocked from LSI (32kHz independent oscillator). Runs in all sleep/standby modes — always active. Cannot be disabled via software once started. Timeout range: 0.125ms – 32.76s. Use for: system-level catch-all protection. WWDG (Window WDT): Must be fed within a WINDOW (not too early, not too late). Catches code running too fast (runaway loops) AND too slow. More deterministic — catches timing deviations. ESP32-S3 RWDT (Real-time WDT): Can be active in deep sleep (feeds from RTC). TWDT (Task WDT): monitors individual tasks/threads. Zephyr maps both to the standard watchdog API. ZEPHYR WATCHDOG PATTERN: #include <zephyr/drivers/watchdog.h> const struct device *wdt = DEVICE_DT_GET(DT_NODELABEL(iwdg)); struct wdt_timeout_cfg cfg = { .window.min = 0, .window.max = 2000, /* 2-second timeout */ .callback = NULL, /* NULL = immediate hardware reset */ }; int ch = wdt_install_timeout(wdt, &cfg); wdt_setup(wdt, WDT_OPT_PAUSE_HALTED_BY_DBG); /* In each critical thread: */ wdt_feed(wdt, ch); /* must call at least every 2 seconds */

# Exercise 01 — GPIO Blink (The Real "Hello World")

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

Blinking an LED in Zephyr teaches the Device Tree (DT) abstraction, the GPIO API, and how Zephyr's build system selects board-specific hardware. Do NOT skip this — every subsequent exercise builds on these concepts.

## 1.1  Project Structure

Project Tree

zephyr_blink/ ├── CMakeLists.txt ├── prj.conf └── src/ └── main.c

## 1.2  CMakeLists.txt

CMakeLists.txt

cmake_minimum_required(VERSION 3.20.0) find_package(Zephyr REQUIRED HINTS $ENV{ZEPHYR_BASE}) project(blink) target_sources(app PRIVATE src/main.c)

## 1.3  prj.conf — Minimal Config

prj.conf

# Nothing extra needed for GPIO blink # Zephyr enables GPIO driver automatically when # board DTS has GPIO nodes. CONFIG_GPIO=y

## 1.4  Source Code — Side by Side

| // main.c (ESP32-S3) #include <zephyr/kernel.h> #include <zephyr/drivers/gpio.h> /* DT alias from board DTS */ #define LED0_NODE DT_ALIAS(led0) static const struct gpio_dt_spec led = GPIO_DT_SPEC_GET(LED0_NODE, gpios); int main(void) { if (!gpio_is_ready_dt(&led)) { return -ENODEV; } gpio_pin_configure_dt(&led, GPIO_OUTPUT_ACTIVE); while (1) { gpio_pin_toggle_dt(&led); k_msleep(500); } return 0; } | // main.c (STM32F446RE) #include <zephyr/kernel.h> #include <zephyr/drivers/gpio.h> /* Nucleo green LED = PA5 via alias */ #define LED0_NODE DT_ALIAS(led0) static const struct gpio_dt_spec led = GPIO_DT_SPEC_GET(LED0_NODE, gpios); int main(void) { if (!gpio_is_ready_dt(&led)) { return -ENODEV; } gpio_pin_configure_dt(&led, GPIO_OUTPUT_ACTIVE); while (1) { gpio_pin_toggle_dt(&led); k_msleep(500); } return 0; } /* Code is IDENTICAL — DTS does the work */ |
| --- | --- |

## 1.5  Build & Flash

| # ESP32-S3 west build -b esp32s3_devkitc \ -- -DBOARD_FLASH_RUNNER=esp32 west flash --runner esp32 west espressif monitor | # STM32F446RE (Nucleo) west build -b nucleo_f446re west flash # ST-Link auto-detected # Serial via /dev/ttyACM0 |
| --- | --- |

## 1.6  Minute Details & Common Pitfalls

| GPIO_DT_SPEC_GET vs GET_BY_IDX | Use GET when DTS has single pin; GET_BY_IDX(node, gpios, 0) for arrays |
| --- | --- |
| gpio_is_ready_dt() | ALWAYS check — returns false if clock/power not enabled |
| GPIO_OUTPUT_ACTIVE | Drives to the "active" logic level from DTS (handles active-low LEDs transparently) |
| GPIO_ACTIVE_LOW in DTS | If DTS has GPIO_ACTIVE_LOW flag, pin is LOW when "active". Code needs no change |
| k_msleep() vs k_busy_wait() | k_msleep releases CPU; k_busy_wait is a spin-loop — use msleep unless sub-ms timing needed |
| ESP32-S3 GPIO38+ | GPIO 35-39 on ESP32-S3 are input-only on some package variants — check your schematic |

# Exercise 02 — UART Console & Shell

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

UART is your primary debug window. Zephyr's shell subsystem gives you a live interactive console — invaluable for testing without re-flashing.

## 2.1  prj.conf — Enable Shell

prj.conf

CONFIG_UART_CONSOLE=y CONFIG_SHELL=y CONFIG_SHELL_BACKEND_SERIAL=y CONFIG_SHELL_PROMPT_UART="zephyr:~$ " CONFIG_LOG=y CONFIG_LOG_BACKEND_UART=y CONFIG_LOG_DEFAULT_LEVEL=3 # 0=off 1=err 2=warn 3=inf 4=dbg

## 2.2  Using printk vs LOG_*

src/main.c

#include <zephyr/kernel.h> #include <zephyr/logging/log.h> LOG_MODULE_REGISTER(myapp, LOG_LEVEL_DBG); int main(void) { /* printk: always compiled in, no module filter */ printk("Boot\n"); /* LOG_*: filtered at compile+runtime per module */ LOG_INF("System started, uptime %u ms", k_uptime_get_32()); LOG_WRN("Low memory warning"); LOG_ERR("This is an error"); LOG_DBG("Debug: cycle count %u", k_cycle_get_32()); return 0; }

## 2.3  Custom Shell Command

src/shell_cmds.c

#include <zephyr/shell/shell.h> static int cmd_hello(const struct shell *sh, size_t argc, char **argv) { shell_print(sh, "Hello, %s!", argc > 1 ? argv[1] : "World"); return 0; } /* Register as: "myapp hello [name]" */ SHELL_CMD_REGISTER(hello, NULL, "Print greeting", cmd_hello);

## 2.4  Board-Specific UART Notes

| # ESP32-S3 UART config # Default console = UART0 (GPIO43/44) # In prj.conf for ESP32-S3 with USB-OTG: CONFIG_USB_DEVICE_STACK=y CONFIG_USB_CDC_ACM=y CONFIG_UART_LINE_CTRL=y # Then connect to /dev/ttyACM0 in WSL2 | # STM32F446RE UART config # Nucleo routes UART2 to ST-Link VCP # PA2=TX, PA3=RX → /dev/ttyACM0 # No extra config needed — board.cmake # already sets CONFIG_UART_CONSOLE=y # Baud: 115200 (Zephyr default) minicom -D /dev/ttyACM0 -b 115200 |
| --- | --- |

⚠ On ESP32-S3 devkit, USB-JTAG/Serial port (GPIO19/20) also works as console. Set CONFIG_ESP_CONSOLE_UART_NUM=0 to keep UART0 vs USB-CDC choice consistent.

# Exercise 03 — Threads, Semaphores & Mutexes

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

This is the heart of Zephyr. Understanding threads, scheduling, and synchronization primitives separates real RTOS developers from bare-metal coders.

## 3.1  Thread Fundamentals

src/main.c — Threads + Semaphore + Mutex

#include <zephyr/kernel.h> #include <zephyr/logging/log.h> LOG_MODULE_REGISTER(threads_ex, LOG_LEVEL_INF); /* Static stack allocation — PREFERRED over dynamic */ #define STACK_SZ 1024 #define PRIORITY 5 K_THREAD_STACK_DEFINE(sensor_stack, STACK_SZ); K_THREAD_STACK_DEFINE(blink_stack, STACK_SZ); struct k_thread sensor_tid, blink_tid; /* ── Semaphore: signal between threads ────────────── */ K_SEM_DEFINE(data_ready_sem, 0, 1); /* args: name, initial_count, limit */ /* ── Mutex: protect shared resource ──────────────── */ K_MUTEX_DEFINE(uart_mutex); static uint32_t shared_value; /* Sensor thread — produces data */ void sensor_thread(void *p1, void *p2, void *p3) { ARG_UNUSED(p1); ARG_UNUSED(p2); ARG_UNUSED(p3); while (1) { /* Simulate sensor read */ shared_value = k_cycle_get_32() & 0xFFFF; LOG_INF("Sensor: value=%u", shared_value); k_sem_give(&data_ready_sem); /* signal */ k_msleep(1000); } } /* Processing thread — consumes data */ void process_thread(void *p1, void *p2, void *p3) { ARG_UNUSED(p1); ARG_UNUSED(p2); ARG_UNUSED(p3); while (1) { /* Block until sensor signals — no CPU spin */ k_sem_take(&data_ready_sem, K_FOREVER); k_mutex_lock(&uart_mutex, K_FOREVER); LOG_INF("Process: got value=%u", shared_value); k_mutex_unlock(&uart_mutex); } } int main(void) { k_thread_create(&sensor_tid, sensor_stack, K_THREAD_STACK_SIZEOF(sensor_stack), sensor_thread, NULL, NULL, NULL, PRIORITY, 0, K_NO_WAIT); k_thread_name_set(&sensor_tid, "sensor"); k_thread_create(&blink_tid, blink_stack, K_THREAD_STACK_SIZEOF(blink_stack), process_thread, NULL, NULL, NULL, PRIORITY + 1, 0, K_NO_WAIT); k_thread_name_set(&blink_tid, "process"); return 0; /* main() becomes idle thread in Zephyr */ }

## 3.2  Priority & Scheduling Rules

| Priority 0 | Highest priority (pre-empts everything) |
| --- | --- |
| Negative priorities | Meta-IRQ threads — only for specific use cases |
| CONFIG_NUM_PREEMPT_PRIORITIES | Default = 15; set in prj.conf to change range |
| k_yield() | Voluntarily give up CPU slice to threads of same priority |
| K_FOREVER | Block indefinitely — safe for semaphore/mutex takes |
| K_NO_WAIT | Return immediately if resource unavailable (returns -EBUSY) |
| K_MSEC(n) | Timeout value: wait up to n milliseconds |

## 3.3  Minute Details — Stack Overflows

prj.conf + debug tip

# prj.conf — Enable stack sentinel (detects overflow) CONFIG_THREAD_STACK_INFO=y CONFIG_STACK_SENTINEL=y CONFIG_THREAD_ANALYZER=y # In code — print thread stats to find correct stack size #include <zephyr/debug/thread_analyzer.h> thread_analyzer_print(); /* call from main after threads start */ /* Output shows: name ¦ unused stack bytes ¦ used ¦ size */

⚠ Rule of thumb: Start with 2048 bytes per thread. Run thread_analyzer after your workload, then reduce to (peak_used + 20% margin). Never go below 512 bytes.

# Exercise 04 — Timers, Work Queues & Delayed Work

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

Timers and work queues are the preferred way to schedule periodic actions without blocking threads — the Zephyr equivalent of FreeRTOS software timers and deferred interrupt handlers.

## 4.1  Kernel Timers

src/main.c — k_timer

#include <zephyr/kernel.h> #include <zephyr/logging/log.h> LOG_MODULE_REGISTER(timers_ex, LOG_LEVEL_INF); /* Timer callback runs in SYSTEM WORK QUEUE context */ void heartbeat_handler(struct k_timer *t) { /* KEEP SHORT — no blocking calls here */ LOG_INF("Heartbeat tick @ %lld ms", k_uptime_get()); } void stop_handler(struct k_timer *t) { LOG_WRN("Timer expired and stopped"); } K_TIMER_DEFINE(heartbeat_timer, heartbeat_handler, /* expiry fn */ stop_handler); /* stop fn */ int main(void) { /* Start: fire after 500ms, repeat every 1000ms */ k_timer_start(&heartbeat_timer, K_MSEC(500), K_MSEC(1000)); k_sleep(K_SECONDS(5)); k_timer_stop(&heartbeat_timer); /* Query remaining time */ uint32_t rem = k_timer_remaining_get( &heartbeat_timer); LOG_INF("Remaining: %u ms", rem); return 0; }

## 4.2  Work Queue (Deferred ISR Pattern)

src/main.c — Work Queue

#include <zephyr/kernel.h> #include <zephyr/drivers/gpio.h> /* Work item for deferred processing */ struct k_work process_work; struct k_work_delayable periodic_work; static void process_handler(struct k_work *work) { /* This runs in system work queue thread */ /* Safe to use LOG, I2C, SPI here */ LOG_INF("Work queue: processing event"); } static void periodic_handler(struct k_work *work) { LOG_INF("Periodic: %lld ms", k_uptime_get()); /* Re-schedule itself */ k_work_reschedule( CONTAINER_OF(work, struct k_work_delayable, work), K_MSEC(500)); } /* GPIO ISR — called in ISR context, CANNOT block */ static void button_isr(const struct device *dev, struct gpio_callback *cb, uint32_t pins) { /* Submit work — don't do anything heavy here */ k_work_submit(&process_work); } int main(void) { k_work_init(&process_work, process_handler); k_work_init_delayable(&periodic_work, periodic_handler); /* Start periodic work after 1s, then self-reschedules */ k_work_reschedule(&periodic_work, K_SECONDS(1)); return 0; }

## 4.3  Custom Work Queue (Separate Thread)

Custom Work Queue

/* Create a dedicated work queue thread — isolate from sys WQ */ K_THREAD_STACK_DEFINE(my_wq_stack, 2048); struct k_work_q my_work_q; void init_custom_wq(void) { k_work_queue_init(&my_work_q); k_work_queue_start(&my_work_q, my_wq_stack, K_THREAD_STACK_SIZEOF(my_wq_stack), 5, /* priority */ NULL); /* no config */ } /* Submit to custom queue, not system queue */ k_work_submit_to_queue(&my_work_q, &process_work);

⚠ Never call k_sleep(), k_sem_take(K_FOREVER) or any blocking API from a timer callback. Timer callbacks run in ISR context. Use k_work_submit() to defer to a thread.

# Exercise 05 — GPIO Interrupts & Callbacks

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

GPIO interrupts in Zephyr use a callback registration model. Understanding the ISR vs thread boundary is critical for reliable, race-condition-free code.

## 5.1  Button Interrupt with Debounce

src/main.c — GPIO IRQ + Debounce

#include <zephyr/kernel.h> #include <zephyr/drivers/gpio.h> #include <zephyr/logging/log.h> LOG_MODULE_REGISTER(gpio_irq, LOG_LEVEL_INF); #define BTN_NODE DT_ALIAS(sw0) static const struct gpio_dt_spec btn = GPIO_DT_SPEC_GET(BTN_NODE, gpios); static struct gpio_callback btn_cb; static struct k_work_delayable debounce_work; /* Debounce handler — runs in work queue thread */ static void debounce_handler(struct k_work *work) { /* Re-read pin to confirm stable */ int val = gpio_pin_get_dt(&btn); if (val == 0) { LOG_INF("Button PRESSED (debounced)"); } } /* ISR — ultra-fast, no logs, no blocking */ static void btn_isr(const struct device *dev, struct gpio_callback *cb, uint32_t pins) { /* Schedule debounce check 20ms later */ k_work_reschedule(&debounce_work, K_MSEC(20)); } int main(void) { if (!gpio_is_ready_dt(&btn)) { return -1; } gpio_pin_configure_dt(&btn, GPIO_INPUT); gpio_pin_interrupt_configure_dt( &btn, GPIO_INT_EDGE_TO_ACTIVE); /* falling edge */ gpio_init_callback(&btn_cb, btn_isr, BIT(btn.pin)); gpio_add_callback(btn.port, &btn_cb); k_work_init_delayable(&debounce_work, debounce_handler); LOG_INF("Waiting for button..."); while (1) { k_msleep(1000); } return 0; }

## 5.2  Interrupt Trigger Options

| GPIO_INT_EDGE_RISING | Trigger on 0→1 transition |
| --- | --- |
| GPIO_INT_EDGE_FALLING | Trigger on 1→0 transition |
| GPIO_INT_EDGE_BOTH | Both edges — good for encoder quadrature |
| GPIO_INT_EDGE_TO_ACTIVE | Edge to "active" level — respects GPIO_ACTIVE_LOW flag |
| GPIO_INT_LEVEL_HIGH | Level-sensitive HIGH (held, re-triggers) |
| GPIO_INT_LEVEL_LOW | Level-sensitive LOW |

## 5.3  Board-Specific Details

| // ESP32-S3 NOTE /* ESP32-S3: All GPIOs support interrupts * EXCEPT GPIO 35-39 (input only, some silicon) * * For esp32s3_devkitc: BOOT button = GPIO0 * DTS alias sw0 → gpio0 pin 0 * * Internal pull-up/down: * GPIO_PULL_UP or GPIO_PULL_DOWN in DTS * OR at runtime: * gpio_pin_configure_dt(&btn, * GPIO_INPUT ¦ GPIO_PULL_UP); */ | // STM32F446RE NOTE /* STM32 uses EXTI lines: GPIO PA0-PA15 share * EXTI0-EXTI15. Only ONE port per EXTI line: * PA0 and PB0 CANNOT both be interrupts! * * Nucleo B1 button = PC13 → EXTI13 * DTS alias sw0 → gpioc pin 13 * * STM32 has no internal pull management * in gpio_pin_configure_dt for ACTIVE_LOW: * Add GPIO_PULL_UP to configure flags */ |
| --- | --- |

⚠ On STM32, EXTI lines are shared between ports. PA5 and PB5 cannot both generate interrupts simultaneously — only one can be configured per EXTI line number.

# Exercise 06 — I2C Sensor Reading (GY-86 / IMU)

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

I2C in Zephyr uses the sensor driver abstraction for supported sensors, or raw i2c_write/read for custom/unsupported devices. This exercise covers both paths — directly relevant to your GY-86 IMU work.

## 6.1  prj.conf — I2C + Sensor

prj.conf

CONFIG_I2C=y CONFIG_SENSOR=y CONFIG_MPU6050=y # for GY-86 MPU6050 accel/gyro CONFIG_HMC5883L=y # for GY-86 compass (if using driver) CONFIG_LOG=y CONFIG_LOG_DEFAULT_LEVEL=3

## 6.2  Device Tree Overlay (both boards)

| // boards/esp32s3_devkitc.overlay /* I2C0 on ESP32-S3: SDA=GPIO8, SCL=GPIO9 * Adjust pins for your wiring */ &i2c0 { status = "okay"; sda-pin = <8>; scl-pin = <9>; clock-frequency = <I2C_BITRATE_FAST>; /* 400kHz */ mpu6050@68 { compatible = "invensense,mpu6050"; reg = <0x68>; /* AD0 low = 0x68 */ label = "MPU6050"; }; }; | // boards/nucleo_f446re.overlay /* I2C1 on Nucleo: PB8=SCL, PB9=SDA */ &i2c1 { status = "okay"; /* STM32 uses pinctrl for pin assignment */ pinctrl-0 = <&i2c1_scl_pb8 &i2c1_sda_pb9>; pinctrl-names = "default"; clock-frequency = <I2C_BITRATE_FAST>; mpu6050@68 { compatible = "invensense,mpu6050"; reg = <0x68>; label = "MPU6050"; }; }; |
| --- | --- |

## 6.3  Using Sensor API (High Level)

src/main.c — Sensor API

#include <zephyr/kernel.h> #include <zephyr/device.h> #include <zephyr/drivers/sensor.h> #include <zephyr/logging/log.h> LOG_MODULE_REGISTER(imu, LOG_LEVEL_INF); int main(void) { const struct device *mpu = DEVICE_DT_GET_ANY(invensense_mpu6050); if (!device_is_ready(mpu)) { LOG_ERR("MPU6050 not ready"); return -ENODEV; } struct sensor_value ax, ay, az; struct sensor_value gx, gy, gz; while (1) { /* Trigger measurement */ sensor_sample_fetch(mpu); sensor_channel_get(mpu, SENSOR_CHAN_ACCEL_X, &ax); sensor_channel_get(mpu, SENSOR_CHAN_ACCEL_Y, &ay); sensor_channel_get(mpu, SENSOR_CHAN_ACCEL_Z, &az); sensor_channel_get(mpu, SENSOR_CHAN_GYRO_X, &gx); /* sensor_value: val1 = integer part, * val2 = fractional (millionths) */ LOG_INF("Accel: %d.%06d, %d.%06d, %d.%06d m/s2", ax.val1, abs(ax.val2), ay.val1, abs(ay.val2), az.val1, abs(az.val2)); k_msleep(50); /* 20 Hz */ } return 0; }

## 6.4  Raw I2C Transactions (When No Driver Exists)

Raw I2C Transactions

#include <zephyr/drivers/i2c.h> #define I2C_NODE DT_NODELABEL(i2c0) const struct device *i2c_dev = DEVICE_DT_GET(I2C_NODE); /* Write single register */ void i2c_write_reg(uint8_t dev_addr, uint8_t reg, uint8_t val) { uint8_t buf[2] = { reg, val }; i2c_write(i2c_dev, buf, 2, dev_addr); } /* Read burst: write reg address, then read n bytes */ void i2c_read_regs(uint8_t dev_addr, uint8_t reg, uint8_t *out, size_t len) { /* Combined write+read in one transaction */ struct i2c_msg msgs[2] = { { .buf = &reg, .len = 1, .flags = I2C_MSG_WRITE }, { .buf = out, .len = len, .flags = I2C_MSG_READ ¦ I2C_MSG_STOP }, }; i2c_transfer(i2c_dev, msgs, 2, dev_addr); }

⚠ sensor_value uses fixed-point: actual_value = val1 + val2 / 1,000,000. To convert to float: sensor_value_to_double(&sv). Avoid float math on Cortex-M4 without FPU flag — enable CONFIG_FPU=y for STM32F446RE.

# Exercise 07 — SPI & ADC

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

## 7.1  SPI Configuration

| // ESP32-S3 SPI overlay /* SPI2 on ESP32-S3 */ &spi2 { status = "okay"; #address-cells = <1>; #size-cells = <0>; cs-gpios = <&gpio0 10 GPIO_ACTIVE_LOW>; /* MOSI=11, MISO=13, CLK=12 */ mosi-pin = <11>; miso-pin = <13>; sclk-pin = <12>; mydevice@0 { compatible = "vnd,spi-device"; reg = <0>; spi-max-frequency = <1000000>; }; }; | // STM32F446RE SPI overlay /* SPI1 on Nucleo: PA5=CLK, * PA6=MISO, PA7=MOSI, PA4=CS */ &spi1 { status = "okay"; pinctrl-0 = <&spi1_sck_pa5 &spi1_miso_pa6 &spi1_mosi_pa7>; pinctrl-names = "default"; cs-gpios = <&gpioa 4 GPIO_ACTIVE_LOW>; mydevice@0 { compatible = "vnd,spi-device"; reg = <0>; spi-max-frequency = <1000000>; }; }; |
| --- | --- |

## 7.2  SPI Read/Write

SPI Full-Duplex Transfer

#include <zephyr/drivers/spi.h> static const struct spi_dt_spec spidev = SPI_DT_SPEC_GET(DT_NODELABEL(mydevice), SPI_OP_MODE_MASTER ¦ SPI_WORD_SET(8) ¦ SPI_TRANSFER_MSB, 0); int spi_transfer(uint8_t *tx, uint8_t *rx, size_t len) { struct spi_buf tx_buf = { .buf = tx, .len = len }; struct spi_buf rx_buf = { .buf = rx, .len = len }; struct spi_buf_set tx_set = { .buffers = &tx_buf, .count = 1 }; struct spi_buf_set rx_set = { .buffers = &rx_buf, .count = 1 }; return spi_transceive_dt(&spidev, &tx_set, &rx_set); }

## 7.3  ADC Sampling

ADC Sampling — STM32F446RE / ESP32-S3

#include <zephyr/drivers/adc.h> /* prj.conf: CONFIG_ADC=y */ #define ADC_NODE DT_NODELABEL(adc1) #define ADC_CHANNEL 0 #define ADC_GAIN ADC_GAIN_1 #define ADC_REF ADC_REF_INTERNAL #define ADC_RES 12 /* bits */ const struct device *adc_dev = DEVICE_DT_GET(ADC_NODE); struct adc_channel_cfg ch_cfg = { .gain = ADC_GAIN, .reference = ADC_REF, .acquisition_time = ADC_ACQ_TIME_DEFAULT, .channel_id = ADC_CHANNEL, }; int16_t raw; struct adc_sequence seq = { .channels = BIT(ADC_CHANNEL), .buffer = &raw, .buffer_size = sizeof(raw), .resolution = ADC_RES, }; int read_adc_mv(void) { adc_channel_setup(adc_dev, &ch_cfg); adc_read(adc_dev, &seq); int32_t mv = raw; /* Convert to mV using Zephyr helper */ adc_raw_to_millivolts( adc_ref_internal(adc_dev), ADC_GAIN, ADC_RES, &mv); return (int)mv; }

| ESP32-S3 ADC | 12-bit SAR, up to 20 channels; GPIO1-10 = ADC1, GPIO11-20 = ADC2. WiFi/BLE interferes with ADC2 |
| --- | --- |
| STM32F446RE ADC | ADC1/2/3 up to 16ch each; 12-bit; PA0 = ADC1_IN0. Enable VDD filter caps |
| adc_raw_to_millivolts() | Converts raw counts to mV accounting for gain and reference voltage |
| DMA ADC (STM32) | Use CONFIG_ADC_STM32_DMA=y for continuous DMA transfer without CPU polling |

# Exercise 08 — DMA Transfers (STM32) & PSRAM (ESP32-S3)

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

This exercise dives into memory and data movement: DMA on STM32F446RE for zero-CPU transfers, and PSRAM/SPIRAM usage on ESP32-S3 for large buffers.

## 8.1  STM32 DMA — UART with DMA

prj.conf (STM32)

# prj.conf CONFIG_UART_ASYNC_API=y CONFIG_DMA=y CONFIG_UART_STM32_DMA=y

DMA UART — STM32F446RE

#include <zephyr/drivers/uart.h> #define UART_NODE DT_NODELABEL(usart2) const struct device *uart = DEVICE_DT_GET(UART_NODE); static uint8_t rx_buf[256]; static uint8_t tx_buf[] = "Hello via DMA!\r\n"; /* Async callback for DMA completion */ static void uart_cb(const struct device *dev, struct uart_event *evt, void *user_data) { switch (evt->type) { case UART_TX_DONE: LOG_INF("DMA TX done"); break; case UART_RX_RDY: LOG_INF("DMA RX: %d bytes", evt->data.rx.len); /* Process evt->data.rx.buf + offset */ break; case UART_RX_STOPPED: uart_rx_enable(dev, rx_buf, sizeof(rx_buf), 100 * USEC_PER_MSEC); break; default: break; } } int main(void) { uart_callback_set(uart, uart_cb, NULL); uart_rx_enable(uart, rx_buf, sizeof(rx_buf), 100 * USEC_PER_MSEC); uart_tx(uart, tx_buf, sizeof(tx_buf), SYS_FOREVER_MS); while (1) { k_msleep(1000); } return 0; }

## 8.2  ESP32-S3 PSRAM (SPIRAM) Usage

prj.conf (ESP32-S3)

# prj.conf — Enable PSRAM on ESP32-S3 CONFIG_ESP_SPIRAM=y CONFIG_SPIRAM_MODE_OCT=y # OPI mode (Octal) CONFIG_SPIRAM_SPEED_80M=y CONFIG_HEAP_MEM_POOL_SIZE=0 # Use SPIRAM for heap

PSRAM Allocation — ESP32-S3

#include <zephyr/kernel.h> #include <stdlib.h> /* malloc uses SPIRAM heap */ /* Large buffer — goes to PSRAM automatically * when CONFIG_ESP_SPIRAM=y */ uint8_t *large_buf; int main(void) { /* Allocate 100KB — impossible in internal SRAM */ large_buf = k_malloc(100 * 1024); if (!large_buf) { LOG_ERR("PSRAM alloc failed"); return -ENOMEM; } LOG_INF("PSRAM buffer @ %p", large_buf); /* Use normally */ memset(large_buf, 0, 100 * 1024); k_free(large_buf); return 0; }

| STM32F446RE DMA streams | Each peripheral has assigned DMA stream+channel; check RM Table 27/28 for UART2 = DMA1 Stream 6, Ch 4 |
| --- | --- |
| DMA burst size | Set burst for cache-line aligned transfers; avoid unaligned DMA addresses on STM32 AHB bus |
| ESP32-S3 PSRAM note | 8MB OPI PSRAM on devkit; cache-coherent at 80MHz; slower than internal SRAM (~40ns vs ~10ns) |
| esp32s3 internal SRAM | 512KB internal SRAM + 16KB RTC SRAM; place ISR handlers in IRAM_ATTR for speed |

# Exercise 09 — BLE GATT Server (ESP32-S3)  &  USB CDC (Both)

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

BLE GATT on ESP32-S3 using Zephyr's Bluetooth stack. This exercise connects directly to your smart shoe gait platform work. STM32 section covers USB CDC-ACM as an alternative serial transport.

## 9.1  prj.conf — BLE

prj.conf (ESP32-S3 BLE)

# ESP32-S3: BLE GATT Server CONFIG_BT=y CONFIG_BT_PERIPHERAL=y CONFIG_BT_DEVICE_NAME="SmartShoe" CONFIG_BT_DEVICE_APPEARANCE=832 /* Running shoe category */ CONFIG_BT_GATT_DYNAMIC_DB=y CONFIG_BT_MAX_CONN=1 CONFIG_BT_GAP_AUTO_UPDATE_CONN_PARAMS=y CONFIG_BT_LOG_LEVEL_INF=y

## 9.2  Custom GATT Service

BLE GATT Service — ESP32-S3

#include <zephyr/bluetooth/bluetooth.h> #include <zephyr/bluetooth/gatt.h> #include <zephyr/bluetooth/uuid.h> /* Custom Service UUID */ #define SHOE_SERVICE_UUID \ BT_UUID_DECLARE_128(BT_UUID_128_ENCODE( \ 0x12345678, 0x1234, 0x5678, \ 0x1234, 0x56789abcdef0)) /* Gait Data Characteristic UUID */ #define GAIT_CHAR_UUID \ BT_UUID_DECLARE_128(BT_UUID_128_ENCODE( \ 0x12345679, 0x1234, 0x5678, \ 0x1234, 0x56789abcdef0)) static uint8_t gait_data[20]; static bool notify_enabled; /* Notification CCC changed callback */ static void ccc_cfg_changed(const struct bt_gatt_attr *attr, uint16_t value) { notify_enabled = (value == BT_GATT_CCC_NOTIFY); LOG_INF("Notify: %s", notify_enabled ? "ON" : "OFF"); } /* Read callback */ static ssize_t read_gait(struct bt_conn *conn, const struct bt_gatt_attr *attr, void *buf, uint16_t len, uint16_t offset) { return bt_gatt_attr_read(conn, attr, buf, len, offset, gait_data, sizeof(gait_data)); } /* Service definition — macro expands to static vars */ BT_GATT_SERVICE_DEFINE(shoe_svc, BT_GATT_PRIMARY_SERVICE(SHOE_SERVICE_UUID), BT_GATT_CHARACTERISTIC(GAIT_CHAR_UUID, BT_GATT_CHRC_READ ¦ BT_GATT_CHRC_NOTIFY, BT_GATT_PERM_READ, read_gait, NULL, gait_data), BT_GATT_CCC(ccc_cfg_changed, BT_GATT_PERM_READ ¦ BT_GATT_PERM_WRITE), ); /* Send notification */ void send_gait_notify(uint8_t *data, size_t len) { if (!notify_enabled) return; memcpy(gait_data, data, MIN(len, sizeof(gait_data))); bt_gatt_notify(NULL, &shoe_svc.attrs[1], /* char attr */ gait_data, sizeof(gait_data)); }

## 9.3  BLE Advertising & Connection

BLE Advertising

static const struct bt_data ad[] = { BT_DATA_BYTES(BT_DATA_FLAGS, BT_LE_AD_GENERAL ¦ BT_LE_AD_NO_BREDR), BT_DATA(BT_DATA_NAME_COMPLETE, CONFIG_BT_DEVICE_NAME, sizeof(CONFIG_BT_DEVICE_NAME) - 1), }; static struct bt_conn_cb conn_cb = { .connected = on_connected, .disconnected = on_disconnected, .le_param_updated = on_param_updated, }; void on_connected(struct bt_conn *conn, uint8_t err) { if (!err) { LOG_INF("Connected: RSSI watch thread here"); } } int main(void) { bt_conn_cb_register(&conn_cb); bt_enable(NULL); /* synchronous init */ bt_le_adv_start( BT_LE_ADV_CONN_NAME, ad, ARRAY_SIZE(ad), NULL, 0); while (1) { k_msleep(1000); } return 0; }

## 9.4  USB CDC-ACM (Both Boards — Virtual COM Port)

prj.conf — USB CDC

# prj.conf CONFIG_USB_DEVICE_STACK=y CONFIG_USB_CDC_ACM=y CONFIG_UART_LINE_CTRL=y CONFIG_LOG=y CONFIG_LOG_BACKEND_USB_CDC_ACM=y

USB CDC DTR Wait

/* Wait for DTR (host opened port) before printing */ const struct device *cdc = DEVICE_DT_GET_ONE(zephyr_cdc_acm_uart); uint32_t dtr = 0; usb_enable(NULL); while (!dtr) { uart_line_ctrl_get(cdc, UART_LINE_CTRL_DTR, &dtr); k_msleep(100); } printk("USB CDC ready!\n");

# Exercise 10 — Power Management & Production Patterns

| 🟢 ESP32-S3 | 🔵 STM32F446RE |
| --- | --- |

Power management is what separates prototype firmware from production-grade IoT devices. Zephyr's PM subsystem abstracts sleep states across both platforms.

## 10.1  prj.conf — Power Management

| # ESP32-S3 PM config CONFIG_PM=y CONFIG_PM_DEVICE=y CONFIG_POWEROFF=y # Light sleep (RAM retained) CONFIG_PM_S2RAM=y # Deep sleep with RTC wakeup CONFIG_ESP_SLEEP_ENABLE_RTC_TIMER=y CONFIG_ESP32_LIGHT_SLEEP_UART_ALLOW=y | # STM32F446RE PM config CONFIG_PM=y CONFIG_PM_DEVICE=y CONFIG_PM_DEVICE_RUNTIME=y # STM32 sleep states: # PM_STATE_RUNTIME_IDLE = WFI # PM_STATE_SUSPEND_TO_IDLE = SLEEP # PM_STATE_STANDBY = STANDBY # PM_STATE_SOFT_OFF = SHUTDOWN |
| --- | --- |

## 10.2  Device Runtime PM

Device Runtime PM

#include <zephyr/pm/pm.h> #include <zephyr/pm/device.h> #include <zephyr/pm/device_runtime.h> /* Suspend a peripheral when not in use */ void use_sensor(const struct device *sensor) { /* Wake up device — increments usage count */ pm_device_runtime_get(sensor); /* ... use sensor ... */ sensor_sample_fetch(sensor); /* Release — driver can now power-gate */ pm_device_runtime_put(sensor); }

## 10.3  System Sleep with Timed Wakeup

System Sleep

#include <zephyr/pm/pm.h> /* Force system into sleep for 5 seconds */ void deep_sleep_5s(void) { /* Create wakeup constraint — prevents sleep * if other threads are still busy */ struct pm_state_info info = { .state = PM_STATE_SOFT_OFF, .substate_id = 0, .min_residency_us = 5000000, /* 5s */ }; LOG_INF("Entering deep sleep..."); /* RTC/timer wakeup handled by board driver */ pm_state_force(0u, &info); /* Does not return for SOFT_OFF */ }

## 10.4  Production Checklist

| CONFIG_RESET_ON_FATAL_ERROR=n | Prevent silent reboots in production — log and halt instead |
| --- | --- |
| CONFIG_WATCHDOG=y | Enable WDT; configure k_wdt_feed() in each main thread |
| CONFIG_STACK_SENTINEL=y | Keep in production for stack overflow detection |
| CONFIG_ASSERT=n | Disable asserts in release builds for performance |
| CONFIG_BOOT_BANNER=n | Suppress boot banner to reduce UART noise in production |
| CONFIG_LOG_DEFAULT_LEVEL=1 | Set to ERR only in production; use LOG_INF for development |
| CONFIG_REBOOT=y | Enable sys_reboot() for controlled restart after errors |
| CONFIG_HWINFO=y | Access chip UID for device identity / provisioning |

⚠ Enable CONFIG_SHELL in development builds only. It adds ~20KB of flash. For production, use a lightweight command parser over UART or strip the shell entirely.

## 10.5  Flash & RAM Optimization

Memory Analysis

# Check memory usage after build: west build ... && \ arm-zephyr-eabi-size build/zephyr/zephyr.elf # Or use Zephyr's built-in: west build -t ram_report west build -t rom_report # Outputs per-symbol breakdown — find memory hogs # Reduce flash: disable unused subsystems CONFIG_PRINTK=n # saves ~2KB CONFIG_CBPRINTF_FP_SUPPORT=n # no float formatting CONFIG_ASSERT=n

# Quick Reference — Commands, Kconfig & Zephyr APIs

## West Commands Cheat Sheet

| west build -b <board> | Build for target board |
| --- | --- |
| west build -b <board> -p always | Pristine build (clean) |
| west flash | Flash using board default runner |
| west flash --runner jlink | Force JLink runner |
| west espressif monitor | Serial monitor (ESP32) |
| west build -t menuconfig | Interactive Kconfig GUI |
| west build -t guiconfig | Graphical Kconfig (requires tkinter) |
| west build -t hardenconfig | Apply security hardening configs |
| west build -t ram_report | Per-symbol RAM breakdown |
| west build -t rom_report | Per-symbol Flash breakdown |
| west debugserver | Start GDB server (OpenOCD/JLink) |
| west attach | Attach GDB to running target |

## Critical Kconfig Options by Category

| CONFIG_GPIO=y | GPIO subsystem — almost always needed |
| --- | --- |
| CONFIG_I2C=y | I2C bus driver |
| CONFIG_SPI=y | SPI bus driver |
| CONFIG_UART_CONSOLE=y | Enable UART console (usually default) |
| CONFIG_BT=y + BT_PERIPHERAL=y | BLE stack + peripheral role |
| CONFIG_SENSOR=y | Sensor abstraction layer |
| CONFIG_DMA=y | DMA subsystem |
| CONFIG_USB_DEVICE_STACK=y | USB device (CDC, HID, etc.) |
| CONFIG_SHELL=y | Interactive shell over UART |
| CONFIG_LOG=y | Logging subsystem |
| CONFIG_THREAD_ANALYZER=y | Thread stack analysis tool |
| CONFIG_PM=y | Power management |
| CONFIG_WATCHDOG=y | Hardware watchdog |
| CONFIG_FPU=y | Float hardware (STM32F4/ESP32-S3 have FPU) |

## Key Kernel API Summary

| k_msleep(ms) | Sleep thread for ms milliseconds |
| --- | --- |
| k_uptime_get() | Uptime in ms since boot (int64) |
| k_cycle_get_32() | CPU cycle counter (fast, for profiling) |
| k_sem_give / k_sem_take | Counting semaphore operations |
| k_mutex_lock / k_mutex_unlock | Mutex lock/unlock |
| k_work_submit(&work) | Submit work to system work queue |
| k_work_reschedule(&dwork, K_MSEC(n)) | Schedule delayed work |
| k_timer_start / k_timer_stop | Software timer control |
| K_THREAD_DEFINE() | Static thread creation macro |
| K_SEM_DEFINE(name, init, max) | Static semaphore definition |
| K_MUTEX_DEFINE(name) | Static mutex definition |
| DEVICE_DT_GET(node) | Get device handle from DT node |
| device_is_ready(dev) | Check if device initialized OK |